| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC- K0353 |
| --- | --- |

| Title: | HEVC encoding of deinterlaced sequences: a preliminary study |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Informative |  |  |
| Author(s) or Contact(s): | Zineb Agyo, Jérôme Viéron, Pierre Larbier, Jean-Marc Thiesse, ATEME 26 Burospace, Route de Gisy, 91570 Bièvres, FRANCE | Tel: Email: | +33 16 935 8998 z.agyo@ateme.com, j.vieron@ateme.com, p.larbier@ateme.com, jm.thiesse@ateme.com |
| Source: | ATEME |  |  |

_____________________________

# Abstract

This contribution presents a preliminary study about HEVC encoding of deinterlaced sequences. The study aims to determine whether deinterlacing is to be used before or after coding.

A first comparison is made between HEVC field coding of interlaced sequences and HEVC frame coding of a deinterlaced version of the sequence. Secondly, HEVC frame coding of deinterlaced sequences is compared to H.264/AVC encoding of interlaced video. Three deinterlacers are used for this study: ATEME’s own deinterlacer, in addition to two state of the art deinterlacers.

As expected, very first results show  that deinterlacing before compressing with HEVC provides a bitrate advantage compared to compressing an interlaced content. Moreover, it is reported that ATEME’s deinterlacer gives better PSNR results in addition to a significant visual quality improvement in compared to other tested deinterlacers.

Apology

The authors sincerely apologize for the tardiness of this contribution and would greatly appreciate its consideration.

# Introduction

Regarding interlaced content coding and broadcasting, two opposite views are defended in the group: interlaced content have to be deinterlaced either before coding (e.g. pre-processing etc.) or after decoding (by the set-top-box,  TV, Screen etc.).

In the current contribution, we propose a very preliminary study comparing the encoding performance of each respective scenario.

# Contribution

The first test opposes HEVC field coding to HEVC frame coding after deinterlacing. Performance is evaluated according to two models: first with taking deinterlacing loss into account, then by evaluating the loss introduced by compression only.

In the second test HEVC frame coding after deinterlacing is compared to AVC interlaced coding.

Three deinterlacers are used for this study: ATEME’s own deinterlacer, and two other deinterlacers referred to in the following by “Deinterlacer 1” and “Deinterlacer 2”.

## HEVC field coding Vs. HEVC deinterlaced frame coding

In the first case, fields of the interlaced source sequence are separated/deinterleaved and then compressed using HEVC with GOP structure described in [1]. In the second case, the source is deinterlaced using one of the deinterlacers above, compressed using HEVC then interlaced back. One must remark that the denterlacing process are performed at double the original interlaced frame rate

## HEVC deinterlaced frame coding Vs. H.264/AVC field coding

HEVC frame coding after deinterlacing compared to compressing interlaced video using ATEME’s H.264/AVC own encoder with both PAFF and MBAFF activated.

# Results

## Experimental settings

Tests have been run on 4 different interlaced HD (1080i @30Hz) sequences [1]:

- “Church_1920x1080i_30Hz”

- “Marching_in_1920x1080i_30Hz”

- “Walk_through_the_square_1920x1080i_30Hz”

- “Whale_show_1920x1080i_30Hz”

Four quantization parameters leading to conventional  broadcast bitrates have been used.

Following encoding configurations are used:

Table 1 Coding configuration.

|  | H.264/AVC | HEVC Field | HEVC deinterlaced frame |
| --- | --- | --- | --- |
| FrameRate | 30 | 60 | 60 |
| SourceHeight | 1088 | 544 | 1088 |
| FrameToBeEncoded | 200 | 100 | 100 |
| IntraPeriod | 48 | 96 | 96 |
| GOPSize | 8 | 16 | 8 |

## Comparison to HEVC field coding

### Taking deinterlacing into account

Table 3 reports the results for coding deinterlaced sequences compared to coding fields with HEVC.

In this first test, the PSNR is computed taking as reference the interlaced source as described in figure 1, this way, we evaluate both the coding and deinterlacing effects.

Results reportedly show that deinterlacing before encoding is more efficient for sequences without motion (or slow motion) such as “Marching” or “Walk through the square”. For this kind of sequences where interlacing is not really useful, deinterlacing before encoding effectively improve coding efficiency. Otherwise, expected losses are observed for BD-Rate that depend on how much the deinterlacer affects the existing lines in the interlaced source while deinterlacing.

In average, deinterlacing leads to 3.71% and 12.42% losses (both coding and deinterlacing losses) for the two firsts deinterlacer and 12.12% bitrate savings for last deinterlacer.

Figure 1:  PSNR computation process while taking deinterlacing into account.

Table 3: Results of BD-Rate comparing HEVC encoding of various deinterlacers to HEVC field coding.

|  | ATEME's Deinterlacer |  |  | Deinterlacer 1 |  |  | Deinterlacer 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Church | 31,0% | -42,2% | -39,0% | 76,1% | -37,5% | -36,9% | 0,2% | -42,8% | -41,5% |
| Marching in | -18,5% | -69,9% | -73,8% | -14,2% | -61,9% | -65,5% | -20,5% | -55,5% | -58,0% |
| Walk through the square | -4,6% | -53,5% | -54,8% | -14,4% | -41,8% | -42,3% | -27,6% | -31,5% | -33,0% |
| Whale show | 6,9% | -60,0% | -59,7% | 2,2% | -58,6% | -59,2% | -0,6% | -53,2% | -52,3% |
| Overall | 3,71% | -56,42% | -56,85% | 12,42% | -49,97% | -50,99% | -12,12% | -45,74% | -46,20% |

### Without taking deinterlacing into account

In this second test, the PSNR is computed taking as reference the interlaced source which was deinterlaced, as described in diagram 2. Basically, we compare the encoder input and output without taking into account the systematic PSNR loss due to deinterlacing.

Results in table 4 assert that deinterlacing before coding provides bitrate savings compared to compressing interlaced content. Losses that were observed in the last test are actually due to deinterlacing. However, even when field coding is used, deinterlacing is inevitable and will introduce a PSNR loss subsequently. These savings are higher while using ATEME’s deinterlacer. Besides, image quality is significantly improved compared to the other deinterlacers.

Diagram 2: PSNR computation process while ignoring deinterlacing effect.

Table 4: Results of BD-Rate comparing HEVC encoding of various deinterlacers to HEVC field coding without taking deinterlacing into account.

|  | ATEME's Deinterlacer |  |  | Deinterlacer 1 |  |  | Deinterlacer 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Church | -57,4% | -48,5% | -51,7% | -21,9% | -49,1% | -51,2% | -40,9% | -60,7% | -64,6% |
| Marching | -60,8% | -56,1% | -55,8% | -40,8% | -68,8% | -69,7% | -44,7% | -75,3% | -74,2% |
| Walk through the square | -42,9% | -32,2% | -33,3% | -27,3% | -46,4% | -47,6% | -34,1% | -42,4% | -42,6% |
| Whale show | -23,6% | -49,9% | -55,3% | -19,4% | -60,8% | -62,9% | -20,6% | -70,0% | -70,8% |
| Overall | -46,18% | -46,67% | -49,04% | -27,35% | -56,28% | -57,84% | -35,07% | -62,09% | -63,04% |

## Comparison to H.264/AVC interlaced coding

Table 5 reports performance of HEVC coding of deinterlaced sequences against AVC coding of interlaced sequences.

Systematic bitrate savings are observed for all tested sequences and all tested deinterlacers with an average of 42.01%, 41.08% and 51.24% bitrate savings for luma respectively for each deinterlacer. These results confirm the relevance of such a solution to deal with interlaced material but this must be confirmed for other most complicated sequences.

Table 5 Results of BD-Rate comparing HEVC encoding of various deinterlacers to H.264/AVC encoding.

|  | ATEME's Deinterlacer |  |  | Deinterlacer 1 |  |  | Deinterlacer 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Church | -51,3% | -80,0% | -78,8% | -34,4% | -78,3% | -78,1% | -63,8% | -80,0% | -79,6% |
| Marching in | -60,1% | -89,8% | -91,0% | -64,1% | -87,0% | -88,3% | -65,2% | -83,7% | -84,9% |
| Walk through the square | -47,1% | -74,4% | -75,2% | -52,5% | -67,7% | -67,9% | -59,8% | -61,8% | -62,7% |
| Whale show | -9,6% | -48,9% | -44,3% | -13,4% | -47,7% | -44,3% | -16,2% | -46,2% | -42,5% |
| Overall | -42,01% | -73,25% | -72,29% | -41,08% | -70,16% | -69,67% | -51,24% | -67,93% | -67,42% |

# Conclusion

It is asserted following the results shown in this contribution that deinterlacing before compression is more effective that compressing interlaced sequences with current HEVC framework since the PSNR loss inherent to deinterlacing step is not taken into account. Therefore, deinterlacing should be used before encoding. Also, a good deinterlacer guarantees a better visual quality that might not appear in PSNR performance.

# Reference(s)

[1]Jérôme Viéron, Jean-Marc Thiesse and Pierre Larbier, “HEVC interlaced coding assessment and Chroma issue consideration”, JCTVC-K0216, October, 2012, Shanghai, CN.

[2] Rec. ITU-R BT.1210-3, "Test materials to be used in subjective assessment".

# Patent rights declaration(s)

ATEME does not have any current or pending patent rights relating to the technology described in this contribution.