| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0354-r4 |
| --- | --- |

| Title: | BoG report on SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrew Segall 5750 NW Pacific Rim Blvd. Camas, WA 98607 USA | Tel: Email: | +1-360-817-8487 asegall@sharplabs.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This is a report on the Breakout Group on the scalable extension of HEVC.  The BoG met from October 11, 2012, to October 18, 2012.  The notes and recommendations from the meeting are included in the report.  In addition, the summary of coding efficiency of tools in the CfP response is included.  Finally, documentation of informal viewing and bit-stream verification activities are provided.

# Introduction

The BoG on SHVC was formed to collect information and study responses from the Call for Proposal responses for the scalable extension of HEVC.  Twenty-one technical responses were provided as input to the meeting.  These responses are identified as document numbers 30 through 52 in the JCTVC document registery.

# Mandates

Mandates for the BoG were to:

- Provide a summary about the various proposals in terms of performance, tools included, etc.

- Study suggested software starting points – investigating whether any of the suggested software frameworks fulfills the needs for further experimentation e.g. in providing suitable hooks for inter-layer motion/mode/partition coding, inter-layer processing, EL difference coding etc.

On Friday (October 12, 2012), the mandates were extended to:

- Performing verification of provided bitstreams

- Arranging for informal viewing of responses

On Saturday (October 13, 2012), the mandates were extended to:

- Establish a list of tools to be further investigated in (tool) experiments

- Decide about a common reference for comparison

- Investigate which software is best fulfilling that purpose

# Summary of Progress

This section provides a high level summary of the progress of the BoG.

- The summary categorization of tools present in responses to the Call for Proposals was completed and agreed in the BoG.  The agreed summary is provided as JCTVC-K0354_tool_summary-r3.xls

- The summary collection of performance characteristics of response to the Call for Proposals was assembled and discussed in the BoG.  This summary contains information on coding efficiency and run-time calculations, as reported by responses to the Call for Proposals.  A draft of this summary is currently available in JCTVC-K0354_performancy_study-r1.xls

- Bit-stream verifications were completed on October 14, 2012, and no issues were identified with the verification.  The instructions provided to CfP proponents are included in the archive JCTVC-K0354_bitstream_verification_instructions.zip.

- Decoder timing testing was completed on October 14, 2012.  Results are provided in JCTVC-K0354_bitstream_verification_summary.xls

- Informal viewing of three responses were performed on October 14, 2012, and October 15, 2012.

- Six tool experiments were identified by the Breakout Group.  These tool experiments were further classified into three categories – A, B and C that correspond to “high” priority, “medium” priority and “low” priority for the next meeting.  Test conditions and anchors for each of the tool experiments was also decided. The tool experiments are:

  - Upsampling filters (Category A)

  - Inter-layer texture prediction signaling (Category A)

  - Combined prediction (Category B)

  - Inter-layer filtering (Category B)

  - Inter-layer syntax prediction using HEVC baselayer (Category C)

  - Inter-layer syntax prediction using AVC baselayer (Category C)

- Seven software proposals were identified as proposals for initial starting points of the SHVC collaborative phase.  These software candidates were

  - JCTVC-K0345

  - JCTVC-K0362

  - JCTVC-K0038

  - JCTVC-K0366

  - JCTVC-K0364

  - JCTVC-K0370

  - JCTVC-K0348

  - Note that JCTVC-K0348 was eventually selected as software to be used for tool experiments on Upsampling Filters, Inter-layer texture prediction signaling, Combined prediction, Inter-layer filtering and Inter-layer syntax prediction using HEVC baselayer

  - Note that JCTVC-K0345 was eventually selected as software to be used for tool experiments on Inter-layer syntax prediction using AVC baselayer

- Three Ad hoc Groups were identified

  - AhG on SHVC tool experiments.

  - AhG on SHVC software.

  - AhG on Upsampling and downsampling filters for SHVC

# Meeting Notes

[ Chair note: The rest of this document contains meeting notes from the Break out Group.  The notes are organized chronologically with topics identified with section headings.  Please note that a topic may be discussed over multiple days. ]

## October 11, 2012

Three input contributions are available providing summaries of the Call for Proposal responses: JCTVC-K0346, K0349, K0352.

### JCTVC-K0346 Summary of tools and performance of scalable video coding technology proposals [P. Lai, T.-D. Chuang, S. Liu, Y.-W.Huang, S. Lei (MediaTek), E. Nassor, E. Francois, F. Le Leannec, P. Onno (Cannon), C.-K. Kim, B.-M. Jeon, J.-Y. Park (LG), J. Chen, X. Li (Qualcomm)] [late]

This document is asserted to provide a summary of broad categories of coding tools.  It is also asserted to provide a summary of results submitted to Call for Proposals on scalable video coding technology.

Document includes two summaries.  First summary provides information on tools present in CfP responses.

One participant commented that it will be necessary to define terms in any categorization.

Second summary provides information on the coding efficiency performance of CfP responses.  Results are for target rates – AI, RA, SNR categories.  AVC baselayer is not provided but authors are willing to incorporate.

On participant questioned about the use of target and/or actual rates for the summarization.

One participant suggested to report both target and actual rates.

One participant suggested to compute the BD-rate for performance of CfP response against each other.  Specifically, the compute the BD-rate for proposal “A” versus the BD-rate for proposal “B”.  The participant suggested to do this for all proposals ideally.  However, an alternative suggestion is to limit the activity to proposals of interest.

One participant questioned if cross checks (to verify that decoder runtime) had been provided.  Additional request supporting this activity.

Decision: Discuss decoder verification during BoG report.  [Chair note: This was subsequently added to the BoG mandates on October 12, 2012.]

On participant questioned if visual quality of the CfP response has been evaluated.  Suggestion to perform visual viewing. Multiple requests supporting this activity.

Decision: Discuss visual quality evaluation during BoG report.  [Chair note: This was subsequently added to the BoG mandates on October 12, 2012.]

One participant suggested identifying if decoder run times correspond to only the enhancement layer or both the enhancement and base layer.

One participant suggested capturing memory usage.  Comment that this was described as part of the CfP but not necessarily quantified.

### JCTVC-K0352 Summary of CfP results on scalable video coding technologies [J. Lainema, K. Ugur (Nokia)] [late]

This document is asserted to contain a summary of the results submitted to Call for Proposals on scalable video coding technology. The provided spreadsheets provide both individual coding efficiency and run time results for each proposal as well as a different summaries and visualizations.

Contribution captures the target and actual rates of the contributions.  Additionally, it captures the deviation from the target rate and encoder/decoder time.  Provides summary plots showing coding performance versus run time.

One participant suggested to not average the AI and RA information and/or report it separately.

One participant suggested clarifying the terms used in the document.

Terms used in the document:

+ Encoding and Decoding times are averages of decoding times compared to reference reported (simulcast anchor)

+ Max deviation from target is the maximum difference between BD-rate using the actual and target bit-rates for multiple categories (for example, RA-1.5 and AI-2).

One participant suggested to capture if target rates were violated.

### JCTVC-K0349 Summary and performance comparison of scalable coding CfP responses [T Wiegand, H Schwarz, H Kirchhoffer, H Lakshman] [late]

The proposal is asserted to provide collated performance comparison of CfP responses to scalability extension of HEVC. Performance numbers based on both actual bitrates of individual submissions and target bitrates for the CfP are tabulated.

Input document provides BD-rate results for the different proposals.  Run-times can also be provided in a revision.

[Chair note:  Following presentation of JCTVC-K0349, the group discussed the list of metrics to be included in the summary. ]

List of metrics to include in the summary:

- Coding Efficiency

  - BD-rate using both actual and target rate (as reported in CfP response)

  - Separate BD-rate for both A and B (actual and target rate)

  - Both enhancement layer and EL+BL results

- Complexity/Run Time

  - Encoder and decoder run time for each category

- Visualization

  - Run time versus coding efficiency for AI, RA, SNR and AVC

List of tool categorization:

- Upsampling filter (switched, adaptive/download)

- Interlayer texture prediction

- Difference domain coding (intra and inter)

- Generalized residual prediction

- Interlayer filtering (DBF/SAO & ALF)

- Interlayer syntax prediction (EL Merge/AMVP, Partition, INTRA).

- Interlayer syntax prediction - AVC

- Single layer tools (EL ALF, EL AMP, EL IBDI, Modified deblocking, Transform, Modified scan, Modified CABAC) Note that AMP is in HEVC main profile.

- Single pass or multi-pass encoder

Agreed.

Second mandate (Study of suggested software starting point)

There was no software offered to the BoG for a starting point.

## October 12, 2012

[Chair note: The BoG met again on October 12, 2012.  The agenda for the meeting was proposed as follows:]

Agenda

+ Plan for bit-stream verification

+ Plan for visual evaluation

+ Software candidate

+ Other items

### Plan for bit-stream verification

Suggestion to have a small, random set of bit-streams to verify

Suggestion to have one or two test points

Suggestion to have a single machine perform the decode

Comment that it is useful to test for mismatch

Comment that it is useful to test for decoder time

Comment that to measure decoder time, it is beneficial to run the anchor

Suggestion to test bit-streams for each configuration (RA, AI, SNR, AVC)

Suggestion to allow additional decoder implementations to be tested

Suggestion to choose the second highest bit-rate for testing

Suggestion to use Class B and not Kimono or BQTerrace

Comment that in Dresden (HEVC CfP), 9 bit-streams were used for decoder run-time

Suggestion to use second smallest QP for base layer

Suggestion to test two points – one for random access and one for all intra

Plan:

- Two test points

  - AI – 2X Cactus (QP26 for BL; second highest rate point for EL) (PnnS5B2E2C1)

  - RA – 1.5x Parkscene (QP26 for BL; second highest rate point for EL) (PnnS4B2E2C4)

- Proponents to provide script, decoder, bit-stream and YUV file

- Test performed on single platform (as close as possible due to OS)

- For proposals that do not have point corresponding to the AI and RA points above, alternative bit-streams (2) can be provided for testing.  Suggested to use the same sequence, BL QP and enhancement layer rate targets.

- Offline activity to provide example script, compute platform and run test.

Agreed for plan above.

### On the topic of visual evaluation

Discussion - what is our goal? Suggestion that there are two goals: (i) determine if a proposal has undesirable visual quality characteristics; (ii) to demonstrate performance of different tool set ‘categories’

Suggestion that any proposal wants to be evaluated it should be possible

Suggestion that configurations representative of common tool sets be selected.  Agreed.

Suggestion of comment tool sets:

- IntraBL/upsampled baselayer only

- High performing without single layer tools

- High performing with single layer tools

Suggestion to test RA-2X.

Suggestion to test RA-SNR.  Some participants voiced comments not supporting this suggestion.

Suggestion to compare with simulcast at similar PSNR

Suggestion to look at HD and lower, or moderates, rates.

Action item (suggestion of Chair): Check with other experts about amount of material that can be tested with system at meetig and report to BoG.

### On software candidates

Participants reported that two software candidates are offered.  One is representative of JCTVC-K0345; one is representative of JCTVC-K0348 (asserted).

For scheduling, one candidate is currently being evaluated by individual experts.  The other to be provided within 24 hours (asserted)

The proponents of the software provided the following status to the group:

JCTVC-K0345: Already distributed to several individual parties beginning Oct. 11, asserted to be identical to JCTVC-K0345, contain hooks for base layer information to be available to the enhancement layer, and contains tools for IntraBL, motion vector prediction, and difference coding mode.  It has support for AVC base layer.

Comment that the performance of the software is not known.

Comment that the software representative of JCTVC-K0345 was not part of the CfP.

Comment that the software representative of JCTVC-K0345 may not contain all the needed hooks for some tools submitted to CfP.  One example is generalized residual prediction.

Comment that the software representative of JCTVC-K0345 is based on HM6.1.

JCTVC-K0348: Based on TI (JCTVC-K0038) interlayer texture prediction tool and HM6.1.  Representative of CfP submission and available Oct. 12 (now).  Tool has been ported to HM8.1 and currently undergoing verification testing and should be available within 24 hours.  Only tool in HM8.1 version is the same interlayer texture prediction tool.

Comment is that the HM6.1 software (above) is based on the JCTVC-K0038 response and that JCTVC-K0038 does not contain any hooks.

Comment is that the HM8.1 software (above) is also based on the JCTVC-K0038 response.

## October 13, 2012

[Chair note: The BoG met again on October 13, 2012.  The agenda for the meeting was proposed as follows:]

Agenda

+ Update on tool summary

+ Update on performance measures

+ Update on bit-stream verification

+ Plan for informal viewing

+ Other items

### Update on tool summary

Reviewed tools in meeting with verbal confirmation of proposals that had not previously responded by e-mail.

Suggestion to add category for combined mode (both intra and inter) – Agreed.

With modifications made during the BoG, tool summary considered complete and to be uploaded as revision to K0354 for review and comment

### Update on performance measures

Question from participants if the scaling of the plots should be changed to min and max.  No consensus

Observation that the summary of EL+BL performance for separate Class A and Class B are not yet available.  Volunteers are being determined.

Suggestion to create four additional columns – one for HEVC-encoder, HEVC-decoder, AVC-encoder, AVC-decoder – to capture if timing is for BL+EL or simply EL encoder/decoder.  Agreed.

Proponents to send information.

Suggestion to incorporate supplemental information capturing additional configurations (not in CfP, but variations of the proposals).  Primarily with the goal of quantifying the benefit EL-only tools.  Agreed.  Off-line determine how to best incorporate the information.

Updated table to be included in the K0354 document package for further review.

### Update on bit-stream verification

Participants reported the following

Instructions sent to the reflector

Six responses have currently submitted the information

Suggestion to set a deadline for the end of the day to deliver data.

Clarification that either encoder or decoder logs may be provided.

Suggestion to verify that the correct baselayer is present in the bit-stream. Suggestion to check the md5sum of the baselayer.

Agree to check the md5sums of the baselayer output.

### Informal viewing

Suggestion to pick three example proposals at different levels of complexity. For example, “low”, “medium”, and “high”.  Identify different bit-streams with similar/matched PSNR and confirm that objective and subjective performance are consistent.  Intention is to confirm that scalable coding did not result in visual artifact.  One benefit may be demonstration of coding efficiency benefit of representative CfP calls.  Possible suggestion is to have three sequences, two QP points.

Suggestion to view sequences from proponents and determine if there are unexpected artifacts.

Suggestion to use specific bit-rates (data already provided) and sub-set of responses.

Suggestion to focus on a sub-set of responses.  Agreed.

Suggestion to use: intraBL only, intraBL+inter-layer tools, and intraBL+inter-layer+single layer tools (e.g., ALF, etc.)

Suggestion to consider JCTVC-K0034 as part of or in addition to the suggestion above.  Agreement that JCTVC-K0034 contains upsampling, motion mapping and enhancement layer filtering.

Suggestion that there is no special status assigned to the candidates selected for viewing.  Agreed.

Suggestion to consider three sequences: park scene, cactus and basketball drive

Suggestion to consider two different bit-rate points per sequence.  QP base 30 and matching the target bit-rate corresponding to the anchor at Qp 32.  Qp base 34 and matching the target bit-rate corresponding to the anchor at Qp 38.  This is for both 2X and 1.5X.

Comment that Qp_delta 2 may be too small of a QP delta.

Suggestion to have pre-viewing to see if anything is visible at these rates.

Suggestion use all five of Class B for informal viewing.

Suggestion to only do 2X initially.

Tentative Plan:

- Class B (all 5 sequences), initial rate points identified above (anticipate refinements), RA-2X, using the representative responses below:

- Representative CfP responses:

  - IntraBL: JCTVC-K0038

  - IntraBL+interlayer: JCTVC-K0040

  - IntraBL+interlayer+EL:  K0036

- Create a list of filenames off-line.  Include additional points around the rate points in the tentative plan.  (Jill to provide.)

Tentative plan is Agreed.

Suggestion to allow other proposals to be viewed at rate points identified after first viewing.  Agreed.

[Chair note: The BoG provided an update to the JCTVC during JCTVC plenary.  The following two comments were captured.]

During plenary report, it was suggested to include a fourth representative category that contains only IntraBL+EL tools.

During plenary report, it was suggested to compute the coding efficiency of CfP responses compared to single layer.

## October 14, 2012

[Chair note: The BoG met again on October 14, 2012.  The agenda for the meeting was proposed as follows:]

Agenda

+ Update on verification

+ Update on informal viewing

+ Discussion of tool experiments

+ Revisit tool summary

+ Other topics

### Update on bit-stream verification

[Chair note: Report from coordinators:]

Verification performed

No mis-match observed.  Achieved by comparing the MD5sums in the bit-streams with the MD5sums output by the decoder.

Base layer MD5sums were compared and all produced the same value

Timing results are being compiled and will be made available as part of BoG document/package.

No issues were identified.  One proponent chose not to participate.

### Update on informal viewing

[Chair note: Report from coordinators:]

Three representative proposals have provided decoded YUV files and now loaded on playback machine.

Suggestion to have a small group view perform an initial screening.

Volunteers for first screening group (for bit-rate confirmation): Interdigital, Nokia, QC, TI, Canon, Sharp.

### Discussion of tool experiments

Comment to not include single layer tools in initial tool experiments. Multiple statements of support.

Suggestion to divide tools into three categories:

+ only use reconstructed picture from BL and only HLS changes in EL

+ only use reconstructed picture from BL and anything allowed in EL

+ use reconstructed picture and syntax from BL and anything allowed in EL

Comment that “single layer” tools that use new inter-layer syntax (such as IntraBL) should not be classified as single layer tools.

Suggestion to focus on “major” tools, e.g. something that would limit the architecture or performance.

Suggestion to identify “major” tools as most common tools in CfP responses.

[ Chair note: Discussion related to the forming of the list also follows list the list ]

Tentative tool experiment list:

- Upsampling filters

- Inter-layer texture prediction signaling

- Combined prediction (intra/inter – this includes any prediction method that combines BL and EL data in some manner – F:K and conceptually D from JCTVC-K0354_tool_summary-r2.xls)

- Inter-layer filtering

- Inter-layer syntax prediction using HEVC baselayer

- Inter-layer syntax prediction using AVC baselayer (no consenus on “priority”; further discussion needed)

- Adaptive loop filtering for EL (no consensus on “priority”; further discussion needed)

- Modified de-blocking filter (suggestion to defer until after inter-layer texture prediction signaling)

Suggestion that it is important to consider and include MVC-like/HLS-only approaches in the tool experiment plan.  May either be a separate tool experiment or components of other tool experiment. Agreed.

Comment that candidate TE1&2 are only ones needed for software.

Comment that candidate TE1&2 are basic elements of design

Comment that candidate TE4 might be treated with lower priority.

Comment that candidate TE3&4 might be treated with lower priority

Comment that the word “comments” above (and below) correspond to comments from individuals and not a conclusion of the BoG.  Multiple comments of support that this is understood.

Multiple comments that inter-layer syntax prediction with an AVC baselayer may be a lower priority item compared to inter-layer syntax prediction with an HEVC baselayer.

Comment that using inter-layer syntax prediction with an AVC baselayer creates difficulties with many, existing implementations due to a need to extract AVC syntax elements (from completed designs).

Comment that some applications and some implementations where this [extracting syntax elements] may not be a problem.

### Update on bit-stream verifications

Report from side activity:

Bit-streams match the results provided by CfP respondents.

Performance measures calculated on Windows x64 based computer.  Timing results calculated.  Fastest proposals are in the ~60s range for random access.

Comment that run time is only an indication of complexity.

CPU usage reported to be small (12-15%)

Spot check of decoded video showed no problems.

### Update on informal viewing

Report from side activity: Two rates points selected (confirmed) as part of side activity

### On software issues

Comment that the software JCTVC-K0033 is available and offered as a starting point.  The software is a sub-set of the K0033 CfP response and includes inter-layer texture prediction and hooks for inter-layer syntax prediction. [Chair note: this was subsequently uploaded as JCTVC-K0370.]

Update on availability:

JCTVC-K0345: Available and distributed

JCTVC-K0362: Available and distributed

JCTVC-K0038: Available and distributed

JCTVC-K0366: Available and distributed

JCTVC-K0364: Available and distributed

JCTVC-K0033(subset): Available [ Chair note: This was subsequently uploaded as JCTVC-K0370.]

## October 14, 2012 (Plenary Report)

[Chair note: The BoG provided an update to the JCTVC during JCTVC plenary on October 15, 2012.  The tentative tool list was further classified as:]

Tentative tool experiment list:

- Category A

  - Upsampling filters

  - Inter-layer texture prediction signaling

- Category B

  - Combined prediction (intra/inter – this includes any prediction method that combines BL and EL data in some manner – F:K and conceptually D)

  - Inter-layer filtering

- Category C

  - Inter-layer syntax prediction using HEVC baselayer

  - Inter-layer syntax prediction using AVC baselayer (no consenus on “priority”; further discussion)

- Category D

  - Adaptive loop filtering for EL (no consensus on “priority”; further discussion)

  - Modified de-blocking filter (suggestion to defer until after inter-layer texture prediction signaling)

## October 16, 2012

[Chair note: The BoG met again on October 16, 2012.  The agenda for the meeting was proposed as follows:]

Agenda

+ Update on informal viewing comments

+ Develop tool experiment descriptions

+ Other open items

### Update on informal viewing (comments)

During the 6PM-7PM viewing session on October 14, 2012, there were 25 viewers.  The following comments were recorded:

- One of the participant mentioned that Chroma setting of the TV might need to be adjusted

- Few of the participant’s commented about the script that HIGH, MEDIUM and LOW should not be used and Instead Method1, Method2 and Method3 be used to enable fair comparison. The commented point was that if the configuration is known the opinion might be biased.

During the 1PM-2PM session on October 15, 2012, there were 12 viewers.  The following comments were recorded:

- Not much difference between the 3 proposals (high, medium, low).

- Please show video of broadcast quality (at least at high).

- Also, at broadcast quality, what is the overhead to single-rate (for BL+EL).

- What are the data rates.

- We should look at this from the point of view of base layer rate, plus additional rate for the enhancement layer.  An important question for a broadcaster is: for BL rate X, what percentage rate surplus is needed in a scalable codec to support a higher resolution, higher quality stream in addition.  Is that 30%, 50%, etc.

During the 2PM-3PM session on October 15, 2012, there were 10 viewers.  The following comments were recorded:

- They gave the feedback on the visual quality. Three of them can see small difference among different complexity levels. Others cannot.

### Discussion of tool experiments

Major topics: Experiment description, anchor identification, test conditions

- Category A

  - Upsampling filters

  - Inter-layer texture prediction signaling

- Category B

  - Combined prediction (intra/inter – this includes any prediction method that combines BL and EL data in some manner – F:K and conceptually D)

  - Inter-layer filtering

- Category C

  - Inter-layer syntax prediction using HEVC baselayer

  - Inter-layer syntax prediction using AVC baselayer (no consenus on “priority”; further discussion)

- Category D

  - Adaptive loop filtering for EL (no consensus on “priority”; further discussion)

  - Modified de-blocking filter (suggestion to defer until after inter-layer texture prediction signaling)

#### Upsampling filters TE

Suggestion to start with high resolution video and no base layer.  Test at four points with initial evaluation using PSNR.

Suggestion to start with intra only

Suggestion to look at different types of upsampling filters, e.g. DCTIF, Gaussian window sync, different taps, different bit-depths, phase derivation, arbitrary scaling

Suggestion to consider random access

Suggestion to consider low delay

Suggestion to use Class A and Class B (and not Class A+).

Multiple suggestions to use Class A+ and Class B. Agreed.

Suggestion that if we/JCTVC study alternative downsamplers, then it is important to evaluate visual quality

Suggestion to fix the method for using the baselayer information for prediction

Suggestion to use a CU level signaling for “IntraBL” mode.  Context coded, with value “1” signaling to use the upsampled BL frame.  Agreed.

Suggestion to use AI, RA, LD for testing.  Agreed.

Suggestion to select between LD-B or LD-P by consulting with other video conferencing experts.

Suggestion to test LD-P, as it may be first configuration used in at least one video conference product.

Suggestion to create an AhG to study different downsampling filters (and upsampling filter). Agreed.

Suggestion to use the CfP downsampler in the TE. Agreed.

Suggestion to add additional test sequences to the test.  Perhaps study between now and next meeting.

Suggestion to reduce the number of rate points

Suggestion to reduce the QP delta range

Suggestion to test two QP deltas (from the CfP test conditions).  Further suggestion to use the middle two of the QP delta set.

Suggestion to test more than two QP deltas (from the CfP test conditions).  Further suggestion to remove AI from the test in order to reduce the number of test points.

Comment that the current delta QPs for Kimono may not be useful.

Suggestion to test two QP deltas (from the CfP test conditions).  Further suggestion to use the extremes of the EL/BL efficiency metric.

Suggestion to test four QP delta points (from the CfP test conditions).

Suggestion to reduce the number of BL QP points (from the CfP test conditions).  This was suggested by multiple participants.

Suggestion to test two BL QP points and four deltaQP points (a total of 8 points).  Agreed.

Suggestion to use the four deltaQP points from the CfP.

Suggestion to use “middle two” BL QP points (26 and 30). Agreed.

Suggestion to have an AhG to study QP points and R-D performance.

Suggestion for IntraBL CU considered as an Intra CU for de-blocking. Agreed.

Suggestion to test both 2x and 1.5x scalability. Agreed.

One participant commented that all software offered (currently) would be suitable for this tool experiment.  Additional participant agreed with this statement.

One participant suggested to select a software package based on HM8.1.  Another participant suggested HM9.0.

Comment that JCTVC-K0366 uses a ref_idx approach.

One participant suggested to select the first software package that is uploaded and only containing the features necessary for the TE.

Suggestion to have only one software for this TE.  Agreed.

#### Inter-layer texture prediction signaling

Comment that a large number of CfP responses used an upsampling filter derived from DCTIF.

Suggestion to use a DCT-IF derived upsampling filter for the test.  Additional participants supported this suggestion. Agreed - contingent on definition of DCT-IF derived upsampling filter.

Discussion of phase shift selection – 1/3 and 2/3 for 1.5x and 1/2 for 2x. (for luma).

Comment that up sampling operation may only be an interpolator.

Comment that the 1/2 phase could correspond to the 1/2-pel filter for motion compensation.

Suggestion to use the 1/2-pel filter from HEVC MC for 2x scalability. Agreed.

For 1/3

[ -1 4 -11 52 26 -8 3 -1]/N

For 2/3

[-1 3 -8 26 52 -11 4 -1]/N

Agreed.

For chroma

[-5 50 22 -3] and [-3 22 50 -5]/N

Suggestion that the chroma filter should not introduce a phase shift (asserted that the filters above introduce a phase shift).  Another participant supported this suggestion.

Suggestion to use JCTVC-K0035 for chroma filters. Filters are reported to be 5 bits.

Offline activity to determine chroma filters. [Chair note: It was subsequently decided to use the filter in JCTVC-K0379.]

Suggestion to average the chroma filters from all CfP responses that use the luma filters (above and that were agreed).  Agreed (in spirit); actual tap values to be calculated off-line and reported back to the group. Offline activity organized by TI and MMobility.  [ Chair note: It was subsequently decided to use the filter in JCTVC-K0379.]

Same downsampling filter as in TE1. Sequences as in TE1. Same rate points as in TE1.  AI, RA, LD-P, as in TE1. Agreed.

Suggestion to use include a description of the encoding process.  Agreed.

Suggestion to treat IntraBL as an inter-coded block for de-blocking.

Agreement to not restrict the value of deblocking filter strength when the enhancement layer predicts from the baselayer in this TE2.

Suggestion to treat ref_idx==BL blocks as intra-coded block

Suggestion to consider AVC baselayer also.  Multiple participants expressed that the AVC simulations may not give us additional information.  It is noted that AVC baselayer performance data could be provided as optional (supplemental) information in a response.  Suggested to use the same conditions as for the AI,RA,LD-P information described above (in this TE).

Request for clarification if skip information is part of the TE, e.g. IntraBL skip mode.  Multiple participants expressed support for including this in the TE.  Agreed (to include).

One participant suggested to request performance information for ref_idx based solutions that when referencing a upsampled BL frame a motion vector is restricted to zero (or not) and/or the encoder selects bi-directional prediction using the upsampled BL frame and a previously reconstructed EL frame. No consensus.

One participant suggested to request block based solution s provide results using BS=1 and BS=2 values for IntraBL blocks. No consensus.

Suggestion to use simulcast as a reference for BD-rate calculations. Agreed.

## October 17, 2012

[Chair note: The BoG met again on October 17, 2012.  The agenda for the meeting was proposed as follows:]

Agenda

+ Develop tool experiment descriptions

+ Other open items

### Discussion of tool experiment descriptions

Major topics: Experiment description, anchor identification, test conditions

- Category A

  - 1. Upsampling filters

  - 2. Inter-layer texture prediction signaling

- Category B

  - 3. Combined prediction (intra/inter – this includes any prediction method that combines BL and EL sample data in some manner – F:K and conceptually D)

  - 4. Inter-layer filtering

- Category C

  - 5. Inter-layer syntax prediction using HEVC baselayer

  - 6. Inter-layer syntax prediction using AVC baselayer (no consenus on “priority”; further discussion)

- Category D

  - Adaptive loop filtering for EL (no consensus on “priority”; further discussion)

  - Modified de-blocking filter (suggestion to defer until after inter-layer texture prediction signaling)

On software, a participant announced that JCTVC-K0348 is now available in the document management system.

#### Combined prediction

(intra/inter – this includes any prediction method that combines BL and EL data in some manner – F:K and conceptually D)

Suggestion to use upsampling filter from TE2 and IntraBL signaling method of TE1 for test. Agree.

Suggestion to use AI for intra-only tools. Agreed.

Suggestion to use same QP points, sequences, sample factors as TE2. Agreed.

Suggestion to use “upsampling filter from TE2 and IntraBL signaling method of TE1” as “anchor” for BD-rate calculations.  Agreed.

#### Inter-layer filtering

Suggestion to have two sub-categories in the TE (adaptive upsampling; IL DBF/SAO/ALF ) [Editorial]

Suggestion to use ref_idx signaling for testing inter-layer filtering tools for test. Optional.  Agreed.

Suggestion to use upsampling filter from TE2 and IntraBL signaling method of TE1 for test.  Multiple participants supported this suggestion. Agreed.

Suggestion to test both methods.  Multiple participants did not support this suggestion.

One participant suggested to test the technologies in “Category B” in a combined way.

Suggestion to use same QP points, sequences, sample factors as TE2. Agreed.

Suggestion to use “upsampling filter from TE2 and IntraBL signaling method of TE1” as “anchor” for BD-rate calculations.  Agreed.

Suggestion to include SNR in the test. Agreed.

Suggestion to use BL Qp points 28 and 36.

Suggestion that use deltaQP of [-2, -4]

Suggestion use four BL Qp points

Suggestion to use “middle two” BL QP points (30 and 34). Agreed.

Suggestion to use deltaQP of [-2, -4, -6, -8] Agreed.

#### Adaptive loop filtering and modified de-blocking

Suggestion on lower category items to put in AhG.  Motivation is that information collected may not be as meaningful due to dependence on “higher” category items.

Suggestion to create AhG on ALF and no TE. Agreed.

Suggestion include include modified deblocking in ALF AhG (above). Agreed.

…

Suggestion to include “example” tool from different categories (e.g, B3 and C5) in this test.

Multiple participants expressed concern about the number of tests required. [ This assumes that this “example” tool would be a second set of simulations. ]

#### Inter-layer texture prediction signaling

Offline activity on chroma filter selection.  Reported issue of selecting either 2 or 5 phase filter due to phase shift between BL and EL.

## October 18, 2012

[Chair note: The BoG met again on October 18, 2012.  The agenda for the meeting was proposed as follows:]

Agenda

+ Develop tool experiment descriptions

+ Other open items

### Discussion of tool experiment descriptions

#### Inter-layer texture prediction signaling

Suggestion to include an additional anchor for the tool experiment to understand tool interactions.

Suggestion that because of tool interactions is may not be preferred to choose a “specific example” tool.

Comment that any proponent is allowed to bring additional information to a tool experiment (optional information).  Additional participant supported this comment.

Suggestion to include multiple anchors, including the one anchor agreed (above), for the tool experiment.  No consensus

Suggestion that any support for multiple anchors could consist of one mandatory anchor and additional optional anchors.

Suggestion that it would be desirable that if additional anchors are used that these show some degree of similarity among different experiments.

Suggestion to study the tool interactions (above) in an AhG.

#### Inter-layer syntax prediction using HEVC baselayer

Suggestion to use upsampling filter from TE2 and IntraBL signaling method of TE1 for test.  Multiple participants supported this suggestion. Agreed.

Suggestion to use same QP points, sequences, sample factors as TE2. Agreed.

Suggestion to use “upsampling filter from TE2 and IntraBL signaling method of TE1” as “anchor” for BD-rate calculations.  Agreed.

Suggestion to include SNR as an optional test. Agreed.

Suggestion to use “middle two” BL QP points (30 and 34). Agreed.

Suggestion to use deltaQP of [-2, -4, -6, -8] Agreed.

Suggestion to have two parts of the TE – one for CU based tools and one for “ref_idx” style tools.  “ref_idx” style proposals to use an anchor of “ref_idx” without inter-layer syntax prediction. Agreed.

Suggestion that the TE shall not allow the enhancement layer to have a parsing dependency on the base layer.  Additional participant supported this statement.  Agreed.

Suggestion that the discussion below (discussed during the previous tool experiment) also applies to this tool experiment where C5 would be replaced by B4:

Suggestion to include “example” tool from different categories (e.g, B3 and C5) in this test.

Multiple participants expressed concern about the number of tests required. [ This assumes that this “example” tool would be a second set of simulations. ]

Suggestion to include an additional anchor for the tool experiment to understand tool interactions.

Suggestion that because of tool interactions is may not be preferred to choose a “specific example” tool.

Comment that any proponent is allowed to bring additional information to a tool experiment (optional information).  Additional participant supported this comment.

Suggestion to include multiple anchors, including the one anchor agreed (above), for the tool experiment.  No consensus

Suggestion that any support for multiple anchors could consist of one mandatory anchor and additional optional anchors.

Suggestion that it would be desirable that if additional anchors are used that these show some degree of similarity among different experiments.

Suggestion to study the tool interactions (above) in an AhG.

#### Inter-layer syntax prediction using AVC baselayer

Discussion if this TE should be an AhG for the next meeting.

Comment that forming an AhG (opposed to a TE) in no way discourages work in the area.  Agreed.

Suggestion that the previous tool experiment (with an HEVC BL) provides sufficient information for inter-layer syntax prediction design (at this time).  An additional participant supported this comment.

Suggestion to not “focus” on AVC base layer between now and next meeting.  Multiple participants supported this comment.

Comment that the following design assumes that a TE will be formed on this subject.  But, there is no consensus on whether that will be the case at this time. [ Note: During reporting to plenary, the TE was confirmed. ]

Suggestion to use upsampling filter from TE2 and IntraBL signaling method of TE1 for test.  Multiple participants supported this suggestion. Agreed.

Suggestion to use same sequences, sample factors as TE2. Agreed.

Suggestion to use QP values of 25 and 29 for BL AVC layer. Agreed.

Suggestion to use base layer anchors from CfP. Agreed.

Suggestion to test AI, RA and LD-P (stay consistent with TEC5). Agreed.

Suggestion to test 1.5x and 2x. Agreed.

Suggestion that the TE shall not allow the enhancement layer to have a parsing dependency on the base layer.  Agreed.

Suggestion to add AVC baselayer test as mandatory for all TE1,TE2,TE3, and TE4. No consensus (see discussion above). [ Editorial: Copy to previous discussion with note of later date.]

Suggestion to not include AVC baselayer test as optional for all TEs (TE1, TE2, TE3, and TE4).

Suggestion to use the same test conditions as TE-C6 for supplemental information.

#### Inter-layer texture prediction signaling (chroma filters)

Revisit of interpolation filter design for chroma for TE2.

Suggestion to use filters using 5-phases so that there is no phase shift in the interpolation filter. Agreement in spirit subject to text.  Agreed.  [Chair note: Filters uploaded as JCTVC-K0378.]

Suggestion to use bi-linear interpolation for the chroma interpolation in TEA2 (and others).

### Clarificiation on TEs and assignment of coordinators

TEA1: Jie Dong, Elena A., Jianle C., Andrew S. (primary: asegall@sharplabs.com)

TEA2: Yong H., D. Kwon., L. Guo (primary: liweig@qti.qualcomm.com) , Jinwen Z., Haricharan L, J. Kang

TEB3: X.Li (lxiang@qti.qualcomm.com), E. Francois, P. Lai, D. Kwon, A. Saxena

TEB4: J. Chen. (primary: cjianle@qti.qualcomm.com) , Elena A., Shan L., J. Park, Jie Dong, Andrew S.

TEC5: Vadim (primary: vseregin@qti.qualcomm.com), P. Onno, Shan L., Tammy, C. Kim, H. Yang, Haricharan L.

TEC6: Haricharan L., Wonkap (primary wonkap@vidyo.com) J., K. Kawamura

AhG ALF/modified deblocking: No chair identified.

AhG upsampling and downsampling filter: Elena A., J. Chen, Andrew S., Pankaj, T.

[October 18, 2012]

[Chair note: The BoG met again on October 18, 2012.  The agenda for the meeting was proposed as follows:]

Agenda

+ Revisit tool experiment descriptions (as needed)

+ Review/clarify tools to be included in the TEs

+ Other

Suggestion to include Class A instead of Class A+ sequences in the defined TEs.  Multiple participants supported this suggestion. Agreed.

Comment that new down sampled sequences for Class A should be created.

Suggestion to include combined tests in the TEs.

Suggestion to have an AhG on combined tests.

Suggestion that any technology proposed in a CfP response is a candidate for TE. Agreed.

Suggestion that any technology proposed in a response is a candidate for a TE. Agreed.

Suggestion to establish an AhG on SHVC tool experiments.  Mandates include establishing configurations (LD-P), generating anchors, reporting XLS, etc.  Make available two weeks after software is available.  X. Li, P. Onno, Y. Ye., Jill Boyce

Suggestion to establish an AhG on SHVC software. Mandates to prepare software (based on HM8.1) for experimentation.  Mandate to determine appropriate software for TEC6.  Make available to TEs within two weeks after the meeting. V. Seregin, D. Kwon, Y. He, T. Chuang, F. Le Leannec

Suggestion that the tool description must be to the coordinators within two weeks of the close of the meeting.  Agreed