| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0358 |
| --- | --- |

| Title: | BoG report on conformance testing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 teruhikos@jp.sony.com |
| Source: | BoG |  |  |

_____________________________

# Agenda

- Reviewing of related contributions

  - JCTVC-K0111, K0162 and K0330

- Bitstream design, features, and the assignment of volunteers

- Plan

- BoG recommendations

# Review of contributions

JCTVC-K0111 AHG5: On HEVC conformance testing [T. Suzuki, A. Tabatabai (Sony)]

(Chaired by W. Wan)

This contribution proposes the followings as a starting point of the discussion on HEVC conformance testing,

-           Define conformance testing similar to AVC conformance testing

-           Clarify what should be tested by using excel spread sheet

-           Set up a repository for conformance/bitstream exchange

More detail parameters of bitstream should be discussed during the Shanghai meeting

An initial draft is provided using the AVC conformance testing document as its initial basis.

The BoG recommends using this draft as a starting point for HEVC conformance.

A question was asked about both the static and dynamic tests being included and if they were included in the AVC conformance specification.  It was confirmed that both tests were included in the AVC conformance specification but also noted that it may not be used in practice due to difficult in testing.  The BoG recommends that the decision to include or remove the dynamic test should be further studied.

JCTVC-K0162 Corner case conformance bitstreams [C. Fogg, A. Wells (??)]

With the aim of improving the chances of interoperability, a list of conformance bitstreams is proposed that test corner cases the authors feel are likely to lead to behaviour mismatch among decoders. Suggested bitstream features are included in the candidate of conformance bitstream list.

JCTVC-K0330 Introduction of temporal_id_type [Arturo Rodriguez (??)] [late]

No presentation

# Conformance bitstreams

|  | Categories | Sub categories | Bitstream features | Volunteers | candidate |
| --- | --- | --- | --- | --- | --- |
| To test basic features | Block structure and partitioning (LCU/CU/PU/TU) |  |  |  | Samsung ? |
|  |  |  | inferred splits along non-CTU multiple picture borders |  |  |
|  | Intra coding | Intra prediction | Functional stage: Decoding of intra PU. Purpose: Check that the decoder can properly decode all values of intraPredMode for different Luma and Chroma transform block sizes. | C.S. Lim, F. Bossen | Nokia, Mitsubishi, NHK |
|  |  | Constraint intra prediction | Functional stage: Decoding of intra PU in non-intra slice when constrained_intra_pred_flag is set to one Purpose: Check that the decoder can properly perform reference sample substitution process. | C.S. Lim, K. Chono |  |
|  | Inter frame coding | AMVP |  | C. Kim, M. Zhou | LG, MediaTek |
|  |  | MVs | maximum number of motion vectors permitted per CTU |  |  |
|  |  |  | vectors reference padded picture edges. | M. Coban |  |
|  |  |  | clipping of motion vector range in MVP/Merge candidates | M. Coban |  |
|  |  |  | motion vectors are random, maximum page DRAM misses / minimize cache hits (according to memory bandwidth model... e.g. JCTVC-H007) |  |  |
|  |  | Merge |  |  |  |
|  |  |  | Merge candidate list index signaling: Check that merge candidate list can deal with every merge index (0..4) | B.Bross |  |
|  |  |  | Parallel merge: test all the possible combinations of log2_parallel_merge_level_minus2 and CTB sizes. | M. Zhou | MediaTek |
|  |  |  | number of maximum merge candidates signalling: test all the values of five_minus_max_merge_cand, i.e. 0, 1, 2, 3, 4 for CTB size of 64x64 | M. Zhou, | Samsung |
|  |  | WP | weighted_pred_flag and/or weighted_bipred_flag are equal to 1. | T. Chujoh | Technicolor |
|  |  |  | various weighted prediction parameter combinations | T. Chujoh | Technicolor |
|  |  | mvd_l1_zero_flag |  | F. Bossen |  |
|  |  | temporal_mvp | Test slice_temporal_mvp_enable_flag |  |  |
|  |  | Collocated ref idx | Test collocated_ref_idx (single slice or multiple slice/picture with different reference lists.) |  |  |
|  | HL syntax | slices, PSs | bitstreams with stuffed extensions (vps_extension_data_flag=1, sps_extension_data_flag=1, pps_extension_flag=1, ..) |  | Qaulcomm, Microsoft, Vidyo, Ericsson LG |
|  |  | dependent slice | Check: - dependent slices - WPP with dependent slices - tiles with dependent slices | B. Bross |  |
|  |  | POC | POC increments > 1 |  | Qaulcomm, Microsoft, Vidyo, Ericsson LG |
|  |  | RPS |  | M. Coban |  |
|  |  |  |  | F. Bossen |  |
|  |  |  | Slice header RPS with short-term pictures | R. Sjöberg |  |
|  |  |  | Slice header RPS with short-term and long-term pictures | R. Sjöberg |  |
|  |  |  | PPS RPS with short-term pictures | R. Sjöberg |  |
|  |  |  | PPS RPS with short-term pictures and inter-RPS prediction | R. Sjöberg |  |
|  |  |  | Mix of PPS and slice header RPS with short-term pictures | R. Sjöberg |  |
|  |  |  | Mix of PPS and slice header RPS with short-and long-term pictures | R. Sjöberg |  |
|  |  | RPL |  | M.Coban |  |
|  |  | RAP | all 6 (currently identified) RAP types | F. Bossen | Sharp Qaulcomm, Microsoft, |
|  |  | HRD | maximum number of pictures in DPB |  | Microsoft |
|  |  |  | bitstream with largest possible CPB delay (maximum number of coded bits per picture but within MinCR) |  | Microsoft |
|  |  |  | bumping DPB |  | Microsoft |
|  |  |  | Low delay |  | Fujutsu, Sharp, Qualcomm |
|  |  |  | Sub picture |  | Fujutsu, Sharp, Qualcomm |
|  | Transform | RQT | Check all depths of inter & intra transform tree: QuadtreeTUMaxDepthInter : 0..4 QuadtreeTUMaxDepthIntra : 0..4 | B. Bross | Motorola |
|  | Quantization | dQP | all depths of CU and diff_cu_qp_delta_depth | W. Wan, T. Suzuki |  |
|  |  | Scaling list | non-default scaling lists | T. Suzuki, M. Zhou |  |
|  | Other coding tools | AMP |  | X. Zheng, C. Kim |  |
|  |  | Transform skip |  | J. Xu | BBC, RIM, Ericsson, MediaTek, Canon |
|  |  | I_PCM | Functional stage: Parsing of I_PCM size information at SPS and I_PCM flag at CU. Purpose: Check that the decoder can properly decode pcm_flag for different intra CU sizes. Related Syntax: log2_min_pcm_luma_coding_block_size_minus3, log2_diff_max_min_pcm_luma_coding_block_size, pcm_flag. | W. Wan, C.S. Lim, K. Chono |  |
|  |  |  | Functional stage: Parsing and decoding of I_PCM CUs. Purpose: Check that the decoder can properly decode I_PCM CUs with different PCM bit depth. Related syntax: pcm_sample_bit_depth_luma_minus1, pcm_sample_bit_depth_chroma_minus1, pcm_flag | W. Wan, C.S. Lim, K. Chono |  |
|  |  |  | Functional stage: Decoding of I_PCM CUs with loop filter skipping. Purpose: Check that the decoder can properly decode I_PCM CUs by skipping loop filter. Related syntax: pcm_loop_filter_disable_flag, pcm_flag. Note: Combination with cu_transquant_bypass_flag will be tested. | W. Wan, C.S. Lim, K. Chono |  |
|  |  | Lossless |  | F. Henry | Huawei |
|  | Parallel processing tools | Tile | Maximum number of tiles in a random, non-uniform arrangement that changes from picture to picture, with random chosen value for loop_filter_across_tiles_enabled_flag, deblocking_filter_override_flag, slice_disable_deblocking_filter_flag, slice_loop_filter_across_slices_enabled_flag | - Fuldseth, M. Zhou | Sharp, eBrisk |
|  |  | WPP | entropy_coding_sync_enabled_flag on. Purpose: Check that the decoder handles wavefront parallel processing | F. Henry | Ericsson, Mediatek, Samsung, Broadcom |
|  |  | WPP and slices | entropy_coding_sync_enabled_flag on, number of slice varies randomly between 2 and maximum number of slices permitted, each non-first slice is either regular or dependent in a random way. Purpose: Check that the decoder handles wavefront parallel processing and slices | F. Henry |  |
|  |  | Tile & QPP | entry point of WPP |  | Sharp |
|  | Temporal scalability |  |  |  | Vidyo, Ericsson |
|  |  |  | range of nested temporal_id's |  |  |
|  | Deblocking filter |  | various cb, cr qp_offsets | T. Suzuki, R. Sjöberg |  |
|  |  |  | various beta_offset_div2, tc_offset_div2 | T. Suzuki, R. Sjöberg |  |
|  |  |  | trigger all deblocking decisions | T.Suzuki, R. Sjöberg |  |
|  |  |  | deblocking + tile, etc |  |  |
|  | SAO |  | all CTU sizes; with max. SAO information rate. | E. Francois | Samsung, MediaTek, Sony, Motorola |
|  |  |  | Random and extreme SAO offsets values | E. Francois |  |
|  |  |  | Random SAO merge left/up flag values | E. Francois |  |
|  |  | Slice- and component-level control | Pictures divided into x slices / randomly enabling SAO Y and/or SAO UV per slice | E. Francois |  |
|  |  | Tile- and component-level control | Pictures divided into x tiles / randomly enabling SAO Y and/or SAO UV per tile | E. Francois |  |
|  | Entropy coding |  |  |  |  |
|  |  | Sign data hiding |  | H. Henry | RIM |
|  |  | High through put binalizaion |  | Qualcomm |  |
|  |  | cabac_init_idc |  |  | Sharp |
|  |  | Max number of bins |  | V. Sze |  |
|  | Video resolution |  | pic_width_in_luma_samples = pic_height_in_luma_samples = (1 << Log2MinCbSize), with various CTU sizes. |  |  |
| Corner case bitstream |  |  | Maximum number of slices permitted for Profile & Level combination; Each slice has unique short_term_ref_pic_set(), ref_pic_list_modification() and pred_weight_table() with maximum num_long_term_pics | C. Fogg |  |
|  |  |  | CTU size is 64x64 with no cu splits, and only one coded transform unit in the lower right 4x4 pixel corner of the CTU with delaQp signalled (as raised by P. Kaspenberg on the JCT reflector 12-Sept-2012) | C. Fogg |  |
|  |  |  | A wide range of values for pic_crop_left_offset, pic_crop_right_offset, pic_crop_top_offset, pic_crop_bottom_offset | C. Fogg |  |
|  |  |  | combinations of transform_skip_enabled_flag, transquant_bypass_enable_flag, cu_transquant_bypass_flag |  |  |
|  |  |  | something to test the vast universe that is: POC signalling |  |  |

Two volunteers for each category is desiable.

Volunteers.

- T. Suzuki (Sony)

- M. Coban (Qualcomm)

- R. Sjöberg (Ericsson)

- C. S. Lim (Panasonic)

- W. Wan (Broadcom)

- M. Zhou (TI)

- C. Fogg (Harmonic)

- S. Lei (Mediatek)

- A. Fuldseth (Cisco)

- K. McCann (Samsung)

- F. Henry (Orange)

- F. Bossen (Docomo Innovations)

- B. Bross (Fraunhofer HHI)

- J. Xu (Microsoft)

- C. Kim (Samsung)

- E. Francois (Canon)

- X. Zheng (Hi-Silicon)

- V. Sze (TI)

- K. Chono (NEC)

- T. Chujoh (Toshiba)

# Plan

## Procedure

- Repository for this activity is necessary (ask Gary to set up ITU site)

- The volunteer upload the followings to the repository

  - bitstream

  - short description

  - yuv

  - trace (optional: useful for debugging. Recommend to include short trace. full trace is optional)

- Acceptance of bitstream

  - Uploaded bitstreams should be verified by two companies ?

    - Verify if it can be decoded

    - Confirm bitstream feature

- Confirm copy right of test materials. Use only the sequences allowed for this purpose.

## Time line

- When should we start ?

  - Start the first exchange based on HM9 (Draft 9 text)

  - Then revise the bitstreams based on final standard after Jan. 2013.

- Consider to remove the tool from the text if no conformance stream

# Recommendations

BoG recommends,

- to review draft conformance spec text proposed by JCTVC-K0111

- to review the table of the candidate conformance bitstreams and its volunteers

- to identify more volunteers, more conformance bitstream features

- to set up repository for this activity

- to remove tools from the spec if no conformance bitstreams