| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0360 |
| --- | --- |

| Title: | Need to code Intra pictures of progressive source as two fields |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arturo A. Rodriguez 5030 Sugarloaf Parkway Lawrenceville GA 30044 USA | Tel: Email: | +1 770 236 6483 rodrigaa@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

Coding of designated Intra pictures of progressive scan video as two fields is commonly practiced as it extends benefits to VOD and PVR applications.  Trick mode operations, such as fast forward or reverse playback, depend heavily on Intra pictures.  However, Intra pictures tend to be significantly larger in size (i.e., number of coded bits) in comparison to other types of pictures.  Coding designed Intra picture as two fields provision trick modes with a higher number of Intra coded pictures without exceeding bit-rate constraints.

Apology

The authors sincerely apologize for the tardiness of this contribution and would greatly appreciate its consideration.

# Requirement for coding Intra pictures as two fields

Extracted sub-sequences for trick mode operations contain a higher number of Intra pictures per second than the coded bitstream.  The extracted sub-sequence must not exceed the maximum allowed bit-rate.  However, in home networks, the issue is exacerbated as the allocated channel bandwidth is much lower than the specified level’s maximum bit-rate.  Home networks allocate bandwidth for normal playback operation - the bitstream’s bit-rate.  Coding a designed Intra picture as two fields allows for a first field to be coded as Intra and the second to be predicted from the first.  It does not typically result in better coding performance but it provides usability of the video stream.  During trick mode operations, an extracted sub-sequence of only Intra coded fields results in lower bit-rate in comparison to providing a full Intra frame.  It also allows for the extracted sub-sequences with a higher number of Intra pictures per second.

The size of Intra pictures in hierarchical GOP structures is significantly disproportionate in comparison to other pictures.   This is a result of the successive QP increments at each successive level of the picture hierarchy.  RDO parameters used for coding Intra pictures also favor less distortion, thus, results in a higher number of bits.  The average size (in number of bits) of the designated Intra pictures in AVC streams coded with one Intra field and a P field ranges from 6.25% to 13.9% of the stream’s bit-rate.  The size of full Intra coded frames ranges from 8.3% to 24% of the stream’s bit-rate. A higher number of Intra pictures per second can be provided for the former case.

We note that a new video coding specification should extend the same level of functionality and usability as found in prior specifications for wider adoption.  If the specification cannot meet application requirements, HEVC will not displace the incumbent video coding specification that meets the application requirements.

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).