| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0362 |
| --- | --- |

| Title: | Canon’s proposal for initial software model for scalable HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | S. Lasserre, F. Le Leannec, J. Taquet, N. Ouedraogo, S. Pautet, C. Gisquet, G. Laroche, T. Poirier, Y. Verdavaine, E. François, P.Onno, E.Nassor | Tel: Email: | +33(0)299876800 eric.nassor@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes a potential software for a starting point for the scalable HEVC proposal.

# Introduction

The choice of a reference software for future Tool or Core Experiments should take into account the following points:

- Common tools proposed by the CfP proponents should be considered.

- Good combination and tuning of the different tools should be selected.

- Such good combination can be found among the best performing proposals.

The proposal JCTVC-K0041 implements tools that are common to many other CfP proposals. This solution offers very good performance and it is proposed to consider a simplified version of this proposal as a possible starting point for the reference software.

# Software Description

JCTVC-K0362 is based on JCTVC-K0041 and has the following features:

- Similar structure as most of the proposals

- multi-loop encoding/decoding

- HM6.1 based

The tools in JCTVC-K0362 are a subset of the inter-layer tools from JCTVC-K0041, common with other proposals (see list of tools JCTVC-K0354)

- IntraBL mode (with simple fixed upsampling filter)

- Inter-Layer Syntax Prediction (motion and partition)

- Generalized Residual Prediction

- Intra Diff coding mode (no intra picture mode)

The tools removed from JCTVC-K041 are the following:

- Low complexity Intra coding: this tool gives a very good compromise efficiency/complexity and could be studied within a CE

- 2 Single layer tools, ALF and IBDI are also removed

The source code has the following characteristics:

- based on HM6.1

- Modified by Vidyo to create a scalable codec

- Modified by Canon

  - To add new tools

  - Tuning of the tools

The proponents consider that it is a good basis for experimentation, because of its sanity and because it includes many tools, including for instance inter-diff mode which is shared by several Cfp proposals.

# Conclusion

This contribution submits new elements for the choice of a software model. It is suggested to:

- Either select a simple solution with minimal common denominator as JCTVC-K0348

  - Each new tool introduced should be validated by a CE

- Or select more complete solution with several inter layer prediction tools

This new contribution provides a selection of several inter layer prediction tools combined in an efficient way. The source code is available upon request.

Aknowledgement: the proponent thanks Vidyo for providing initial source code based on HM6.1.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).