| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0365 |
| --- | --- |

| Title: | BoG report on range extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | David Flynn |  | dflynn@rim.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

A break-out group was held on 13th September 2012 to examine documents relating to range extensions proposed for version two of HEVC.  Documents considering test sequences, methods to address one colour sampling format by using another, review of the current AHG software (results and proposed principles), and new coding tools.

A number of recommendations are made relating to modifications of the current HEVC draft and software model to support non-4:2:0 chroma formats.  Principal of which is to follow a 4:2:0-like design principals unless demonstrable benefits are provided.

A side activity is continuing to produce working draft text based upon the current recommended design.

# Timeline

The current proposed timescale is:

- PDAM January 2013

- DAM July 2013

- FDAM January 2014

# Software models

Currently there is a software version as a branch of HM-8.0.  How should we proceed with the software model?

- The current model has a large amount of refactoring, should this be merged into HM-9?  If any merging takes place it should be structured as a number of commits to aid review and understanding.

- The current model has a large number of options, how should these be preserved?

- Suggestion that a simple minimal version is made as a baseline (which could be suitable for merging) and used for comparison of all tools.

- Should anything be “accepted” at this meeting, or should there be a software adoption of anything more complicated than the essential features?

- There are a large number of changes relating to 4:2:2, should 4:4:4 be merged first and 4:2:2 be kept as a development branch?

# Test material

## Contributions

JCTVC-K0207 AHG7: Update on full-chroma (YUV444) screen content test sequences of JCTVC-H0294 [Shuhui Wang, Tao Lin, Kailun Zhou (Tongji U)]

Provides five sequences for screen content that are quite different from the current JCT-VC/HM Class F sequences.  All sequences have been captured in 1080p 8-bit full-range RGB 4:4:4 from the HDMI output of a graphics card at 20fps.  The material contains very high frequency features that would normally be considered inappropriate in television systems.  Some sequences have mixed screen and video content.  This material has resolved previously highlighted potential copyright issues.

The proponent will make the sequences available for viewing during this meeting.  It is noted that the sequences are longer than 10 seconds and will need to be trimmed.

Comment: It was pointed out that the RGB to Y’CbCr conversion has used the Rec.601 matrix which isn’t the native colour space for modern computer systems.

The proponent still has the RGB source material that can be used to perform a more desirable conversion using the Rec.709/sRGB matrix.

Comment: the capture is at 20fps, is the native video content in the sequence temporally aliased?

Yes, this would be the case, but that is not a major issue for the codec.

Comment: the sequence containing embedded video uses a highly compressed sequence with obvious coding artefacts.

It would be trivial to embed a new sequence in that region of the content without requiring a new capture.

Comment: what target rates for screen content?

Examples: wifi display and screen casting on services such as youtube.

Comment: The terms are R&D only, does that include use in journals for example?

Some participants commented that they would like a more general non-commercial licence that would allow use in trade shows etc.,.

Contributor states that these are the terms from the legal department

JCTVC-K0246 AHG7: The need for screen-content/graphics test material for extended chroma formats [R. Cohen, A. Vetro (MERL)]

Informational document identified that features such as transform skip are not strongly exercised in the AHG7 test sequences, demonstrating very similar gains to class B in HEVC common conditions.

Effects of disabling transform skip for 4:4:4 sequences

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 93% |  |  | 92% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Effects of disabling transform skip for 4:2:2 sequences

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 91% |  |  | 90% |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  |

Question: What high-resolution images have a large number of 4x4 blocks?

Question: Will the new proposed material assist?  Proponent will talk offline to perform a single frame test of the proposed sequences with TS enabled/disabled.  New information: K0338 repeats this test on a single frame of the K0207 sequences.

JCTVC-K0368 Impact of transform skip on screen content sequences from JCTVC-K0207 [R. Cohen (MERL),T. Lin (Tongji U)] [late]

In JCTVC-K0246, it was shown that with the current set of AHG7 4:4:4 and 4:2:2 test sequences, enabling/disabling transform skip yielded BD-Rate changes of 0.0%. The purpose of this document is to report the effects of transform skip on the set of five 4:4:4 screen-content being offered for use by JCTVC-K0207. The first frame from each of the five sequences was coded using HM-8.0-dev-ahg7, with transform skip enabled then disabled. The resulting changes in BD-Rate ranged from 23.9% to 35.1%, with an average of 28.2%.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Overall | 28.2% | 34.5% | 31.9% | 28.2% | 34.3% | 31.7% |
| Enc Time[%] | 92% |  |  | 91% |  |  |
| Dec Time[%] | 101% |  |  | 109% |  |  |

It should be noted that this test is performed on a single input frame per sequence.

JCTVC-K0296 AHG7: Additional test sequences (computer graphics) [M. Mrak, G. Thomas] [late]

This contribution provides five 1080p CG rendered sequences in 8-bit 4:4:4 RGB format of flypasts through buildings.  All sequences contain similar content with minor changes in camera paths.  Since the sequences are of static buildings and environments they are frame-rate agnostic to some extent.

The sequences are of 150-300 frames in length.

It was suggested to take a 300 frame sequence for playback at 30fps as this is a clean multiple of the frame rate used for playback by the equipment used at JCT-VC meetings.

Comment: The sequences are only 8 bit, can it be re-rendered as 10-bit?

The proponent has offered to provide alpha channels used as masks in the subsequent production process for compositing overlays.

Comment: The content appears to contain a large amount of green detail, is there much left in Cb and Cr after conversion?

The proponent doesn’t have any other material that is less “green”

## Other remarks

The proponents of K0211 has suggested that they have RGB test material available in higher bit-depths that could be shared with the group.

The proponents of K0181 would like to make some changes to some sequences to reduce length and runtime.

The EBU has now published a set of 4k 10bit RGB test material, two sequences of which are available under a Creative Commons Attribution-NonCommercial-NoDerivs 3.0 licence.

Discussions have suggested that the EBU may be willing to provide access to select 1080p 4:2:2 sequences.  The previous meeting identified some sequences that may be of interest and the BoG recommends a liaison statement to request cooperation for performing experiments using these sequences.

Viewing of the above sequences will be arranged during the meeting.

# Methods to address a sampling format using a different sampling format

JCTVC-K0211 AHG7: Colour Spaces and Chroma Sampling Methods [Pankaj Topiwala, Wei Dai, Madhu Krishnan (FastVDO)]

Techniques for integer color transforms and spatial resampling are developed for HEVC applications, using tools embedded in HM 8.0 (AHG7 branch).  Low-complexity coding in 4:4:4 format is performed starting/ending with RGB data, and coding performance is measured in terms of average PSNR in RGB. Our tools convert the RGB data to one of several (YUV-like) integer color spaces, and then down sample the data to 4:2:0 prior to encoding/decoding. The decoded data is converted back to RGB 4:4:4, where performance quality is measured. This approach for a baseline for 4:4:4 coding, against which more sophisticated, native 4:4:4 techniques can be measured in their complexity/performance tradeoffs.

This contribution has two parts: 1) sampling, 2) colourspace transformations.

The first part compares 4:4:4 coding using the AHG7 software vs. converting to 4:2:0 using just HM-8.0.  Using the following method: RGB -> Colourspace transform -> 4:2:0 -> end/dec -> 4:4:4 -> Colourspace transform -> RGB

Proponent suggests that any BD rate measurements are unreliable since the QPs used do not overlap in terms of bitrate.

The comparisons are using averaged RGB PSNR.

It was observed that some sequences showed no substantive difference in performance between the 4:4:4 and 4:2:0 paths.

Comment: Which sequences? The SVT sequences, other sequences did not exhibit this behaviour.

Comment: Which conversion was used for 4:2:0 were there differences in performance?  Proponent thinks it was Y’CbCr only

The proponent views up/down sampling being a normative part of the decoding process.

Comment: but this isn’t in the loop at the moment

The proponent asserted that this could be used to produce multiple outputs from a single stream (eg, 4:4:4 and 4:2:0)

Comment: how aliased is the down converted version (ie the proposed 4:2:0 output)?  Proponent agrees that, for display one would have to do some post processing.

The contribution also presents variations of colour transforms using 4:4:4 only.  This would be VUI only at the moment.

It is asserted that the combination provides a low complexity approach to achieving 4:4:4 compression.

Comment: it would be nice to see the behaviour on screen content with high frequency colour information.

JCTVC-K0302 AHG7: On processing 4:2:2 chroma format [A. Gabriellini, M. Mrak (BBC)] [late]

This document presents the results of an experiment aimed at investigating the efficiency of coding 4:2:2 chroma formats using AHG7 software (branch HM-8.0-dev-ahg7). The modified codec takes a 4:2:2 input and it upconverts it to 4:4:4 before starting processing it. Thus the bitstream generates a 4:4:4 signal. Both locally decoded output (on the encoder side) and the decoder’s output are downconverted to 4:2:2 before writing to file to obtain a sequence the same chroma format as the input. The aim is to avoid having to tackle the potential issues generated by the 4:2:2 format, where non square chroma blocks must be processed. The experiment was limited to intra and low delay test points. The experiment provides a benchmark for the potential compression and complexity penalty associated with processing 4:2:2 sequences as 4:4:4 for further study.

When looking at the AHG7 software that supports 4:2:2 and 4:4:4 formats the proponent observes 4:2:2 support requires a larger number of changes and introduces tools to use non-square transform units for chroma.

An experiment is performed that uses a very simple upsampling filter to convert 4:2:2 to 4:4:4, performs an encode/decode and then down-converts using a simpler filter to obtain 4:2:2.

Comparisons were performed against the JM18.4 anchor (4:2:2) as used by the AHG.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| 4:2:2 upconverted to 4:4:4 vs JM 18.4 4:2:2 (8-bit processing) | -12.3% | -15.6% | -19.7% | -14.6% | -20.2% | -27.3% |
| 4:2:2 upconverted to 4:4:4 vs 4:2:2 anchors | 2.1% | 14.8% | 13.0% | 1.8% | 11.7% | 7.6% |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| 4:2:2 upconverted to 4:4:4 vs JM 18.4 4:2:2 (8-bit processing) | -37.4% | -40.6% | -40.5% | -40.0% | -48.8% | -51.3% |
| 4:2:2 upconverted to 4:4:4 vs 4:2:2 anchors | 10.4% | -1.0% | 1.6% | 10.5% | -7.7% | -7.0% |

Comment: bitrates in non-intra sequences are very low, between 1 and 8 Mbit/sec.

The proponent suggests that a non-native 4:2:2 solution should be considered.

Comment: 4:2:2 systems are very prevalent in the broadcast industry, and multi-generation performance is a key consideration in the broadcast chain.  Suggest that that the proponent examines the multi-generation performance of such a system.

Comment: Is this approach something that is effective?

Comment: Is this a patch while those issues are being worked out?

It was suggested that this could be similar to the interlace issue.

Comment: it isn’t quite the same, in the interlace model we are preserving the ability to carry interlaced content, whereas this doesn’t preserve the ability to carry 4:2:2 content natively.

The proponent suggests that this would save implementation complexity.

# Non-4:2:0 by extension of current design

## Contributions

JCTVC-K0181 AHG7: Options present in Extended Chroma Format model [K. Sharman, N. Saunders, J. Gamei (Sony)]

In this proposal, the 4:2:2 / 4:4:4 AHG7 model, which was previously proposed in JCT-VC J0191 and extended from the last meeting to also include the functions of JCT-VC J0357, is examined. Many of the available options are individually tested to obtain BD-rate changes and processing times, using the test material agreed upon at the previous meeting (JCT-VC J0581). Benchmarks against AVC JM18.4 are also given for comparison.

It is proposed that the committee examine these results to help agree on a base 4:2:2 and 4:4:4 (and 4:0:0) system, which may then be further developed. It is also proposed that the committee consider the operating points and test conditions for these new formats.

Due to the large number of test configurations, comparisons were performed in the HE10 configuration only.  Of the current model, 33 of 44 options have been tested individually (with the addition of some combinations).  Not tested: 32x32 scaling list for chroma, MDCS limits

There are many changes that can be made: changes to context modelling can give gains for some cost (eg, trebling context variables can give 0.2--0.3% gains, proponent would like to have the current  NxN chroma PU split option for 4:4:4 as the new default, and also adopt the usage of the  intra reference sample filtering option as new default.

There were issues with RDOQ, and an updated test model has been used to address some issues.  The proponent would like more attention given to what RDOQ is doing in HM.  Some possible bugs in HM8 have been pointed out on the mailing list,although no response on these issues has been received.

For 4:2:2, nothing stands out as deviations from the current AHG common condition.  For 4:4:4 there are some benefits with some changes.

As other contributors have comments on methodology and design goals, recommend to revisit this document after those to review the details.

The tests were reviewed by the BoG with the objective to maintain 4:2:0-style processing unless there was a good reason to deviate from this.

Contributor will upload new results based upon HM8.1 with bugfixes and increased support for higher bit depths:

| Test | Remarks | Recommendation |
| --- | --- | --- |
| 01a: 4x4 TU in chroma gives -0.6%/+0.5/+0.7 Luma gain | Comment: is there any net gain? Recommend: 4:2:0-like mode (8x8 chroma)? Comment: no RGB results? This might be a feature of the soft material Comment: it doesn’t make sense, would like 4x4 TU's (i.e. by not selecting this option). Recommend: 4:4:4-like mode (4x4 chroma) Comment: does this change RQT? No, chroma is still following luma tree. Comment: this would be confusing by allowing chroma to split to 4x4 but not allowing different modes. | 4:4:4-like mode (4x4 chroma) Not Adopt |
| 01b: changes PU structure to give more signalling to give 4 PUs for chroma. | Gives -0.2%/-0.65%/-0.79%. Comment: this affects not just 4x4 and 8x8, in general this can affect other things. Comment: does this change RQT for chroma? No, this is the same tree: chroma and luma will be exactly the same. | Adopt |
| 01c: Entropy coding using double height coefficient groups for non-square transforms (4:2:2) | Losses 0.17/-0.14/0.39. Currently using two 4x4 groups. | Keep current |
| 2a: Use only DM_CHROMA. 2b: Alternative intra prediction mode (either scale the angle or change the source) | Losses when changing current default NB, this uses rectangular blocks | Keep current |
| 3a: Take into account chroma cost while doing luma | Comment: encoder decision option Luma loss, large encoding time increase +40% | Keep current |
| 3b: similar to the above, preestimate with DM chroma | Better than 3a, but large increase in encoding time | Keep current |
| 3c: as 3b, but constrains search to always be DM_CHROMA | +20/30% encoding time | Keep current |
| 3d: combined set to reproduce K0192 | +0.47/-0.67/-0.48 4:4:4, encoding time increase +50% Ie, to process all channels the same way, gives possible loss, and encoding time increase due to joint optimisation. | Keep current |
| 4a: modify filtering for DC/EDGE filtering of chroma (K0192) | Marginal as to any change in performance | leave for now |
| 4b: as 4a, but for 4:2:2 |  | leave for now |
| 4c: Vertical edge/DC filter in 4:2:2 |  | leave for now |
| 4d: Do same reference sample filtering on all components of intra prediction: | Gives gains in all components (0.2%) NB: adds a change relative to 4:2:0 Comment: is already mode-dependent switched in luma Comment: lack of compelling evidence, test set may be too weak Comment: but there is a reason that we do this for luma, that will exist in RGB Comment: is that the same filter as luma Yes. | adopt |
| 4e: Should 4d also apply to 4:2:2? | Comment: nb, already mode dependent, so can be switched anyway depending upon chroma_format_idc Comment: align to 4:4:4 Comment: Is a profiling issue, if separate or not. Comment: if same profile as 4:4:4, it would be for free. | not for now (opposition wasn’t strong) |
| 4f: |  | Redundant |
| 5a: uses luma inter pred filter for chroma | Strange results (Cr performs opposite to Cb) Losses Comment: might be beneficial to RGB | Further study |
| 6a: DST for chroma | Small bitrate reduction, loss for chroma. Comment: not surprising given that we don’t do dst on chroma in 4:2:0 Comment: might be worth reconsidering when there is RGB test material in the test set | Further study |
| 6b-g: changes in quantiser. | Comment: RDOQ HM needs to be revisited Author recommends not changing any of these. 6b has a 10% luma loss for not compensating for sqrt(2). Comment: only matters for non-square transforms. Comment: What should chroma weighting in BD rates be? Eg in 4:2:0 an order of magnitude change is important.. Comment: in this case there is no change in luma PSNR. Comment: how is the adjustment performed in the case of transform skipping? Note: transform skip applies to any transform with a 4 in it. Answer: no adjustment, not needed. Comment: are shifts modified? No. | Keep current |
| 6f/g: doing sqrt(2) mul/div in quantiser | Comment: extra complexity of calculation per coeff. Tiny gain in luma and small loss in chroma. | Keep current |
| 6h: single transform skip flag for all channels | 4:4:4 only, no change in performance | Keep current 4:2:0-like behaviour |
| 7a: use same chroma CBF context selection for chroma as luma (and saves a context) | Gain is nothing, possible oddness in V Comment: try swapping components to see if it moves Comment: do we care about a single context? Comment: if unified 4:2:2 and 4:4:4 nothing is saved | Keep current 4:2:0-like behaviour |
| 7b: for 4x8 using stretched sig map contexts | Small choma gain, nothing in luma. Comment: given this is strictly coding efficiency and we see no gain in luma, it isn’t significant. Comment: goes against earlier desire to have 4:2:0-like encoding | Keep current 4:2:0-like behaviour |
| 7c: see 7b. |  | Keep current 4:2:0-like behaviour |
| 7d: Use luma context selection for chroma, increases the number of contexts | Some gains in 4:2:2, mixed in 4:4:4 Comment: goes against earlier desire to have 4:2:0-like encoding | Keep current 4:2:0-like behaviour |
| 7e: allow two chroma to have independent contexts following the chroma rules | Cb gain, Cr loss | Keep current 4:2:0-like behaviour |
| 7f: combination of 7d+7e, completely independent using luma selection | Mixed gains/loss | Keep current 4:2:0-like behaviour |
| 7g: use separate contexts for cb and cr. | Cb gain, Cr loss | Keep current 4:2:0-like behaviour |
| 7h: when neighbouring group is unavailable assume is same as current | All losses. Comment: what was the motivation for this, not related to 4:4:4 or 4:2:2 Can’t remember Recommend: no change Comment: comparison with JM, V component always has bigger gains compared to U. Comment: this could be because the contexts adapt to the first component that trains the second | Keep current |
| 8a: different MDCS size limits | Mostly losses | Keep current |
| Untested: QP mapping table extension for chroma. | Untested due to QPs in use. | Adopt |
| 9a: Shows 01b,04d,04e | Combinations of adoptions Gives gains and no enc/dec time change Ignore 4:2:2 results (as we rejected 4e for 4:2:2) |  |

Some tests demonstrate non-equal gains for chroma components, with Cr having worse performance than Cb.  It was suggested to re-encode a sample sequence and reverse the chroma components to see if the change is reflected in the results.  This test was performed and the expected change occurred, suggesting that this is not a bug in the software.

It was noted that the base system used in for the production of these results uses non-square transforms, with no results for using square for 4:2:2.

If keeping the 4:2:0-like behaviour for using only square transforms, some simulations should be run during the meeting to confirm the desired behaviour.

Comment: does such a vertical splitting use one or two Pus?

Answer: One PU

Comment: what should we pick given that we don’t now restrict that in 4:4:4

Comment: Pick what we will have results for and recommend testing of alternative.

JCTVC-K0171 AHG7: Transforms for extended chroma formats [C. Rosewarne (CiSRA), M. Maeda (Canon)]

This contribution presents an approach to transforms for the extended chroma formats that avoids the introduction of 32-point transforms for both the 4:2:2 and 4:4:4 chroma format (in either horizontal or vertical directions) by splitting the region in chroma into multiple 16x16 regions, thus reusing existing 16-point transform logic.  It is asserted that this is beneficial for implementations having separate luma and chroma processing paths, as 32-point transform logic does not need to be introduced into the chroma processing path.

Design choices for 4:2:2 and 4:4:4.  Expresses a concern that implementations will have different logic paths for luma and chroma.  eg., larger transforms allow reusing the existing transform logic in the chroma path.  Suggests all transforms in chroma are limited to 16x16 (including converting the non-square 4:2:2 transform to two square transforms, although non-square transforms smaller than 16x16 are permitted).

Results for HM-8.0.

|  | All Intra HE10 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U |  | V |
| Overall(4:2:2) | 1.2% | 2.0% |  | 2.8% |
| Overall(4:4:4) | 1.2% |  | 2.0% | 2.5% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 112% |  |  |  |

Comment: this is very important, it may not be so straightforward for non-square transforms in hardware (which also has lots of knock-on effects for QP normalisation)

Comment: it is a significant change to use non-square.

Comment: 32x32 might be okay for chroma, but non-square probably not

Comment: need to look more into this.  Non-square transform continues to provide games.

Comment: square transform blocks are forced for some sizes, have they experimented using square blocks for all sizes?

Answer: no not tried for all sizes.

JCTVC-K0338 AHG7: Cross-verification of JCTVC-K0171, Transforms for extended chroma formats [R. Cohen (Mitsubishi)] [late]

Everything matched.  Reports that decoding times in K0171 were spurious.

JCTVC-K0322 AHG7: Comments on 422 and 444 coding tools and software [R. Joshi, G. Van Der Auwera, J. Sole, M. Karczewicz (Qualcomm)] [late]

AHG7 recently released software that adds capability to code extended chroma formats (422 and 444) to HM8.0. The software contains a number of coding options that can be configured using #defines. This document presents some observations regarding the software and choice of default coding tools. It recommends that by default, the 422 and 444 chroma processing should be aligned with 420 chroma processing. It is remarked that more time and better documentation is needed to understand the AHG7 software and extension of coding tools to 422. Finally, a bugfix for planar mode intra prediction calculation for 422 chroma formats is reported. For the intra_he10 configuration, a BD-rate of -0.6%, -0.3% and -0.4% is reported for Y, Cb and Cr components, respectively, with respect to the AHG7 anchor.

Informative contribution on 4:2:2 and 4:4:4 coding tools and software.

Background: AHG7 software contains lots of options, exhaustive testing is very hard.  Contribution proposes some of the options to pick.

Regarding 4:2:2 and 4:4:4, could align the processing to 4:2:0 processing, or in 4:4:4 can align to luma.  For example:

- CABAC contexts, smaller number of contexts used for chroma than luma.  Should that be the same for 4:2:2 and 4:4:4?

- Choice for using 4:2:0 TU structure, eg 8x8 chroma block in 4:4:4 cannot be split to 4x4.

- 4:2:2 and 4:4:4 should be aligned to 4:2:0.  Will need to support 4:2:0 in same system, so differences in processing is additional logic.

Suggests to use 4:2:0 compatibility as a base and then evaluate improvements to that.  Current defaults tend to satisfy this criteria.

Eg, coefficient coding. 4:2:0 chroma blocks are subdivided into 4x4 subblocks.  Same is possible in 4:2:2 and 4:4:4 (cf. NSQT)

Question: are there any changes that you suggest should be the base configuration?  A: Unsure if some macros will actually achieve that effect.  Suggests to go through macro by macro to check.  For the majority this is already the case.  NB, there may be something not guarded by a macro that may be affected.

Comment: thinks the suggestion is good.  4:4:4 10bit will be in consumer market.  So should try to minimize changes in 4:2:0 design.

Recommend: base model should be aligned to 4:2:0-style processing where possible, anything that deviates should have demonstrable benefits over that.

Comment: everything has been done in YCbCr.  But in RGB chroma QP clipping would need to be modified, so some modifications

AHG7 software:

- Participants have put a lot of energy into production of the software

- Huge amount of changes looking at a simple diff.  many of these are cosmetic changes that use for loops rather than copy and paste.  Some refactoring is more substantial, eg using a TU structure.  Would like a two step process, eg one version that just includes the refactoring.  Then it is much easier to know what changes relate to a specific choice (which is currently hard).

- Some things are not under macros, and a refactoring process would help in that respect.

- Some processing that isn’t directly under a macro is affected by a macro and can be unexpected.
4:2:0 TU structure macro: delocking is performed at 16x16 block size for chroma. Otherwise it ix 8x8.

- Some more documentation on what is being affected by a particular macro.

- Bug in planar intra prediction 4:2:2 is being calculated incorrectly. Leading to a -0.6/-0.3/-0.3% BD-rate loss

JCTVC-K0192 Chroma coding structure [K. Kawamura (KDDI)]

This contribution proposes the consistent coding structure and tools for all planes. Since 4:4:4 chroma format has the same resolution for all planes, some limitations can be relaxed. Furthermore, different structure and prediction mode between each plane cause the visual artifact in RGB 4:4:4 format.

Focuses only on 4:4:4. Using the same structure for all colour planes.  RGB colour format to always use same prediction mode for all planes, otherwise a coding artefact may be visible.

Comment: we should be measuring performance end-to-end in RGB.

Refinement: we should measure the performance in the source space, not the Y’C*C* space.

Comment: how much is encoder only vs structure? A: Some can be achieved as encoder only restriction, some is a structural change

Note: this conflicts with earlier proposal on 4:2:0-like design decisions

Proponent suggests it would be nice for 4:4:4 though.

JCTVC-K0196 Crosscheck of chroma coding structure (JCTVC-K0192) [S. Matsuo, M. Matsumura, H. Fujii, S. Takamura, A. Shimizu (NTT)] [late]

Not present

# New coding tools

JCTVC-K0253 AHG7: performance of extended chroma mode for non 4:2:0 format [J. Kim, B.Jeon (LGE)]

This contribution reports the performance of chroma prediction which predicts chroma samples using linear combination of luma samples. The average gains of HEVC-inherited LM chroma are -2.8%, -5.8%, -5.0% for the case of all intra high efficiency in chroma 4:2:2 sampling and -3.1%, -4.4%, -5.1% in chroma 4:4:4 sampling. With HEVC-inherited LM chroma, the average gains for the case of all intra main are -2.5%, -5.3%, -4.4% in chroma 4:2:2 sampling and -2.8%, -4.1%, -4.5% in chroma 4:4:4 sampling. It also proposes that the parameters for the linear combination are acquired by each PU to reduce the number of the derivation to get them. It reports additional gain of -0.1%, -0.3%, -0.3% for 4:2:2 sampling and 0.0%, 0.0%, -0.1% for 4:4:4 sampling in the case of all intra high efficiency and -0.1%, -0.3%, -0.3% for 4:2:2 sampling and 0.0%, 0.0%, 0.0% for 4:4:4 sampling in the case of all intra main.

This proposal is identical to the first part of K190.

Proponent of an identical proposal for the first part agrees with such a system.

Comment: LM had an issue with decoding and encoding dependency

Comment: doesn’t know if this will be any better now, worst case complexity is the same

Comment: high bitrates, would imply smaller blocks and more worse case for software

Comment: baseline version needs to work on many devices, this is more specialised and could possibly afford it.

Comment: is there such a thing as a professional profile?

Comment: might we have two profiles?

Comment: improvement is nice for now

Comment: we have already seen RDOQ issues and bugfixes in planar prediction

There are updated results: to be uploaded

Comment: with LM mode on do you see any subjective improvement? A: No subjective test performed

Comment: LM has been studied extensively so far, this understood and mature

Comment: but not in this context.

Comment: Do you have text? A: No.

Comment: Integration, not adopt now as we need a stable base for which to make comparisons and provide a PDAM text for the next meeting, but later?

Proposal (2) is simpler than (1) but also gives a gain.

Recommendation: further study in AHG, based on baeline version.

JCTVC-K0190 On luma-chroma mode support [K. Kawamura (KDDI)]

Identical to the above, no need to present

This document provides RGB results for proposal (1) of K0253.

JCTVC-K0333 Cross-check of extended chroma mode for non 4:2:0 format (JCTVC-K0253) [K. Kawamura, T. Yoshino, S. Naito (KDDI)] [late]

In agreement

JCTVC-K0194 Crosscheck of luma-chroma mode support (JCTVC-K0190) [S. Matsuo, M. Matsumura, H. Fujii, S. Takamura, A. Shimizu (NTT)] [late]

In agreement

JCTVC-K0191 Inter-plane intra coding [K. Kawamura (KDDI)]

This contribution proposes an intra coding approach based on inter-plane prediction of residual signal as HEVC fidelity range extension. This method reduces correlation of residual signals between two planes based on linear prediction model. Prediction parameter is calculated by the sum of the estimated parameter and the estimation error which is encoded as an additional information. The BD-rate gains for Y/U/V in AI-HE10 configuration achieve 1.5%/4.7%/3.9% in YUV422 case.

Presenter will upload revised document that includes presentation.

Question: complimentary to LM mode, but only tested individually. Suggests testing to compare the combined effect.

Suggestion: take LM and only RM if it demonstrates further gains.

Presenter asserts that gains shown in 4:2:0 previously that the gains were additive, suggesting a 17-18% gain being likely.

Recommend: further study

Question: Is this expected to work in 4:2:0 sequences that operate in a 4:4:4 decoder?

JCTVC-K0195 Crosscheck of inter-plane intra coding (JCTVC-K0191) [S. Matsuo, M. Matsumura, H. Fujii, S. Takamura, A. Shimizu (NTT)]

Not present

JCTVC-K0193 Colour-space transformation [K. Kawamura (KDDI)]

This contribution proposes the adaptive color-space transformation of residual signals for HEVC FrExt. This approach adaptively transforms prediction error signals from 4:4:4 chroma format to a sub-optimal colour space which reduce redundancy of pixel value between planes. The transformation matrix is derived for each coding unit by a singular-value-decomposition. The colour-space transformation is applied to prediction error of both intra and inter mode. Simplified singular-value-decomposition is realized only by integer multiplication and shift operation with limited iterations. Additional information is not necessary because residual signals are always transformed. The BD-rate gains for YUV 444 sequences are 6.7%/7.2%/8.7% for all intra / random access / low delay B with 10bit configuration, respectively.

Motivation is to exploit inter-colour-plane redundancy.  Singular value decomposition for each TU.

Transform matrix derived via singular value decomposition of locally decoded picture, not requiring any additional syntax.  In inter pictures, derivation is achieved by using an extended region of the motion compensated prediction.

Presenter will upload presentation

The proposal shows very large gains for RGB sequences, with substantial encoding time increases attributed to the need to perform a joint optimisation of intra RDO decisions across all three planes.  This proposal is based upon the model configuration in Kxxxx where all planes use the same CU/PU/TU structure, same intra prediction mode and requires the use of DM_MODE only.  Results using the new conditions are not available.

Comment: A new matrix is computed for every TU?  Yes.

Comment: results are not using AHG7 sequences (due to lack of RGB content) all sequences were SVTs.

Comment: Gains are impressive in RGB, do we need a tool that does an automatic colour conversion?  Gains would diminish greatly if a perceptual colour space were used for plane coding.

Comment: Matrix is trained on reconstructed data, do you get improvements at higher rates than lower rates?  Looking at the results, it appears to become more valuable at lower bitrates (in Y’CbCr).

Comment: How much can this be done at the picture level?  Ie, only changing the matrix once per picture.

Comment: Why is it taking longer on the decoder in the Y’CbCr case? 2% gain for 16% run time

Comment: This tool adds dependencies across colour planes, which may be undesirable.

Comment: How about a small dictionary of transforms?

Comment: It would be interesting to see how much the matrix is changing

The contributor says that the source code is available upon request.

Recommend: Further study

JCTVC-K0198 Crosscheck of colour-space transformation (JCTVC-K0193) [S. Matsuo, M. Matsumura, H. Fujii, S. Takamura, A. Shimizu (NTT)] [late]

Not present: results match and run times are representative.

JCTVC-K0133 AHG7: Full-chroma (YUV444) dictionary+hybrid dual-coder extension of HEVC [Tao Lin, Shuhui Wang, Peijun Zhang, Kailun Zhou (Tongji U)]

This contribution presents full-chroma (YUV444) dictionary+hybrid dual-coder extension of HEVC. A YUV444 dictionary-entropy coder is added to the traditional YUV444 hybrid coder. It is reported that BD-rate reduction is 34%~85% to code the five YUV444 SCC test sequences of JCTVC-H0294 (re-submitted in JCTVC-K0207).

Comment: it would be nice to have a thorough explanation of the dictionary construction and referencing process.

The feature is enabled or disabled in SPS. When enabled, a per-CU flag is introduced, allowing either the normal leaf coding of HEVC, or offering a dictionary based method to produce the reconstructed pels.  A dictionary is defined to be an n-byte string of the most recently reconstructed pixels of the current picture, in effect forming a sliding window on the reconstruction process.  Reconstruction of each CU is performed column wise, by copying a substring from the dictionary.  The fundamental memory footprint of the decoder is not changed, as the dictionary is in-effect a reference to the reconstructed pels.

It was observed that the engine has been updated to use CABAC since the original proposal in previous meetings, introducing XX contexts.

At the encoder, a hash table is used to provide an efficient method of searching the dictionary for matching substrings.

Software is released in K0134.

The reported performance increases are impressive, offering a ~60% BD-rate “gain” (Transform skipping is enabled in this configuration) with a 64kiB dictionary (and encoder-only 138kiB hash table).

Size of dictionary can be customised from 4k to 4MiB.  There is a 20-30% performance difference between the smallest and largest.

Comment: Are the BD rate measurements still meaningful with such a tool or large gain?

After examining the results and rate-distortion graphs, it was noted that there is no overlap between the anchor and the configuration under test.

Comment: A better comparison may be against lossless.  A test would be to code the entire frame losslessly and switch this tool on and off, to measure the true gain since it is in essence a lossless tool.

Lossless coding: In the AHG7 software, there is a switch to turn lossless on and off everywhere, replaced CuTransquantBypassFlag with *False, still enableflag/.

Comment: it is a complicated direct mode

Comment: a lot of the content is an exact match, might be desirable to encode first frame losslessly (or at least in part).

Comment: if we’re serious about coding screen content, we need some method for coding that type of content.

Comment: an interesting way to do lossless coding of screen content.

Comment: will work well with images with parallel lines, will it work well with diagonals?

Comment: In worst case this could be predicting one byte at a time?

Comment: How would this work in a 10bit system?

Comment: how does this compare with gif or png?

Comment: there are usecases for this sort of hybrid system.

Comment: this codec is difficult to rate-control?

Comment: could this be modified to allow an inexact match and non-lossless behaviour?

Comment: What is the implication when tiles or slices are enabled?  It was suggested that each tile or slice would have its own dictionary.

What next?

Serious tests, with better conditions defined?

Question: Are we allowed to develop a more efficient lossless coding mode?

Recommend: Serious further study

Comment: trying to mix too many things together.  What tools should we use in frext?  Frext may need different types of tools for different types of content.  May need two different AHGs.  If developing specific F class tools.  Some form of clear demarcation would be good.

Comment: do you have to touch every part of the codec? No, just CU

Comment: it could be possible to study at CU level separate bits (ie not frext)

Comment: There are cost constraints in consumer application that need to be considered

Comment: how does the system work in a 4:2:0 configuration?

JCTVC-K0134 AHG7: HM software implementation and source code for JCTVC-K0133 [Peijun Zhang, Tao Lin, Xianyi Chen, Xiaojuan Jin (Tongji U)]

Confirmed the BD rate but didn’t perform code analysis.  Presenter mentions that the dictionary is not always small.

JCTVC-K0329 Cross-check of JCTVC-K0133 ( dictionary+hybrid dual-coder extension of HEVC) [M. Budagavi (TI)] [late]

JCTVC-K0312 AHG7: Cross-verification of JCTVC-K0302, On processing 4:2:2 chroma format [R. Cohen (MERL)] [late]

# Common conditions

## Test material

A viewing session was held to view the new sequences proposed at this meeting.  In addition, static frames of the EBU’s 4k test material were shown.

There have been a number of comments on the test sequences arising from the work conducted during the previous meeting cycle:

- DucksAndLegs too long to encode, could it be reduced in length? 30fps or a crop? It takes 28h at QP22.  If lower testing of lower QPs is desired, this could become more challenging to perform experiments.
Recommend: reduce to 30fps since the sequence is varied and cutting the sequence will loose some of that utility.

- BirdsInCage is quite static, giving very low bitrates at high QPs.  Should we keep the sequence?

- RGB test material is needed for some experiments.  Some of the RGB source has now been uploaded to the FTP server.  New sequences are also available in RGB

Regarding the new sequences:

- It would be desirable to pick one of the Wimbledon CGI sequences

- Taking two of the screen content sequence and merging them into a single sequence would be desirable.  Concerns were raised about the copyright permissions regarding representation of the user interface and any modification.
Comment: Should future screen-content sequences contain transparency effects as rendered by modern operating systems (NB: this is not the use of alpha channels)
Comment: One of the desirable characteristics is to have a mixed content, eg teleconference with slides and presenter side-by-side.
Comment: How about dragging of windows?  Perhaps this just tests the motion prediction in an integer pel manner, however if it were alpha blended that might be more challenging.
Comment: How about changing the magnification of a document?

A proposed set of codec-input test sequences was made:

XXX: word crashed again, please verify that the table is still accurate (I know it has some errors)

XXX: please review assignments according to sequence complexity and performance gains.  It is possible undesirable to have multiple sequences from the same source in the same test set.

| 4:4:4 RGB | 4:4:4 YC*C* | 4:2:2 YCbCr |
| --- | --- | --- |
| Traffic (12-bit) | Traffic (n-bit Y’CoCg?) | Traffic (n-bit) |
| Kimono | Kimono | Kimono |
| EBU-LupoCandlelight (pan&scan) | EBU-LupoCandlelight (downconv) | EBU-Horse |
| EBU-RainFruits (pan&scan) | EBU-RainFruits (downconv) | EBU-Vegies |
| Wimbledon (8bit) | Wimbledon (8bit) | EBU-WaterRocksClose |
| DucksAndLegs (5s) | BirdsInCage | EBU-KidsSoccer |
| OldTownCross | CrowdRun | Seeking |
| ParkScene |  |  |
| Screen captured sequence |  |  |

Comment: proponent of Wimbledon sequence suggests that a 10bit version is being investigated.  If it is produced it should be used instead.

Comment: there is a noticeable cliff-edge in performance with BirdsInCage and OldTownCross, where ld_he10 has very high bitrates at QP22 and quickly drops by an order of magnitude.

Comment: DucksTakeOff might be a desirable sequence

## Test conditions (in progress)

Comment: it was suggested that tests should be evaluated by considering the end-to-end performance in the RGB domain.  To some extent, consider the colourspace as part of the test conditions.

Comment: should we add YCoCg to the YUV category?

Comment: appropriate formats should be used to understand the new tools.

Comment: could colour space conversion could be performed by the encoder and decoder?  A drawback is then testability and reliance on the internal tools of the HM software.  It is not expected to be a burden to have multiple input sequences.

Comment: some lower QP values would be desired?

Comment: it was noted that it would be possible to provide some bitrate targets and derive suitable QP values to achieve suitable coverage.

Comment: Remove QP 37 and add 17?  Another expert commented that this would be a good suggestion and that it could be tested after the meeting, further changes would only be necessary if it did not meet expectations.

Current systems: Intra rate: 220/440/880Mbit/sec  (professional format (D16), eg: 880Mbit is at 4:4:4 1080p 60P, objective would be to see how HEVC can reduce these rates).

Comment: test conditions would target halving these rates.

Comment: for comparison it would be good to see the performance against MPEG4-part 2 simple studio profile.

Current systems: Contribution: Typically between 20 and 40 Mbit/sec using AVC, low-delay configurations.

Comment: it may be desirable to use a separate set of configurations for screen-content style applications, where the target bitrates are radically lower than used for professional applications.

Comment: all these systems are typically low delay (with random access of some form, but no, or little, picture reordering).  Should we change the low delay configuration?

Comment: a participant suggested that they could provide JM 18.4 anchors.

Comment: another participant suggested that they could provide MPEG4:part2 simple studio profile anchors (Intra only).  NB, there are certain constraints with such a configuration.

Recommendations: QPchange

Comment: it may be desirable to see the impact of introducing random access into the low delay configuration as this is a typical operating point.

Comment: could have AI/LB/LBwithRA(optional) rather than AI/LB/LP(optional)/RA(optional)

Comment: Why LB not LP? At high operating points B PUs are not such a difficult problem.

Comment: consumer operating point at 4:2:2?  Eventually this should be considered.

Question: do we drop 8bit tests? Do we keep it just for screen content conditions?

Comment: he10 keep same QPs as current 37—22 change chroma format; replace main with he*pro with QPs as 32—17.

Recommend this.

NB: there was a bug (probably) in HM’s RDOQ that might skew the gains at 12bits and it might have been fixed we think, maybe.  Note this should be fixed in HM9 too.

## Software model

So far, there is a release of HM8.0+frext and HM8.1+frext.  People will want to see the changes between the two versions.  It is suggested to commit two bug fixes and updates concerning higher bitdepth support to the HM-8.0-dev-ahg7 branch.  The version of HM8.1+frext (available in K0181) contains all of these.

Suggest development model is, rather than to have separate branches for each release and consequential difficulties in observing differences, it would be more manageable to have a frext-dev branch where HM is periodically merged in to update.

Proposed merge of HM-8.2 and HM-9.0.  Following this, it is recommended to remove the macros of features that are no longer actively interesting.

A side activity (which can occur at any point) is to consider how to merge (and refactor) the code into a future HM10.1 release and at what level of granularity.

Comment: A number of potential HM bugs were discovered and notified to the mailing list, but no activity to address these concerns has happened.  It would be useful if more eyes could be applied to validate these.

Comment: can LM mode be kept in the HM derived model?  It is suggested to remove the macro last in the frext development version so that a contributor can revert the change for their work.

Which macros should we remove?  Big ones relate to entropy coding, we should remove then and realign with HM (since this is in line with the guiding principal recommended earlier).  Many of these are related to inline functions and it is easy to remove the macros.

Comment: some would like to remove more “macro”-effects (a lot of the time this relates to an inline function that would now statically return a constant value).  Some concern was expressed at this.

The following table of modifications to the software model are recommended.  A participant has volunteered to make these changes.

| Macro name | Macro | Inline | Remarks |
| --- | --- | --- | --- |
| ECF__ALL_CHROMA_FORMATS_USE_SAME_TU_STRUCTURE_AS_420 | remove | Keep | inline function indicates points of divergence, doesn't do much in and of itself |
| ECF__INTRA_NxN_CU_CHROMA_PU_SPLIT_MODE | remove | Keep |  |
| ECF__DOUBLE_HEIGHT_COEFFICIENT_GROUPS_422 | remove | remove |  |
| ECF__REDUCED_CHROMA_INTRA_MODE_SET |  | Keep | dmchroma only mode |
| ECF__COMBINED_LUMA_CHROMA_INTRA_MODE_SEARCH |  | Keep | bluma only mode, removing this would remove support for a broken mode in HM |
| ECF__ENCODER_INITIAL_INTRA_MODE_PREEST_DMCHROMA |  | remove |  |
| ECF__ENCODER_FAST_INTRA_MODE_SEARCH_OVER_ALL_COMPONENTS |  | remove |  |
| ECF__ENCODER_FULL_RATE_DISTORTION_SEARCH_OVER_ALL_COMPONENTS |  | remove |  |
| ECF__ADDITIONAL_TRIAL_ENCODE_CHROMA_INTRA_MODE_SEARCH |  | remove |  |
| ECF__CHROMA_INTRA_REFERENCE_SAMPLE_FILTERING |  | Keep | has different bahaviour for different formats |
| ECF__GET_444_LMCHROMA_REFERENCE_SAMPLES_FROM_1ST_COLUMN |  | remove |  |
| ECF__CHROMA_422_INTRA_ANGLE_SCALING |  | remove |  |
| ECF__CHROMA_422_INTRA_DC_DOUBLE_WEIGHT_ABOVE_SAMPLES |  | remove |  |
| ECF__CHROMA_422_INTRA_PLANAR_SINGLE_STAGE_CALCULATION |  | remove |  |
| ECF__SET_INTRA_CHROMA_EDGE_FILTER_422 |  | Keep | acts as a tidyup |
| ECF__SET_INTRA_CHROMA_DC_FILTER_422 |  | Keep | acts as a tidyup |
| ECF__SET_INTRA_CHROMA_EDGE_FILTER_444 |  | Keep | acts as a tidyup |
| ECF__SET_INTRA_CHROMA_DC_FILTER_444 |  | Keep | acts as a tidyup |
| ECF__USE_LUMA_FILTER_FOR_CHROMA_QUARTER_SAMPLE_INTERPOLATION |  | remove |  |
| ECF__ENABLE_MDDT_FOR_444_CHROMA |  | Keep | acts as a tidyup |
| ECF__SINGLE_TRANSFORM_SKIP_FLAG_FOR_ALL_CHANNELS_444 |  | remove |  |
| ECF__CHROMA_422_QUANTISER_ADJUSTMENT |  | remove |  |
| ECF__CHROMA_422_QUANTISER_ADJUSTMENT_METHOD |  | remove |  |
| ECF__ADDITIONAL_CHROMA_QP_MAPPING_TABLES |  | remove | (always enabled) |
| ECF__INCREASE_NUMBER_OF_SCALING_LISTS_FOR_CHROMA | no action | no action | undecidable for now |
| ECF__USE_TRANSFORM_DEPTH_FOR_444_CHROMA_CBF_CONTEXT_SELECTION |  | Keep | acts as a tidyup |
| ECF__CHROMA_422_SIGNIFICANCE_MAP_CONTEXT_GRID |  | remove |  |
| ECF__PATTERNSIGCTX_MISSING_GROUPS_SAME_AS_AVAILABLE_GROUPS |  | remove |  |
| ECF__SIGNIFICANCE_MAP_CONTEXT_CHANNEL_SEPARATION |  | remove |  |
| ECF__LAST_POSITION_CONTEXT_CHANNEL_SEPARATION |  | remove |  |
| ECF__C1_C2_CONTEXT_CHANNEL_SEPARATION |  | remove |  |
| ECF__CBF_CONTEXT_CHANNEL_SEPARATION |  | remove |  |
| ECF__EXTENDED_CHROMA_SIGNIFICANCE_MAP_CONTEXT |  | remove |  |
| ECF__EXTENDED_CHROMA_LAST_POSITION_CONTEXT |  | remove |  |
| ECF__EXTENDED_CHROMA_C1_C2_CONTEXT |  | remove |  |
| ECF__LUMA_MDCS_MODE | rename | - | Magic constant removal |
| ECF__LUMA_MDCS_ANGLE_LIMIT | rename | - | Magic constant removal |
| ECF__LUMA_MDCS_MAXIMUM_WIDTH | rename | - | Magic constant removal |
| ECF__LUMA_MDCS_MAXIMUM_HEIGHT | rename | - | Magic constant removal |
| ECF__CHROMA_MDCS_MODE | rename | - | Magic constant removal |
| ECF__CHROMA_MDCS_ANGLE_LIMIT | rename | - | Magic constant removal |
| ECF__CHROMA_MDCS_MAXIMUM_WIDTH | rename | - | Magic constant removal |
| ECF__CHROMA_MDCS_MAXIMUM_HEIGHT | rename | - | Magic constant removal |
| ECF__NON_SUBSAMPLED_CHROMA_USE_LUMA_MDCS_SIZE_LIMITS |  | Keep | acts as a tidyup |
| ECF__BACKWARDS_COMPATIBILITY_HM | keep | - |  |
| ECF__BACKWARDS_COMPATIBILITY_HM_TRANSQUANTBYPASS | keep | - |  |
| ECF__CHROMA_NxN_PU_CAN_HAVE_4_PARTS |  | redundant |  |
| ECF__NON_SCALED_INTRA_CHROMA_422_ENABLED |  | redundant |  |
| ECF__EXTENDED_QP_TABLES |  | redundant |  |
| ECF__MULTIPLE_CHROMA_QP_MAPPING_TABLES |  | redundant |  |
| ECF__EXTENDED_SIZE_COEFFICIENT_GROUPS |  | redundant |  |
| ECF__CHROMA_SIGNIFICANCE_MAP_CONTEXT_SAME_AS_LUMA |  | redundant |  |
| ECF__CHROMA_LAST_POSITION_CONTEXT_SAME_AS_LUMA |  | redundant |  |
| ECF__CHROMA_C1_C2_CONTEXT_SAME_AS_LUMA |  | redundant |  |

Comment: Anything that controls large amounts of code that we are not concerned about should be removed, but anything controlling one or two lines of an inline function is not so great an issue.

Comment: There were some other things that the BoG was considering which may not be complete, we should have some plan.  Eg, If you are going to process chroma in 4x4 subblocks, little needs to be changed except for the order in which it is scanned.  This should be handled by the AHG during the meeting cycle.  A similar issue is the square vs. non-square.

Recommend that the AHG study these issues.

# Other matters

Question: Profiles?  While not in the remit of the BoG, it is obvious that there is speculation on how the extensions will be profiled, for example, will there be a consumer 4:4:4 profile that is different to a professional 4:4:4 profile.  How should 4:2:2 be scoped?  What about higher orders of bitdepth?  Some guidance may be beneficial.

CEs: there had been some comment that there is an important question concerning the transform shapes to use.  When (and should) a CE occur?

Time scales, some concerns had been raised over the time scales for achieving high quality 4:4:4/4:2:2 support.  It was asked if given that there is likely to be a Main 10 profile, is the same time scale required for 4:4:4 and 4:2:2?

The BoG would like to encourage experts to study the higher bit-depth aspects of the system.