| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0366 |
| --- | --- |

| Title: | Proposal of Scalable HEVC Initial Test Model |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Y. Ye, A. Fuldseth, P. Yin, C. Cui | Tel: Email: | +1-858-210-4803 yan.ye@interdigital.com |
| Source: | InterDigital, Cisco, Dolby, ASTRI |  |  |

_____________________________

# Abstract

This contribution proposes a potential software as the starting point for the development of HEVC scalable extension.

# Introduction

Most of the responses to the CfP on the HEVC SVC extension use similar coding tools, such as inter layer texture/motion/mode predictions and differential domain prediction. Within one coding tool category, the performances of the individual ones from different responses have not been investigated yet. Therefore, a simple software providing basic scalability is highly desired as the starting point for the development of HEVC scalable extension, upon which each individual coding tools can be easily integrated and fairly evaluated. For this purpose, we propose a software using only picture-level inter layer texture prediction and limiting the changes to HEVC codec within high level syntax.

# Software Description

Figure 1 shows the block diagram of the proposed system at the encoder side. The picture level inter layer processing block in Figure 1 generates the inter layer reference picture based on the base layer reconstruction, which is then used as an additional reference picture to predict the corresponding enhancement layer picture and put at the end of the reference picture list. The enhancement layer encoder operations remain identical to those of a single layer HEVC encoder. Developed based on HM6.1, very small changes are made in the software and are now migrating to HM8.1, which will be available soon. One familiar with HM software can understand the proposed software easily.

The two-layer coding is performed on a picture basis, meaning all the pictures with the same time instances are coded from low to high layers sequentially. By doing this, the memory requirement and delay caused by scalable coding is not increased much.

The software also supports multi-standard scalability, including H.264/AVC and HEVC, where the base layer reconstructed pictures from legacy bitstreams are upsampled as inter layer reference pictures inserted into the reference picture list.

In addition, the software includes a VC10 build on Windows used specifically to check memory problems, such as memory leaks, and a bitstream extractor used to extract a certain layer bitstream from a scalable bitstream.

## Fixed Upsampling Filters

The fixed upsampling filters in the proposed software that generate the inter layer reference (ILR) pictures for 1.5X and 2X spatial scalability have the following features:

- Derived from Gaussian windowed Sinc function.

- 2-D separable.

- For pixels in the ILR picture that correspond to pixels at integer pixel locations in the base layer picture, they are directly copied from the base layer without filtering.

- For pixels in the ILR picture that correspond to pixels at factional pixel locations in the based layer picture, they are filtered using 8-tap filters for the luma component and 4-tap filters for the chroma components.

- The precision of all the upsampling filter coefficients is 8-bit, and filtering is realized by 16-bit implementation.

The fixed upsampling filter coefficients are provided in Table 1.

Table 1 The fixed upsampling filter coefficients

| Luma | 2X |  | [-3 | 8 | -21 | 80 | 80 | -21 | 8 | -3]/128 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.5X |  | [-5 | 11 | -24 | 107 | 53 | -18 | 8 | -4]/128 |
|  |  |  | [-4 | 8 | -18 | 53 | 107 | -24 | 11 | -5]/128 |
| Chroma | 2X | Vertical | [-17 | 100 | 56 | -11]/128 |  |  |  |  |
|  |  |  | [-3 | 15 | 126 | -10]/128 |  |  |  |  |
|  |  | Horizontal | [-15 | 79 | 79 | -15]/128 |  |  |  |  |
|  | 1.5X | Vertical | [-20 | 122 | 38 | -12]/128 |  |  |  |  |
|  |  |  | [-20 | 71 | 101 | -24]/128 |  |  |  |  |
|  |  |  | [-4 | 12 | 128 | -8]/128 |  |  |  |  |
|  |  | Horizontal | [-23 | 113 | 54 | -16]/128 |  |  |  |  |
|  |  |  | [-16 | 54 | 113 | -23]/128 |  |  |  |  |

Conclusion

Figure 1 Block diagram of the proposed scalable system

We proposed a potential software as the starting point for the development of HEVC scalable extension, which has the following features:

- HM6.1 based (HM8.1 based software will be available soon)

- Minimal change to enhancement layer codec: The changes are only limited within high level syntax, and there are no changes to EL block level operations or prediction structure.

- Only one coding tool: Base layer reconstruction is upsampled by fixed upsampling filters and used as an additional reference picture for enhancement layer coding.

- Low delay coding structure: All the pictures with the same time instances are coded from lowest to highest layers sequentially. By doing this, the memory requirement and delay caused by scalable coding is not increased much.

- Support multiple standard scalability.

- Additional development tools: Two tools are provided for memory leak check and bitstream extracting, respectively.

We believe the benefits of this approach are as follows:

- There are small changes on HM and anyone familiar with HM can understand the software easily.

- The software is clean to be used as a starting point for further development.

- Base layer coding information is ready for implementing block level coding tools.

- Easily to be extended to support any legacy bitstreams as the base layer, as long as the base layer reconstruction pictures are available.

- Convenient for future development and debugging.

# Patent rights declaration(s)

InterDigital Communications, LLC, may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).