| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0367 |
| --- | --- |

| Title: | BoG report on high level parallelism |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrew Segall 5750 NW Pacific Rim Blvd. Camas, WA 98607 USA | Tel: Email: | +1-360-817-8487 asegall@sharplabs.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This is a report on the Breakout Group on High Level Parallelism.  The BoG met on October 13, 2012.  The notes and recommendations from the meeting are included in this report.

# Review of contributions

JCTVC-K0106 On Tile Processing Order [P. Kapsenberg (Intel), W. Zhang (Intel)]

This contribution claims that the current raster ordering of tiles within a picture poses a minor but unnecessary burden on hardware decoders. A solution is proposed that has no coding performance impact and reduces memory footprint; it is proposed that tiles should be processed column by column instead of row by row (vertical raster). This contribution includes a suggested HM-8.0 patch.

Identifies a potential issue for single threaded encoders.

Non-normative solutions are possible but have additional complexity.

Proposes to order vertical raster scan by columns instead of rows.

Software provided.

Question about impact on low latency encoder applications.  Specifically, it is no longer possible to encode data in camera scan line order with the proposed changes.

Comment that this change may not be needed in the spirit of stability.

Comment that this may not benefit some single core decoder architectures

The BoG recommended no action.

JCTVC-K0244 Cross check of JCTVC-K0106 [G. Van der Auwera (Qualcomm)] [late]

Not presented due to no action on JCTVC-K0106.

JCTVC-K0108 On parallel decoding with SEI message containing Reference Dependency Tree [W. W. Ro, M. Kim, D. Kim, K. Kim (Yonsei Univ.), C. Kim, Y. Park, K. Choi (Samsung)]

In this contribution, a parallel decoding scheme for random access profile is proposed. Such parallel algorithm is implemented by using a simple data structure called Reference Dependency Tree (RDT), indicating the order of decoding with parallelism. RDT is built at the encoder side and is designed to be contained in the SEI message portion of the bitstream to be transmitted to the decoder side. The decoder replaces the former Reference Picture Set (RPS) with RDT for parallel decoding of the frames by resolving the reference dependency. The experimental results show an average of 35.1% decrease in decoding time, thus efficiently improving the performance of the decoder.

One participant commented that this functionality may be possible with the SOP SEI

One participant commented that this information may be available with RPS.  Additional discussion that the slice level RPS data may not enable decoder parallelism in practice.

After discussion offline, proponent was satisified with the use of the SOP SEI for the envisioned application.

The BoG recommended no action.

JCTVC-K0135 AHG4: Independent parsing for WPP substreams [T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S. Lei (MediaTek)]

Withdrawn

JCTVC-K0261 Cross-verification of JCTVC-K0135 on Independent parsing for WPP substreams [M. Coban (Qualcomm)] [miss]

Not presented due to withdrawl of JCTVC-K0135.

JCTVC-K0182 CABAC termination for the end of tile/WPP [K. Terada, H. Sasai (Panasonic)]

At the end of tile/WPP sub-stream in WD8, the function byte_alignment() is called, except for the end of slice. However, is is asserted that there is no description for CABAC termination in the function, which was reported in ticket #768. This contribution proposes to add a new syntax element at the end of tile/WPP sub-stream for solution of this defect.

Comment of support for the approach.

Suggestion that an alternative solution would be to replace “end_of_slice_flag” with “end_of_substream_flag”

Question about need to conditionally transmit a bit that is always 1

Agree that there is a mis-match between the text and software

Comment that this may not be a complete solution.

Upon revisit, previous expressed concerns that the solution may not be complete were withdrawn.  Additional comments of support provided.

The BoG recommended adoption of the proposal.

JCTVC-K0183 AHG4: Dependent slices restriction [S. Esenlik, M. Narroschke, T. Wedi (Panasonic)]

The current HEVC standard includes two restrictions related to the coexistence of slices and WPP/tiles. However, it is asserted that there is no separation between normal and dependent slices in the context of the restrictions. As a result, it is claimed that the recently adopted dependent slices partially defeat the intended purposes of the restrictions that are imposed previously. In the document, it is proposed to add limitations on the use of dependent slices in order to extend the scope of the existing two restrictions. The proposed limitations are asserted to guarantee that when a parent slice and all following dependent slices are seen as a single slice, the already existing two restrictions are satisfied.

Noted that the Hi-Level Syntax (HLS) AhG recommended that dependent slices should be considered to belong to the base slice (full slice)

Comment that the problems identified in the contribution are recognized problems and related to how base slices and dependent slices are defined in the text.

Proponent: Goal of the proposal is to make tile and WPP restrictions consistentent with HLS AhG recomendations.

The BoG recommended that the JCTVC make sure that modified text for dependent slices resolves the issues identiifed in JCTVC-K0183.

JCTVC-K0200 AHG4: Sub-stream entry points SEI message [M. Zhou (TI)]

In the current HM8.0 design, sub-stream entries are signaled in slice header only to facilitate sub-stream switching of decoding parallelized streams with single core decoders. No entry points are available in the bitstream to assist parallel decoding on multiple core platforms. This contribution advocates signaling of sub-stream entry points in a SEI message as well to facilitate parallel decoding. Each sub-stream entry point shall be a parallel decoding entry point and start with a slice header with dependent_slice_flag set equal to 0. It is also signaled whether sub-streams have approximately same size in terms of CTBs or compressed bitstream bytes. The number of sub-stream entry points signaled in the SEI message is not necessarily equal to the number of WPPs, tiles or slices in the picture associated with the SEI message. It is recommended to add this SEI message to the spec to facilitate parallel decoding in environments in which encoder and decoder negotiation might be possible, and provides extendibility of supporting parallel decoding in which parallel tools might need to be mandated.

Question about how entry points offsets account for multiple NAL units

Comment that this (accounting for multiple NAL units) is typically handled at system layer

Statement of support (multiple)

Text provided

Suggestion (editorial concern) to use a different word than sub-stream

Suggestion to clarify sub_stream_type flag, specifically uniform spacing

Suggeston to clarify the offset length calculation in relation to multiple NALUs

Suggestion to clarify what the offset calculation is in reference to

The BoG recommended adoption of the proposal subject to addressing above suggestions and text review

JCTVC-K0236 Decoder parallelism indication [J. Samuelsson, R. Sjöberg, M. Westerlund, B. Burman, P. Fröjdh (Ericsson)]

This document proposes a syntax elements, threads_idc, to indicate the degree of parallelism provided in a bit-stream.  The proposed parameter is transmitted in the video usability information (VUI).  The proposal requires bit-stream restrictions when the indicator is not zero.  It is asserted that the indicator can be used together with profile and level to indicate the capabilities of a decoder.  For example, it is claimed that some decoder implementations may be able to decoder a higher profile levels as the indicator increases.

The proposal is a resubmission of JCTVC-J0249 with one major difference: The indicator is now proposed to be included in the VUI.  In JCTVC-J0249, it was proposed to be in the SPS.

Question about the use of a fractional thread_idc value

Comment: It may be possible to combine with previous proposal (JCTVC-K0200)

Comment: JCTVC-K0236 provides clarification of uniform spacing in JCTVC-K0200

Comment: The proposed information may not be sufficient for parallel decoder

Concern: Number of threads does not guarantee an ability to decode with a given parallel architecture

Statement of support: It is useful to have information to complement capability exchange. Provides additional responsibilities.

<Meeting closed at 8:15PM due to bus schedule>

The BoG recommended revisiting the proposal.