| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0368 WG11 Number: m27191 |
| --- | --- |

| Title: | Impact of transform skip on screen content sequences from JCTVC-K0207 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway, Cambridge, MA 02139 U.S.A. Tao Lin Tongji University, Shanghai, China | Tel: Email: Tel: Email: | +1-617-621-7503 cohen@merl.com 86-21-5103-0961 lintao@tongji.edu.cn |
| Source: | Mitsubishi Electric Research Laboratories and Tongji University |  |  |

_____________________________

# Abstract

In JCTVC-K0246, it was shown that with the current set of AHG7 4:4:4 and 4:2:2 test sequences, enabling/disabling transform skip yielded BD-Rate changes of 0.0%. The purpose of this document is to report the effects of transform skip on the set of five 4:4:4 screen-content being offered for use by JCTVC-K0207. The first frame from each of the five sequences was coded using HM-8.0-dev-ahg7, with transform skip enabled then disabled. The resulting changes in BD-Rate ranged from 23.9% to 35.1%, with an average of 28.2%.

# Introduction

It was shown in JCTVC-K0246 that the set of sequences [1] used by AHG7 for testing extended chroma formats do not significantly exercise the transform skip functionality of HM-8.0-dev-ahg7 [2]. Those sequences are not considered as being screen-content material. For the current JCT-VC meeting, five new screen-content 4:4:4 sequences are being offered for use by JCTVC-K0207 [3]. In order to help the BoG on range extensions decide whether to use these sequences for future activities, we coded the first frame of each of the sequences with transform skip enabled then disabled.

# Performance of transform skip

We generated anchors for the HM-8.0-dev-ahg7 software using the configurations from the common conditions [4], for the first frame of each of the five 4:4:4 sequences described in JCTVC-K0207. We then generated results using the same conditions, except that transform skip was disabled. BD-Rate results are included in the following table:

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| sc1CAD_waveform | 28.6% | 32.0% | 32.8% | 28.2% | 32.0% | 32.7% |
| sc2CAD_waveform | 23.9% | 28.3% | 28.9% | 23.2% | 28.1% | 28.3% |
| sc3PCB_layout | 35.1% | 43.3% | 39.5% | 35.2% | 42.6% | 39.5% |
| sc4_PCB_schematic | 27.3% | 29.0% | 27.1% | 28.2% | 29.1% | 27.4% |
| sc5_ppt_doc_xls | 26.2% | 40.1% | 31.2% | 26.3% | 39.6% | 30.7% |
| Overall | 28.2% | 34.5% | 31.9% | 28.2% | 34.3% | 31.7% |
| Enc Time[%] | 92% |  |  | 91% |  |  |
| Dec Time[%] | 101% |  |  | 109% |  |  |

# Discussion and conclusions

For this brief test using the first frame from each sequence, the impact of transform ranged from 23.9% to 35.15%, with an average of 28.2%. For this particular set of sequences, the type of content in each frame is generally consistent over the entire sequence, so this range of values implies that the sequences from JCTVC-K0207 do indeed significantly exercise the transform skip functionality of the codec.

# References

- Online: ftp://hevc@ftp.tnt.uni-hannover.de/testsequences/FrExt-candidate-sequences/

- Online: https://hevc.kw.bbc.co.uk/svn/jctvc-hm/branches/HM-8.0-dev-ahg7

- S. Wang, T. Lin, and K. Zhou, “AHG7: Update on full-chroma (YUV444) screen content test sequences of JCTVC-H0294,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J1100, Shanghai, CN, October 2012.

- F. Bossen, “HM 8 Common Test Conditions and Software Reference Configurations,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J1100, Stockholm, SE, July 2012.