| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0369 |
| --- | --- |

| Title: | BoG report on subjective viewing test for deblocking filter proposals |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrey Norkin Kenneth Andersson | Email: | andrey.norkin@ericsson.com kenneth.r.andersson@ericsson.com |
| Source: | BoG on subjective viewing |  |  |

_____________________________

This contribution is a report on informal subjective viewing for the deblocking filtering that was held during the Shanghai meeting on October 12, 2012. The subjective viewing was conducted according to the recommendations of BoG on subjective viewing set-up for deblocking filtering JCTVC-K0342 [2]. The goal of informal subjective viewing test was to determine how the deblocking filter proposals affect the subjective quality. In total, 7 proposals were evaluated in the subjective test.

# Test procedure

Seven proposals were tested. The proposals were based on HM8.0. The proposals in each test session were evaluated by a group of 4-6 experts. In total, five viewing sessions have been held. The identities of the proposals and the anchor were hidden (the test subjects were shown labels A or B instead).

The following six sequences have been used in the test.

- Riverbed, Qp=32, RA.

- Riverbed, Qp=37, RA

- WestWindEasy, Qp=37, LDB

- DucksTakeOff, Qp=37, LDB

- ChinaSpeed, Qp=37, LDB

- RedKayak, Qp=37, RA

Anchor: HM8 with maximum tc offset + maximum beta offset

ABAB test was used. The same sequence and the anchor were shown one after another. The order of proposals was randomized for every test and sequence and identities of the proposals are hidden. Test subjects were asked to rate each proposal on a scale from -2 to 2. The results were later calculated as well as 95-percent confidence intervals for every proposal and test sequence.

The length of one session was about 35 minutes. In total, 24 test subjects participated in scoring. The test sessions were conducted by Andrey Norkin. Processing of score sheets and calculations of the results were done by Andrey Norkin and Kenneth Andersson (Ericsson) and cross-verified (obtained in parallel) by Shuo Lu and Ohji Nakagami (Sony).

# Test equipment

Equipment provided by meeting organizers was used. Samsung UA40ES6100J display with 40'' diagonal was used in the subjective tests.

# Proposals

Test 1 from CE1: maximum beta and tc offsets are sent.

Beta table modification in the high QP range, maximum tc and beta_ ffsets are sent.

Test 4 from CE1: no offsets.

Modification of strong filter and its decisions, intra boundary smoothing restricted, intra_tc_offset = 3

Test 5 from CE1: maximum beta and tc offsets are sent.

Test 4 + beta table modification from Test 1, maximum tc and beta offsets are sent

K0150: no offsets.

Increasing of tc- and beta- dependent thresholds of strong filter differently for inter and intra blocks, intra-boundary smoothing restricted.

K0269: maximum beta and tc offsets for TU of maximum size.

Maximum tc and beta offsets are sent for the TU of max size.

Test 4 + K289: tc_ffsets: 0, 2, 3, 4.

Test 4 + sending tc_offset depending on the picture depth in GOP

HM8.0 + K289: tc_ffsets: 0, 2, 3, 4.

HM8.0  + sending tc_offset depending on the picture depth in GOP

# Test results

The graphs below show the test results. Values with overlapping CI values cannot be rated as distinguishable.

# Outcome of the test

Most test points have overlapped confidence intervals with the anchor (HM8.0 with maximum offsets). However, in some cases the proposals have non-overlapped confidence intervals with the anchor and with each other. These cases are listed below:

HM8.0_K0289 is statistically worse than the anchor in three cases.

K150 and K269 are statistically worse than the anchor each in one case.

Test4_K0289 and K0269 are statistically better than the anchor each in one case.

Test4_K0289 is statistically better than HM8.0_K0289 in two cases.

Test4, Test5, and K0150 are statistically better than HM8.0_K0289 each in one case.

Test 4 is statistically better than K0269 in one case.

# References

- A. Norkin, T. Suzuki, M. Budagavi, G. Van der Auwera, "Summary report of Core Experiment on deblocking filtering", JCTVC-K0021, Shanghai, Oct. 2012.

- T. Yamakage, "BoG report on subjective viewing set up for deblocking filter", JCTVC-K0342, Shanghai, Oct. 2012.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-J1100, Stockholm, July 2011.