| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10-19 Oct., 2012 | Document: JCTVC-K0370 |
| --- | --- |

| Title: | Suggested design of initial software model for scalable HEVC extension proposal by LG Electronics and MediaTek Inc. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chulkeun Kim, Joonyoung Park, Jungsun Kim, Hendry, Seungwook Park, Yongjoon Jeon, Jaehyun Lim, Naeri Park, Byeongmoon Jeon LG Electronics #221 Yangjae-dong, Seocho-gu, Seoul 137-130, Korea Shan Liu, Ximin Zhang, Mei Guo, Zhenzhong Chen MediaTek USA 2860 Junction Ave, San Jose, CA 95134, USA Tzu-Der Chuang, Shih-Ta Hsiang, Ching-Yeh Chen, Chia-Yang Tsai, Chih-Ming Fu, Chih-Wei Hsu, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Byeongmoon Jeon +82-10-82031126 {chulkeun.kim, jy.park, jungsun.kim, hendry.hendry, seungwook.park, yongjoon.jeon, jaehyun.lim, naeri.park, bm.jeon}@lge.com Shan Liu +1-408-5261899 ext. 88129 {shan.liu, ximin.zhang, mei.guo, zhenzhong.chen, peter.chuang, shih-ta.hsiang, chingyeh.chen, chiayang.tsai, chihming.fu, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | LG Electronics and MediaTek Inc. |  |  |

_____________________________

# Abstract

This proposal provides a suggestion for an initial software model for the scalability extension of HEVC. It is based on merging subsets of the software implementations from LG Electronics and MediaTek Inc..

# Contents

Abstract	1

Contents	2

1	Introduction	3

2	Algorithm description	3

2.1	CTU/TU/PU partitioning	3

2.2	Up-sampling filters	3

2.2.1	Up-sampling filters	3

2.3	Inter-layer prediction	4

2.3.1	Inter-layer texture (intra) prediction	4

2.3.2	Inter-layer motion vector prediction	5

2.3.3	Inter-layer intra mode prediction	7

2.4	Motion/mode/parameter coding	8

2.4.1	Intra prediction mode coding	8

3	Syntax and semantics description	9

3.1	NAL unit syntax	9

3.2	NAL unit header SVC extension syntax	10

3.3	Subset sequence parameter set RBSP syntax	10

3.4	Sequence parameter set SVC extension syntax	10

3.5	Slice layer extension RBSP syntax	10

3.6	Slice header in scalable extension syntax	10

3.7	Slice data in scalable extension syntax	11

3.8	Coding tree in scalable extension syntax	12

3.9	Coding unit in scalable extension syntax	12

4	Compression performance discussion	13

4.1	Category 1 (HEVC base layer)	13

4.1.1	Spatial scalability	13

4.1.2	SNR scalability	13

4.1.3	Overall	13

5	Complexity analysis	13

5.1	Encoding time and measurement methodology	13

5.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM	13

5.3	Description of computing platform used to determine encoding and decoding times reported in Sections 5.1 and 5.2	14

6	Patent rights declaration(s)	14

References	14

# Introduction

This proposal covers the categories which are checked below.

 	Category 1 (HEVC base layer) spatial scalability

 	Category 1 (HEVC base layer) intra-only spatial scalability

 	Category 1 (HEVC base layer) SNR scalability

 	Category 2 (AVC base layer) spatial scalability

 	The proposal obeys the constraints under Section 5 of the CfP (if box is not ticked, explain cases where constraints are violated)

# Algorithm description

## CTU/TU/PU partitioning

The basic structure of coding unit / transform unit / prediction unit (CU/TU/PU) partitioning is the same as that is used in the “Main” common test conditions of HEVC test model version 6.1 (HM6.1) [2], where asymmetric motion partitioning (AMP) and non-square quadtree transform (NSQT) are disabled.

## Up-sampling filters

### Up-sampling filters

The up-sampling filters are discrete cosine transform interpolation filtering (DCT-IF) in horizontal and vertical directions. Both directions use the identical 1-D DCT-IF. An 8-tap DCT-IF is used for luma, and a 4-tap DCT-IF is used for chroma. The filter coefficients are shown in Table 1 and Table 2. The phase of down-sampled sequences can be shifted according to the down-sampling method. To consider this aspect, position and filter coefficients for up-sampling are determined by the phase of the down-sampled sequence. Interpolation positions according to the shifted phase are shown in Table 3.

Table 1.  8-tap DCT-IF coefficients for luma interpolation

| Position | Filter coefficients |
| --- | --- |
| 1/12 | { -1, 1, -4, 63, 6, -2, 1, 0 } |
| 2/12 | { -1, 3, -7, 61, 12, -5, 2, -1} |
| 3/12 | { -1, 4, -10, 57, 19, -7, 3, -1 } |
| 4/12 | { -1, 4, -11, 52, 26, -8, 3, -1} |
| 5/12 | { -1, 5, -12, 47, 32, -9, 3, -1 } |
| 6/12 | { -1, 4, -11, 40, 40, -11, 4, -1} |

Table 2.  4-tap DCT-IF coefficients for chroma interpolation

| Position | Filter coefficients |
| --- | --- |
| 1/12 | { -2, 62, 5, -1} |
| 2/12 | { -4, 59, 11, -2 } |
| 3/12 | { -4, 54, 16, -2 } |
| 4/12 | { -5, 50, 22,-3 } |
| 5/12 | {-5, 43, 30, -4 } |
| 6/12 | {-4, 36, 36, -4 } |

Table 3.  Interpolation positions according to the shifted phase

|  | 2x up-sampling | 1.5x up-sampling |
| --- | --- | --- |
|  | Position | Position |
| No phase shift | 0/12, 6/12 | 0/12, 8/12, 4/12 |
| 1/2 phase shift | 9/12, 3/12 | 10/12, 6/12, 2/12 |

## Inter-layer prediction

### Inter-layer texture (intra) prediction

The base layer (BL) reconstructed texture can be used to reduce the redundant texture information of the enhancement layer (EL).

#### Inter-layer texture (intra) prediction

When a coding unit (CU) in the EL is coded as inter-layer texture prediction, the corresponding texture in the BL is up-sampled and used as texture predictors for the CU in the EL, as shown in Figure 2. The up-sampling procedure of generating the inter-layer texture prediction samples can be referred to Section 2.2.1.

When a CU is coded as inter-layer texture prediction, the prediction mode of the CU is set equal to MODE_INTRA, and the intra luma prediction mode is set equal to the intra luma prediction mode of the corresponding block in the BL.

Figure 1.  Inter-layer texture prediction

#### Syntax and context formation of inter-layer texture (intra) prediction

Inter-layer texture prediction is applied to the current CU when inter_layer_intra_prediction_enabled_flag and base_pred_flag[x0][y0] are equal to 1. The inter_layer_intra_prediction_enabled_flag is signalled in slice header and is to indicate whether the inter-layer texture prediction can be enabled or not. The base_pred_flag is signalled after the skip flag. The base_pred_flag[x0][y0] for inter-layer texture prediction is present when all of the following conditions are satisfied.

- inter_layer_intra_prediction_enabled_flag == 1

- skip_flag[x0][y0] != 1

- diff_coding_flag[x0][y0] != 1

Two context models are used for base_pred_flag in CABAC. The information of the left CU is used for the context formation of base_pred_flag. The context index of base_pred_flag is equal to the value of the base_pred_flag of the left CU.

### Inter-layer motion vector prediction

When a prediction unit (PU) in the EL is coded as inter mode, motion information from the corresponding PU in the BL is added to the advanced motion vector prediction (AMVP) candidate list for Inter mode and the merging candidate list for Merge mode if it is available.

The motion information of the BL is derived as follows.

- For the derivation of motion information, the corresponding positions IL-C0 and IL-H in the BL are derived as specified in Section 2.3.3.1.

- The motion vector at the corresponding positions is scaled by proportion between the resolution of the BL and the EL.

In Section 2.3.3.2, it is described how the scaled motion information is added to the merging candidate list. In Section 2.3.3.3, it is described how the scaled motion information is added to the AMVP candidate list. In Section 2.3.3.4, it is described when to compress the motion data in the proposed method.

#### Derivation process for corresponding positions IL-C0 and IL-H in base layer

In our proposal, two corresponding positions IL-C0 and IL-H are used for inter-layer motion candidates.

In Figure 2, the coordinate for IL-C0 is (xR,C0,yR,C0), and the coordinate for IL-H is (xR,H,yR,H) . In order to calculate those coordinates, a scale_factor should be pre-defined. In our proposal, the factor is defined as below.

scale_factor = ( picture_width_of_enhancement_layer << 8 ) / picture_width_of_base_layer

Figure 2.  Derivation of corresponding positions for inter-layer motion candidates

When the coordinate for the upper-left sample of the center position in the current PU of the EL is given as (xE,C0,yE,C0), the coordinate for IL-C0 is calculated as below.

(xR,C0,yR,C0) = ( ( xE,C0 << 8 ) / scale_factor, ( yE,C0 << 8 )_/ scale_factor)

When the coordinate for the bottom-right sample outside the current PU of the EL is given as (xE,H,yE,H), the coordinate for IL-H is calculated as below.

(xR,H,yR,H) = ( ( xE,H << 8 ) / scale_factor, ( yE,C0 << 8 )_/ scale_factor)

The motion information of (xR,C0,yR,C0) and (xR,H,yR,H) is scaled and added to the AMVP candidate list and the merging candidate list. The MV scaling function is identical to the MV scaling function in HEVC, but the scaling factor is replaced with the scale_factor, which is the resolution proportion between the resolution of the BL and that of the EL.

#### Inter-layer merging candidate

In Table 4, the left column shows the merging candidate list in HM 6.1, and the right column shows the proposed merging candidate list for EL.

Table 4.  Merging candidate list

| HM6.1 | Proposed |
| --- | --- |
| - A1 - B1 - B0 - A0 - if(N < 4), B2 - Col - first pruning - if(N < Max.), Combined bi-pred. candidates - if(N < Max.), Zero MV candidates | - IL-C0 - A1 - B1 - IL-H - if(N < 4), B0 - if(N < 4), A0 - if(N < 4), B2 - Col - first pruning - if(N < Max.), Combined bi-pred. candidates - if(N < Max.), Zero MV candidates |

#### Inter-layer AMVP candidate

In Table 5, the left column shows the AMVP candidate list in HM6.1, and the right column shows the proposed AMVP candidate list for EL.

Table 5.  AMVP candidate list

| HM6.1 | Proposed |
| --- | --- |
| - A - B - Col - first pruning - if(N < Max.), Zero MV candidates | - IL-C0 - A - B - Col - first pruning - if(N < Max.), Zero MV candidates |

IL-C0 is marked as available only when its reference list and reference picture are the same as those of the current PU in the EL.

#### Access unit based motion data compression

In HEVC, motion data (i.e., motion vectors, reference indexes, and prediction modes) are compressed after decoding a picture. Compressed motion data can make inter-layer motion prediction inefficient. In our model, uncompressed motion data of the BL is used for the EL. After decoding all the pictures of all layers in an access unit, motion data compression is performed all together.

### Inter-layer intra mode prediction

In order to exploit the correlation between the intra modes from different layers, we propose to predict the intra mode at EL mainly from the intra mode of the collocated unit at BL. Furthermore, Planar and DC modes are less useful at EL than they are at BL, since the uniform and flat regions are efficiently compressed with inter-layer texture prediction method. Inspired by that, we also propose to remove DC and Planar modes at EL.

Both Planar and DC modes are excluded from intra coding at EL. Then, there are 33 angular intra prediction modes available in intra coding for luma component. During the intra mode coding of luma component at EL, one most probable mode (denoted as MPM_EL) is used, so that the number of remaining modes would be the power of 2 and the remaining modes could be coded with 5 bits of fixed-length binarization.

The most probable mode MPM_EL is derived for each luma PU from the intra mode of the corresponding part at BL (denoted as candIntraPredModeBL), the intra mode of neighboring PU to the left of the current PU (denoted as candIntraPredModeA), and the one of the neighboring PU to the top of the current PU (denoted as candIntraPredModeB).

The intra mode of the corresponding part at BL (i.e., candIntraPredModeBL) is derived as follows. At first, a sample P_EL is selected in the current unit at EL. If the current unit is 4x4, the sample with the coordinate (2,2) could be selected as P_EL, as illustrated in Figure 3. If the current unit is larger than 4x4, a 4x4 unit A, within the current unit is chosen as shown in Figure 4. After that, the sample with the coordinate (2,2) within the selected 4x4 unit A is selected as P_EL. Second, the corresponding pixel of P_EL at BL is calculated as follows.

scale_factor = ( picture_width_of_enhancement_layer << 8 ) / picture_width_of_base_layer

PosY_BL = (PosY_EL << 8) / scale_factor

PosX_BL = (PosX_EL << 8) / scale_factor

In the above equations, PosY_EL and PosX_EL denote the coordinates of P_EL, PosY_BL and PosX_BL denote the coordinates of the corresponding sample of P_EL at BL, and picture_width_of_base_layer and picture_width_of_enhancement_layer denote the width of the picture at BL and that at EL respectively. Finally, the intra mode of the sample (PosX_BL, PosY_BL) at BL is used as candIntraPredModeBL.

Figure 3.  Illustration of coordinates in a 4x4 block

Figure4.  Illustration of a selected 4x4 block inside the current unit larger than 4x4

If the left or top PU of the current unit is coded with inter-layer texture prediction, candIntraPredModeA or candIntraPredModeB is set equal to the intra mode of the collocated PU of the left or top PU at BL. The derivation is the same as the method that is used to achieve candIntraPredModeBL.

The most probable mode MPM_EL is derived as follows:

- If the corresponding sample of P_EL at BL is located in an intra-coded unit and the intra mode candIntraPredModeBL is neither Planar nor DC,

MPM_EL= candIntraPredModeBL;

- Otherwise, if the left PU is available, it is intra coded, and candIntraPredModeA is neither Planar nor DC,

MPM_EL = candIntraPredModeA;

- Otherwise, if the top PU is available, it is intra coded, and candIntraPredmodeB is neither Planar nor DC,

MPM_EL = candIntraPredModeB;

- Otherwise,

MPM_EL = Intra_Angular(26); (the vertical intra prediction mode)

By removing Planar and DC modes, the chroma PU has 3 candidate modes, are shown in Table 6.

Table 6.  Candidates of intra chroma prediction modes according to the corresponding intra luma prediction mode (IntraPredMode) after removing DC and Planar modes

| Intra_chroma_pred_mode | IntraPredMode |  |  |
| --- | --- | --- | --- |
|  | 26 | 10 | X (0<=X<35) |
| 0 | 34 | 26 | 26 |
| 1 | 10 | 34 | 10 |
| 2 | 26 | 10 | X |

## Motion/mode/parameter coding

### Intra prediction mode coding

When an intra mode of a luma PU (denoted as IntraPredMode ) is coded at enhancement layer (EL), the most probable mode, MPM_EL, is first derived as shown in Section 2.3.4. If IntraPredMode is equal to MPM_EL, a prev_intra_pred_flag equal to 1 is transmitted; otherwise the flag prev_intra_pred_flag equal to 0 is sent. If the flag prev_intra_pred_flag is equal to 0, which means IntraPredMode is not identical to MPM_EL, a remaining mode of that (denoted as rem_intra_luma_pred_mode) is generated as follows.

rem_intra_luma_pred_mode = IntraPredMode;

- If rem_intra_luma_pred_mode is larger than MPM_EL,

rem_intra_luma_pred_mode--;

- If rem_intra_luma_pred_mode is larger than 1 (the index of DC mode),

rem_intra_luma_pred_mode--;

- If rem_intra_luma_pred_mode is larger than 0 (the index of Planar mode),

rem_intra_luma_pred_mode--;

Afterwards, the remaining mode rem_intra_luma_pred_mode is fixed-length coded.

When an intra mode of a chroma PU (denoted as IntraPredModeC) is coded at EL, it is first represented as the binarization shown in Table 7. Then the binary bins are coded one by one.

Table 7.  Specification of prefix and suffix parts for intra_chroma_pred_mode binarization after removing DC and Planar modes

| Value of intra_chroma_pred_mode | prefix | suffix |
| --- | --- | --- |
| 2 | 0 | n/a |
| 0 | 1 | 0 |
| 1 | 1 | 1 |

# Syntax and semantics description

Syntax and semantics which are not described in this section are the same as those in HM6.1. The highlighted descriptions in this section are different from those in HM6.1.

## NAL unit syntax

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_unit_type | u(6) |
| NumBytesInRBSP = 0 |  |
| nalUnitHeaderBytes = 1 |  |
| if( nal_unit_type = = 1 ¦ ¦ nal_unit_type = = 4 ¦ ¦ nal_unit_type = = 5 ¦¦ nal_unit_type = = SVC_NAL ) { |  |
| temporal_id | u(3) |
| output_flag | u(1) |
| } |  |
| if( nal_unit_type = = SVC_NAL ) |  |
| nal_unit_header_svc_extension( ) |  |
| reserved_one_4bits | u(4) |
| for( i = nalUnitHeaderBytes; i < NumBytesInNALunit; i++ ) { |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| } |  |
| } |  |

SVC_NAL is a NAL unit type defined to indicate NAL units involved in high-layer coding.

## NAL unit header SVC extension syntax

| nal_unit_header_svc_extension( ) { | Descriptor |
| --- | --- |
| idr_flag | u(1) |
| dependency_id | u(3) |
| quality_id | u(4) |
| } |  |

## Subset sequence parameter set RBSP syntax

| subset_seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| seq_parameter_set_rbsp( ) |  |
| if( profile_idc = = SVC_PROFILE ) { |  |
| seq_parameter_set_svc_extension( ) |  |
| } |  |
| } |  |

SVC_PROFILE is a profile index defined to indicate the profile index of enhancement layer. The subset_seq_parameter_set_rbsp( ) is expected to be updated to the video_parameter_set_rbsp( ) as defined in the HEVC DIS.

## Sequence parameter set SVC extension syntax

| seq_parameter_set_svc_extension( ) { | Descriptor |
| --- | --- |
| reserved_flag_for_singleloop_decoding | u(1) |
| upsampling_filter_idc | u(2) |
| } |  |

reserved_flag_for_singleloop_decoding is reserved for indicating the usage of single loop decoding.

upsampling_filter_idc specifies the phase used for up-sampling the reconstructed samples of base layer as specified in Section 2.2. The phase according to the value of upsamping_filter_idc is specified as follows.

| upsampling_filter_idc | filter-tap length | phase |
| --- | --- | --- |
| 0 | 8-tap | 0/4, 2/4 |
| 1 | 8-tap | 1/4, 3/4 |

## Slice layer extension RBSP syntax

| slice_layer_extension_rbsp( ) { | Descriptor |
| --- | --- |
| slice_header_in_scalable_extension( ) |  |
| slice_data_in_scalable_extension( ) |  |
| rbsp_slice_trailing_bits( ) |  |
| } |  |

## Slice header in scalable extension syntax

inter_layer_intra_prediction_enabled_flag equal to 1 specifies that inter-layer intra prediction (i.e., intra-layer texture prediction) is used as specified in Section 2.3.1; equal to 0 specifies that inter-layer intra prediction is not used. When this flag is not present, it is inferred to 0.

## Slice data in scalable extension syntax

| slice_data_in_scalable_extension( ) { | Descriptor |
| --- | --- |
| CurrTbAddr = LCUAddress |  |
| moreDataFlag = 1 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = -1 |  |
| do { |  |
| XLCU = HorLumaLocation( CurrTbAddr ) |  |
| YLCU = VerLumaLocation( CurrTbAddr ) |  |
| moreDataFlag = coding_tree_in_scalale_extension( XLCU, YLCU, Log2TbSize, 0 ) |  |
| CurrTbAddr = NextTbAddress( CurrTbAddr ) |  |
| if( CurrTbAddr = = firstTbInTileAddr ) |  |
| rbsp_trailingbits( ) |  |
| } while( moreDataFlag ) |  |
| } |  |

## Coding tree in scalable extension syntax

| coding_tree_in_scalable_extension( x0, y0, log2CUSize, cuDepth ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CUSize ) <= PicWidthInSamplesL && y0 + ( 1 << log2CUSize ) <= PicHeightInSamplesL && cuAddress( x0, y0 ) >= SliceAddress && log2CUSize > Log2MinCUSize ) { |  |
| split_coding_unit_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) { |  |
| if( cuDepth <= alf_cu_control_max_depth ) |  |
| if( cuDepth == alf_cu_control_max_depth ¦¦ |  |
| split_coding_unit_flag[ x0 ][ y0 ] == 0 ) |  |
| AlfCuFlagIdx++ |  |
| } |  |
| if( cu_qp_delta_enabled_flag && log2CUSize >= log2MinCUDQPSize ) |  |
| IsCuQpDeltaCoded = 0 |  |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if( cuAddress( x1, y0 ) > SliceAddress ) |  |
| moreDataFlag = coding_tree_in_scalable_extension ( x0, y0, log2CUSize – 1, cuDepth + 1 ) |  |
| if( cuAddress( x0, y1 ) > SliceAddress && moreDataFlag && x1 < PicWidthInSamplesL ) |  |
| moreDataFlag = coding_tree_in_scalable_extension ( x1, y0, log2CUSize − 1, cuDepth + 1 ) |  |
| if( cuAddress( x1, y1 ) > SliceAddress && moreDataFlag && y1 < PicHeightInSamplesL ) |  |
| moreDataFlag = coding_tree_in_scalable_extension ( x0, y1, log2CUSize − 1, cuDepth + 1 ) |  |
| if( moreDataFlag && x1 < PicWidthInSamplesL && y1 < PicHeightInSamplesL ) |  |
| moreDataFlag = coding_tree_in_scalable_extension ( x1, y1, log2CUSize − 1, cuDepth + 1 ) |  |
| } else { |  |
| if(adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlag[ x0 ][ y0 ] = alf_cu_flag[ AlfCuFlagIdx ] |  |
| coding_unit_in_scalable_extension ( x0, y0, log2CUSize ) |  |
| if( granularity_block_boundary( x0, y0, log2CUSize ) ) { |  |
| end_of_slice_flag | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |
| } else |  |
| moreDataFlag = 1 |  |
| } |  |
| return moreDataFlag |  |
| } |  |

## Coding unit in scalable extension syntax

base_pred_flag[ x0 ][ y0 ] equal to 1 specifies that inter-layer intra prediction (i.e., intra-layer texture prediction) as specified in Section 2.3.1 is used for decoding the current coding unit; equal to 0 specifies that inter-layer intra prediction is

# Compression performance discussion

## Category 1 (HEVC base layer)

### Spatial scalability

### SNR scalability

### Overall

The proposed scalable codec achieves approximately xx%, xx%, xx%, xx%, and xx% BD-rate savings on average for category 1 – 2x spatial scalability, category 1 – 1.5x spatial scalability, category 1 – 2x intra-only spatial scalability, category 1 – 1.5x intra-only spatial scalability, and category 1 – SNR scalability, respectively, compared with CfP “EL-only rate” anchors.

The proposed scalable codec achieves approximately xx%, xx%, xx%, xx%, and xx% BD-rate savings on average for category 1 – 2x spatial scalability, category 1 – 1.5x spatial scalability, category 1 – 2x intra-only spatial scalability, category 1 – 1.5x intra-only spatial scalability, and category 1 – SNR scalability, respectively, compared with CfP “EL+BL rate” anchors.

Please note that the actual rates of all the test points are within x.x% variation of corresponding target rates.

# Complexity analysis

## Encoding time and measurement methodology

The encoding time of the anchor is the run time of encoding single layer sequences of the enhancement layer (EL) spatial resolution by using HM6.1. The encoding time of the proposed scalable video encoder is the run time of encoding both base layer (BL) and EL sequences by using the proposed encoder software.

The encoding time starts after parsing the configuration file at the beginning and ends after recording coded bits for the last picture. For the proposed encoder software, both reconstructed YUVs of BL and EL are output. Also for the anchor, the reconstructed YUVs are output.

The run times of the proposed encoder software are shown in Table 8. Compared with the HM6.1 single layer encoding, the encoding times are x%, x%, x%, x% and x% for category 1 – 2x spatial scalability (namely RA-2x, where RA means “random access”), category 1 – 1.5x spatial scalability (namely RA-1.5x), category 1 – 2x intra-only spatial scalability (namely AI-2x, where AI means “all intra”), category 1 – 1.5x intra-only spatial scalability (namely AI-1.5x), and category 1 – SNR scalability (namely RA-SNR), respectively.

Table 8.  The encoding time of the proposed encoder software compared with HM6.1 anchor

|  | RA-2x | RA-1.5x | AI-2x | AI-1.5x | RA-SNR |
| --- | --- | --- | --- | --- | --- |
| Encoding Time | x% | x% | x% | x% | x% |

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM

Comparison with HM 6.1 should be set to “YUV output enabled” and “reference input disabled.”

The decoding time of the anchor is the run time of decoding single layer sequences of the enhancement layer (EL) spatial resolution by using HM6.1. The decoding time of the proposed scalable video decoder is the run time of decoding both base layer (BL) and EL sequences by using the proposed decoder software.

The decoding time starts after parsing bitstream at the beginning and ends after recoding the last reconstructed YUV output before releasing all memories. For the proposed decoder software, the reconstructed YUVs of EL are output. For the anchor, the reconstructed YUVs of single layer decoding (at enhancement resolution) are output.

The run times of the proposed decoder software are shown in Table 9. Compared with the HM6.1 single layer decoding, the decoding times are x%, x%, x%, x%, and x% for category 1 – 2x spatial scalability (namely RA-2x, where RA means “random access”), category 1 – 1.5x spatial scalability (namely RA-1.5x), category 1 – 2x intra-only spatial scalability (namely AI-2x, where AI means “all intra”), category 1 – 1.5x intra-only spatial scalability (namely AI-1.5x), and category 1 – SNR scalability (namely RA-SNR), respectively.

Table 9.  The decoding time of the proposed decoder software compared with HM6.1 anchor

|  | RA-2x | RA-1.5x | AI-2x | AI-1.5x | RA-SNR |
| --- | --- | --- | --- | --- | --- |
| Decoding Time | x% | x% | x% | x% | x% |

## Description of computing platform used to determine encoding and decoding times reported in Sections 5.1 and 5.2

Computing platforms and assigned simulations are summarized in Table 10.

Table 10.  Summary of computing platform

| CPU | Memory | OS | Storage |
| --- | --- | --- | --- |
| Intel Xeon Quad Core E31230 (3.2GHz) | 12GB DDR3 | Red Hat 4.1.2-50 | NAS (EP-2123-F8S6) |

# Patent rights declaration(s)

LG Electronics may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- ISO/IEC JTC1/SC29/WG11 (MPEG) and ITU-T SG16 WP3 (VCEG), “Joint Call for Proposals on Scalable Video Coding Extensions of High Efficiency Video Coding (HEVC),” ISO/IEC JTC1/SC29/WG11 N12957, July, 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, February, 2012.