| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0371 |
| --- | --- |

| Title: | AHG9: SEI payload extension mechanism |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 5775 Morehouse Dr San Diego, CA 92121, USA Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland Gary J. Sullivan One Microsoft Way Redmond WA 98052-6399, USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com +358 7180 08000 miska.hannuksela@nokia.com +1-425-703-5308 garysull@microsoft.com |
| Source: | Qualcomm Incorporated, Nokia Corporation, Microsoft Corporation |  |  |

_____________________________

# Abstract

This contribution proposes an SEI payload extension mechanism, which came out from an offline discussion in response to proposal item 3 in subsection 1.8 of JCTVC-K0120.

# Proposed spec text changes

Add the following in subclause 7.2 (Specification of syntax functions and descriptors):

more_data_in_sei_payload_syntax_structure( ) is specified as follows.

–	If byte_aligned( ) is equal to TRUE and there is no more data in the sei_payload( ) syntax structure, the return value of more_data_in_sei_payload_syntax_structure( ) is equal to FALSE.

–	Otherwise, the return value of more_data_in_sei_payload_syntax_structure( ) is equal to TRUE.

more_sei_payload_data( ) is specified as follows.

–	The sei_payload( ) syntax structure data is searched for the last (least significant, right-most) bit equal to 1 that is present in the sei_payload( ) syntax structure. Given the position of this bit, which is the bit_equal_to_one syntax element of the sei_payload( ) syntax structure, the following applies.

–	If there is more data in the sei_payload( ) syntax structure before the bit_equal_to_one syntax element, the return value of more_sei_payload_data( ) is equal to TRUE.

–	Otherwise, the return value of more_sei_payload_data( ) is equal to FALSE.

In subclause D.1, change the SEI payload syntax as follows:

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| if( more_data_in_sei_payload_syntax_structure( ) ) { |  |
| if( more_sei_payload_data( ) ) |  |
| reserved_sei_extension_data | u(v) |
| if( !byte_aligned( ) ) { |  |
| bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

In subclause D.2, add the following semantics:

reserved_sei_extension_data may have any value. Decoders shall ignore the value of reserved_sei_extension_data. The length, in bits, of reserved_sei_extension_data is is equal to the value of the variable seiExtLen, which is derived as follows:

seiExtLen = payloadSize * 8 − nEarlierBits − nByteAlignmentBits	(D-x)

where nEarlierBits is the number of bits in the SEI payload before the syntax element reserved_sei_extension_data, and nByteAlignmentBits minus 1 is the number of zero-valued bits at the end of the sei_payload( ) syntax structure.

In subclause D.1.21, change the active parameter sets SEI message syntax as follows:

| active_parameter_sets( payloadSize ) { | Descriptor |
| --- | --- |
| active_vps_id | u(4) |
| active_sps_id_present_flag | u(1) |
| if( active_sps_id_present_flag ) |  |
| active_seq_param_set_id | ue(v) |
| active_param_set_sei_extension_flag | u(1) |
| } |  |

In subclause D.2.21, remove the semantics for the syntax element active_param_set_sei_extension_flag.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Microsoft Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).