| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0372 |
| --- | --- |

| Title: | Encoder solutions for low bitrate contouring artefacts |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jani Lainema Kemal Ugur | Tel: Email: | jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution presents a modification to HM 8.0 encoder that is reported to reduce contouring effects present at the very low bitrates. The encoder algorithm discussed in this contribution classifies the intra slice LCUs into two categories - “flat” and “texture”. The texture LCUs are coded utilizing the selected slice QP, while the QP for the flat LCUs is clipped to have a maximum value of 27 in order to improve reconstruction on those areas. It is claimed that the low bitrate contouring artefacts are reduced to a similar level as what was achieved by normative modifications proposed in JCTVC-K0139. Objective coding results of the encoder only modifications are reported to be +0.1% and +0.0% for AI Main and AI HE10 configurations, respectively.

# Encoder algorithm description

The encoder algorithm discussed in this document classifies the LCUs in intra slices into the following two categories - “flat” and “texture”. The classification is done by finding the following for the LCU:

- Maximum sample value (max)

- Minimum sample value (min)

- Maximum difference between sample values of any two horizontally adjacent samples (maxH)

- Maximum difference between sample values of any two vertically adjacent samples (maxV)

The LCU is classified as “flat” under the following conditions:

Bool bFlat = maxH < 10 && maxV < 10 && (max - min) < 20;

Otherwise the LCU is considered to belong to the “texture” category.

In the case the LCU analyzed belongs to the flat category, the QP is clipped to have a maximum value of 27. The encoder also adjusts the lambda parameters to match the QP selected.

It is also noted that the problem usually starts happening on the first row and column of LCUs within a picture and then propagates from there to the smooth areas. Thus, it may also be sufficient just to apply the QP modification to the top and left LCUs within one picture. The attached software includes a switch to enable the discussed approach only for those LCUs, but that variant has not been studied further at the time of writing this contribution.

# Subjective results

The following pictures demonstrate the effects caused by the current HM 8.0 encoder and how the encoder algorithm discussed in this document affects contouring.

Figure 1: Extracts of first frame of Johnny and KristenAndSara sequences coded with all intra main configuration at QP=37 (HM 8.0)

Figure 2: Comparison of (from left to right) anchor, JCTVC-K0139 normative solution and an encoder solution with flat area QP of 27 (extract of first frame of KristenAndSara sequence coded with all intra main configuration at QP=37 (HM 8.0))

Figure 3: Comparison of (from left to right) anchor, JCTVC-K0139 normative solution and an encoder solution with flat area QP of 27 (extract of first frame of Johnny sequence coded with all intra main configuration at QP=37 (HM 8.0))

# Objective results

The objective results for the common conditions are shown below. Encoder times are from a cluster with different node and thus should not be considered representative.

Table 1: Objective results of the described encoder solution compared to HM8.0 (flat area QP set to 27 if slice QP is larger than 27)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.3% | -0.4% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.4% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class F | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | 0.0% |
| Overall | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.1% | -0.3% | -0.2% |
| Class B | 0.0% | 0.0% | 0.1% |
| Class C | 0.1% | 0.1% | 0.1% |
| Class D | 0.1% | -0.1% | -0.1% |
| Class E |  |  |  |
| Overall | 0.1% | -0.1% | 0.0% |
|  | 0.1% | -0.1% | 0.0% |
| Class F | 0.0% | 0.0% | -0.2% |

|  | Low delay B Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% |
| Class C | 0.1% | 0.2% | 0.3% |
| Class D | 0.1% | 0.5% | 0.1% |
| Class E | 0.2% | -0.6% | 0.7% |
| Overall | 0.1% | 0.1% | 0.2% |
|  | 0.1% | 0.1% | 0.2% |
| Class F | 0.2% | 0.0% | 0.5% |

Table 2: Objective results of the described encoder solution compared to HM8.0 (flat area QP set to 24 if slice QP is larger than 27)

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.4% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.7% | 0.2% | 0.2% |
| Overall | 0.2% | 0.1% | 0.0% |
|  | 0.2% | 0.1% | 0.0% |
| Class F | 0.2% | 0.2% | 0.4% |

# Draft Text

# Conclusions

This contribution identifies one possible encoder side solution to improve the HM 8.0 performance for low bitrate sequences with gradually changing smooth areas. It is proposed to consider the potential of non-normative approaches when deciding on possible improvements for contouring issues that appear to present at low bitrates.

# Patent rights declaration(s)

Nokia, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).