| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0380_r1 |
| --- | --- |

| Title: | BoG report on subjective viewing test for deblocking filter proposals CE1 Test 6 and K0289 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrey Norkin Kenneth Andersson | Email: | andrey.norkin@ericsson.com kenneth.r.andersson@ericsson.com |
| Source: | BoG on subjective viewing |  |  |

_____________________________

This contribution is a report on informal subjective viewing for the deblocking filtering that was held during the Shanghai meeting on October 17, 2012. The goal of informal subjective viewing test was to determine if the intra-boundary smoothing restriction investigated in the CE1 Test 6 is useful for improvement of subjective quality on problematic sequences. In total, 3 combinations were evaluated in the subjective test against the anchor (HM8.0 in common test conditions).

# Test procedure

Seven proposals were tested. The proposals were based on HM8.0. The proposals in each test session were evaluated by a group of 3 experts. In total, seven viewing sessions have been held. The identities of the proposals and the anchor were hidden (the test subjects were shown labels A or B instead).

The following six sequences have been used in the test.

- Riverbed, Qp=32, RA.

- Riverbed, Qp=37, RA

- WestWindEasy, Qp=37, LDB

- DucksTakeOff, Qp=37, LDB

- ChinaSpeed, Qp=37, LDB

- RedKayak, Qp=37, RA, frames 327…419.

One sequence from Type 2 sequences was used to make sure the tool does not result in degradation of the subjective quality on CTC sequences.

1. Johnny, Qp = 32, LDB

Anchor: HM8.0 in common test conditions.

ABAB test was used. The same sequence and the anchor were shown one after another. The order of proposals was randomized for every test and sequence and identities of the proposals are hidden. Test subjects were asked to rate each proposal on a scale from -2 to 2. The results were later calculated as well as 95-percent confidence intervals for every proposal and test sequence.

The length of one session was about 20 minutes. In total, 21 test subjects participated in scoring. The test sessions were conducted by Andrey Norkin. Processing of score sheets and calculations of the results were done by Andrey Norkin and Kenneth Andersson (Ericsson). Processing of score sheets was cross-verified by Shuo Lu (Sony).

# Test equipment

Equipment provided by meeting organizers was used. Samsung UA40ES6100J display with 40'' diagonal was used in the subjective tests.

# Proposals

Test 6 from CE1

Remove intra boundary smoothing (used in DC, horizontal and vertical modes) in prediction units of size 32x32

| - DC prediction mode, block 32x32, intra-filtering along a block boundary creates a singularity in prediction. | - Same block after overlaying the residual error, intra-prediction singularity creates an artifact at block boundary |
| --- | --- |
| - Remaining artifact after deblocking |  |

Figure 1 Riverbed. Example of blocking artifacts caused by intra prediction filtering.

HM8.0 + K289: tc_offsets: 0, 2, 3, 4.

HM8.0 + sending tc_offset depending on the picture depth in GOP

tc_offset_div2 = 3
beta_offset_div2 = 0

I

B

B

B

B

B

B

B

B

tc_offset_div2 = 2
beta_offset_div2 = 0

tc_offset_div2 = 0
beta_offset_div2 = 0

Test 6 + K289: tc_ffsets: 0, 2, 3, 4.

Test 6 + sending tc_offset depending on the picture depth in GOP

# Bitrates of the tested sequences

| Proposal | Riverbed, Qp=32, RA | Riverbed, Qp=37, RA | WestWindEasy,Qp37, LDB | DucksTakeOff, Qp=37, LDB | ChinaSpeed, Qp=37, LDB | RedKayak, Qp=37, RA |
| --- | --- | --- | --- | --- | --- | --- |
| Test 6 | 4775.91 | 2358.17 | 1680.29 | 5112.68 | 611.47 | 1717.33 |
| HM8.0 + K0289 | 4768.35 | 2354.45 | 1676.00 | 5108.50 | 610.46 | 1714.71 |
| Test 6 + K0289 | 4774.88 | 2356.50 | 1678.70 | 5108.63 | 612.15 | 1717.17 |
| HM8.0 CTC (for reference) | 4768.56 | 2354.45 | 1679.87 | 5113.19 | 612.05 | 1714.38 |

# Test results

The graphs below show the test results. Values with overlapping CI values cannot be rated as distinguishable.

# Conclusions

In the following cases the proposals have non-overlapped confidence intervals with the anchor and with each other. These cases are listed below:

Test 6 is statistically better than anchor in two cases, on Riverbed RA Qp32 and ChinaSpeed LDB Qp 37

K0289 is statistically better than the anchor in three cases

Test6+K0289 is better than the anchor in two cases.

# References

- A. Norkin, T. Suzuki, M. Budagavi, G. Van der Auwera, "Summary report of Core Experiment on deblocking filtering", JCTVC-K0021, Shanghai, Oct. 2012.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-J1100, Stockholm, July 2011.