| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0382 |
| --- | --- |

| Title: | Edits reflecting decisions recorded for frame packing arrangement SEI messages and display/cropping windows |  |  |
| --- | --- | --- | --- |
| Status: | Report of editing side activity |  |  |
| Purpose: | Editorial input of draft text to reflect recorded decisions |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan | Tel: Email: | +1 425 703 5308 garysull@microsoft.com |
| Source: | Microsoft |  |  |

_____________________________

- Abstract

This document provides draft text produced in side activity conducted to reflect decisions recorded during the meeting in relation to frame packing arrangement SEI messages and display/cropping window aspects. The text provided herein was initially drafted by Marco Arena (RAI) and Giovanni Ballocca (Sisvel), and was then refined by the submitter.

#### In subclause 7.3.2.2, apply the following changes (highlighted in yellow).

#### 7.3.2.2	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_bit | u(1) |
| profile_and_level( 1, sps_max_sub_layers_minus1 ) |  |
| seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| } |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| for( i = 0; i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase[ i ] | ue(v) |
| } |  |
| restricted_ref_pic_lists_flag | u(1) |
| if( restricted_ref_pic_lists_flag ) |  |
| lists_modification_present_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| if( pcm_enabled_flag ) { |  |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_coding_block_size | ue(v) |
| } |  |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag | u(1) |
| if( scaling_list_enable_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| if( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| sps_temporal_id_nesting_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_temporal_mvp_enable_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

[...]

In subclause 7.4.2.2, apply the following changes (highlighted in yellow).

#### 7.4.2.2	Sequence parameter set RBSP semantics

video_parameter_set_id refers to the active video parameter set. The value of video_parameter_set_id shall be in the range of 0 to 15, inclusive.

sps_max_sub_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in each coded video sequence referring to the sequence parameter set. The value of sps_max_sub_layers_minus1 shall be in the range of 0 to 6, inclusive.

sps_reserved_zero_bit shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value 1 for sps_reserved_zero_bit is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of sps_reserved_zero_bit.

sequence_parameter_set_id identifies a sequence parameter set. The value of seq_parameter_set_id shall be in the range of 0 to 31, inclusive.

chroma_format_idc specifies the chroma sampling relative to the luma sampling as specified in subclause Error: Reference source not found. The value of chroma_format_idc shall be in the range of 0 to 3, inclusive. When chroma_format_idc is not present, it is inferred to be equal to 1 (4:2:0 chroma format).

separate_colour_plane_flag equal to 1 specifies that the three colour components of the 4:4:4 chroma format are coded separately. separate_colour_plane_flag equal to 0 specifies that the colour components are not coded separately. When separate_colour_plane_flag is not present, it is inferred to be equal to 0. When separate_colour_plane_flag is equal to 1, the coded picture consists of three separate components, each of which consists of coded samples of one colour plane (Y, Cb or Cr) that each use the monochrome coding syntax. In this case, each colour plane is associated with a specific colour_plane_id value.

NOTE 1 – There is no dependency in decoding processes between the colour planes having different colour_plane_id values. For example, the decoding process of a monochrome picture with one value of colour_plane_id does not use any data from monochrome pictures having different values of colour_plane_id for inter prediction.

Depending on the value of separate_colour_plane_flag, the value of the variable ChromaArrayType is assigned as follows:

–	If separate_colour_plane_flag is equal to 0, ChromaArrayType is set equal to chroma_format_idc.

–	Otherwise (separate_colour_plane_flag is equal to 1), ChromaArrayType is set equal to 0.

pic_width_in_luma_samples specifies the width of each decoded picture in units of luma samples. pic_width_in_luma_samples shall not be equal to 0 and shall be an integer multiple of ( 1 << Log2MinCbSize ).

pic_height_in_luma_samples specifies the height of each decoded picture in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of ( 1 << Log2MinCbSize ).

conformance_window_flag equal to 1 indicates that the conformance cropping window offset parameters follow next in the sequence parameter set. conformance_window_flag equal to 0 indicates that the conformance cropping window offset parameters are not present.

conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, and conf_win_bottom_offset specify the samples of the pictures in the coded video sequence that are output from the decoding process, in terms of a rectangular region specified in picture coordinates for output. When conformance_window_flag is equal to 0, the values of conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, and conf_win_bottom_offset are inferred to be equal to 0.

The variables WinUnitX and WinUnitY are derived as follows:

–	If ChromaArrayType is equal to 0, WinUnitX and WinUnitY are derived as:

WinUnitX = 1		(7-1)
WinUnitY = 1		(7-2)

–	Otherwise (ChromaArrayType is equal to 1, 2, or 3), WinUnitX and WinUnitY are derived as:

WinUnitX = SubWidthC		(7-3)
WinUnitY = SubHeightC		(7-4)

The conformance cropping window contains the luma samples with horizontal picture coordinates from WinUnitX * conf_win_left_offset to pic_width_in_luma_samples − ( WinUnitX * conf_win_right_offset + 1 ) and vertical picture coordinates from WinUnitY * conf_win_top_offset to pic_height_in_luma_samples − ( WinUnitY * conf_win_bottom_offset + 1 ), inclusive. It is a requirement of bitstream conformance that the value of conf_win_left_offset shall be in the range of 0 to ( pic_width_in_luma_samples / WinUnitX ) − ( conf_win_right_offset + 1 ), inclusive; and the value of conf_win_top_offset shall be in the range of 0 to ( pic_height_in_luma_samples / WinUnitY ) − ( conf_win_bottom_offset + 1 ), inclusive.

When ChromaArrayType is not equal to 0, the corresponding specified samples of the two chroma arrays are the samples having picture coordinates ( x / SubWidthC, y / SubHeightC ), where ( x, y ) are the picture coordinates of the specified luma samples.

NOTE 2 – The conformance cropping window parameters are only applied at the output. All internal decoding processes are applied to the uncropped picture size.

In subclause C.2.2, apply the following changes (highlighted in yellow).

- C.2.2	Picture output

The following happens instantaneously at the CPB removal time of access unit n, tr( n ).

Picture n is considered as decoded after the last decoding unit of the picture is decoded.

When picture n has PicOutputFlag equal to 1, its DPB output time to,dpb( n ) is derived by

to,dpb( n ) = tr( n ) + tc * dpb_output_delay( n )		(C-5)

where dpb_output_delay( n ) is the value of dpb_output_delay specified in the picture timing SEI message associated with access unit n.

The output of the current picture is specified as follows.

–	If PicOutputFlag is equal to 1 and to,dpb( n ) = tr( n ), the current picture is output.

–	Otherwise, if PicOutputFlag is equal to 0, the current picture is not output, but will be stored in the DPB as specified in subclause Error: Reference source not found.

–	Otherwise (PicOutputFlag is equal to 1 and to,dpb( n ) > tr( n ) ), the current picture is output later and will be stored in the DPB (as specified in subclause Error: Reference source not found) and is output at time to,dpb( n ) unless indicated not to be output by the decoding or inference of no_output_of_prior_pics_flag equal to 1 at a time that precedes to,dpb( n ).

When output, the picture shall be cropped, using the conformance cropping window specified in the active sequence parameter set.

When picture n is a picture that is output and is not the last picture of the bitstream that is output, the value of Dto,dpb( n ) is defined as:

Dto,dpb( n ) = to,dpb( nn ) − to,dpb( n )		(C-6)

where nn indicates the picture that follows after picture n in output order and has PicOutputFlag equal to 1.

[…]

In subclause C.5.2.1, apply the following changes (highlighted in yellow).

- C.5.2.1	"Bumping" process

The "bumping" process is invoked in the following cases.

–	The current picture is an IDR or a BLA picture and no_output_of_prior_pics_flag is not equal to 1 and is not inferred to be equal to 1, as specified in subclause Error: Reference source not found.

–	The number of pictures in the DPB that are marked "needed for output" is greater than sps_max_num_reorder_pics[ TemporalId ], as specified in subclause Error: Reference source not found.

–	The number of pictures in the DPB with TemporalId lower than or equal to the TemporalId of the current picture is equal to sps_max_dec_pic_buffering[ TemporalId ], as specified in subclause Error: Reference source not found.

The "bumping" process consists of the following ordered steps:

- The picture that is first for output is selected as the one having the smallest value of PicOrderCntVal of all pictures in the DPB marked as "needed for output".

- The picture is cropped, using the conformance cropping window specified in the active sequence parameter set for the picture, the cropped picture is output, and the picture is marked as "not needed for output".

- If the picture storage buffer that included the picture that was cropped and output contains a picture marked as "unused for reference", the picture storage buffer is emptied.

[Ed. Note (GJS): Similarly search for other uses of the string "crop" and replace as necessary to account for the name change (assuming that we want to change the name, which is only an editorial matter).]

In subclause E.1.1, apply the following changes (highlighted in yellow).

E.1.1	VUI parameters syntax

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = Extended_SAR ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| } |  |
| overscan_info_present_flag | u(1) |
| if( overscan_info_present_flag ) |  |
| overscan_appropriate_flag | u(1) |
| video_signal_type_present_flag | u(1) |
| if( video_signal_type_present_flag ) { |  |
| video_format | u(3) |
| video_full_range_flag | u(1) |
| colour_description_present_flag | u(1) |
| if( colour_description_present_flag ) { |  |
| colour_primaries | u(8) |
| transfer_characteristics | u(8) |
| matrix_coefficients | u(8) |
| } |  |
| } |  |
| chroma_loc_info_present_flag | u(1) |
| if( chroma_loc_info_present_flag ) { |  |
| chroma_sample_loc_type_top_field | ue(v) |
| chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| neutral_chroma_indication_flag | u(1) |
| field_seq_flag | u(1) |
| default_display_window_flag | u(1) |
| if ( default_display_window_flag ) { |  |
| def_disp_win_left_offset | ue(v) |
| def_disp_win_right_offset | ue(v) |
| def_disp_win_top_offset | ue(v) |
| def_disp_win_bottom_offset | ue(v) |
| } |  |
| hrd_parameters_present_flag | u(1) |
| if( hrd_parameters_present_flag ) |  |
| hrd_parameters( 1, sps_max_sub_layers_minus1 ) |  |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| tiles_fixed_structure_flag | u(1) |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_mincu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| } |  |

[...]

In subclause E.2.1, add the following semantics (highlighted in yellow).

E.2.1 	VUI parameters semantics

[...]

default_display_window_flag equal to 1 indicates that the default display window parameters follow next in the VUI. default_display_window_flag equal to 0 indicates that the default display window parameters are not present. The default display window parameters identify the area within the conformance rectangle that is suggested to be displayed in the absence of any alternative indication (provided within the bitstream or by external means) of preferred display characteristics.

def_disp_win_left_offset, def_disp_win_right_offset, def_disp_win_top_offset, and def_disp_win_bottom_offset specify the samples of the pictures in the coded video sequence that are within the default display window, in terms of a rectangular region specified in picture coordinates for display. When default_display_window_flag is equal to 0, the values of def_disp_win_left_offset, def_disp_win_right_offset, def_disp_win_top_offset, and def_disp_win_bottom_offset are inferred to be equal to 0.

The following variables are derived from the default display window parameters.

leftOffset = conf_win_left_offset + def_disp_win_left_offset	(E-7)
rightOffset = conf_win_right_offset + def_disp_win_right_offset	(E-8)
topOffset = conf_win_top_offset + def_disp_win_top_offset	(E-9)
bottomOffset = conf_win_bottom_offset + def_disp_win_bottom_offset	(E-10)

The default display window contains the luma samples with horizontal picture coordinates from WinUnitX * leftOffset to pic_width_in_luma_samples − ( WinUnitX * rightOffset + 1 ) and vertical picture coordinates from WinUnitY * topOffset to pic_height_in_luma_samples − ( WinUnitY * bottomOffset + 1 ), inclusive. It is a requirement of bitstream conformance that the value of leftOffset shall be in the range of 0 to ( pic_width_in_luma_samples / WinUnitX ) − ( rightOffset + 1 ), inclusive; and the value of topOffset shall be in the range of 0 to ( pic_height_in_luma_samples / WinUnitY ) − ( bottomOffset + 1 ), inclusive.

When ChromaArrayType is not equal to 0, the corresponding specified samples of the two chroma arrays are the samples having picture coordinates ( x / SubWidthC, y / SubHeightC ), where ( x, y ) are the picture coordinates of the specified luma samples.

In subclause D.1.25, apply the following changes (highlighted in yellow).

D.1.25	Frame packing arrangement SEI message syntax

| frame_packing_arrangement( payloadSize ) { | Descriptor |
| --- | --- |
| frame_packing_arrangement_id | ue(v) |
| frame_packing_arrangement_cancel_flag | u(1) |
| if( !frame_packing_arrangement_cancel_flag ) { |  |
| frame_packing_arrangement_type | u(7) |
| quincunx_sampling_flag | u(1) |
| content_interpretation_type | u(6) |
| spatial_flipping_flag | u(1) |
| frame0_flipped_flag | u(1) |
| field_views_flag | u(1) |
| current_frame_is_frame0_flag | u(1) |
| frame0_self_contained_flag | u(1) |
| frame1_self_contained_flag | u(1) |
| if ( !quincunx_sampling_flag && frame_packing_arrangement_type != 5 ) { |  |
| frame0_grid_position_x | u(4) |
| frame0_grid_position_y | u(4) |
| frame1_grid_position_x | u(4) |
| frame1_grid_position_y | u(4) |
| } |  |
| frame_packing_arrangement_reserved_byte | u(8) |
| frame_packing_arrangement_repetition_period | ue(v) |
| } |  |
| upsampled_aspect_ratio_flag | u(1) |
| } |  |

[Ed. Note (GJS): Note that the extension flag at the end has been removed, as we now have an alternative extension mechanism for SEI messages.]

In subclause D.2.25, apply the following changes (highlighted in yellow).

D.2.25 Frame packing arrangement SEI message semantics

This SEI message informs the decoder that the output cropped decoded picture contains samples of multiple distinct spatially packed constituent frames that are packed into one frame using an indicated frame packing arrangement scheme. This information can be used by the decoder to appropriately rearrange the samples and process the samples of the constituent frames appropriately for display or other purposes (which are outside the scope of this Specification).

[Ed. Note (GJS): Consider and clarify, as necessary, the use of the term "frame" vs. "picture" in this SEI message name, syntax, and semantics. Frame units are suggested for the clarification.]

This SEI message may be associated with pictures that are either frames or fields. The frame packing arrangement of the samples is specified in terms of the sampling structure of a frame in order to define a frame packing arrangement structure that is invariant with respect to whether a picture is a single field of such a packed frame or is a complete packed frame.

frame_packing_arrangement_id contains an identifying number that may be used to identify the usage of the frame packing arrangement SEI message. The value of frame_packing_arrangement_id shall be in the range of 0 to 232 − 2, inclusive.

Values of frame_packing_arrangement_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of frame_packing_arrangement_id from 256 to 511 and from 231 to 232 − 2 are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore (remove from the bitstream and discard) all frame packing arrangement SEI messages containing a value of frame_packing_arrangement_id in the range of 256 to 511 or in the range of 231 to 232 − 2, and bitstreams shall not contain such values.

frame_packing_arrangement_cancel_flag equal to 1 indicates that the frame packing arrangement SEI message cancels the persistence of any previous frame packing arrangement SEI message in output order. frame_packing_arrangement_cancel_flag equal to 0 indicates that frame packing arrangement information follows.

frame_packing_arrangement_type indicates the type of packing arrangement of the frames as specified in Table D-8.

Table D-8–Definition of frame_packing_arrangement_type

| Value | Interpretation |
| --- | --- |
| 0 | Each component plane of the decoded frames contains a "checkerboard" based interleaving of corresponding planes of two constituent frames as illustrated in Error: Reference source not found. |
| 1 | Each component plane of the decoded frames contains a column based interleaving of corresponding planes of two constituent frames as illustrated in Error: Reference source not found and Error: Reference source not found. |
| 2 | Each component plane of the decoded frames contains a row based interleaving of corresponding planes of two constituent frames as illustrated in Error: Reference source not found and Error: Reference source not found. |
| 3 | Each component plane of the decoded frames contains a side-by-side packing arrangement of corresponding planes of two constituent frames as illustrated in Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found. |
| 4 | Each component plane of the decoded frames contains a top-bottom packing arrangement of corresponding planes of two constituent frames as illustrated in Error: Reference source not found and Error: Reference source not found. |
| 5 | The component planes of the decoded frames in output order form a temporal interleaving of alternating first and second constituent frames as illustrated in Error: Reference source not found. |

NOTE 1 – Error: Reference source not found to Error: Reference source not found provide typical examples of rearrangement and upconversion processing for various packing arrangement schemes. Actual characteristics of the constituent frames are signalled in detail by the subsequent syntax elements of the frame packing arrangement SEI message. In Error: Reference source not found to Error: Reference source not found, an upconversion processing is performed on each constituent frame to produce frames having the same resolution as that of the decoded frame. An example of the upsampling method to be applied to a quincunx sampled frame as shown in Error: Reference source not found or Error: Reference source not found is to fill in missing positions with an average of the available spatially neighbouring samples (the average of the values of the available samples above, below, to the left and to the right of each sample to be generated). The actual upconversion process to be performed, if any, is outside the scope of this Specification.

NOTE 2 – When the output time of the samples of constituent frame 0 differs from the output time of the samples of constituent frame 1 (i.e., when field_views_flag is equal to 1 or frame_packing_arrangement_type is equal to 5) and the display system in use presents two views simultaneously, the display time for constituent frame 0 should be delayed to coincide with the display time for constituent frame 1. (The display process is not specified in this Recommendation | International Standard.)

NOTE 3 – When field_views_flag is equal to 1 or frame_packing_arrangement_type is equal to 5, the value 0 for fixed_frame_rate_flag is not expected to be prevalent in industry use of this SEI message.

NOTE 4 – frame_packing_arrangement_type equal to 5 describes a temporal interleaving process of different views.

All other values of frame_packing_arrangement_type are reserved for future use by ITU-T | ISO/IEC. It is a requirement of bitstream conformance that the bitstreams shall not contain such other values of frame_packing_arrangement_type.

upsampled_aspect_ratio_flag equal to 1 indicates that the sample aspect ratio (SAR) indicated by the VUI parameters of the sequence parameter set identifies the SAR of the samples after the application of an upconversion process to produce a higher resolution frame from each constituent frame as illustrated in Error: Reference source not found to Error: Reference source not found. upsampled_aspect_ratio_flag equal to 0 indicates that the SAR indicated by the VUI parameters of the sequence parameter set identifies the SAR of the samples before the application of any such upconversion process.

NOTE 5 – The default display window parameters in the VUI parameters of the sequence parameter set can be used by an encoder to indicate to a decoder that does not interpret the frame packing arrangement SEI message that the default display window is an area within only one of the two constituent frames.

NOTE 6 – The SAR indicated in the VUI parameters should indicate the preferred display picture shape for the packed decoded frame output by a decoder that does not interpret the frame packing arrangement SEI message. When upsampled_aspect_ratio_flag is equal to 1, the SAR produced in each upconverted colour plane is indicated to be the same as the SAR indicated in the VUI parameters in the examples shown in Error: Reference source not found to Error: Reference source not found. When upsampled_aspect_ratio_flag is equal to 0, the SAR produced in each colour plane prior to upconversion is indicated to be the same as the SAR indicated in the VUI parameters in the examples shown in Error: Reference source not found to Error: Reference source not found.

- Patent rights declaration(s)

Microsoft Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).