| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K1004 |
| --- | --- |

| Title: | HEVC conformance testing draft 1 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Draft |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 teruhikos@jp.sony.com |
|  | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA | Tel: Email: | +1 425 703 5308 garysull@microsoft.com |
|  | Wade Wan 5300 California Ave Irvine, CA 92617 USA | Tel: Email: | wwan@broadcom.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document is a draft 1 of conformance test of HEVC.

- Conformance specification for HEVC

# Scope

This Recommendation | International Standard specifies a set of tests and procedures designed to indicate whether encoders or decoders meet the normative requirements specified in ITU-T H.HEVC | ISO/IEC 23008-2.

# Normative references

## General

The following Recommendations and International Standards contain provisions which, through reference in this text, constitute provisions of this Recommendation | International Standard. At the time of publication, the editions indicated were valid. All Recommendations and Standards are subject to revision, and parties to agreements based on this Recommendation | International Standard are encouraged to investigate the possibility of applying the most recent edition of the Recommendations and Standards listed below. Members of IEC and ISO maintain registers of currently valid International Standards. The Telecommunication Standardization Bureau of the ITU maintains a list of currently valid ITU-T Recommendations.

## Identical Recommendations | International Standards

–	None.

## Paired Recommendations | International Standards equivalent in technical content

–	ITU-T H.HEVC (in force), High efficiency coding.

ISO/IEC 23008-2: in force, Information technology – High efficiency video coding and media delivery in heterogeneous environment – Part 2: High Efficiency Video Coding.

–	ITU-T H.HEVC.2 (in force), Reference software for H.HEVC advanced video coding.

ISO/IEC 14496-5: in force, Information technology – Coding of audio-visual objects – Part 5: Reference software.

## Additional references

–	None.

# Definitions

For the purposes of this Recommendation | International Standard, the terms, definitions, abbreviations and symbols specified in ITU-T H.HEVC | ISO/IEC 23008-2 (particularly in clauses 3) apply. The following terms are further clarified for purposes herein as follows.

3.1	bitstream: An ITU-T H.HEVC | ISO/IEC 23008-2 video bitstream. A bitstream may contain I, P and B slices.

3.2	decoder: An ITU-T H.HEVC | ISO/IEC 23008-2 video decoder, i.e., an embodiment of the decoding process specified by ITU-T H.HEVC | ISO/IEC 23008-2. The decoder does not include the display process, which is outside the scope of this Recommendation | International Standard.

3.3	reference software decoder: The software decoder contained in ITU-T H.HEVC.2 | ISO/IEC 14496-5.

# Abbreviations and acronyms

For the purposes of this Recommendation | International Standard, relevant abbreviations and acronyms are specified in clause 4 of ITU-T H.HEVC | ISO/IEC 23008-2.

# Conventions

For the purposes of this Recommendation | International Standard, relevant conventions are specified in clause 5 in ITU-T H.HEVC | ISO/IEC 23008-2.

# Conformance for ITU-T H.HEVC | ISO/IEC 23008-2

## Introduction

The following clauses specify the normative tests for verifying conformance of video bitstreams as well as decoders. Those normative tests make use of test data (bitstream test suites) provided as an electronic annex to this Recommendation | International Standard and the reference software decoder with source code included in electronic format.

## Bitstream conformance

The bitstream conformance of ITU-T H.HEVC | ISO/IEC 23008-2 is specified by clause C.4 of ITU-T H.HEVC | ISO/IEC 23008-2.

## Decoder conformance

The decoder conformance of ITU-T H.HEVC | ISO/IEC 23008-2 is specified by clause C.5 of ITU-T H.HEVC | ISO/IEC 23008-2.

## Procedure to test bitstreams

A bitstream that claims conformance with ITU-T H.HEVC | ISO/IEC 23008-2 shall pass the following normative test:

The bitstream shall be decoded by processing it with the reference software decoder. When processed by the reference software decoder, the bitstream shall not cause any error or non-conformance messages to be reported by the reference software decoder. This test should not be applied to bitstreams that are known to contain errors introduced by transmission, as such errors are highly likely to result in bitstreams that lack conformance to ITU-T H.HEVC | ISO/IEC 23008-2.

Successfully passing the reference software decoder test provides only a strong presumption that the bitstream under test is conforming to the video layer, i.e., that it does indeed meet all the requirements for the video layer (except Annexes C, D and E) specified in ITU-T H.HEVC | ISO/IEC 23008-2 that are tested by the reference software decoder.

Additional tests may be necessary to more thoroughly check that the bitstream properly meets all the requirements specified in ITU-T H.HEVC | ISO/IEC 23008-2 including the hypothetical reference decoder (HRD) conformance (based on Annexes C, D and E). These complementary tests may be performed using other video bitstream verifiers that perform more complete tests than those implemented by the reference software decoder.

ITU-T H.HEVC | ISO/IEC 23008-2 contains several informative recommendations that are not an integral part of that Recommendation | International Standard. When testing a bitstream for conformance, it may also be useful to test whether or not the bitstream follows those recommendations.

To check correctness of a bitstream, it is necessary to parse the entire bitstream and to extract all the syntax elements and other values derived from those syntactic elements and used by the decoding process specified in ITU-T H.HEVC | ISO/IEC 23008-2.

A verifier may not necessarily perform all stages of the decoding process specified in ITU-T H.HEVC | ISO/IEC 23008-2 in order to verify bitstream correctness. Many tests can be performed on syntax elements in a state prior to their use in some processing stages.

## Procedure to test decoder conformance

### Conformance bitstreams

A bitstream has values of general_profile_idc, general_level_idc corresponding to a set of specified constraints on a bitstream for which a decoder conforming to a specified profile and level is required in Annex A of ITU-T H.HEVC | ISO/IEC 23008-2 to properly perform the decoding process.

### Contents of the bitstream file

The conformance bitstreams are included in this Recommendation | International Standard as an electronic attachment. The following information is included in a single zipped file for each such bitstream.

–	bitstream;

–	decoded pictures or hashes of decoded pictures (may not be present);

–	short description of the bitstream;

–	trace file (the bitstream in ASCII format).

In cases where the decoded pictures or hashes of decoded pictures are not available, the reference software decoder shall be used to generate the necessary reference decoded pictures from the bitstream.

### Requirements on output of the decoding process and timing

Two classes of decoder conformance are specified:

–	output order conformance; and

–	output timing conformance.

The output of the decoding process is specified in clauses 8 and Annex C of ITU-T H.HEVC | ISO/IEC 23008-2.

For output order conformance, it is a requirement that all of the decoded pictures specified for output in Annex C of ITU-T H.HEVC | ISO/IEC 23008-2 shall be output by a conforming decoder in the specified order and that the values of the decoded samples in all of the pictures that are output shall be (exactly equal to) the values specified in clause 8 of ITU-T H.HEVC | ISO/IEC 23008-2.

For output timing conformance, it is a requirement that a conforming decoder shall also output the decoded samples at the rates and times specified in Annex C of ITU-T H.HEVC | ISO/IEC 23008-2.

The display process, which ordinarily follows the output of the decoding process, is outside the scope of this Recommendation | International Standard.

### Recommendations (informative)

This clause does not form an integral part of this Recommendation | International Standard.

In addition to the requirements, it is desirable that conforming decoders implement various informative recommendations specified in ITU-T H.HEVC | ISO/IEC 23008-2 that are not an integral part of that Recommendation | International Standard. This clause lists some of these recommendations.

It is recommended that a conforming decoder be able to resume the decoding process as soon as possible after the loss or corruption of part of a bitstream. In most cases it is possible to resume decoding at the next start code or slice header. It is recommended that a conforming decoder be able to perform concealment for the macroblocks or video packets for which all the coded data has not been received.

### Static tests for output order conformance

Static tests of a video decoder require testing of the decoded samples. This clause will explain how this test can be accomplished when the decoded samples at the output of the decoding process are available. It may not be possible to perform this type of test with a production decoder (due to the lack of an appropriate accessible interface in the design at which to perform the test). In that case this test should be performed by the manufacturer during the design and development phase. Static tests are used for testing the decoding process. The test will check that the values of the samples decoded by the decoder under test shall be identical to the values of the samples decoded by the reference decoder. When a hash of the values of the samples of the decoded pictures is attached to the bitstream file, a corresponding hash operation performed on the values of the samples of the decoded pictures produced by the decoder under test shall produce the same results.

### Dynamic tests for output timing conformance

Dynamic tests are applied to check that all the decoded samples are output and that the timing of the output of the decoder's decoded samples conforms to the specification of clauses 8; and Annex C of ITU-T H.HEVC | ISO/IEC 23008-2, and to verify that the HRD models (as specified by the CPB and DPB specification in Annex C of ITU-T H.HEVC | ISO/IEC 23008-2) are not violated when the bits are delivered at the proper rate.

The dynamic test is often easier to perform on a complete decoder system, which may include a systems decoder, a video decoder and a display process. It may be possible to record the output of the display process and to check that display order and timing of fields or frames are correct at the output of the display process. However, since the display process is not within the normative scope of ITU-T H.HEVC | ISO/IEC 23008-2, there may be cases where the output of the display process differs in timing or value even though the video decoder is conforming. In this case, the output of the video decoder itself (before the display process) would need to be captured in order to perform the dynamic tests on the video decoder. In particular the field or frame order and timing shall be correct.

If buffering period SEI and picture timing SEI are included in the test bitstream, HRD conformance shall be verified using the values of initial_cpb_removal_delay, initial_cpb_removal_delay_offset, cpb_removal_delay and dpb_removal_delay that are included in the bitstream.

If buffering period SEI and picture timing SEI are not included in the bitstream, the following inferences shall be made to generate the missing parameters:

–	fixed_pic_rate_flag shall be inferred to be 1.

–	low_delay_hrd_flag shall be inferred to be 0.

–	cbr_flag shall be inferred to be 0.

–	The frame rate of the bitstream shall be inferred to be the frame rate value specified in the corresponding table of clause 6.7, where the bitstream is listed. If this is missing, then a frame rate of either 25 or 30000 ÷ 1001 can be inferred.

–	time_scale shall be set to 90,000 and the value of num_units_in_tick shall be computed based on field rate (twice the frame rate).

–	The bit rate of the bitstream shall be inferred to be the maximum value for the level specified in Table A-1 in ITU-T H.HEVC | ISO/IEC 23008-2.

–	CPB and DPB sizes shall be inferred to be the maximum value for the level specified in Table A-1 in ITU-T H.HEVC | ISO/IEC 23008-2.

With the above inferences, the HRD shall be operated as follows.

–	The CPB is filled starting at time t = 0, until it is full, before removal of the first access unit. This means that the initial_cpb_removal_delay shall be inferred to be equal to the total CPB buffer size divided by the bit rate divided by 90000 (rounded downwards) and initial_cpb_removal_delay_offset shall be inferred to be equal to zero.

–	The first access unit is removed at time t = initial_cpb_removal_delay ÷ 90000 and subsequent access units are removed at intervals based on the frame distance, i.e., 2 * (90000 ÷ num_units_in_tick) or the field distance, i.e., (90000 / num_units_in_tick), depending on whether the access unit is coded as a frame picture or field picture.

–	Using these inferences, the CPB will not overflow or underflow and the DPB will not overflow.

### Decoder conformance test of a particular profile-and-level

In order for a decoder of a particular profile-and-level to claim output order conformance to ITU-T H.HEVC | ISO/IEC 23008-2 as specified by this Recommendation | International Standard, the decoder shall successfully pass the static test specified in clause 6.5.5 with all the bitstreams of the normative test suite specified for testing decoders of this particular profile-and-level.

In order for a decoder of a particular profile and level to claim output timing conformance to ITU-T H.HEVC | ISO/IEC 23008-2 as specified by this Recommendation | International Standard, the decoder shall successfully pass both the static test specified in clause 6.5.5 and the dynamic test specified in clause 6.5.6 with all the bitstreams of the normative test suite specified for testing decoders of this particular profile-and-level. Tables 1 through 5 specify the normative test suites for each profile-and-level combination. The test suite for a particular profile-and level combination is the list of bitstreams that are marked with an 'X' in the column corresponding to that profile-and-level combination.

'X' indicates that the bitstream is designed to test both the dynamic and static conformance of the decoder.

The bitstream column specifies the bitstream used for each test.

A decoder that conforms to the Main profile at a specific level shall be capable of decoding the specified bitstreams in Tables 1.

## Specification of the test bitstreams

Some characteristics of each bitstream listed in Tables 1 are specified in this clause. In Tables 1, the value "29.97" shall be interpreted as an approximation of an exact value of 30000 ÷ 1001 and the value "59.94" shall be interpreted as an approximation of an exact value of 60000 ÷ 1001.

### Test bitstreams – General

[Ed.Note: The bitstream specifications will be added here.]

## Normative test suites for ITU-T H.HEVC | ISO/IEC 23008-2

Legend:

- X – Bitstream is for static and dynamic test

| Table 1 – Bitstreams for the Main profiles |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Categories | Bitstream | Donated by | File name |  |  |  | Main | Level | Frame rate (Frames/sec) |
| General | HEVCNL-1 |  |  |  |  |  | X | 1.2 and higher | 15 |

[Ed.Note: The above table will be updated during bitstream exchange activity and bitstream generation process.

The following bitstreams and volunteers are identified.

|  | Categories | Sub categories | Bitstream features | Volunteers | candidate |
| --- | --- | --- | --- | --- | --- |
|  | Intra coding | Intra prediction | Functional stage: Decoding of intra PU. Purpose: Check that the decoder can properly decode all values of intraPredMode for different Luma and Chroma transform block sizes. | C.S. Lim, F. Bossen, J. Lainema, K. Sugimoto | NHK |
|  |  | Constraint intra prediction | Functional stage: Decoding of intra PU in non-intra slice when constrained_intra_pred_flag is set to one Purpose: Check that the decoder can properly perform reference sample substitution process. | C.S. Lim, K. Chono |  |
|  | Inter frame coding | AMVP | Check whether motion vector prediction candidate generation and signalling perform correctly. Check the process when neighboring PUs have various partition mode, prediction mode, reference index, inter_pred_idc, etc.. Related syntax element: mvp_l0_flag, mvp_l1_flag | C. Kim, M. Zhou, Y. -W. Huang | LG, MediaTek |
|  |  | MVs | maximum number of motion vectors permitted per CTU |  |  |
|  |  |  | vectors reference padded picture edges. | M. Coban |  |
|  |  |  | clipping of motion vector range in MVP/Merge candidates | M. Coban |  |
|  |  |  | motion vectors are random, maximum page DRAM misses / minimize cache hits (according to memory bandwidth model... e.g. JCTVC-H007) |  |  |
|  |  | Merge | Merge candidate list index signaling: Check that merge candidate list can deal with every merge index (0..4) | B.Bross |  |
|  |  |  | Parallel merge: test all the possible combinations of log2_parallel_merge_level_minus2 and CTB sizes. | M. Zhou | MediaTek |
|  |  |  | number of maximum merge candidates signalling: test all the values of five_minus_max_merge_cand, i.e. 0, 1, 2, 3, 4 for CTB size of 64x64 | M. Zhou, | Samsung |
|  |  | WP | weighted_pred_flag and/or weighted_bipred_flag are equal to 1. | T. Chujoh | Technicolor |
|  |  |  | various weighted prediction parameter combinations | T. Chujoh | Technicolor |
|  |  | mvd_l1_zero_flag | Test mvd_l1_zero_flag | F. Bossen |  |
|  |  | temporal_mvp | Test slice_temporal_mvp_enable_flag | Microsoft |  |
|  |  | Collocated ref idx | Test collocated_ref_idx (single slice or multiple slice/picture with different reference lists.) |  | Broadcom |
|  | HL syntax | slices, PSs | bitstreams with stuffed extensions (vps_extension_data_flag=1, sps_extension_data_flag=1, pps_extension_flag=1, ..) | W. Jang, Qualcomm | Microsoft, Ericsson LG |
|  |  | dependent slice | Check: - dependent slices - WPP with dependent slices - tiles with dependent slices | B. Bross |  |
|  |  | POC | POC increments > 1 | Qualcomm | Microsoft, Vidyo, Ericsson LG |
|  |  | RPS |  | M. Coban |  |
|  |  |  |  | F. Bossen |  |
|  |  |  | Slice header RPS with short-term pictures | R. Sjöberg |  |
|  |  |  | Slice header RPS with short-term and long-term pictures | R. Sjöberg |  |
|  |  |  | PPS RPS with short-term pictures | R. Sjöberg |  |
|  |  |  | PPS RPS with short-term pictures and inter-RPS prediction | R. Sjöberg |  |
|  |  |  | Mix of PPS and slice header RPS with short-term pictures | R. Sjöberg |  |
|  |  |  | Mix of PPS and slice header RPS with short-and long-term pictures | R. Sjöberg |  |
|  |  | RPL |  | M.Coban |  |
|  |  | RAP | all 6 (currently identified) RAP types | F. Bossen | Sharp Qaulcomm, Microsoft, |
|  |  | HRD | maximum number of pictures in DPB | TBD | Microsoft |
|  |  |  | bitstream with largest possible CPB delay (maximum number of coded bits per picture but within MinCR) | TBD | Microsoft |
|  |  |  | bumping DPB | TBD | Microsoft |
|  |  |  | Low delay, Sub picture | K. Kazui | Fujutsu, Sharp, Qualcomm |
|  | Transform | RQT | Check all depths of inter & intra transform tree: QuadtreeTUMaxDepthInter : 0..4 QuadtreeTUMaxDepthIntra : 0..4 | B. Bross | Motorola |
|  | Quantization | dQP | all depths of CU and diff_cu_qp_delta_depth | W. Wan, T. Suzuki |  |
|  |  | Scaling list | non-default scaling lists | T. Suzuki, M. Zhou |  |
|  | Other coding tools | AMP | Check all asymetric partition mode enable | X. Zheng, C. Kim |  |
|  |  | Transform skip |  | J. Xu | BBC, RIM, Ericsson, MediaTek, Canon |
|  |  | I_PCM | Functional stage: Parsing of I_PCM size information at SPS and I_PCM flag at CU. Purpose: Check that the decoder can properly decode pcm_flag for different intra CU sizes. Related Syntax: log2_min_pcm_luma_coding_block_size_minus3, log2_diff_max_min_pcm_luma_coding_block_size, pcm_flag. | W. Wan, C.S. Lim, K. Chono |  |
|  |  |  | Functional stage: Parsing and decoding of I_PCM CUs. Purpose: Check that the decoder can properly decode I_PCM CUs with different PCM bit depth. Related syntax: pcm_sample_bit_depth_luma_minus1, pcm_sample_bit_depth_chroma_minus1, pcm_flag | W. Wan, C.S. Lim, K. Chono |  |
|  |  |  | Functional stage: Decoding of I_PCM CUs with loop filter skipping. Purpose: Check that the decoder can properly decode I_PCM CUs by skipping loop filter. Related syntax: pcm_loop_filter_disable_flag, pcm_flag. Note: Combination with cu_transquant_bypass_flag will be tested. | W. Wan, C.S. Lim, K. Chono |  |
|  |  | Lossless | transquant_bypass_enable_flag on, cu_transquant_bypass_flag is set to 0 and 1 randomly at CU level Purpose: Check that the decoder handles lossless coding. | F. Henry | Huawei |
|  | Parallel processing tools | Tile | Maximum number of tiles in a random, non-uniform arrangement that changes from picture to picture, with random chosen value for loop_filter_across_tiles_enabled_flag, deblocking_filter_override_flag, slice_disable_deblocking_filter_flag, slice_loop_filter_across_slices_enabled_flag | - Fuldseth, M. Zhou | Sharp, eBrisk |
|  |  | WPP | entropy_coding_sync_enabled_flag on. Purpose: Check that the decoder handles wavefront parallel processing | F. Henry | Ericsson, Mediatek, Samsung, Broadcom |
|  |  | WPP and slices | entropy_coding_sync_enabled_flag on, number of slice varies randomly between 2 and maximum number of slices permitted, each non-first slice is either regular or dependent in a random way. Purpose: Check that the decoder handles wavefront parallel processing and slices | F. Henry |  |
|  |  | Tile & QPP | entry point of WPP | Hendry | Sharp |
|  | Temporal scalability |  |  | W. Jang | Ericsson |
|  |  |  | range of nested temporal_id's |  |  |
|  | Deblocking filter |  | various cb, cr qp_offsets | T. Suzuki, R. Sjöberg |  |
|  |  |  | various beta_offset_div2, tc_offset_div2 | T. Suzuki, R. Sjöberg |  |
|  |  |  | trigger all deblocking decisions | T.Suzuki, R. Sjöberg |  |
|  |  |  | deblocking + tile, etc |  |  |
|  | SAO |  | Check all CTU sizes with max. SAO information rate. | E. Francois | Samsung, MediaTek, Sony, Motorola |
|  |  |  | Random and extreme SAO offsets values | E. Francois |  |
|  |  |  | Random SAO merge left/up flag values | E. Francois |  |
|  |  | Slice- and component-level control | Pictures divided into x slices / randomly enabling SAO Y and/or SAO UV per slice | E. Francois |  |
|  |  | Tile- and component-level control | Pictures divided into x tiles / randomly enabling SAO Y and/or SAO UV per tile | E. Francois |  |
|  | Entropy coding |  |  |  |  |
|  |  | Sign data hiding | sign_data_hiding_flag on, transquant_bypass_enable_flag on, cu_transquant_bypass_flag is set to 0 and 1 randomly at CU level Purpose: Check that the decoder handles sign data hiding with lossless coding. | H. Henry | RIM |
|  |  | High through put binalizaion |  | Qualcomm |  |
|  |  | cabac_init_idc |  | - Segall | Sharp |
|  |  | Max number of bins |  | V. Sze |  |
|  | 10 bit |  |  | - Luthra, A. Duenas |  |
| Corner case bitstream |  |  | Maximum number of slices permitted for Profile & Level combination; Each slice has unique short_term_ref_pic_set(), ref_pic_list_modification() and pred_weight_table() with maximum num_long_term_pics | C. Fogg |  |
|  |  |  | CTU size is 64x64 with no cu splits, and only one coded transform unit in the lower right 4x4 pixel corner of the CTU with delaQp signalled (as raised by P. Kaspenberg on the JCT reflector 12-Sept-2012) | C. Fogg |  |
|  |  |  | A wide range of values for pic_crop_left_offset, pic_crop_right_offset, pic_crop_top_offset, pic_crop_bottom_offset | C. Fogg |  |
|  |  |  | inferred splits along non-CTU multiple picture borders |  |  |
|  |  |  | combinations of transform_skip_enabled_flag, transquant_bypass_enable_flag, cu_transquant_bypass_flag |  |  |
|  |  |  | something to test the vast universe that is: POC signalling |  |  |
|  | Video resolution |  | pic_width_in_luma_samples = pic_height_in_luma_samples = (1 << Log2MinCbSize), with various CTU sizes. |  |  |

Two volunteers for each category are desiable.

Volunteers:

- T. Suzuki (Sony)

- M. Coban (Qualcomm)

- R. Sjöberg (Ericsson)

- C. S. Lim (Panasonic)

- W. Wan (Broadcom)

- M. Zhou (TI)

- C. Fogg (Harmonic)

- S. Lei (Mediatek)

- A. Fuldseth (Cisco)

- K. McCann (Samsung)

- F. Henry (Orange)

- F. Bossen (Docomo Innovations)

- B. Bross (Fraunhofer HHI)

- J. Xu (Microsoft)

- C. Kim (Samsung)

- E. Francois (Canon)

- X. Zheng (Hi-Silicon)

- V. Sze (TI)

- K. Chono (NEC)

- T. Chujoh (Toshiba)

- J. Lainema (Nokia)

- K. Sugimoto (Mitsubishi)

- Y. -W. Huang (MediaTek)

- W. Jang (Vidyo)

- K. Kazui (Fujitsu)

- Hendry (LG)

- A. Segall (Sharp)

- A. Luthra (Motorola)

- A. Duenas (NGCodec)

]

This Recommendation | International Standard includes an electronic attachment containing the conformance bitstreams identified within the text. The bitstreams can also be downloaded from the ITU-T Test Signal Database.