| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K1201 |
| --- | --- |

| Title: | Description of Tool Experiment A-1: Upsampling filter |  |  |
| --- | --- | --- | --- |
| Status: | Output document of JCT-VC |  |  |
| Purpose: | TE description |  |  |
| Author(s) or Contact(s): | Andrew Segall (Chair) Jianle Chen Jie Dong Elena Alshina | Email: | asegall@sharplabs.com |
| Source: | TE coordinators |  |  |

_____________________________

# Abstract

This document provides the description of Tool Experiment A1.  The topic of the Tool Experiment is testing Upsampling filters for SHVC.

# Introduction

During the 11th JCT-VC meeting in Shanghai, multiple proposals for upsampling filters for the SHVC design were proposed.  It was then decided to coordinate for further study and testing of these filters in a tool experiment.

# Participants

| Nr. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 | Andrew Segall (Chair) Jianle Chen (Co-chair) Jie Dong (Co-chair) Elena Alshina (Co-chair) Do-Kyoung Madhukar Budagavi Ankur Saxena Zhan Ma Krishna Rapaka Jinwen Zan Shan Liu Shih-Ta Hsiang Tzu-Der (Peter) Chuang Yu-Wen Huang Zhenzhong Chen Cheung Auyeung Shuo Lu Chuo-Hao Yeo Kiran Misra Patrice Onno Edouard Francois Fabrice Leleannec Yan Ye Xianghui Wei | Sharp Qualcomm Interdigital Samsung TI TI Samsung Samsung Qualcomm Huawei MediaTek MediaTek MediaTek MediaTek MediaTek SONY SONY I2R Sharp Canon Canon Canon Interdigital Huawei | asegall@sharplabs.com cjianle@qti.qualcomm.com Jie.Dong@InterDigital.com elena_a.alshina@samsung.com d-kwon@ti.com madhukar@ti.com asaxena@sta.samsung.com zhan.ma@sta.samsung.com krapaka@qti.qualcomm.com JINWEN.ZAN@huawei.com shan.liu@mediatek.com shih-ta.hsiang@mediatek.com peter.chuang@mediatek.com yuwen.huang@mediatek.com zhenzhong.chen@mediatek.com Cheung.Auyeung@am.sony.com Shuo.Lu@jp.sony.com chyeo@i2r.a-star.edu.sg misrak@sharplab.com Patrice.onno@crf.canon.fr Edouard.francois@crf.canon.fr Fabrice.leleannec@crf.canon.fr Yan.Ye@InterDigital.com weixianghui@huawei.com |  |

# Experimental Conditions

  - Test Sequences, Bit Rates and Coding Conditions

The TE will use the Class A and Class B sequences that were considered in the SHVC Call for Proposals.  All frames shall be processed, and the lower resolution representations of the sequences shall correspond to the same lower resolution representations as considered in the SHVC Call for Proposals.

The TE shall test the AI, RA and LD-P configuration for both 2x and 1.5x spatial scalability scenarios.  It is mandatory to report the performance of tested tools for these three configurations.  Proponents shall use the prediction structures provided by the AhG on SHVC tool experiments (by November 16, 2012).

The TE shall use CU based signaling for indicating the use of upsampled baselayer data.  The CU based signaling will be provided via software to the TE.

The TE shall test combinations where the baselayer QP is 26 and 30, and the difference between the baselayer QP and enhancement QP are -2, 0, 2 or 4.

The TE shall consider a block of upsampled base layer data as an “Intra” block for the purpose of de-blocking.  This will be provided via software to the TE.

The TE shall consider simulcast as the anchor for quantitative measurements.

  - Software

Software provided in JCTVC-K0348 and available on November 2, 2012.

  - Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures, complexity, and subjective quality. Complexity assessment is performed on the base of decoding complexity measurement.

### Complexity assessment

Both HW and SW complexity of proposed tools for both encoder and decoder will be evaluated.

To measure software run time, software with the anchor configuration and the proposal implemented on the software shall be used. To measure decoding time, the decoding shall be executed on the same machine without output of decoded YUV data. The computational time must be measured for each test sequence and test case for both anchor and proposal. Relative computational time calculated against the anchor must be presented.

### Subjective tests at the meeting

An informal subjective viewing by experts during the Geneva meeting should be performed and results provided during the meeting in order to ensure tested proposals do not cause degradation of subjective quality on common test condition sequences and improve subjective quality on the additional sequences. Test sequences should be determined before the beginning of cross-check. Proponents are not allowed to participate in the test. It is recommended that the number of test subjects is over 20.

The following test procedure will be used. ABAB test is used. The same sequence and the anchor are shown one after another. The order of proposals is randomized for every test and sequence and identities of the proposals are hidden. Test subjects are asked to rate each proposal on a scale from -2 to 2. The results are later calculated as well as 95-percent confidence intervals for every proposal and test sequence.

### Sequences to be used in subjective test

To be determined before beginning of cross-check.

# Tools to test

## JCTVC-K0378 “Suggested up-sampling filter design for tool experiments on HEVC scalable extension”

Performance of up-sampling filter used for testing in TE-A2 will be evaluated in this test. Phase derivation process is described in the document JCTVC-K0378. Coefficients of up-sampling filter for this test are listed in Tables 1 and 2.

Table 1. Luma up-sampling filters for Test 4.1.

| Phase shift | Luma up-sampling filter |
| --- | --- |
| 0 | { 0, 0, 0, 64, 0, 0, 0, 0}/64 |
| 1/3 | {-1, 4, -11, 52, 26, -8, 3, -1}/64 |
| 1/2 | {-1, 4, -11, 40, 40, -11, 4, -1}/64 |
| 2/3 | {-1, 3, -8, 26, 52, -11, 4, -1}/64 |

Table 2. Chroma up-sampling filters for Test 4.1.

| Phase shift | Chroma up-sampling filter |
| --- | --- |
| 0 | {0, 64, 0, 0}/64 |
| 1/4 | {-4, 54, 16, -2}/64 |
| 1/3 | {-5, 50, 22, -3}/64 |
| 3/8 | {-6, 46, 28, -4}/64 |
| 1/2 | {-4, 36, 36, -4}/64 |
| 7/12 | {-4, 30, 43, -5}/64 |
| 2/3 | {-3, 22, 50, -5}/64 |
| 7/8 | {-2, 10, 58, -2}/64 |
| 11/12 | {-1, 5, 62, -2}/64 |

## JCTVC-K0034 “Description of scalable video coding technology proposal by InterDigital Communications”

The fixed upsampling filters that generate the inter layer reference pictures for 1.5X and 2X spatial scalability have the following features:

- Derived from Gaussian windowed Sinc function.

- 2-D separable.

- For pixels in the ILR picture that correspond to pixels at integer pixel locations in the base layer picture, they are directly copied from the base layer without filtering.

- For pixels in the ILR picture that correspond to pixels at factional pixel locations in the based layer picture, they are filtered using 8-tap filters for the luma component and 4-tap filters for the chroma components.

- The precision of all the upsampling filter coefficients is 8-bit, and filtering is realized by 16-bit implementation.

The fixed upsampling filter coefficients are provided in Table 3.

Table 3. The fixed upsampling filter coefficients  for Test 4.2

| Luma | 2X |  | [-3 | 8 | -21 | 80 | 80 | -21 | 8 | -3]/128 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.5X |  | [-5 | 11 | -24 | 107 | 53 | -18 | 8 | -4]/128 |
|  |  |  | [-4 | 8 | -18 | 53 | 107 | -24 | 11 | -5]/128 |
| Chroma | 2X | Vertical | [-17 | 100 | 56 | -11]/128 |  |  |  |  |
|  |  |  | [-3 | 15 | 126 | -10]/128 |  |  |  |  |
|  |  | Horizontal | [-15 | 79 | 79 | -15]/128 |  |  |  |  |
|  | 1.5X | Vertical | [-20 | 122 | 38 | -12]/128 |  |  |  |  |
|  |  |  | [-20 | 71 | 101 | -24]/128 |  |  |  |  |
|  |  |  | [-4 | 12 | 128 | -8]/128 |  |  |  |  |
|  |  | Horizontal | [-23 | 113 | 54 | -16]/128 |  |  |  |  |
|  |  |  | [-16 | 54 | 113 | -23]/128 |  |  |  |  |

In TE-A1, we will also study alternatives of the filters in Table 1 for example, the same filters but with lower precisions.

## JCTVC-K035 “Description of scalable video coding technology proposal by Qualcomm (configuration 1)”

The fixed upsampling filters in the document JCTVC-K0035 will be evaluated in this test. Coefficients of up-sampling filter for this test are listed in the following tables. The Luma and Chroma filters could be tested separately.

Table 4. Luma up-sampling filters for Test 4.3.

| Phase shift | Luma up-sampling filter |
| --- | --- |
| 0 | { 0, 0, 0, 64, 0, 0, 0, 0}/64 |
| 1/3 | {-1, 4, -11, 53, 25, -8, 3, -1}/64 |
| 1/2 | {-1, 4, -11, 40, 40, -11, 4, -1}/64 |
| 2/3 | {-1, 3, -8, 25, 53, -11, 4, -1}/64 |

Table 5. Chroma up-sampling filters for Test 4.3.

| Phase shift | Chroma up-sampling filter |
| --- | --- |
| 0 | { 0, 64, 0, 0}/64 |
| 1/4 | {-4, 54, 16, -2}/64 |
| 1/3 | {-6, 52, 20, -2}/64 |
| 3/8 | {-6, 46, 28, -4}/64 |
| 1/2 | {-4, 36, 36, -4}/64 |
| 7/12 | {-4, 30, 42, -4}/64 |
| 2/3 | {-2, 20, 52, -6}/64 |
| 7/8 | {-2, 10, 58, -2}/64 |
| 11/12 | { 0, 4, 62, -2}/64 |

## JCTVC-K0044 “ Description of high efficiency scalable video coding technology proposal by Samsung and Vidyo”

An effect of accurate phase alignment in vertical Chroma interpolation will be evaluated in this test. This test has the only one difference from Test 4.1. Accurate Chroma alignment will not be considered in phase derivation process. As result phase derivation process for Luma and Chrom, vertical and horizontal interpolation will use the same formula. Filter coefficients for Luma up-sampling are the same with Test 4.1.    The table of filter coefficients needed for Chroma up-sampling is shorter, since phase shifts ¼,3/8,7/12,7/8,11/12 are not needed in this design.

Table 6. Chroma up-sampling filters for Test 4.4.

| Phase shift | Chroma up-sampling filter |
| --- | --- |
| 0 | {0, 64, 0, 0}/64 |
| 1/3 | {-5, 50, 22, -3}/64 |
| 1/2 | {-4, 36, 36, -4}/64 |
| 2/3 | {-3, 22, 50, -5}/64 |

## JCTVC-K0031 “Description of scalable video coding technology proposal by Sharp”

In this test the performance of the fixed separable up-samplers described in JCTVC-K0031 (and listed in Table 7) will be evaluated for spatial scalability factor 2 and spatial scalability factor 1.5. Note, for each color component, the “support size” and “position of supporting pixels with respect to the pixel being up-sampled” is kept the same within a horizontal or vertical filtering stage. The support size (number of tap values used) is 8 for luma and 4 for chroma. The same up-sampler is used for both the chroma components.

Table 7 Luma/Chroma up-samplers for spatial scalability factor 2 and 1.5

| Spatial scalability factor 2 | Spatial scalability factor 1.5 |
| --- | --- |
| Luma (Y): h0 = { 0, 0, 0, 0, 64, 0, 0, 0}/64 h1 = { -1, 4, -11, 40, 40, -11, 4, -1 }/64 v0 = { 0, 0, 0, 0, 64, 0, 0, 0}/64 v1 = { -1, 4, -11, 40, 40, -11, 4, -1 }/64 | Luma (Y): h0 = { 0, 0, 0, 0, 64, 0, 0, 0 }/64 h1 = { -1, 5, -12, 52, 26, -9, 4, -1 }/64 h2 = { -1, 4, -9, 26, 52, -12, 5, -1 }/64 v0 = {0, 0, 0, 0, 64, 0, 0, 0}/64 v1 = {-1, 5, -12, 52, 26, -9, 4, -1}/64 v2 = { -1, 4, -9, 26, 52, -12, 5, -1 }/64 |
| Chroma (U/V): h0 = { 0, 0, 64, 0}/64 h1 = {-4, 36, 36, -4}/64 v0 = { 0, 0, 64, 0}/64 v1 = {-4, 36, 36, -4}/64 | Chroma (U/V): h0 = { 0, 0, 64, 0}/64 h1 = { -7, 51, 25, -5 }/64 h2 = { -5, 25, 51, -7}/64 v0 = { 0, 0, 64, 0}/64 v1 = { -7, 51, 25, -5 }/64 v2 = { -5, 25, 51, -7}/64 |

# Cross Check Assignments

| Test | Document # | Test will be provided by | Cross-checker |
| --- | --- | --- | --- |
| 4.1 | JCTVC-K0378 | Samsung (elena_a.alshina@samsung.com) | Qualcomm (cjianle@qti.qualcomm.com ) |
| 4.2 | JCTVC-K0034 | InterDigital ( Jie.Dong@InterDigital.com) | Samsung (elena_a.alshina@samsung.com) |
| 4.3 | JCTVC-K0035 | Qualcomm (cjianle@qti.qualcomm.com ) | Sharp (misrak@sharplabs.com) Samsung (elena_a.alshina@samsung.com) |
| 4.4 | JCTVC-K0044 | Samsung (elena_a.alshina@samsung.com) | TI (madhukar@ti.com) |
| 4.5 | JCTVC-K0031 | Sharp (misrak@sharplabs.com) | Samsung (elena_a.alshina@samsung.com) |

# Time-line and Responsibilities

2012-Nov-2:	TE-A1 description finalized and uploaded.

2012-Nov-2:	Software distributed with CU based inter-layer prediction signaling and agreed de-blocking behavior

2012-Nov-16:	Configuration files and anchor performance distributed by AhG on SHVC tool experiments

2012-Dec-3:	Cross-verification begins, proponents provide software, draft of contribution text and results to cross-verifiers(s) and TE A1 coordinators.

2012-Jan-7:	Cross-verifiers report results to TE participants (2 weeks from cross-check start)

2012-Jan-7:	Contribution documents uploaded, the document number and title are reported to coordinators.

2012-Jan-14:	Summary report uploaded

# References