| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K1102 |
| --- | --- |

| Title: | Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Output document of JCT-VC |  |  |
| Purpose: | TE description |  |  |
| Author(s) or Contact(s): | Liwei Guo (Chair) Yong He Do-Kyoung Kwon Jinwen Zan Haricharan Lakshman Jung Won Kang | Email: | liweig@qti.qualcomm.com Yong.He@InterDigital.com d-kwon@ti.com JINWEN.ZAN@huawei.com haricharan.lakshman@hhi.fraunhofer.de jungwon@etri.re.kr |
| Source: | TE coordinators |  |  |

_____________________________

# Abstract

This document provides the description of Tool Experiment A2. The topic of the Tool Experiment is testing inter-layer texture prediction signaling.

# Introduction

This document defines Tool Experiment A2 (TE A2) on inter-layer texture prediction signaling, to be performed for the upcoming January 2013 JCT-VC meeting.

# Participants

| No. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Liwei Guo Vadim Seregin Jianle Chen Geert Van Der Auwera | Qualcomm | liweig@qti.qualcomm.com vseregin@qti.qualcomm.com cjianle@qti.qualcomm.com geertv@qti.qualcomm.com | Proponent |
| 2 | Yong He Yan Ye Xiaoyu Xiu | InterDigital | Yong.He@InterDigital.com Yan.Ye@InterDigital.com Xiaoyu.Xiu@InterDigital.com | Proponent |
| 3 | Do-Kyoung Kwon Madhukar Budagavi | Texas Instruments | d-kwon@ti.com madhukar@ti.com | Cross-checker |
| 4 | Jinwen Zan Xianghui Wei Haoping Yu Haitao Yang | Huawei | JINWEN.ZAN@huawei.com weixianghui@huawei.com haoping.yu@huawei.com haitao.yang@huawei.com | Proponent |
| 5 | Haricharan Lakshman | HHI | haricharan.lakshman@hhi.fraunhofer.de |  |
| 6 | Jung Won Kang Jinho Lee Hahyun Lee | ETRI | jungwon@etri.re.kr jinosoul@etri.re.kr hanilee@etri.re.kr | Proponent |
| 7 | Kawamura Kei | KDDI | kei@kddilabs.jp |  |
| 8 | Tmoyuki Yamamoto Takeshi Tsukuba Andrew Segall Kiran Misra Jane Zhao | Sharp | yamamoto.tomoyuki@sharp.co.jp tsukuba.takeshi@sharp.co.jp asegall@sharplabs.com misrak@sharplabs.com jzhao@sharplabs.com | Proponent |
| 9 | Wenhao Zhang | Intel | wenhao.zhang@intel.com | Proponent |
| 10 | Edouard Francois Fabrice Le Léannec Patrice Onno | Canon | Fabrice.leleannec@crf.canon.fr Edouard.francois@crf.canon.fr Patrice.onno@crf.canon.fr | Proponent |
| 11 | Tzu-Der Chuang Yu-Wen Huang Shan Liu Polin Lai Shih-Ta Hsiang Ximin Zhang | MediaTek | Peter.chuang@mediatek.com yuwen.huang@mediatek.com shan.liu@mediatek.com polin.lai@mediatek.com shih-ta.hsiang@mediatek.com ximin.zhang@mediatek.com | Proponent |
| 12 | Chulkeun Kim Joonyoung Park Jungsun Kim | LG | chulkeun.kim@lge.com jy.park@lge.com jungsun.kim@lge.com | Proponent |
| 13 | O. Bici D. Bugdayci K. Ugur J. Lainema | Nokia | ext-oguz.bici@nokia.com ext-done.bugdayci@nokia.com Kemal.Ugur@nokia.com Jani.Lainema@nokia.com | Proponent |
| 14 | Kazushi Sato J. Xu S. Lu | Sony | Kazushi.Sato@jp.sony.com jun.xu@am.sony.com Shuo.Lu@jp.sony.com |  |
| 15 | Ankur Saxena Zhan Ma Tammy Lee Elena Alshina | Samsung | asaxena@sta.samsung.com zhan.ma@sta.samsung.com tammy.lee@samsung.com elena_a.alshina@samsung.com | Proponent |
| 16 | Y. H. Tan | I2R | yhtan@i2r.a-star.edu.sg |  |

# Tools to test

## CU Based approach

### CU level Intra BL Mode (Qualcomm, JCTVC-K0035/K0036)

Inter-layer texture prediction mode is signaled at CU level. A flag intra_bl_skip_flag is first signaled. If intra_bl_skip_flag = 1, reconstructed base layer pixel values are used as the prediction and no residue is transmitted. In the case that intra_bl_skip_flag = 0, syntax intra_bl_flag is signaled. If intra_bl_flag = 1, reconstructed base layer pixel values are used as the prediction. The deblocking for Intra BL mode and IntraBL Skip mode was specified in JCTVC-K0035/K0036.

An additional test (optional) to encode Intra BL block residue as Inter block will be evaluated.

Results for combination with other deblocking strengh may be provided as additional information.

### PU level I_BL signaling (Sharp, JCTVC-K0032)

For NxN PU, Inter-layer pred mode is signaled in combination of CU level flag (inter_layer_pred_flag) and IntraPredMode in HEVC. When inter_layer_pred_flag equals to 1 and IntraPredMode equals to 1 (DC mode), corresponding prediction block is predicted using base-layer reference. Otherwise, prediction blocks are predicted using HEVC intra prediction corresponding to IntraPredMode. For 2Nx2N CU, same signaling with anchor is used.

Table 1 Signaling for Test 3.1.2

| inter_layer_pred_flag (CU level) | IntraPredMode (Pred. block level) | Associated prediction mode |
| --- | --- | --- |
| 0 | 0..34 | HEVC intra prediction |
| 1 | 0, 2..34 | HEVC intra prediction |
| 1 | 1 | Inter-layer texture prediction |

### Inter-layer texture prediction mode signaling (Huawei, JCTVC-K0046)

This inter-layer texture prediction mode is signaled on the CU basis in both inter and intra pictures in the enhancement layer, with partition size 2Nx2N. A flag, named inter_layer_texture_prediction_flag, is inserted after the skip_flag to indicate whether this mode is selected.

The residual coding of CUs in this mode follows that of the HEVC.

### Inter-layer texture prediction mode with skip signaling (Huawei, JCTVC-K0046)

In case the inter-layer texture prediction results in all DCT coefficients quantized to 0s, the inter-layer texture skip mode could be signaled. It is noted that in case a CU is signaled as the inter-layer texture prediction mode, but not skipped, the no_residue_syntax_flag could be inferred to be 1, thus there is no need to code this flag.

The proposed inter-layer texture skip mode applies to both intra and inter slices in the enhancement layer.

In HEVC, the skip_flag is signaled at the beginning of a CU in P/B slices, but not in I slices. In K0046, the skip_flag is reused in EL I slices, to indicate wheather the proposed inter-layer texture skip is used or not.

In the case of enhancement layer P/B slice, the skip_flag is used to indicate the usage of the skip mode, so it cannot be used to signal the proposed Inter-layer texture skip mode. Besides, the proposed inter-layer texture skip mode needs to be differentiated from the ordinary skip mode. For this purpose, an addition flag inter_layer_texture_skip_flag is added right after the skip_flag. Only when skip_flag is 1, is this added flag parsed.

### ILSkip mode and ILDirect mode (Intel, JCTVC-K0039)

JCTVC-K0039 applies inter-layer texture prediction at CU level by a signal flag, denoted as inter_layer_skip_flag, to be added after the split_cu_flag to CU syntax table to indicate if the current CU to apply inter-layer pixel prediction or not. When inter_layer_skip_flag is equal to 1, there is a flag, denoted as inter_layer_direct_flag, to indicate if residual signal exists or not for current CU. With these two flags, two new coding modes are proposed for CU coding of EL video:

ILSkip mode: inter-layer pixel prediction is used without any residual transmitted;

ILDirect mode: inter-layer pixel prediction is used with residual to be decoded.

Figure 1 Flowchart of CU decoding process with inter-layer pixel prediction

The decoding process is provided in Figure 1.

### Inter-layer modes signaling as non-inter modes (Canon, JCTVC-K0041)

The inter-layer texture prediction can be controlled at the slice and PU level. Two inter-layer modes are supported:

- IntraBL, enabling texture prediction from the upsampled BL texture of the collocated block;

- BaseMode, mixing inter-layer texture prediction (for BL blocks coded as intra), and temporal prediction using BL motion data (for BL blocks coded as inter). In addition a deblocking filter is applied to the prediction samples on the border of the inherited BL partitions.

Only IntraBL mode is considered in this TE and results using IntraBL only will be provided.

In the EL slice header, one or more flags are inserted to signal the use of specific coding modes, including the intra prediction and the inter-layer coding modes, in the considered enhancement slice. A flag inserted in the slice header (enable_intra_pred_flag) controls the usage of the inter-layer texture prediction for the EL slice. In K0041, some other inter-layer modes are also possibly controlled by slice level flags (enable_grilp_flag, enable_inter_layer_flag) but are not considered in this TE.

At the PU level, inter-layer texture prediction modes are inserted in the syntax tree in the non inter PU branch.

When enabled at the slice level, a flag inter_layer_flag syntax element is inserted at PU level, as a non-inter mode, to control the inter layer modes (“IntraBL” and possibly “BaseMode”). If only inter-layer texture prediction is enabled for the EL slice as inter layer mode, this flag directly enables or disables the IntraBL mode. Otherwise, if inter-layer texture prediction modes (namely IntraBL and possibly BaseMode) are enabled for the EL slice as inter layer modes, and when inter_layer_flag is equal to 1, an additional flag inter_layer_mode is inserted to indicate if the mode is IntraBL or BaseMode.

This is illustrated in the figure below, in three different cases: intra prediction and inter-layer IntraBL enabled in the EL slice, intra prediction disabled and inter-layer IntraBL enabled in the EL slice, intra prediction enabled and inter-layer IntraBL disabled in the EL slice. Only left part of the shown decoding trees relates to the inter-layer texture syntax signalling.

intra prediction and inter-layer IntraBL enabled

intra prediction disabled and inter-layer IntraBL enabled

intra prediction enabled and inter-layer IntraBL disabled

Figure 2 Syntax Tree for Test 3.1.6

### Inter-layer texture prediction signaling (MediaTek and LG, JCTVC-K0033)

An Intra_BL_flag is signaled at coding unit (CU) level to indicate whether the current CU uses Intra_BL or not. The information of neighbouring blocks is used for the context formation of the Intra_BL_flag. Furthermore, a no_residual_syntax_flag is used to indicate whether the CU has at least a nonzero coefficient. Deblocking filter for Intra BL mode will be investigated in this test.

### PU based inter-layer texture prediction signaling (Nokia, JCTVC-K0040)

In this tool, the inter-layer texture prediction is signaled at the PU level. The flag implies that both luma and chroma textures are to be copied.

The behavior differs for intra and inter slices as follows:

- Intra slices: For each intra PU, a new flag at the beginning is inserted to define whether the prediction is to be copied from co-located upsampled base layer texture. All the CUs in intra slices are treated as intra mode. This also implies that all deblocking decisions are taken according to being intra mode. Additionally, a skip mode for the largest CU size is introduced where no additional syntax elements are coded and the block is fully predicted using the upsampled base layer and the residual decoding process is omitted.

  - The behavior of deblocking filter will also be studied.

- Inter slices: For each inter PU, before the merge flag, a new flag is inserted to define whether the prediction is to be copied from co-located upsampled base layer texture. The behavior of intra CU’s are not modified. The CUs that contain PU’s that are predicted from the base layer are treated as inter mode. This also implies that all deblocking decisions for those CUs are taken according to being inter mode.

  - It should be noted that in a CU partitioned into two PUs, setting both of the PU’s to the base layer prediction would be redundant since the same prediction functionality can be achieved by having a single PU for the CU, which is predicted as base layer. Therefore, if the first PU is signaled to be base prediction, it is inferred that the base prediction flag is not to be signaled for the second PU.

  - The behavior of deblocking filter will also be studied.

When coding the base prediction flag with CABAC, contexts are assigned according to the base prediction flag values of the left and above neighbor blocks. Another property in this technology is that a new set of contexts is used for the cbf flag, if the prediction is signaled to be from the base layer.

### Proposal of Inter-layer Texture Prediction Signaling from Samsung (Samsung, K0045)

When the CU is coded in Intra_BL mode, no spatial intra directional prediction is performed, and only inter-layer pixel prediction is used. In this mode, the corresponding BL block of reconstructed pixels are scaled to the EL CU resolution and used as the predictor for the 2Nx2N PU of the CU. The resulting residual samples of the CU are coded following non-scalable HEVC's inter residual sample coding process with its prediction mode set to intra DC mode by default.

When the CU is coded in Intra_BL mode, due to the efficiency of the mode, the residual coefficients often get quantized to zero.  As a result, a special mode of Intra_BL – Intra_BL Skip mode – is introduced. In this case the CU is implied to have no significant coefficient.

## Reference Picture Approach

### Inter-layer reference picture placement (InterDigital, JCTVC-K0034)

The base layer reconstructed picture is up-sampled and inserted into the enhancement layer DPB for enhancement layer motion compensated prediction. The inter layer reference picture (ILRP) is added as an additional entry into the reference picture list(s) and referred to through the use of ref_idx. By default, the inter layer reference picture is placed at the end of the reference picture lists. Alternative inter-layer reference picture placement using single pass encoding will be evaluated in TE-A2.

### Inter-layer reference picture placement (ETRI, JCTVC-K0037)

A reconstructed picture of the reference layer is used as the reference picture for the corresponding picture of the enhancement layer. For the inter-layer texture prediction, the reconstructed picture of the reference layer, whose spatial resolution is the same as that of the enhancement layer, is added into the reference picture lists, L0 and L1, for the corresponding picture of the enhancement layer although it is not a temporal reference picture of the enhancement layer.

In detail, the process for reference picture list construction of the enhancement layer is as follows.

- An Inter-layer reference picture (ILRP) is generated by reconstructing and up-sampling (if necessary) the corresponding picture of the reference layer.

- The reference picture lists, L0 and L1, are constructed as signaled in slice header of the enhancement layer.

- For non-random access picture, the ILRP, that is assigned to the reference index of “1”, is added into the reference picture lists, L0 and L1 of the enhancement layer while reference index of reference picture whose reference index was originally greater than or equal to “1” is increased by 1 if there is any.

- For random access picture, the ILRP, that is assigned to the reference index of “0”, is added into the reference picture lists, L0 and L1 of the enhancement layer

### Infer zero motion vector when referencing co-located up-sampled base layer picture (Sharp, JCTVC-K0031)

This test will evaluate the performance of placing the co-located up-sampled base layer picture within the reference picture list. When referencing the co-located up-sampled base layer picture the motion vector is inferred to be equal to zero. The “zero motion vector” results in the merge and AMVP process altering the temporal candidate selection to use a replacement if the originally constructed temporal candidate references an up-sampled base layer picture:

Merge process modification

The temporal merge list candidate is constructed using a reference index and the temporal motion vector predictor. If the reference index of this candidate points to the up-sampled base layer picture then the candidate is modified to refer to an enhancement layer picture. The enhancement layer picture is co-located with the base layer reference picture of the co-located (base layer) PU. Below is a description of the process:

For both list 0 and list 1 the temporal merge list candidate is not allowed to point to the up-sampled base layer picture co-located with the current picture under consideration. If the temporal merge list candidate constructed using base layer construction process points to the up-sampled base layer picture co-located with the current picture under consideration then the reference index for the corresponding list is replaced. For a given reference picture list, the reference index is modified to point to the enhancement layer picture co-located with the base-layer reference picture. The base layer reference picture corresponds to the reference picture of the base layer picture co-located with the enhancement layer picture under consideration. The same reference picture list is considered for the base layer picture while determining the reference picture as was being considered for the enhancement layer picture.

AMVP modification

If the temporally co-located PU contains a reference index pointing to the up-sampled base layer picture then the motion vector component of this PU is undefined for motion vector prediction. An alternative motion vector is then defined by using the co-located base layer PU’s motion vector and used as the temporal motion vector predictor. Below is a description of the process:

If the temporal motion vector candidate constructed (using the base layer construction process) points to the base layer picture co-located with the temporal enhancement layer picture being considered then this candidate is replaced. The replacement motion vector candidate corresponds to the motion vector derived from the co-located PU of the base layer picture co-located with the current enhancement layer picture. The derived motion vector is scaled and used as the replacement motion vector candidate.

## Slice-level approach

### Enhancement layer skipped slice (InterDigital, JCTVC-K0034)

Enhancement layer skipped slice mode uses up-sampled base layer reconstructed picture directly as the enhancement layer reconstruction. The skip slice mode can sometimes offer better rate distortion performance. When such is the case, only inter layer processing parameters, used to generate the inter layer reference, need to be sent at picture (or slice) level, and no additional coding of the blocks in the enhancement layer picture is performed, which significantly reduces decoding complexity as well. EL skipped slice mode will be evaluated in TE-A2.

# Experimental Conditions

  - Test Sequences, Bit Rates and Coding Conditions

The TE will use the Class A and Class B sequences that were considered in the SHVC Call for Proposals.  All frames shall be processed, and the lower resolution representations of the sequences shall correspond to the same lower resolution representations as considered in the SHVC Call for Proposals.

The TE shall test the AI, RA and LD-P configuration for both 2x and 1.5x spatial scalability scenarios.  It is mandatory to report the performance of tested tools for these three configurations. Proponents shall use the prediction structures provided by the AhG on SHVC tool experiments (by November 16, 2012).

The TE shall use upsampling filters suggested in JCTVC-K0378 [1]. The upsampling will be provided via software to the TE.

The TE shall test combinations that baselayer QPs are 26 and 30, and the difference between the baselayer QP and enhancement QP are -2, 0, 2 and 4.

The TE shall use simulcast as a reference for BD-rate calculations.

  - Software

The software provided in JCTVC-K0348 shall be used in TE2. The software will be release by AhG on Nobember 2, 2012 with upsampling filters specified in JCTVC-K0378 integrated.

  - Evaluation of TE Results

Results of the TE will be evaluated on the basis of BD-measures and complexity. Complexity assessment is performed on the base of decoding complexity measurement. The TE report should include a description of the encoding process.

### Complexity assessment

Both HW and SW complexity of proposed tools for both encoder and decoder will be evaluated.

To measure software run time, software with the anchor configuration and the proposal implemented on the software shall be used. To measure decoding time, the decoding shall be executed on the same machine without output of decoded YUV data. The computational time must be measured for each test sequence and test case for both anchor and proposal. Relative computational time calculated against the anchor must be presented.

# Cross-checking

The package released for cross-checking shall include both software and test results. Software only without test results should not be considered as ready for cross-check release.

The following table provides the cross-checker assignment for test tools.

| Test | Document No. | Proponent | Cross-checker |
| --- | --- | --- | --- |
| 3.1.1 | JCTVC-K0035/0036 | Qualcomm | ETRI (jungwon@etri.re.kr) Huawei (JINWEN.ZAN@huawei.com) |
| 3.1.2 | JCTVC-K0032 | Sharp | Intel (Wenhao.zhang@intel.com) |
| 3.1.3 | JCTVC-K0046 | Huawei | KDDI(kei@kddilabs.jp) |
| 3.1.4 | JCTVC-K0046 | Huawei | InterDigital(Yong.He@InterDigital.com) |
| 3.1.5 | JCTVC-K0039 | Intel | Sharp (yamamoto.tomoyuki@sharp.co.jp) |
| 3.1.6 | JCTVC-K0041 | Canon | Sharp (misrak@sharplabs.com) |
| 3.1.7 | JCTVC-K0033 | MediaTek | KDDI(kei@kddilabs.jp) |
| 3.1.8 | JCTVC-K0040 | Nokia | LG(chulkeun.kim@lge.com) |
| 3.1.9 | JCTVC-K0045 | Samsung | Nokia (Kemal.Ugur@nokia.com) MediaTek (Peter.Chuang@mediatek.com) |
| 3.2.1 | JCTVC-K0034 | InterDigital | Qualcomm (liweig@qti.qualcomm.com) Samsung (elena_a.alshina@samsung.com) ETRI (jungwon@etri.re.kr) |
| 3.2.2 | JCTVC-K0037 | ETRI | Intel (Wenhao.zhang@intel.com) InterDigital(Yong.He@InterDigital.com) |
| 3.2.3 | JCTVC-K0031 | Sharp | TI (d-kwon@ti.com) Canon (edouard.francois@crf.canon.fr) |
| 3.3.1 | JCTVC-K0034 | InterDigital | Huawei (JINWEN.ZAN@huawei.com) |

# Time-line and Responsibilities

2012-Nov-2:	TE-A2 description finalized and uploaded.

2012-Nov-2:	Software distributed by AhG on SHVC Software

2012-Nov-16:	Configuration files and anchor performance distributed by AhG on SHVC tool experiments

2012-Dec-3:	Cross-verification begins, proponents provide software, draft of contribution text and results to cross-verifiers(s) and TE-A2 coordinators.

2012-Jan--7:	Cross-verifiers report results to CE participants (5 weeks from cross-check start)

2012-Jan-7:	Contribution documents uploaded, the document number and title are reported to coordinators.

2012-Jan-14:	Summary report uploaded

# References

- ”Suggested up-sampling filter design for tool experiments on HEVC scalable extension”, E. Alshina, H. Lakshman, J.Dong, J.Chen, A.Luthra, JCTVC-K0378, Oct. 2012, Shanghai, China.