| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K1103 |
| --- | --- |

| Title: | Description of Tool Experiment B3: Combined Prediction in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Output Document to JCT-VC |  |  |
| Purpose: | TE Description |  |  |
| Author(s) or Contact(s): | X. Li E. Francois P. Lai D. Kwon A. Saxena | Email: | lxiang@qti.qualcomm.com edouard.francois@crf.canon.fr Polin.Lai@mediatek.com d-kwon@ti.com asaxena@sta.samsung.com |
| Source: | TE coordinators |  |  |

_____________________________

# Abstract

This document is a description of Tool Experiment 3 on Combined Prediction in SHVC.

# Introduction

This document defines Tool Experiment 3 (TE3) on combined prediction in SHVC, to be performed for the 12th JCT-VC meeting. The TE consists of proposals which use combined base layer and enhancement layer information to predict enhancement layer.

# Participants

| Company | Name & Email |
| --- | --- |
| Qualcomm | X. Li (lxiang@qualcomm.com), V. Seregin (vseregin@qti.qualcomm.com), K. Rapaka (krapaka@qti.qualcomm.com), J. Chen (cjianle@qti.qualcomm.com) |
| Canon | F. Le Leannec (fabrice.leleannec@crf.canon.fr), E. Francois (edouard.francois@crf.canon.fr) |
| MediaTek | P. Lai (Polin.Lai@mediatek.com), T.-D. Chuang (peter.chuang@mediatek.com), Y.-W. Huang (yuwen.huang@mediatek.com), S. Liu (shan.liu@mediatek.com), S.-T. Hsiang (shih-ta.hsiang@mediatek.com), X. Zhang (Ximin.zhang@mediatek.com), Z. Chen (Zhenzhong.chen@mediatek.com) |
| TI | D. Kwon (d-kwon@ti.com) M. Budagavi (madhukar@ti.com) |
| Samsung | A. Saxena (asaxena@sta.samsung.com), E. Alshina (elena_a.alshina@samsung.com), T. Lee (tammy.lee@samsung.com); C. Kim (dionism@samsung.com), A.Alshin (alexander_b.alshin@samsung.com) |
| Sharp | T. Yamamoto (yamamoto.tomoyuki@sharp.co.jp) |
| Intel | Y.-J. Chiu (yi-jen.chiu@intel.com), W. Zhang (wenhao.zhang@intel.com), X. Cai (xiaoxia.cai@intel.com) |
| Nokia | J.Lainema (Jani.Lainema@nokia.com) |
| LG | J. Park (jy.park@lge.com) |
| Vidyo | W. Jang (wonkapjang@gmail.com) |

Assignment of crosschecking

| Section | Technique | Proponent | Crosschecker |
| --- | --- | --- | --- |
| 4.1.1 | JCTVC-K0032 | T. Yamamoto (yamamoto.tomoyuki@sharp.co.jp) |  |
| 4.1.2 | JCTVC-K0040 | J.Lainema (Jani.Lainema@nokia.com) | K. Rapaka (krapaka@qti.qualcomm.com) X. Li (lxiang@qti.qualcomm.com) |
| 4.1.3 | JCTVC-K0033 | J. Park (jy.park@lge.com) P. Lai (Polin.Lai@mediatek.com) | T. Yamamoto (yamamoto.tomoyuki@sharp.co.jp) |
| 4.2.1 | JCTVC-K0040 | J.Lainema (Jani.Lainema@nokia.com) | T.-D. Chuang (peter.chuang@mediatek.com) |
| 4.2.2 | JCTVC-K0036 | K. Rapaka (krapaka@qti.qualcomm.com) | E.Alshina (elena_a.alshina@samsung.com) J. De Cock (jan.decock@ugent.be) |
| 4.2.3 | JCTVC-K0033 | J. Park (jy.park@lge.com) P. Lai (Polin.Lai@mediatek.com) | K. Rapaka (krapaka@qti.qualcomm.com) |
| 4.2.4 | JCTVC-K0044 | W. Jang (wonkapjang@gmail.com) E. Alshina (elena_a.alshina@samsung.com) |  |
| 4.3.1 | JCTVC-K0033 | J. Park (jy.park@lge.com) P. Lai (Polin.Lai@mediatek.com) | D.-K. Kwon (d-kwon@ti.com) M. Budagavi (madhukar@ti.com ) |
| 4.4.1 | JCTVC-K0033 | J. Park (jy.park@lge.com) P. Lai (Polin.Lai@mediatek.com) | X. Li (lxiang@qti.qualcomm.com) |
| 4.4.2 | JCTVC-K0044 | W. Jang (wonkapjang@gmail.com) E. Alshina (elena_a.alshina@samsung.com) |  |
| 4.5.1 | JCTVC-K0039 | W. Zhang (wenhao.zhang@intel.com) | T.-D. Chuang (peter.chuang@mediatek.com) |
| 4.6.1 | JCTVC-K0041 | F. Le Leannec (fabrice.leleannec@crf.canon.fr) | A.Alshin (alexander_b.alshin@samsung.com) |
| 4.6.2 | JCTVC-K0036 | X. Li (lxiang@qti.qualcomm.com), V. Seregin (vseregin@qti.qualcomm.com) | J. W. Kang (jungwon@etri.re.kr) C. Kim (chulkeun.kim@lge.com) |
| 4.6.3 | JCTVC-K0040 | J.Lainema (Jani.Lainema@nokia.com) | X. Li (lixiang@qti.qualcomm.com) E.Alshina (elena_a.alshina@samsung.com) |

# Experimental Conditions

  - Test Sequences, Bit Rates and Coding Conditions

The test conditions for this TE are based on [1]. The TE will use the Class A and Class B sequences that were considered in the SHVC Call for Proposals. All frames shall be processed, and the lower resolution representations of the sequences shall correspond to the same lower resolution representations as considered in the SHVC Call for Proposals.

The TE shall test following cases.

|  | AI | RA | LD-P |
| --- | --- | --- | --- |
| Scalability | 2x, 1.5x | 2x, 1.5x, and SNR | 2x, 1.5x, and SNR |

It is mandatory to report the performance of tested tools for these three configurations if the tool affects both intra and inter. For intra only coding tools, RA and LD-P configurations are not mandatory. For inter only coding tools, AI tests are not required. Proponents shall use the prediction structures provided by the AhG on SHVC tool experiments (by November 16, 2012).

The TE shall test points where baselayer QP is 26 and 30 for spatial cases and 30, 34 for SNR cases, and the difference between the base layer QP and enhancement QP is either -2, 0, 2 or 4 for spatial cases and -2, -4, -6, -8 for SNR cases. Further details should be available once AhG on SHVC tool experiments releases configuration files.

The TE shall consider the software provided to the TE as the anchor.

  - Software

Software based on JCTVC-K0348 and available on November 2, 2012.

  - Evaluation of TE Results

Results of the TE will be evaluated on the basis of BD-measures, and complexity. Complexity assessment is performed on the base of encoding and decoding complexity measurement.

### Complexity assessment

Both HW and SW complexity of proposed tools for both encoder and decoder will be evaluated.

To measure software run time, software with the anchor configuration and the proposal implemented on the software shall be used. To measure decoding time, the decoding shall be executed on the same machine without output of decoded YUV data. The computational time must be measured for each test sequence and test case for both anchor and proposal. Relative computational time calculated against the anchor must be presented.

# Tools to test

## Intra prediction based on reconstructed base layer

### JCTVC-K0032 [2](Sharp)

Hybrid intra and inter-layer prediction (HIILP) is a prediction mode where the prediction image created by either Planar or Angular mode is modified based on the average pixel values of the reference region on base layer reference picture. The modification is done with a unit such as 4x4.

#### Inter-layer prediction

Only reconstructed pixel data is used for inter-layer prediction. Inter-layer texture prediction (ILTP) and Hybrid intra and inter-layer prediction (HIILP) are introduced. In intra CU, ILTP and HIILP can be selected in addition to original intra modes (i.e. DC, Planar, and Angular modes). A new flag, inter_layer_pred_flag, is added to intra CU syntax. Signaling of prediction modes in intra CU is summarized in Table 1. Whereas IntraPredMode is signaled per prediction block, inter_layer_pred_flag is signaled per CU. A flag, inter_layer_pred_flag, is encoded using CABAC with one single context.

Table 1 Signaling of prediction modes in intra CU

| inter_layer_pred_flag | IntraPredMode | Associated prediction mode |
| --- | --- | --- |
| 0 | 0 | Planar |
| 0 | 1 | DC |
| 0 | 2..34 | Angular |
| 1 | 1 | ILTP |
| 1 | 0, 2..34 | HIILP |

##### Inter-layer texture prediction (ILTP)

Inter-layer texture prediction (ILTP) is a prediction mode where base layer reference picture is directly used as prediction image. The pixel values of the reference region on base layer reference picture may be filtered with Adaptive Filter. The reference region is collocated to the target prediction block thus additional information, such as the displacement vector, is not needed.

##### Hybrid intra and inter-layer prediction (HIILP)

Hybrid intra and inter-layer prediction (HIILP) is a prediction mode where the prediction image created by either Planar or Angular mode is modified based on the average pixel values of the reference region on base layer reference picture. The modification is done by 4x4 unit basis. Conceptually, the original prediction image is modified so that the DC component of each 4x4 unit is replaced by the DC component of the corresponding 4x4 unit on base layer reference picture. Specifically, the prediction image of HIILP mode, P(x,y), is derived by the following equation.

P(x,y) = Clip3(0, 255, Po(x,y) - Avg4x4(Po, x,y) + Avg4x4(R, x,y))

Where Po (x,y) is pixel values of prediction image by either Planar or Angular modes. The mode used to derive Po (x,y) is determined based on IntraPredMode value. Avg4x4(Po, x,y) is the average pixel value of 4x4 region on prediction image Po. The top-left pixel position of the 4x4 region is ((x>>2)<<2, (y>>2)<<2). Similarly, Avg4x4(R, x, y) is the average pixel value of 4x4 region on base layer reference picture R.

### JCTVC-K0040 [3](Nokia)

DC adjustment is applied for intra prediction blocks of size 8x8 and larger in order to adjust the enhancement layer intra predicted sample values in a way that the block’s DC is made to match that of the corresponding base layer block. The process can be represented as follows:

P(x,y) = P’(x,y) + dc_delta

Where:

P(x,y) denotes the sample to be predicted in the enhancement layer picture.

P’(x,y) denotes a predicted enhancement layer sample after HEVC (version 1) intra prediction process

dc_delta denotes the difference of base layer prediction block’s DC and P’(x,y) block’s DC component.

### JCTVC-K0033 [5] (LG/MTK)

If there is any unavailable reference sample during intra prediction at enhancement layer (EL), the corresponding sample at base layer (BL) will replace it. For example, in Figure 1, left-bottom reference samples are unavailable and then are replaced with corresponding samples at BL.

Figure 1 Filling unavailable reference samples in intra prediction at enhancement layer

## Intra prediction based on differential picture

### JCTVC-K0040 [3] (Nokia)

Gradient based enhancement of intra prediction utilizes the base layer sample information to refine the directly horizontal and vertical intra prediction modes. There is no additional syntax or signalling associated with the proposal. Instead, the intra prediction process at enhancement layer is modified as follows for the mentioned intra prediction directions:

P(x,y) = PR(xR,yR) + B(x,y) - BR(xR,yR)

Where:

P(x,y) denotes the sample to be predicted in the enhancement layer picture.

PR(xR,yR) denotes the enhancement layer reference sample in the enhancement layer picture based on the directionality of the intra prediction mode

B(x,y) in the base layer picture with location corresponding to the enhancement layer sample P(x,y)

BR(xR,yR) denotes the base layer reference sample in the base layer picture based on the directionality of the intra prediction mode

Additional test measures performance of combining gradient based enhancement of intra prediction (JCTVC-K0040) and DC adjustment for intra prediction blocks (JCTVC-K0040). Gradient based enhancement is applied for directly horizontal and vertical intra prediction modes, DC adjustment for the rest of the intra predicted blocks.

### JCTVC-K0036 [4] (Qualcomm)

In the intra residual prediction mode, as shown inFigure 2, the difference between the pixels of current neighbors and that of collocated BL neighbors are used to generate a difference prediction based on the intra prediction mode. The generated difference prediction signal is added to the collocated BL block signal to form the final prediction.

The intra prediction method for difference signal remains unchanged with respect to HEVC except for the planar mode. After intra prediction is performed to get the difference prediction signal, the bottom-right portion of the difference prediction for the planar mode is set to zero (that is for each pixel(x, y) satisfying the condition (x + y) >= N-1, where N is the width of the current block).

The intra residual prediction mode is indicated by a flag intra_resi_pred_flag at CU level.

Due to the high frequency nature of the difference signals, HEVC mode dependent intra smoothing process is disabled in intra residual prediction mode.

Figure 2 Intra residual prediction

In intra residual prediction, the MPM candidate list derivation process of HEVC is modified. The MPM derivation process is refined in a way that the horizontal or vertical mode has higher priority than the DC mode. Table 2 illustrates only the refinements in MPM list derivation process.

Table 2 MPM list refinement for intra residual prediction

|  | candModeList[0] | candModeList[1] | candModeList[2] |
| --- | --- | --- | --- |
| (LeftIntraDir == AboveIntraDir) && (LeftIntraDir == Not Angular) | Planar | Vertical | Horizontal |
| LeftIntraDir != AboveIntraDir | Angular | Planar | If Mode[0] == Vertical DirSet then Horizontal else Vertical |
|  | Planar | Angular | If Mode[1] == Vertical DirSet then Horizontal else Vertical |

### JCTVC-K0033 [5] (LG/MediaTek)

When the current CU is intra-coded with differential coding mode, we need to obtain the reference sample values in the differential domain. If the block which the reference samples belong to is also coded with differential coding mode, the reference sample values can be obtained directly from the differential domain. Otherwise, “up-sampled reconstructed sample values of the BL with in-loop filtering” and “reconstructed sample values without in-loop filtering” are used for obtaining differential reference sample values.

In the differential domain, samples are coded in the same way as in the normal domain except the sample clipping process. The sample values in the differential domain are clipped to reduce the amount of memory usage. For example, when we code 8-bit samples without internal bit depth increase, the differential sample values are clipped from [-255, 255] into [-128, 127].

### JCTVC-K0044/JCTVC-K0045 [6] (Vidyo/Samsung)

In the intra difference coding mode, difference samples for the samples of the neighboring blocks are derived by subtracting the co-located base layer samples from the reconstructed enhancement layer samples. The intra prediction is then done using the difference samples. The final prediction signal for a block is obtained by adding the (upsampled) reconstructed base layer signal to the difference prediction signal.

The same intra prediction modes as in conventional intra prediction are used. The difference mode is signaled at CU level.

The difference intra prediction is illustrated in Figure 3.

Figure 3 Illustration of intra difference coding mode

## Inter prediction based on reconstructed base layer

### JCTVC-K0033 [5] (MediaTek/LG)

In this test, base layer (BL) reconstructed textures are utilized to refine enhancement layer (EL) motion compensated predictors. For the EL blocks, the difference between the BL textures and the EL motion compensated predictors are used to derive adaptive offsets. The adaptive offsets are added on the EL motion compensated predictors conditionally. This test also includes simplification of the adaptive offsets derivation process.

## Inter prediction based on differential picture

### JCTVC-K0033 [5] (MediaTek/LG)

In the differential domain, samples are coded in the same way as in the normal domain except the sample clipping process. The sample values in the differential domain are clipped to reduce the amount of memory usage. For example, when we code 8-bit samples without internal bit depth increase, the differential sample values are clipped from [-255, 255] into [-128, 127].

### JCTVC-K0044/JCTVC-K0045 [6] (Vidyo/Samsung)

In the inter difference coding mode, motion compensation is not done using reconstructed reference pictures, but using difference reference pictures. A difference reference picture represents the difference of the reconstructed picture and the (upsampled) base layer for the corresponding picture.

The motion vector coding is not modified for the difference mode, but sub-sample positions are interpolated using a bi-linear filter instead of the 8-tap filter for luma and the 4-tap filter for chroma (which are used for standard inter modes).

The difference mode is signaled at CU level.

The difference inter prediction is illustrated in Figure 7.

Figure 4 Illustration of inter difference coding mode

## SVC style residual prediction

### JCTVC-K0039 [7] (Intel)

The residual value of BL can be used to refine the residual of the intra/inter prediction for coding the video in EL. The magnitude of predicting error of EL can be can be reduced by adding the residual in BL to the prediction signal in EL. With the inter-layer residual refinement at EL encoder, the residual refined by the residual of BL will be transformed, quantized and entropy coded into bitstream. The integration of inter-layer residual refinement is dependent on the coding mode of the CU. When the CU in EL is coded by using intra modes, this contribution applies inter-layer residual refinement on CU level with a flag, denoted as inter_layer_residual_refine_for_intra_flag, to be added in syntax table to indciate if current intra CU uses inter-layer residual refinement or not. Figure 5 provides the flowchart of intra CU decoding process with inter-layer residual refinement to be enabled in a SVC structure.

Figure 5 Flowchart of intra CU decoding process with inter-layer residual refinement

On the other hand, if the CU in EL is coded by using inter modes, this contribution applies inter-layer residual refinement on PU level with a flag, denoted as inter_layer_residual_refine_for_inter_flag, to be added in syntax table to indciate if current inter PU uses inter-layer residual refinement or not. Figure 6 provides the flowchart of inter PU decoding process with inter-layer residual refinement to be enabled in a SVC structure.

Figure 6 Flowchart of inter PU decoding process with inter-layer residual refinement

The availability of inter-layer residual refinement is conditional to the coding modes of the collocated BL blocks. When current EL PU is inter coded, this contribution only applies inter-layer residual refinement when the collocated blocks in BL are all coded in inter coding mode. When current EL PU is intra coded, this contribution only applies inter-layer residual refinement when the collocated blocks in BL are all coded in intra coding mode. If the collocated blocks in BL are mixed by inter and intra coded, the inter-layer residual refinement is not applied to the EL PU.

## Generalized residual prediction

### JCTVC-K0041 [8] (Canon)

Generalized Residual Inter-Layer Prediction (GRILP) mode consists in generating, given a Prediction Unit and motion vector MV being considered, a corresponding temporal residual CU in the base layer, that is added to enhancement layer temporal prediction PU.

The GRILP prediction mode is selected on a CU basis. It can be activated inside Inter CUs and is signaled by a CU flag. When activated, it is used for all PUs of the CU. The GRILP prediction mode proceeds as follows.

The prediction signal is derived as follows (shown for the mono-prediction case, the concept is easily generalized for the bi-prediction case):

PRED = MC[REFEL, MVEL] + UP[RECBL] - MC[UP[REFBL], MVEL]

where PRED is the prediction signal, REFEL and REFBL are the reference pictures in the Enhancement and Base layers, RECBL is the base layer reconstructed picture corresponding to the current enhancement layer picture, UP[.] is the upsampling operator, MC[.,MV] is the motion compensation using the motion vector MV.

In JCTVC-K0041, the interpolation filters for MC are those of the HEVC core design. The upsampling interpolation filters are those defined in the TE 2.

Signaling at CU or PU basis will be studied.

### JCTVC-K0036 [4] (Qualcomm)

#### Generalized residual prediction

In this technique, inter predicted residue of the collocated BL block is used to predict that of the current block. This method is applied to inter CUs, and skip mode CUs. The framework of this method is shown in Figure 7 where the case of uni-prediction is illustrated.

Let Be and Bb denote the current block in the EL picture and its collocated BL block, respectively. Let Pe0 denote the temporal prediction for the block Be obtained by using the motion vector MVe0 (where sub-index 0 refers to reference list 0). Similarly, Pb0 represents the temporal prediction for the block Bb obtained by using the same motion vector MVe0 in the BL reference picture.

Then, the inter predicted residue of the BL block  is obtained as

Considering the temporal prediction Pe0 for block Be, the final uni-prediction P for block Be is

,

where w is a the weighting factor, which takes the values 0, 0.5, or 1.

When extending this method to the bi-predictional case, the final prediction P for Be is formed as

where sub-index 1 indicates reference list 1.

The weighting factor w is signaled at CU level as a weighting index. The weighting index 0, 1 and 2 are used to indicate the weighting factors 0, 0.5 and 1, respectively. A truncated unary code in regular CABAC mode is used for the signaling.

Figure 7 Inter residual prediction for uni-prediction

In addition, different filters [4] may be employed to generate upsampled base layer block when generalized residual prediction is used.

#### Combined mode

An inter combined mode can be further applied on top of GRP. The combined prediction mode is applied to both intra and inter predicted CUs. The prediction in this mode is defined as an average of two predictions which are done in intra-BL mode and enhancement layer intra or inter prediction mode. Let Pb denote intra-BL prediction and Pe denote enhancement layer inter/intra prediction. The final prediction P for the CU is defined as

In addition to the combined_mode_flag which indicates the usage of the mode at CU level, the intra/inter prediction information are also signaled to decoder. This method is applied only to the luma component. The enhancement layer intra/inter prediction is used for chroma components when the combined prediction mode is enabled.

The combination of combined mode and generalized residual prediction will also be studied

### JCTVC-K0040 [3] (Nokia)

Base-layer enhanced motion compensation (BEMCP) can be classified to belong to the category of differential inter coding approaches. Some variants are also known as generalized residual prediction. The technology to be tested in this TE can be characterized as follows:

- PU level signaling of the weight of the base layer component to be added to the enhancement layer prediction (0, 0.5 or 1).

- Performing a base layer motion compensation with the enhancement layer motion parameters and adding the difference between base layer reconstruction and the motion compensation result to the enhancement layer prediction signal (weighting it as indicated above)

As a result the enhancement layer motion compensated prediction is defined as follows:

P0(x,y) = P’(x,y), for weight = 0

P1(x,y) = Clip ( P’(x,y) + B(x,y) - B’(x,y) ), for weight = 1

P2(x,y) = Clip ( P’(x,y) + ( ( B(x,y) - B’(x,y) ) >> 1 ) ), for weight = 0.5

where P’(x,y) corresponds a predicted pixel from enhancement layer motion compensation, B’(x,y) a predicted pixel utilizing enhancement layer motion information but upsampled base layer reference frames and B(x,y) the upsampled reconstructed base layer block co-locating with the enhancement layer block.

# Time-line and Responsibilities

2012-Nov-2:	TE-3 description finalized and uploaded.

2012-Nov-2:	Software distributed by AhG on SHVC Software

2012-Nov-16:	Configuration files and anchor performance distributed by AhG on SHVC tool experiments

2012-Dec-10:	Cross-verification begins, proponents provide software, draft of contribution text and results to cross-verifiers(s) and TE coordinators.

2013-Jan-7:	Cross-verifiers report results to TE participants (2 weeks from cross-check start)

2013-Jan-7:	Contribution documents uploaded, the document number and title are reported to coordinators.

2013-Jan-14:	Summary report uploaded

# References

- A. Segall, “BoG report on SHVC”, JCTVC-K0354, Shanghai, China, Oct 2012.

- T. Yamamoto, Y. Yasugi, H. Kumai, M. Takahashi, “Description of scalable video coding technology proposal by Sharp (proposal 2)”, JCTVC-K0032, Shanghai, China, Oct 2012.

- O. Bici, D. Bugdayci, A. Aminlou, A. Hallapuro, M. Hannuksela, J. Lainema, K. Ugur, “Description of scalable video coding technology proposal by Nokia (encoder configuration 1)”, JCTVC-K0040, Shanghai, China, Oct 2012.

- J. Chen, K. Rapaka, X. Li, V. Seregin, L. Guo, M. Karczewicz, G. Van der Auwera, J. Sole, X. Wang, C. J. Tu, Y. Chen, “Description of scalable video coding technology proposal by Qualcomm (configuration 2)”, JCTVC-K0036, Shanghai, China, Oct 2012.

- C. Kim, J. Park, J. Kim, Hendry, S. Park, Y. Jeon, J. Lim, N. Park, B. Jeon, S. Liu, X. Zhang, M. Guo, Z. Chen, T.-D. Chuang, S.-T. Hsiang, C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, C.W. Hsu, Y.-W. Huang, S. Lei, “Description of scalable video coding technology proposal by LG Electronics and MediaTek (differential coding mode on)”, JCTVC-K0033, Shanghai, China, Oct 2012.

- K. McCann, J. H. Park, J. Kim, C. Kim, J.-H. Min, E. Alshina, A. Alshin, I.-K. Kim, T. Lee, B. Choi, Y. Piao, S. Jeong, S. Lee, Y Cho, J. Y. Choi, F. C. A. Fernandes, Z. Ma (Samsung), J. Boyce, D. Hong, W. Jang, A. Abbas, S. Reddy (Vidyo), “Description of high efficiency scalable video coding technology proposal by Samsung and Vidyo”, JCTVC-K0044, Shanghai, China, Oct 2012.

- Y. Chiu, W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, H. Jiang, “Description of scalable video coding technology proposal by Intel”, JCTVC-K0039, Shanghai, China, Oct 2012.

- S. Lasserre, F. Le Léannec, E. Nassor, J. Taquet, N. Ouedraogo, S. Pautet, C. Gisquet, G. Laroche, T. Poirier, Y. Verdavaine, E. François, “Description of the scalable video coding technology proposal by Canon Research Centre France”, JCTVC-K0041, Shanghai, China, Oct 2012.