| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K1104 |
| --- | --- |

| Title: | Description of Tool Experiment B4: Inter-layer filtering |  |  |
| --- | --- | --- | --- |
| Status: | Output document of JCT-VC |  |  |
| Purpose: | TE description |  |  |
| Author(s) or Contact(s): | Jianle Chen (Chair) Andrew Segall Elena Alshina Shan Liu Jie Dong Joonyoung Park | Email: | cjianle@qti.qualcomm.com asegall@sharplabs.com elena_a.alshina@samsung.com shan.liu@mediatek.com jie.dong@interdigital.com jy.park@lge.com |
| Source: | TE coordinators |  |  |

# Abstract

This document provides the description of Tool Experiment B4. The topic of the Tool Experiment is testing Inter-layer filters for SHVC.

# Introduction

During the 11th JCT-VC meeting in Shanghai, multiple proposals for Inter-layer and/or adaptive filters for the SHVC design were proposed.  It was then decided to coordinate for further study and testing of these filters in a tool experiment.

# Participants

| Nr. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Krishna Rapaka Liwei Guo Wei Pu Jianle Chen | Qualcomm | krapaka@qti.qualcomm.com liweig@qti.qualcomm.com wpu@qti.qualcomm.com cjianle@qti.qualcomm.com | P, C |
| 2 | Andrew Segall Tomoyuki Yamamoto Kiran Misra Zhao, Jie | Sharp | asegall@sharplabs.com yamamoto.tomoyuki@sharp.co.jp misrak@sharplabs.com jzhao@sharplabs.com | P, C |
| 3 | Elena Alshina Ankur Saxena | Samsung | elena.alshina@gmail.com elena_a.alshina@samsung.com asaxena@sta.samsung.com | P, C |
| 4 | Shang Liu Mei Guo Ximin Zhang Polin Lai Shih-Ta Hsiang Tzu-Der (Peter) Chuang Yu-Wen Huang CW Hsu ChingYeh Chen Chiayang Tsai Zhenzhong Chen | MediaTek | shan.liu@mediatek.com mei.guo@mediatek.com ximin.zhang@mediatek.com polin.lai@mediatek.com shih-ta.hsiang@mediatek.com peter.chuang@mediatek.com yuwen.huang@mediatek.com cw.hsu@mediatek.com chingyeh.chen@mediatek.com chiayang.tsai@mediatek.com zhenzhong.chen@mediatek.com | P, C |
| 5 | Jie Dong Yuwen He Yan Ye | Interdigital | jie.dong@interdigital.com yuwen.he@interdigital.com yan.ye@interdigital.com | P, C |
| 6 | Le Leannec Fabrice Francois Edouard Onno Patrice | Canon | fabrice.leleannec@crf.canon.fr edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr | P, C |
| 7 | Madhukar Budagavi Do-Kyoung Kwon | TI | madhukar@ti.com d-kwon@ti.com |  |
| 8 | Chulkeun Kim Joonyoung Park | LGE | chulkeun.kim@lge.com jy.park@lge.com | P, C |
| 9 | Yi-jen Chiu Wenhao Zhang | Intel | yi-jen.chiu@intel.com wenhao.zhang@intel.com | P, C |
| 10 | Jinwen Zan Haitao Yang Haoping Yu | Huawei | jinwen.zan@huawei.com haitao.yang@huawei.com haoping.yu@huawei.com |  |
| 11 | Lu, Shuo Ikeda, Masaru Auyeung, Cheung | Sony | shuo.lu@jp.sony.com Masaru.Ikeda@am.sony.com Cheung.Auyeung@am.sony.com |  |
| 12 | Tan Yih Han | I2R | yhtan@i2r.a-star.edu.sg |  |
| 13 | Jan De Cock Sebastiaan Vanleuven Glenn Van Wallendael | Ugent | jan.decock@ugent.be sebastiaan.vanleuven@ugent.be Glenn.VanWallendael@elis.UGent.be |  |

# Experimental Conditions

  - Test Sequences, Bit Rates and Coding Conditions

The test conditions for this TE are determined based on the decision in [1]. The TE will use the Class A and Class B sequences that were considered in the SHVC Call for Proposals.  All frames shall be processed, and the lower resolution representations of the sequences shall correspond to the same lower resolution representations as considered in the SHVC Call for Proposals.

The TE shall test the AI, RA and LD-P configurations for both 2x and 1.5x spatial scalability scenarios and RA and LD-P configurations for SNR scalability scenario. It is mandatory to report the performance of tested tools for these configurations. Proponents shall use the prediction structures provided by the AhG on SHVC tool experiments (by November 16, 2012).

For 2x and 1.5x spatial scalability case, the TE shall test combinations where a baselayer QP is either 26 or 30, and the difference between the baselayer QP and enhancement QP is either -2, 0, 2 or 4.

For SNR scalability case, the TE shall test combinations where a baselayer QP is either 30 or 34, and the difference between the baselayer QP and enhancement QP is either -2, -4, -6 or -8.

The TE shall consider the software provided to the TE as the anchor, wherein upsampling filter from TE2 and IntraBL signaling method of TE1 shall be used.

Optionally, the TE can use ref_idx signaling for indicating the use of upsampled baselayer data. The ref_idx signaling will be provided via software to the TE.

  - Software

Software provided in JCTVC-K0348 and available on November 2, 2012.

  - Evaluation of TE Results

Results of the TE will be evaluated on the basis of BD-measures, complexity. Complexity assessment is performed on the base of encoding/decoding complexity measurement.

Both HW and SW complexity of proposed tools for both encoder and decoder will be evaluated.

To measure software run time, software with the anchor configuration and the proposal implemented on the software shall be used. To measure decoding time, the decoding shall be executed on the same machine without output of decoded YUV data. The computational time must be measured for each test sequence and test case for both anchor and proposal. Relative computational time calculated against the anchor must be presented.

# Tools to test

The tools in this TE are classified into five categories:

- TE4.1 Switchable up-sampling filters

- TE4.2 Adaptive up-sampling filter

- TE4.3 Interlayer SAO

- TE4.4 Interlayer ALF

- TE4.5 Interlayer de-blocking

Summary of the testing tools is provided in the following table.

| Section | Technique | Proponent | Crosschecker |
| --- | --- | --- | --- |
| 4.1.1 | JCTVC-K0034 | InterDigital (jie.dong@interdigital.com) | Qualcomm (krapaka@qti.qualcomm.com) |
| 4.1.2 | JCTVC-K0034 | InterDigital (jie.dong@interdigital.com) | Samsung (elena_a.alshina@samsung.com) |
| 4.2.1 | JCTVC-K0035 | Qualcomm (cjianle@qti.qualcomm.com) | Samsung (elena_a.alshina@samsung.com) |
| 4.2.2 | JCTVC-K0031 | Sharp (misrak@sharplabs.com) | LGE (chulkeun.kim@lge.com) |
| 4.3.1 | JCTVC-K0033 | MediaTek (ximin.zhang@mediatek.com) | Intel (wenhao.zhang@intel.com) |
| 4.3.2 | JCTVC-K0044 | Samsung (elena_a.alshina@samsung.com) | Qualcomm (krapaka@qti.qualcomm.com) |
| 4.4.1 | JCTVC-K0032 | Sharp (yamamoto.tomoyuki@sharp.co.jp) | InterDigital (jie.dong@interdigital.com) |
| 4.4.2 | JCTVC-K0033 | MediaTek (ximin.zhang@mediatek.com) | Intel (wenhao.zhang@intel.com) |
| 4.4.3 | JCTVC-K0039 | Intel (wenhao.zhang@intel.com) | MediaTek (ximin.zhang@mediatek.com) |
| 4.5.1 | JCTVC-K0033 | LGE (chulkeun.kim@lge.com) | Samsung (elena_a.alshina@samsung.com) |

## Sub-test1: switchable up-sampling filters

### Selectable up-sampling filters from K0034 [2]

Four selectable upsampling filters for each of the 1.5X and 2X cases will be evaluated in TEB4. These are used to provide a wider range of filter frequency response characteristics. They have the following characteristics:

- They are 2-D non-separable.

- Each SUF consists of a set of phase filters, and each phase filter has sizes 7×7, 7×6, 6×7, or 6×6, depending on to which phase the filter applies.

- All the filter coefficients can be represented by 8-bit integers.

Figure 1 and Figure 2 show the impulse responses of the four SUFs for 1.5X and 2X upsampling, respectively. SUF filters are applied to luma samples only. One of the SUF filters can be selected at the picture level.

Figure 1 Impulse responses of the four SUFs for 2X upsampling

Figure 2 Impulse responses of the four SUFs for 1.5X upsampling

### Intra mode dependent directional filters (IMDDF) from K0034 [2]

Figure 3 shows all the intra prediction modes supported in HEVC. For intra coded blocks, their prediction modes are highly correlated to the direction of the textures/edges in the block. In the case of scalable coding, when the corresponding base layer blocks are intra coded, the directionality information provided by their prediction modes can be used to improve the inter layer prediction process. IMDDF is specifically designed to take advantage of this “free” information offered by the base layer to improve enhancement layer coding.

Figure 3 Intra prediction modes in HEVC

As show in Figure 4, there are 4 and 9 relative phases for 2x and 1.5x ratios, respectively. IMDDF consists of a set of poly-phase filters that correspond to each prediction mode. As examples, Figure 5 and Figure 6 show the impulse responses of the IMDDF filters for the horizontal, vertical, and diagonal modes for 2x and for 1.5x, respectively. IMDDF filter sizes for 2x ratio and for 1.5x ratio are given in Table 1 and Table 2, respectively.

- 2x ratio 					(b) 1.5x ratio

Figure 4 Upsampling filter phases for luma samples

- 2x IMDDF for horizontal mode (mode 10)     (b) 2x IMDDF for vertical (mode 26)

(c)  2x IMDDF 135⁰ diagonal (mode 2)           (d) 2x IMDDF: 45⁰ diagonal (mode 18)

Figure 4 Examples of IMDDF impulse responses for 2x scaling ratio

- 1.5x IMDDF for horizontal mode (mode 10)    (b) 1.5x IMDDF for vertical (mode 26)

(c)  1.5x IMDDF 135⁰ diagonal (mode 2)        (d) 1.5x IMDDF: 45⁰ diagonal (mode 18)

Figure 5 Examples of IMDDF impulse responses for 1.5x scaling ratio

Table 1 IMDDF filter size for 2x ratio

| Filter phase | Filter size (H×V) |
| --- | --- |
| 0 | 7×7 |
| 1 | 8×7 |
| 2 | 7×8 |
| 3 | 8×8 |

Table 2 IMDDF filter size for 1.5x ratio

| Filter phase | Filter size (H×V) |
| --- | --- |
| 0 | 7×7 |
| 1 | 8×7 |
| 2 | 8×7 |
| 3 | 7×8 |
| 4 | 8×8 |
| 5 | 8×8 |
| 6 | 7×8 |
| 7 | 8×8 |
| 8 | 8×8 |

Currently, IMDDF is only applied when the corresponding base layer picture is intra coded, and is only applied to the luma samples. In TE-B4, we will also study the simplifications of the filters, for example, reducing the number of filters and/or reducing filter sizes.

## Sub-test2: Adaptive up-sampling filter

### Adaptive up-sampling filters from K0035 [3]

A 2D separable adaptive filter used as up-sampling filter will be tested in this test. The filtering process is also applied to the integer sample positions. As a result, filtering process is used in both SNR and spatial scalability cases.

In this test, the filter set for intra-BL prediction can be switched between the adaptive filter set and the fixed filter set. A flag is signaled to the decoder at picture level to indicate the choice of the filter. The filter coefficients are quantized to six bits, with the constraint that the sum of the coefficients is equal to 64. The quantized coefficients are predicted before coding. The horizontal filter coefficients are predicted by the corresponding fixed filter. The vertical filter coefficients are predicted by the horizontal filter coefficients. The magnitude of the predicted residue for each coefficient is coded with an Exponential-Golomb code. A sign flag is signaled if the predicted residue is non-zero.

### Adaptive up-sampling from K0031 [4]

In this test, when base layer picture is used for reference the filter set can be selected from two given sets of fixed up-samplers. The first set of up-samplers are DCTIF based separable up-samplers. This set contains 8-tap filters for luma and 4-tap filters for chroma; one filter is defined for each phase; and one for each filtering direction. The second set of up-samplers use 2-D kernels. The filter sets are as defined in JCTVC-K0031.

Signaling of up-sampler index: An up-sampler index is added to each merge list candidate. The motion equality check is modified to include the up-sampler index. A merge list candidate based on motion data in co-located base layer picture and an up-sampler index derived from neighbors is added to the merge list. Additional merge list candidates with zero motion vectors, referencing co-located base layer picture and up-sampler indices not yet added to the merge list are appended after addition of spatial, temporal and base layer candidates. When the merge/skip process is not used then an up-sampler index is conditionally signaled and context coded.

This setup will be tested with both CU and reference index based signalling as well as other alternative up-samplers. The up-sampler index will be signalled at the PU level. An optional test with CU level up-sampler index for intra BL based approach may be performed.

## Sub-test3: Interlayer SAO

### Interlayer SAO from K0033 [5]

The up-sampled base layer (BL) samples can be used as the predictors of the enhancement layer (EL) samples by inter-layer texture prediction. In order to reduce the distortion between the original EL samples and the up-sampled BL samples, inter-layer sample adaptive offset (IL-SAO) is proposed to apply to the up-sampled BL samples that are generated by the up-sampling filters.

The IL-SAO can adaptively split one picture of a color component into four regions or treat the entire picture of a color component as one region. The picture-level on/off decision of IL-SAO is done before encoding any coding tree unit (CTU) of the EL picture, so the encoding flow of the EL picture is single-pass.

### Inter-layer sample adaptive offset from K0044 [6]

In this test SAO process specified in the current non-scalable HEVC specification is applied to up-sampled BL reconstructed pictures to minimize the difference between up-sampled BL reconstructed pictures and original EL input pictures. This allows correction of errors introduced by interpolation without changing up-sampling filter. Picture based vs LCU based inter-layer SAO parameter selection will be compared in this test.

## Sub-test3: Interlayer ALF

### Adaptive filter prior to inter-layer prediction from K0032 [7]

Adaptive Filter (AF) is applied to the up-sampled base-layer reconstruction. In AF, 5x5 square shape filter characterized by 13 coefficients is used to each pixel. Filtering can be turned on and off in a unit called AF unit. The AF units are represented by quad-tree structure, of which max depth is equal to 3. The syntax for AF consists of split flags for AF unit, filter coefficients, and filter-enabling flags. Those flags are signaled at each color component independently.

### Inter layer ALF from K0033 [5]

In order to provide better BL derived predictors, inter-layer adaptive loop filter (IL-ALF) is proposed to apply to the up-sampled BL samples that are generated by the up-sampling filters or IL-SAO output samples.

The IL-ALF is based on the adaptive loop filter (ALF) in HM. For a luma picture, IL-ALF classifies 4x4 blocks of IL-SAO output samples into 16 categories, and basically one filter is derived for each category. Then different categories can be merged for reducing the number of filters. The category merging information and filter coefficients are sent to the decoder. The picture-level on/off decision of IL-ALF is done before encoding any CTU of the EL picture, so the encoding flow of the EL picture is single-pass.

### Sub-test3: Interlayer refining filter from K0039 [8]

With the inter-layer texture prediction to produce the predicted pixel samples based on the reconstructed pixels from BL, the predicted pixel samples can be further processed by an inter-layer refining filter. This refining filter can improve the accuracy of inter-layer prediction to achieve better compression performance for video in EL. This contribution applies a 13-tap 2D filter on each predicted pixel sample value. The refining filter has the diamond shape as shown in the following figureFigure ,

.

Figure 3 Filter window of inter-layer refining filter

P0 to P12 indicate the predicted pixel samples before filtering. Assume P0’ is the refined value of the center pixel P0, the filtering process can be expressed as the following equation,

P0’ = C0* P0 + C1* P1 + … + C12* P12 	(1)

where C0 to C12 are the filter coefficients. The filter coefficients are produced by encoder, and are adaptively refreshed to decoder according to the characteristics of the input video. This contribution uses picture-level refreshing of the filter by coding the coefficients in the first slice header of each picture in EL video.

In order to reduce the overhead of transmitting the coefficients, the refining filter is implemented in symmetry. Specifically, the filter taps whose positions are with respect to center with symmetry share the same coefficients, as described in following equations,

C0

C1 = C2;

C3 = C4;

Filter coefficients include:	C5 = C6;	(2)

C7 = C8;

C9 = C10;

C11 = C12.

With symmetry, the number of filter coefficients is reduced from 13 to 7. In this contribution, two sets of refining filter are used by Luma component and Chroma components, respectively. The total number of coefficients for each picture of EL video is equal to 14.

## Sub-test3: Interlayer de-blocking

### Inter layer filtering of boundary between EL and BL from JCTVC-K0033 [5]

To remove blocking artifacts between the prediction block from the BL and the neighboring reconstructed region in the EL, filtering of boundary between EL and BL is to be tested. Filtering on/off decision method is basically the same as that of deblocking filter for luma in HEVC.  It is regarded as variant of IL-deblocking Filter.

Horizontal and vertical directions use a common filter is as follows.

# Time-line and Responsibilities

2012-Nov-2:	TE-B4 description finalized and uploaded.

2012-Nov-2:	Software distributed with CU based inter-layer prediction signaling and agreed de-blocking behavior.

2012-Nov-16:	Configuration files and anchor performance distributed by AhG on SHVC tool experiments.

2012-Dec-3:	Cross-verification begins, proponents provide software, draft of contribution text and results to cross-verifiers(s) and TE-B4 participants.

2013-Jan-7:	Cross-verifiers report results to TE-B4participants.

2013-Jan-7:	Contribution documents uploaded, the document number and title are reported to coordinators.

2013-Jan-14:	Summary report uploaded.

# References

- A. Segall, “BoG report on SHVC”, JCTVC-K0354, Shanghai, China, Oct 2012.

- J. Dong, Y. He, G. McCllelan, E.-S. Ryu, X. Xiu, Y. Ye, “Description of scalable video coding technology proposal by InterDigital Communications”, JCTVC-K0034, Shanghai, China, Oct 2012

- J. Chen, K. Rapaka, X. Li, V. Seregin, L. Guo, M. Karczewicz, G. Van der Auwera, J. Sole, X. Wang, C. J. Tu, Y. Chen, “Description of scalable video coding technology proposal by Qualcomm (configuration 1)”, JCTVC-K0035, Shanghai, China, Oct 2012.

- K. Misra, J. Zhao, S. H. Kim, S. Deshpande, A. Segall, “Description of scalable video coding technology proposal by Sharp (proposal 1)”, JCTVC-K0031, Shanghai, China, Oct 2012

- C. Kim, J. Park, J. Kim, Hendry, S. Park, Y. Jeon, J. Lim, N. Park, B. Jeon, S. Liu, X. Zhang, M. Guo, Z. Chen, T.-D. Chuang, S.-T. Hsiang, C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, C.W. Hsu, Y.-W. Huang, S. Lei, “Description of scalable video coding technology proposal by LG Electronics and MediaTek (differential coding mode on)”, JCTVC-K0033, Shanghai, China, Oct 2012.

- K. McCann, J. H. Park, J. Kim, C. Kim, J.-H. Min, E. Alshina, A. Alshin, I.-K. Kim, T. Lee, B. Choi, Y. Piao, S. Jeong, S. Lee, Y Cho, J. Y. Choi, F. C. A. Fernandes, Z. Ma (Samsung), J. Boyce, D. Hong, W. Jang, A. Abbas, S. Reddy (Vidyo), “Description of high efficiency scalable video coding technology proposal by Samsung and Vidyo”, JCTVC-K0044, Shanghai, China, Oct 2012.

- T. Yamamoto, Y. Yasugi, H. Kumai, M. Takahashi, “Description of scalable video coding technology proposal by Sharp (proposal 2)”, JCTVC-K0032, Shanghai, China, Oct 2012.

- Y. Chiu, W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, H. Jiang, “Description of scalable video coding technology proposal by Intel”, JCTVC-K0039, Shanghai, China, Oct 2012.