| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K1105 |
| --- | --- |

| Title: | Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer |  |  |
| --- | --- | --- | --- |
| Status: | Output document of JCT-VC |  |  |
| Purpose: | TE description |  |  |
| Author(s) or Contact(s): | Vadim Seregin Patrice Onno Shan Liu Tammy Lee Chulkeun Kim Haitao Yang Haricharan Laksman | Email: | vseregin@qti.qualcomm.com patrice.onno@crf.canon.fr shan.liu@mediatek.com tammy.lee@samsung.com chulkeun.kim@lge.com haitao.yang@huawei.com haricharan.lakshman@hhi.fraunhofer.de |
| Source: | TE coordinators |  |  |

_____________________________

# Abstract

This document provides the description of Tool Experiment C5 on inter-layer syntax prediction using HEVC as a base layer in SHVC.

# Introduction

During the 11th JCT-VC meeting in Shanghai, multiple proposals using inter-layer syntax prediction for the SHVC design were proposed. It was then decided to coordinate for further study and testing of these methods in a tool experiment.

# Participants

| Participant | Person | Email address | P | C |
| --- | --- | --- | --- | --- |
| Qualcomm | V. Seregin K. Rapaka L. Guo J. Chen X. Li | vseregin@qti.qualcomm.com krapaka@qti.qualcomm.com liweig@qti.qualcomm.com cjianle@qti.qualcomm.com lxiang@qti.qualcomm.com | X | X |
| Canon | P. Onno G. Laroche F. Le Leannec E. Francois | patrice.onno@crf.canon.fr guillaume.laroche@crf.canon.fr Fabrice.LeLeannec@crf.canon.fr Edouard.francois@crf.canon.fr | X | X |
| MediaTek | P. Chuang Y.W. Huang S. Liu M. Guo Shih-Ta Hsiang | Peter.Chuang@mediatek.com yuwen.huang@mediatek.com shan.liu@mediatek.com mei.guo@mediatek.com Shih-ta.Hsiang@mediatek.com | X | X |
| Samsung | T. Lee A. Saxena Z. Ma E. Alshina | tammy.lee@samsung.com asaxena@sta.samsung.com zhan.ma@sta.samsung.com elena_a.alshina@samsung.com | X | X |
| LGE | C. Kim J. Park Hendry J. Kim | chulkeun.kim@lge.com jy.park@lge.com hendry.hendry@lge.com jungsun.kim@lge.com | X | X |
| Huawei | H. Yang X. Wei J. Zan H. Yu | haitao.yang@huawei.com weixianghui@huawei.com jinwen.zan@huawei.com haoping.yu@huawei.com | X | X |
| HHI | H. Laksman | haricharan.lakshman@hhi.fraunhofer.de |  |  |
| InterDigital | Y. He Y. Ye J. Dong X. Xiu | Yong.He@InterDigital.com Yan.Ye@InterDigital.com Jie.Dong@InterDigital.com Xiaoyu.Xiu@interDigital.com | X | X |
| KDDI | K. Kawamura | kei@kddilabs.jp | X | X |
| Intel | W. Zhang Y.-J. Chiu Z. Deng | wenhao.zhang@intel.com yi-jen.chiu@intel.com zhipin.deng@intel.com | X | X |
| Leibniz University Hannover | Z. Zhao H. Meuel T. Laude | zhao@tnt.uni-hannover.de meuel@tnt.uni-hannover.de laude@tnt.uni-hannover.de | X |  |
| TI | M. Budagavi D.-K. Kwon | madhukar@ti.com d-kwon@ti.com |  |  |
| ETRI | J. Lee J. Kang H. Lee | jinosoul@etri.re.kr jungwon@etri.re.kr hanilee@etri.re.kr | X | X |
| Vidyo | W. K. Jang | wonkap@vidyo.com | X |  |
| Nokia | O. Bici D. Bugdayci K. Ugur J. Lainema | ext-oguz.bici@nokia.com ext-done.bugdayci@nokia.com Kemal.Ugur@nokia.com Jani.Lainema@nokia.com | X | X |
| Sony | K. Sato C. Auyeung J. Xu | Kazushi.Sato@jp.sony.com Cheung.Auyeung@am.sony.com Jun.Xu@am.sony.com |  | X |
| Sharp | A. Segall K. Misra S.-H. Kim | asegall@sharplabs.com misrak@sharplabs.com kimse@sharplabs.com | X | X |
| I2R | Y. H. Tan | yhtan@i2r.a-star.edu.sg |  |  |
| JVC Kenwood | H. Nakamura | nakamura.hiroya@jvckenwood.com |  | X |

# Experimental Conditions

  - Test Sequences, Bit Rates and Coding Conditions

The TE will use the Class A and Class B sequences that were considered in the SHVC Call for Proposals.  All frames shall be processed, and the lower resolution representations of the sequences shall correspond to the same lower resolution representations as considered in the SHVC Call for Proposals.

The TE shall test the AI, RA and LD-P configuration for both 2x and 1.5x spatial scalability scenarios. It is mandatory to report the performance of tested tools for these three configurations, while SNR configuration is optional.  Proponents shall use the prediction structures provided by the AhG on SHVC tool experiments (by November 16, 2012).

The TE shall use CU based signaling for indicating the use of upsampled baselayer data. The CU based signaling will be provided via software to the TE. TE shall not allow the enhancement layer to have a parsing dependency on the base layer.

The TE shall test combinations where a baselayer QP is either 26 or 30, and the difference between the baselayer QP and enhancement QP is either -2, 0, 2 or 4 for AI, RA and LD-P configurations. And for SNR optional configuration, a baselayer QP is either 30 or 34, and the difference between the baselayer QP and enhancement QP is either -2, -4, -6 or -8.

The TE shall consider a block of upsampled base layer data as an “Intra” block for the purpose of de-blocking.  This will be provided via software to the TE.

The TE shall consider the software provided to the TE as the anchor.

  - Software

Software provided in JCTVC-K0348 and available on November 2, 2012.

  - Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures and complexity. Complexity assessment is performed on the base of decoding complexity measurement.

### Complexity assessment

Both HW and SW complexity of proposed tools for both encoder and decoder will be evaluated.

To measure software run time, software with the anchor configuration and the proposal implemented on the software shall be used. To measure decoding time, the decoding shall be executed on the same machine without output of decoded YUV data. The computational time must be measured for each test sequence and test case for both anchor and proposal. Relative computational time calculated against the anchor must be presented.

# Description of Experiments

The tools in this TE are classified into five categories:

- TE5.1 Inter-layer intra mode prediction.

- TE5.2 Inter-layer motion prediction including merge and AMVP modes.

- TE5.3 Inter-layer inferred prediction mode (also known as InterBL or Base Mode). In this mode, co-located base layer block is divided into sub-blocks and motion information associated with each sub-block is used to form a prediction of the corresponding block in the enhancement layer.

- TE5.4 Motion mapping for HLS approach (also known as ”ref_idx” approach), where reconstructed upsampled base layer picture is inserted into DPB buffer and there is no low level block modifications.

The following experiments will be conducted and evaluated in TE5.

The least number of estimated experiments are summarised in the next table.

| Category | Number of tests | Configuration |
| --- | --- | --- |
| TE5.1 | 6 | AI |
| TE5.2 Merge | 9 | RA, LD-P |
| TE5.2 AMVP | 10 | RA, LD-P |
| TE5.3 | 6 | RA, LD-P |
| TE5.4 | 1 | RA, LD-P |

Summary of the testing tools is provided in the below table.

| Test | Technique | Proponent | Cross-checker |
| --- | --- | --- | --- |
| TE5.1 | ## Inter-layer intra mode prediction |  |  |
| 5.1.1 | JCTVC-K0033 | MediaTek, LGE | Nokia |
| 5.1.2 | JCTVC-K0036 | Qualcomm | MediaTek |
| 5.1.3 | JCTVC-K0040 | Nokia | Qualcomm |
| 5.1.4 | JCTVC-K0045 | Samsung | LGE |
| 5.1.5 | JCTVC-K0238 | Leibniz University Hannover | JVC Kenwood |
| TE5.2 | Inter-layer motion vector prediction |  |  |
| 5.2.1 | JCTVC-K0033 | LGE, MediaTek | Canon |
| 5.2.2 | JCTVC-K0036 | Qualcomm | Nokia |
| 5.2.3 | JCTVC-K0037 | ETRI | InterDigital, Sony |
| 5.2.4 | JCTVC-K0039 | Intel | Huawei |
| 5.2.5 | JCTVC-K0040 | Nokia | Qualcomm |
| 5.2.6 | JCTVC-K0041 | Canon | MediaTek |
| 5.2.7 | JCTVC-K0046 | Huawei | Intel |
| 5.2.8 | JCTVC-K0052 | KDDI | Sharp |
| 5.2.9 | JCTVC-K0031 | Sharp | KDDI |
| 5.2.10 | JCTVC-K0045 | Vidyo, Samsung |  |
| TE5.3 | Inter-layer inferred prediction mode (InterBL mode) |  |  |
| 5.3.1 | JCTVC-K0036 | Qualcomm | Samsung, LGE, ETRI |
| 5.3.2 | JCTVC-K0039 | Intel | Huawei |
| 5.3.3 | JCTVC-K0041 | Canon | Samsung |
| 5.3.4 | JCTVC-K0045 | Samsung | Canon |
| 5.3.5 | JCTVC-K0046 | Huawei | Intel |
| TE5.4 | Motion mapping |  |  |
| 5.4.1 | JCTVC-K0034 | InterDigital | Huawei |

## TE5.1 Inter-layer intra mode prediction

### JCTVC-K0033 (MediaTek, LGE)

The intra mode at EL is mainly predicted from the intra mode of the co-located unit at BL. Besides, Planar and DC modes are removed at EL. The luma component has access to 33 angular intra prediction modes, while the chroma component has 3 candidate modes ( i.e., DM, VER, HOR). During the intra mode coding of luma component at EL, one most probable mode (denoted as MPM_EL) is used, and the remaining modes are coded with 5 bits of fixed-length binarization.

The most probable mode MPM_EL is derived for each luma PU from the intra mode of the corresponding unit at BL (denoted as candIntraPredModeBL), the intra mode of neighboring PU to the left of the current PU (denoted as candIntraPredModeA), and the one of the neighboring PU to the top of the current PU (denoted as candIntraPredModeB).

The most probable mode MPM_EL is derived as follows:

- If the co-located PU at BL is intra coded and candIntraPredModeBL is neither Planar nor DC,

MPM_EL= candIntraPredModeBL;

- Otherwise, if the left PU is available, and is intra coded,

MPM_EL = candIntraPredModeA;

- Otherwise, if the top PU is available, and is intra coded,

MPM_EL = candIntraPredModeB;

- Otherwise,

MPM_EL = Intra_Angular(26); (the vertical intra prediction mode)

### JCTVC-K0036 (Qualcomm)

As part of inter-layer syntax prediction in this proposal, the intra prediction mode of the co-located BL block is used to refine the final Most Probable Mode (MPM) list. The intra prediction mode of the co-located BL block, iColBaseDir, is added to the final MPM list, candModeList[ x ] x=0..2, as shown below:

if iColBaseDir is not one of the candModeList[ x ] x=0..2 and iColBaseDir is not equal to Intra_DC then,

candModeList[0]=iColBaseDir
                                candModeList[1]=candModeList[0]
                                candModeList[2] = candModeList[1]

The total number of candidates in the MPM list is limited to three as that in HEVC.

### JCTVC-K0040 (Nokia)

Intra prediction mode of enhancement layer is indicated by coding the difference with respect to the mode of base layer. Since the base layer intra prediction mode information is already present in the decoder side, the actual enhancement layer intra prediction mode information can be reconstructed by adding this difference to the base layer intra prediction mode. Probability distribution of the difference values can be exploited during binarization in order to improve the coding efficiency as follows:

- First bin indicates whether the difference is equal to zero, i.e. base layer intra prediction mode is equal to enhancement layer intra prediction mode or not.

- Second bin indicates the sign of the difference (either positive or negative)

- Third bin indicates whether the absolute value of the difference is equal to 1 or not.

- Fourth bin indicates whether the absolute value of the difference is equal to 2 or not.

- Since the difference values ranges from -34 to 34 and values 0, +-1 and +-2 are covered by the previous bins and the sign of the difference value is indicated by the second bin, there are a total of 32 absolute values that the difference can take. The remaining values can be coded by assigning absolute value of the difference minus 3 to 5 bins with equal probability (i.e. in bypass mode).

### JCTVC-K0045 (Samsung)

Intra modes from the co-located base-layer, the left neighbor and the above neighbor are initially listed as MPM0, MPM1 and MPM2. HEVC method is used to derive the MPM list when duplication happens.

In the hook of K0348, the necessary code change is only from

*piMode = 2;

to

*piMode = 3;

when MPM0, MPM1 and MPM2 are distinct.

### JCTVC-K0238 (Leibniz University Hannover)

Base layer based MPM coding (BL-MPM)

A flag, bl_mode_mpm_flag, is introduced to indicate whether the intra prediction mode of the co-located PU in the base layer is used as the intra prediction mode of the current PU in the enhancement layer. The bl_mode_mpm_flag is coded in the coding unit syntax only if prev_intra_luma_pred_flag is false. If the encoder cannot find a matched intra prediction mode in the list of 3MPM candidates for the current PU, prev_intra_luma_pred_flag is set to false and the intra prediction mode of the co-located PU in the base layer is checked whether it is equal to the intra prediction mode of the current PU in the enhancement layer. If the intra prediction mode of the current PU is equal to the intra prediction mode of the co-located PU in the base layer, bl_mode_mpm_flag is set to true and rem_intra_luma_pred_mode does not need to be coded any more. Otherwise, Bl_mode_mpm_flag is set to false and rem_intra_luma_pred_mode is coded as the single layer HM coding.

Inter-layer intra prediction mode coding (ILIPM)

In this approach, a new mode, called inter-layer intra prediction mode (ILIPM), is added as an extra intra prediction mode for an intra coded CU in the enhancement layer. ILIPM is only evaluated if the current PU size of the enhancement layer is twice the co-located PU size of the base layer in terms of RD cost. In the IPILM mode, the current PU utilizes the intra prediction mode of the co-located PU in the base layer as its intra prediction mode. A flag, intra_bl_mode_flag, is introduced before prev_intra_luma_pred_flag in the coding unit syntax. Intra_bl_mode_flag is only coded in the coding unit syntax if the current PU size of the enhancement layer is twice the co-located PU size of the base layer. If intra_bl_mode_flag is true, prev_intra_luma_pred_flag and possible rem_intra_luma_pred_mode are not coded any more. If the current PU size of the enhancement layer is twice the co-located PU size of the base layer and IPILM is not chosen as the intra prediction mode for the current PU, intra_bl_mode_flag is set to false and MPM coding as in the single layer coding is performed.

## TE5.2 Inter-layer motion vector prediction

### 5.2.1 JCTVC-K0033 (LGE, MediaTek)

When a prediction unit (PU) in the enhancement layer (EL) is coded as inter mode, motion information from the corresponding PU in the base layer (BL) is added to the advanced motion vector prediction (AMVP) candidate list for Inter mode and the merging candidate list for Merge mode if it is available. Two categories of test are included. One is for merge candidate derivation, and the other is for AMVP candidate derivation.

EL merge mode

In this test, two corresponding positions, IL-C0 and IL-H, as shown in Figure 1, are used for inter-layer motion candidates. The IL-C0 candidate is placed before the spatial candidates in the merging candidate list, and the IL-H is placed between two spatial candidates. In the spatial scalability case, if the inter prediction direction and reference indices of a spatial candidate are the same as those of the IL-C0 and the motion vector of the spatial candidate is similar to that of the IL-C0 (i.e., the MV difference is within 1/4 pel), the IL-C0 candidate is replaced with the spatial candidate. All spatial candidates and the IL-H candidate are checked whether they are the same as the IL-C0 candidate.

EL AMVP mode

In this test, the IL-C0 is used for inter-layer motion candidates. The IL-C0 candidate is placed before the spatial candidates in the AMVP candidate list. All spatial candidates are checked whether they are the same as the IL-C0 candidate.

Figure 1. Derivation of corresponding positions for inter-layer motion candidates

### 5.2.2 JCTVC-K0036 (Qualcomm)

In HEVC, motion vectors are compressed after being coded and such compressed motion vectors can be later used as TMVP for coding other frames. In this proposal, the uncompressed base layer MVs, obtained at a location co-located to the center block in EL, which is the same as that used for TMVP, are used in enhancement layer coding. When the spatial resolutions of BL and EL pictures are different, the base layer MVs are scaled according to the spatial resolution ratio between the two layers.

EL merge mode

The derived base layer MV is inserted as the first candidate in merge list, additional pruning is applied between spatial merge candidates and the base layer MV. TMVP if available should be always present in the merge list, for this purpose third spatial candidate is conditionally added.

EL AMVP mode

The derived base layer MV is added after the TMVP candidate in AMVP list in case the AMVP list is not complete.

The maximum number of merge and AMVP motion vector candidates is kept the same as in HEVC, i.e. five for merge and two for AMVP.

### 5.2.3 JCTVC-K0037 (ETRI)

Since motion information of the current PU in the enhancement layer is likely to be similar to the co-located PU in the base layer, motion information of the co-located PU in the base layer is employed as one of candidates for merge/skip mode and motion vector prediction. When an uncompressed motion vector of the co-located PU in the base layer is used as merge/skip or motion vector prediction candidate, it is scaled by the spatial resolution factor. In addition, the maximum number of motion candidates of merge and AMVP is same as the one in HEVC, and the position of co-located PU in base layer is same as the center position of TMVP.

EL Merge mode

The motion information of the co-located PU in the base layer is added as the first merge candidate. The maximum number of candidates before deriving TMVP is four, which is same as the one in HEVC.

EL AMVP mode

The motion information of the co-located PU in the base layer is added as the first motion vector prediction candidate.

### 5.2.4 JCTVC-K0039 (Intel)

The motion parameters of BL such as the inter prediction direction, reference index and motion vectors, can be used as the additional candidate information to encoder for the inter coding for EL video. For Merge mode and AMVP mode, the motion information of BL, which aligns to the right-bottom corner of current PU in EL, is extracted and is placed as the first entry in the candidate list. With this motion information from the co-located BL added to the set of the motion information from EL, the maximum number of motion candidates is increased from 5 to 6 for Merge PUs and is increased from 2 to 3 for AMVP PUs.

### 5.2.5 JCTVC-K0040 (Nokia)

Motion vector of the co-located base layer block is used as a candidate motion vector predictor for the enhancement layer. Co-located base layer block is found by taking the center coordinates of the current block and scaling it according to scalability factor. Base layer motion vector is used in enhancement layer after being scaled according to the scalability factor and POC differences.

EL AMVP mode

In the AMVP list construction, if available, the co-located motion vector in the base layer (BMVP) is placed as the first candidate in the list. In addition, base layer MVP does not participate in pruning process to reduce the complexity of list construction.

EL Merge mode

Merge list construction is modified as follows to include BMVP in the merge list;

- Place the base layer MVP in the beginning of the list

- Select maximally 3 spatial MVP candidates.

- Place the temporal MVP to the list.

- Base layer MVP does not participate in redundancy check process to reduce complexity.

### 5.2.6 JCTVC-K0041 (Canon)

In this contribution, the MERGE derivation predictors and the AMVP process specified for HEVC are adapted to the enhancement layer prediction in order to improve the coding efficiency of the Inter, Merge, Merge Skip and GRILP modes.

EL merge mode

The derivation process of the Merge, Merge Skip and Merge GRILP Modes in JCTVC-K0041 has been modified compared to the HM6.1, the number of candidates and the order of candidates are modified. Moreover, new candidates are added for the enhancement layer. These adaptations are listed below:

•	The number of candidates in the final list is increased.

•	The temporal candidate is now the first candidate in the list. Only the centered position is considered for the enhancement layer.

•	The spatial candidates are added in the list after the temporal candidate. The derivation process of spatial candidates is the same as for the base layer. In addition, if the neighboring block is encoded with the base mode this candidate is not added in the Merge list.

•	Then the motion vector of the co-located base layer is added to the list of candidates.

•	The base motion vector is followed by 4 offset candidates derived according to this base motion vector candidate. The generation of these candidates consists in adding one offset (offset value: 4 or -4) alternatively to the horizontal or to the vertical component of the motion vector from list L0 and its inverse value to components of motion vector from list L1.

A duplicate check is used on this list of maximum 10 candidates. Then the combined candidates derivation and the zero candidates derivation are applied if at least one position is available.

EL AMVP mode

As for the MERGE mode candidate derivation, the number of predictors and the order of predictors are modified and a new predictor is added for AMVP in the enhancement layer compared to HM6.1 as described in the following:

- The number of predictors in the final list is increased from 2 to 3. The base layer motion vector located at the center of considered enhancement CU is added to the motion prediction list. The co-located motion vector in the base picture, re-scaled to the resolution of the enhancement layer, is added to the list of predictors. The base layer motion vector that is used here is located at the center of considered enhancement CU.

- The temporal is the first candidate of the list instead of the last one for the base layer. Moreover, only the centered position of the temporal predictor is used.

The availability check of the spatial predictor is modified in order to select the motion vectors of neighboring blocks which were encoded with the base mode.

The following tests regarding the candidates list derivation presented in JCTVC-K0041 will be tested in the TE5:

- Modify the AMVP/MERGE candidates list ordering

- Change the AMVP/MERGE candidate list  checking

- Add new AMVP/MERGE candidates (as offset predictors in JCTVC-K0041)

- Combination of the three above tests with the base mode will be performed as additional data.

### 5.2.7 JCTVC-K0046 (Huawei)

EL AMVP mode

An additional AMVP candidate, i.e. the base layer AMVP candidate, is added in the first place in the AMVP candidate list. The motion information of each SPU (Smallest Prediction Unit) inside the current PU is derived from the base layer as described in section 2.3.5.2 in JCTVC-K0046_r1.doc. SPU size is set to 4x4 in this TE. Then the motion information of the center SPU is used to derive the base layer AMVP candidate. Specifically, the motion vector of the center SPU is scaled according to the POC difference, in the same way as the scaling operation on the temporal co-located motion vector for the derivation of the temporal AMVP candidate in the base layer. Motion compensation is performed for the current PU using the derived base layer AMVP candidate. The max number of the AMVP candidate is set to 2. The construction of the AMVP candidate list is described in section 2.3.5.5 in JCTVC-K0046_r1.doc.

### 5.2.8 JCTVC-K0052 (KDDI)

In this contribution, inter-layer motion-vector prediction is proposed for both merge list and AMVP. In the HEVC specification of the merge list construction, predicted motion vectors (PMVs) are listed in the following order; left, up, upper left, and so on. In the proposed scheme, PMVs are listed in the following order; left, up, inter-layer, upper left, and so on.

Therefore the maximum number of merge mode candidates becomes six. In the HEVC specification of AMVP, PMVs are listed in the following order; left blocks, upper blocks, and temporal blocks. The number of PMVs is then truncated to two. In the proposed scheme, PMVs are listed in the following order; left blocks, upper blocks, inter-layer blocks, and temporal blocks.

The number of PMVs in this list is then truncated to three.

### 5.2.9 JCTVC-K0031 (Sharp)

In this test the merge list and motion vector list are populated with motion information from the base layer as described in JCTVC-K0031 and the performance evaluated.

EL merge mode

A merge list candidate is added to the merge list based on the motion information corresponding to a co-located PU in the base layer picture co-located with the current picture. The reference index of the base layer merge list candidates are set to point to enhancement layer picture co-located with the base-layer reference picture. The base layer reference picture corresponds to the reference picture of the base layer picture co-located with the enhancement layer picture under consideration. The same reference picture list is considered for the base layer picture while determining the reference picture as was being considered for the enhancement layer picture. The motion vector corresponding to the co-located PU in the base layer picture co-located with the current enhancement layer picture is scaled.

EL AMVP mode

A primary motion vector predictor is generated for every reference picture using the base layer motion vector. More specifically, a base layer motion vector is derived from the co-located PU from the base layer picture co-located with the current enhancement layer picture. The derived motion vector is scaled and used as the primary motion vector predictor. A context coded flag mvp_lX_bl_flag is used to signal if the primary motion vector predictor is to be used for the current PU. If the mvp_lX_bl_flag is false the advanced motion vector prediction is used to generate the predictor.

### 5.2.10 JCTVC-K0045 (Vidyo, Samsung)

The BL’s motion information is added as a motion prediction candidate to the candidate list.  The BL’s motion information is added in the last position in the list; however, placement of the BL’s information can be controlled in slice header in the future version.  When determining the corresponding BL PU (or a block in the case of mixed-codec scalability), there is no required correlation between the CU or PU sizes for the BL and the EL.  The EL simply selects the position corresponding to the center of the EL PU, and the BL PU that includes the corresponding sample (after properly scaling) is used.  The BL’s information is currently stored in the granularity of 4x4 blocks; however, if memory is an issue, the future version can be modified to store the information in the granularity of 8x8 blocks.

## TE5.3 Inter-layer inferred prediction mode (InterBL mode)

### 5.3.1 JCTVC-K0036 (Qualcomm)

An inferred base layer mode is introduced and applied to enhancement layer coding. In this mode, prediction information for a current enhancement layer CU is inferred from its co-located base layer blocks.

For a CU with this mode, its motion information (which includes inter prediction direction, reference index and motion vectors) is not signaled. Instead, for each sub-block in the CU, its motion information is derived from its co-located base layer block. In the case that the motion information of a co-located base layer block is unavailable (e.g., the co-located base layer block is intra predicted), the sub-block is predicted using reconstructed pixels value from the co-located base layer block.

In deriving motion information, when the spatial resolutions of BL and EL pictures are different, the base layer MVs need to be scaled according to the spatial resolution ratio between the two layers.

This mode is signaled using a flag baselayer_mode_flag at CU level. The sub-block with size as 4x4 and 8x8 (optional) will be studied in this test.

### 5.3.2 JCTVC-K0039 (Intel)

Inter-layer pattern/mode prediction utilizes the splitting pattern and prediction information of BL to encode video in EL. To encode a CU in EL, the information of sub-CU quadtree structure, PU partitions and intra/inter prediction modes can be derived from co-located blocks in BL when all of the co-located BL blocks belong to a single CU. With the quadtree pattern projection from BL to EL, the partition sizes and prediction modes of each node CU in BL can be copied to the co-located CU in EL. Inter-layer pattern prediction transmits only the transform parameter and residual for the target CU in EL. Inter-layer pattern/mode prediction is an additional CU coding mode to code video in EL. A CU level control flag, denoted as inter_layer_pattern_mode_pred_flag, is added in CU syntax table to indicate if current CU uses inter-layer pattern/mode prediction or not. With this flag, a new coding mode called ILPMPred mode, is proposed for CU coding of EL video.

### 5.3.3 JCTVC-K0041 (Canon)

In JCTVC-K0041, an inter-layer prediction mode is proposed for the coding/decoding of enhancement inter picture. This mode, called the “BaseMode” prediction mode consists in predicting an entire EL CU from some so-called BaseMode prediction texture data. The BaseMode prediction data is computed through either temporal prediction of derived Inter CUs (with derived BL motion info) or Intra BL prediction derived from Intra CUs, in combination with the compensation of enhancement reference pictures. The BaseMode prediction mode is signaled on the CU level in the enhancement bit-stream.

Two distinct Inter-layer predictions of CU/PU/TU partitions are used for spatial ratio 2x and for ratio 1.5x as described in JCTVC-K0041.

The following two tests are optional/additional for this TE. The goal is to provide further results in order to demonstrate the good synergy when combining the BaseMode with other tools of JCTVC-K0041 (GIRLP, DBF) related to others TEs.

- In JCTVC-K0041, a combination of this BaseMode prediction with inter-layer residual prediction is introduced. This consists in employing the Generalized Residual Inter-Layer Prediction (GRILP) mechanism, proposed in SHVC TE3, in order to apply the inter-layer predicted residual onto MC predicted CUs in the construction of BaseMode prediction data. The GRILP mechanism to generate the BaseMode prediction data significantly improves the quality of the BaseMode predictor, hence overall coding efficiency.

In JCTVC-K0041, a combination of this BaseMode prediction with an Inter-layer de-blocking filtering step is also performed. It consists in applying a de-blocking filter onto the reconstructred texture used to predict a given enhancement CU, through the above “BaseMode” prediction mode. This de-blocking step is a simplified version of the common de-blocking filtering, i.e. it operates only at CU and PU boundaries.

### 5.3.4 JCTVC-K0045 (Samsung)

When the CU is coded in Inter_BL mode, all motion information for each 8x8 block of the CU is derived from the corresponding BL block’s motion information.  For example, if the CU is a 32x32 block, then each of the sixteen 8x8 blocks of the CU gets its own motion information derived from the corresponding BL. When determining the corresponding BL motion information, the BL PU (or a block in case of mixed-codec scalability) that includes the corresponding sample (after proper scaling) of the left-top corner of each 8x8 block is selected.  Hence, each 8x8 block of the CU may get motion compensated blocks from different locations. If the corresponding BL motion information of an 8x8 block is not available, the 8x8 block’s motion is set by the previous 8x8 block’s motion in z-scan order or it is set to zero motion if it is the first 8x8 block inside the CU. Inter-BL mode is applied to not only pixel-coding mode but also difference-coding mode.

### 5.3.5 JCTVC-K0046 (Huawei)

CU level signaling

The proposed inter-layer motion copy mode is applied to CUs in enhancement layer P/B slices. A flag is put into the CU header, right after the flag for the inter-layer texture prediction mode, to indicate whether this mode is used. The motion information of all SPU (Smallest Prediction Unit) in the CU is derived from the base layer as described in section 2.3.5.2 in JCTVC-K0046_r1.doc. Then the motion compensation is performed separately for each SPU in the CU. SPU size is set to 8x8 in this TE. The residual coding part is the same as the ordinary inter coded CUs in the base layer.

PU level signaling

An additional merge candidate, i.e. the base layer merge candidate, is added in the first place in the merge candidate list. The motion information of the base layer merge candidate is derived from the base layer as described in section 2.3.5.2 in JCTVC-K0046_r1.doc. SPU size is set to 8x8 in this TE. Note that the motion information of different SPUs (Smallest Prediction Unit) in the base layer merge candidate for the current PU can be different. Using the derived base layer merge candidate, the motion compensation is performed for each SPU if the current PU size is bigger than or equal to the SPU size. Otherwise, the motion compensation is performed for the current PU. The max number of the merge candidate is set to 5. The construction of the merge candidate list is described in section 2.3.5.4 in JCTVC-K0046_r1.doc. The binarization of the merge index is the same as the base layer. The context modeling of the first bin indicating the usage of the base layer merge candidate is similar to that for the skip mode flag, while the context modeling for other bins is the same as the base layer. Other PU level signaling method will also be studied.

## TE5.4 Motion mapping

### 5.4.1 JCTVC-K0034 (InterDigital)

Inter-layer motion field prediction is carried out at the picture level. Specifically, picture level processing of the base layer pictures in DPB in Figure 3 also includes mapping the base layer motion information to form motion field for the inter layer reference picture. This allows the inter layer reference to be used as a co-located picture for enhancement layer TMVP.

Figure 3. Example relationship between the SPUs of upsampled base layer and the original base layer

Motion field mapping will be evaluated in TE-C5.

To evaluate inter-layer motion field prediction, the performance of the reference picture index signaling framework in the reference software will be used as the anchor, where the processed BL picture is placed at the end of the reference picture lists. This means that in the reference software, the EL TMVP will always use a temporal reference picture as the collocated picture. Then, non-normative change will be made to slice header signaling in reference software to always use the processed BL picture for EL TMVP. In this way, the performance with and without inter-layer motion field prediction can be fairly compared. In addition, both the 1.5X and 2X spatial scalability cases will be tested for inter-layer motion field prediction method.

# Time-line and Responsibilities

2012-Nov-2:       TEC5 description finalized and uploaded.

2012-Nov-2:       Software distributed by AhG on SHVC Software

2012-Nov-16:    Configuration files and anchor performance distributed by AhG on SHVC tool experiments

2012-Dec-10:      Cross-verification begins, proponents provide software, draft of contribution text and results to cross-verifiers(s) and TE coordinators.

2013-Jan-7:        Cross-verifiers report results to TE participants (2 weeks from cross-check start)

2013-Jan-7:        Contribution documents uploaded, the document number and title are reported to coordinators.

2013-Jan-14:      Summary report uploaded

# References

- A. Segall, “BoG report on SHVC”, JCTVC-K0354, Shanghai, China, Oct 2012.