| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K1106 |
| --- | --- |

| Title: | TE6: Inter-layer syntax prediction from AVC base layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Kawamura Kei Haricharan Lakshman | Email: | jill@vidyo.com ki-kawamura@kddi.com haricharan.lakshman @hhi.fraunhofer.de |
| Source: TE |  |  |  |

_____________________________

# Abstract

This document provides the description of Tool Experiment C6 on inter-layer syntax prediction from an AVC base layer.

# Introduction

During the 11th JCT-VC meeting in Shanghai, multiple proposals for inter-layer syntax prediction from the AVC base layer for the SHVC hybrid codec mode.  It was then decided to coordinate for further study and testing of inter-layer syntax prediction tools in a tool experiment.

# Participants

| Participant | Person | Email address | P | C |
| --- | --- | --- | --- | --- |
| Vidyo | J. Boyce W. Jang | jill@vidyo.com wonkap@vidyo.com | X |  |
| HHI | H. Laksman | haricharan.lakshman@hhi.fraunhofer.de |  | X |
| KDDI | K. Kawamura | ki-kawamura@kddi.com | X |  |
| Canon | P. Onno F. Le Leannec E. Francois | patrice.onno@crf.canon.fr Fabrice.LeLeannec@crf.canon.fr Edouard.francois@crf.canon.fr |  | X |
| Huawei | J. Zhan H. Yu | jinwen.zan@huawei.com haoping.yu@huawei.com |  | X |
| Mediatek | Y. Huang | yuwen.huang@mediatek.com |  | X |
| Samsung | - Saxena E. Alshina | asaxena@sta.samsung.com elena_a.alshina@samsung.com | X |  |
| Sony | K. Sato C. Auyeung | Kazushi.Sato@jp.sony.com Cheung.Auyeung@am.sony.com |  | X |
| TI | M. Budagavi | madhukar@ti.com |  | X |

The proponent and cross-checkers for the sub-tests are listed in the table below.

| Test | Tool | Proponent | Cross-Checker |
| --- | --- | --- | --- |
| 6.1 | AMVP from K0045 | Vidyo/Samsung | Canon |
| 6.2 | Merge from K0052 | KDDI | Mediatek |

Anchors will be provided by Huawei.

# Experimental Conditions

  - Test Sequences, Bit Rates and Coding Conditions

The TE will use the Class A and Class B sequences.  The lower resolution representations of the sequences shall correspond to the same lower resolution representations as considered in the SHVC Call for Proposals.

The TE shall test the AI, RA and LD-P configuration for both 2x and 1.5x spatial scalability scenarios.  It is mandatory to report the performance of tested tools for these three configurations.  Proponents shall use the prediction structures provided by the AhG on SHVC tool experiments (by November 30, 2012).

The TE shall test combinations where an AVC baselayer QP is either 25 or 29, and the difference between the baselayer QP and enhancement QP is either -1, 1, 3 or 5.

The TE shall consider the software provided to the TE as the anchor.

The tested tools will have no parsing dependencies of the HEVC EL on the AVC BL.

  - Software

Software provided based on JCTVC-K0348 and available on Nov 14, 2012.

  - Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures, complexity, and suitability for use with legacy base layer AVC decoders.  Complexity assessment is performed on the base of decoding complexity measurement.

Two sets of anchors will be used.  First, AVC anchors such as those used in the CfP.  Second, the K0348 based software for an AVC base layer, using upsampling filter and intraBL signaling.

# Description of Experiments

The tools in this TE are classified into two categories:

- TE5.1 Inter-layer motion prediction for AMVP

- TE5.2 Inter-layer motion prediction for Merge

# Items to test

## BL motion vector addition to AMVP list

### K0045

The BL’s motion information is added as a motion prediction candidate to the candidate list.  In the current implementation, the BL’s motion information is added in the last position in the list; however, placement of the BL’s information can be controlled in slice header in the future version.  When determining the corresponding BL PU (or a block in the case of mixed-codec scalability), there is no required correlation between the CU or PU sizes for the BL and the EL.  The EL simply selects the position corresponding to the center of the EL PU, and the BL PU that includes the corresponding sample (after properly scaling) is used.  The BL’s information is currently stored in the granularity of 4x4 blocks; however, if memory is an issue, the future version can be modified to store the information in the granularity of 8x8 blocks.

## BL motion vector addition to merge list

### K0052

In the HEVC specification of the merge list construction, predicted motion vectors (PMVs) are listed in the following order; left, up, upper left, and so on. In the proposed scheme, PMVs are listed in the following order; left, up, inter-layer, upper left, and so on. Therefore the maximum number of merge mode candidates becomes six.

# Time-line and Responsibilities

2012-Nov-2:	TE6 description finalized and uploaded.

2012-Nov-16:	Software distributed with CU based inter-layer prediction signaling, and AVC base layer support

2012-Nov-30:	Configuration files and anchor performance distributed by AhG on SHVC tool experiments

2012-Dec-10:	Cross-verification begins, proponents provide software, draft of contribution text and results to cross-verifiers(s) and TE6 coordinators.

2012-Jan-7:	Cross-verifiers report results to TE participants (2 weeks from cross-check start)

2012-Jan-14:	Contribution documents uploaded, the document number and title are reported to coordinators.

2012-Jan-21:	Summary report uploaded