| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K1121 |
| --- | --- |

| Title: | HEVC Range Extensions Core Experiment 1: Square and non-square transforms for 4:2:2 chroma format coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Joel Sole | Tel: +1-858-845-2429 Email: joels@qti.qualcomm.com |  |
| Source: | CE coordinator |  |  |

_____________________________

# Abstract

This document is a description of the HEVC Range Extensions Core Experiment 1 on square/non-square transforms for the 4:2:2 chroma format.

# Introduction

AHG7 released software that adds capability to code extended chroma formats (4:2:2 and 4:4:4) to HM-8.0 and, as described in JCTVC-K0365, plans to release a simplified version of that software for HM-9.0. The software is primarily based on JCTVC-K0181, which is to be simplified following the design principle of selecting the 4:2:0 chroma processing for 4:2:2 and 4:4:4 unless required for proper handling of the extended chroma. JCTVC-K0365 describes the functionalities and corresponding macros to be enabled in the software.

A design choice is to be made in 4:2:2 with respect to the usage of square or non-square transforms: BD-rate impact and associated complexity of both options are to be studied in this CE.

# Participants

| Participant | Person | Email address | P | C |
| --- | --- | --- | --- | --- |
| BBC | M. Mrak | Marta.Mrak@bbc.co.uk |  |  |
| Canon | C. Rosewarne | chris.rosewarne@cisra.canon.com.au | x | x |
| Hisilicon | X. Zheng | zhengxiaozhen@huawei.com |  | x |
| Huawei | H. Yu | haoping.yu@huawei.com |  |  |
| JVC Kenwood | H. Nakamura | nakamura.hiroya@jvckenwood.com |  | x |
| KDDI | K. Kawamura | ki-kawamura@kddi.com |  |  |
| MediaTek | S. Liu S.-T. Hsiang | shan.liu@mediatek.com Shih-ta.Hsiang@mediatek.com |  |  |
| Qualcomm | J. Sole R. Joshi | joels@qti.qualcomm.com rajanj@qti.qualcomm.com |  | x |
| RIM | D. Flynn | dflynn@rim.com |  |  |
| Samsung | A. Saxena | asaxena@sta.samsung.com |  |  |
| Sony | N. Saunders K. Sharman | nick.saunders@eu.sony.com karl.sharman@eu.sony.com | x |  |
| TI | M. Budagavi V. Sze | madhukar@ti.com sze@ti.com |  |  |

# Experiments

  - Tests

Tests are to be done for the 4:2:2 case by enabling square or non-square transforms for the chroma components. Three configurations are to be tested:

- Non-square transforms: method presented in JCTVC-K0181

  - Proponent: Sony

  - Cross-checker: Qualcomm, Canon

- Mix of square and non-square transforms: method presented in JCTVC-K0171

  - Proponent: Canon

  - Cross-checker: Hisilicon

- Square transforms: disallow non-square transforms for all PU/TU sizes in JCTVC-K0171

  - Proponent: Canon

  - Cross-checker: JVC Kenwood

  - Software

HM-9.0-AhG7 shall be used (will be provided on November 12th, 2012).

  - Test Coding Conditions

The recommended sequences, configurations and test cases described in JCTVC-K1006 will be used. Results for all mandatory test cases shall be provided.

  - Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures and complexity. Complexity assessment is performed on the base of hardware and encoding/decoding complexity.

### Complexity assessment

To measure software run time, HM9.0-AhG7 software with the anchor configuration (which is equivalent to test 1) and the proposal implemented on the HM9.0-AhG7 software shall be used. The computational time must be measured for each test sequence and test case for both anchor and proposal. Relative computational time calculated against the anchor must be presented.

Additional evaluation of the HW and SW complexity of the proposed tools for both encoder and decoder is encouraged.

# Timeline

- 05-Nov-2012: HM-9.0 available.

- 09-Nov-2012: CE description finalized and uploaded.

- 12-Nov-2012: HM-9.0-AhG7 available.

- 10-Dec-2012: Cross-check begins: Proponents provide software, draft of contribution text and results to CE participants.

- 04-Jan-2013: Input document and summary report uploaded.