| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0006_r1 |
| --- | --- |

| Title: | JCT-VC AHG report: HEVC in-loop filtering (AHG6) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | T. Yamakage A. Norkin |  | tomoo.yamakage@toshiba.co.jp andrey.norkin@ericsson.com |
| Source: | AHG co-chairs |  |  |

_____________________________

# Abstract

The following summarizes the HEVC in-loop filtering Ad hoc activities between the 11th JCT-VC meeting in Shanghai, CN (10 to 19 October, 2012) and the current 12th JCT-VC meeting in Geneva, CH (14 to 23 January, 2013).

# Mandate

| AHG Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| In-loop filtering (jct-vc@lists.rwth-aachen.de) - Clean up and stabilize the HM software, the draft text and the HM encoder description on in-loop filtering. - Study and consider improvements to the common test conditions by signalling deblocking filter control parameters. - Identify test sequences, methodology, and test parameters for testing deblocking filter behaviour. | T. Yamakage and A. Norkin (co-chairs) | N |

# Activity and list of related input documents

## Activity

There are a few email exchanges about deblocking filtering on the JCT-VC main reflector.

Mandate 1: A patch to fix Ticket #485 for HM (HM software does not cope with different values of tc / beta across slices) has been provided by an expert and the AHG co-chairs reviewed and confirmed the patch seems fine.  The patch has already been integrated to HM 9.1.  There are several open tickets on the HEVC Text Specification Draft 9 (SoDIS) about in-loop filtering (#881, #882 and #883 for deblocking filter, and #879 and #900 for SAO), which should be addressed during the meeting.

Mandate 2: JCTVC-L0232 and JCTVC-L0386 studied how to set the deblocking filter control parameters at encoder.  JCTVC-L0232 studied a static approach and JCTVC-L0386 studied an adaptive approach.

Mandate 3: It is suggested to evaluate proposals separately based on the approach (i.e, normative changes or non-normative changes).  Their combination may be evaluated.

It is suggested to use two types of sequences.  Type1 sequences are used to evaluate the proposed deblocking modifications on the critical sequences.  Type2 sequences are used to ensure there is no subjective quality loss on the common test set.

It is also suggested to use the same test sequences and evaluation method (i.e., scoring) used for subjective viewing at the Shanghai meeting (see JCTVC-K0369 and JCTVC-K0380) and in CE1 deblocking filtering (JCTVC-J1101):

Type 1 sequences (six test points): "Riverbed (Qp=32 and 37, RA)", "West Winds Easy (Qp=37, LDB)", "Ducks Take Off @ 25fps (Qp=37, LDB)", "China Speed (Qp=37, LDB)", "Red Kayak (Qp=37, RA)"

Type 2 sequences (five test points): "Kimono", "Cactus", "BasketballDrillText", "BQMall", and "Johnny", QP=32, LDB for all sequences

Test subjects will be asked to rate each proposal on a scale from -2 to 2 (“A” is worse, slightly worse, no difference, slightly better, or better than “B”).  The results will be later calculated as well as 95-percent confidence intervals for every proposal and test sequence.

As for the test procedure, the ABAB test against the common anchor is to be used.  The anchor is HM9.0 in common test conditions.  The proposals are tested on Type 1 sequences first. If some proposals show improvements on Type 1 sequences, they will be tested on Type 2 sequences in the second stage of the testing.  The test persons should preferably be seated in one row in front of the display in order to have a good view of the screen. At least 20 people should participate in the test and they should be non-proponents.

The following intra period parameters should be used for non-common test condition sequences:

RedKayak 32

Riverbed 24

In case DucksTakeOff25fps and WestWindsEasy are to be encoded in RA, the following parameters can be used.

DucksTakeOff25fps 24

WestWindsEasy 32

A suggestion for the subjective viewing candidates is as follows:

Anchor: HM9.0 common test conditions

Test1: L0232-1 (modified config (non-normative))

Test2: L0386 (non-normative)

Test3: L0232-1 + L0232-3 (strong filter modification (normative))

Test4: L0232-1 + L0232-2 (TU size restriction (non-normative)) + L0232-3

Test5: L0232-1 + L0232-2 (if possible)

## List of contributions to AHG6

- JCTVC-L0232 [Ericsson] AHG6: On deblocking filter and parameters signaling

  - JCTVC-L0404 [Samsung] AhG6: Cross-check for deblocking filter process and parameter modifications suggested in JCTVC-L0232

- JCTVC-L0386 [Qualcomm] AHG6: On HEVC block artifact reduction

  - JCTVC-L0397 [Sony] AHG6: cross check of JCTVC-L0386 On HEVC block artifact reduction

# Recommendations

To review the open tickets and address them during the meeting.

To conduct subjective viewing for contributions listed in section 2.2.

If the viewing results provide an evidence to improve the subjective picture quality, to study the qualified contribution(s) for adoption to the HEVC Specification Draft or HM.