| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0008r1 |
| --- | --- |

| Title: | JCT-VC AHG report: Screen Content Coding (AHG8) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Wen Gao Huawei Technologies, Madhukar Budagavi Texas Instruments Inc., USA Robert Cohen Mitsubishi Electric Research Laboratories Alberto Duenas NGCodec Tao Lin Tongji University Jizheng Xu Microsoft | Email: Email: Email: Email: Email: Email: | wen.gao@huawei.com madhukar@ti.com cohen@merl.com alberto@ngcodec.com lintao@tongji.edu.cn jzxu@microsoft.com |
| Source: | AHG8 |  |  |

_____________________________

# Abstract

This report summarizes activities related to AHG on Screen Content Coding (AHG8) between the 11th and the 12th JCT-VC meeting.

# Mandates

| Screen content coding (AHG8) (jct-vc@lists.rwth-aachen.de) - Consider needs for lossless coding and screen content coding support in the range extensions. - Study (lossy and lossless) coding tools and performance of HEVC and its range extensions on screen content - Evaluate and identify test material appropriate for screen content coding - Make recommendations for test conditions for screen content coding | W. Gao (chair), M. Budagavi, R. Cohen, A. Duenas, T. Lin, J. Xu (vice-chairs) | N |
| --- | --- | --- |

# Status

During the interim period between the 11th and 12th JCT-VC meetings, AHG8 made good progress in test materials. In addition, the AHG8 also recommended reference software and test conditions.

## Test Materials

In the 11th JCT-VC meeting, four sequences were contributed and can be used as test sequence candidates for screen content coding. The four sequences are

- VenueVu_1920x1080_30 (BBC)

- cad_waveform_1920x1080_20 (Tongji Univ.)

- pcb_layout_1920x1080_20 (Tongji Univ.)

- ppt_doc_xls_1920x1080_20 (Tongji Univ.)

Note that the for the four sequences, both 8-bit RGB format and 8-bit YUV444 format are provided except for VenueVu, 10-bit YUV format is provided, in JCT-VC ftp server.  During the interim period, six more screen captured sequences are provided:

- video_conferencing_doc_sharing_1280x720_30 (Tongji Univ.)

- web_browsing_1280x720_30 (Tongji Univ.)

- cg_twist_tunnel_1280x720_30 (Tongji Univ.)

- wordEditng_1280x720_60 (Huawei & MERL)

- programming_1280x720_60 (Huawei & MERL)

- map_1280x720_60 (Huawei & MERL)

Note that for the six new sequences, both 8-bit RGB format and 8-bit YUV444 format are provided and uploaded in the JCTVC ftp server. For purpose of illustration, one sample frame from each of the six new sequences is shown below.

Figure 1- Frame 0 of video_conferencing_doc_sharing_1280x720_30 sequence

Figure 2- Frame 0 of web_browsing_1280x720_30 sequence

Figure 3 - Frame 0 of cg_twist_tunnel_1280x720_30 sequence

Figure 4 - Frame 100 of programming_1280x720_60_8bit sequence

Figure 5 - Frame 580 of wordEditing_1280x720_60_8bit sequence

Figure 6 - Frame 50 of map_1280x720_60_60bit sequence

In short, we have total 10 sequences in both RGB and YUV444 format. In addition, we also have four Class F sequences in the YUV420 format. Thus total 14 sequences are available for SCC test. Since each sequence has different characteristics and is useful to evaluate coding tools for screen content coding, we recommend using all of them in the AHG8 experiments.

## Reference software and Test Conditions

First, we decide to use HM-range-extensions branch (revision 3055, based on HM-9.0.1) as the base software for AhG8.

Regarding test configurations, we recommend to only test AI-main and LP-main to reduce the simulation load and to allow us to investigate the low delay performance, which is one of the important characteristics of screen content coding. The rest of 6 configurations will be optional.

We also have chosen to use QP=17,22,27,32 for all simulation. If lossless coding is tested, QP is set to 22 to allow for fair comparison.

As for the test anchor, we recommend using the base software to encode the 10 RGB sequences in GBR channel order. Thus for these 10 sequences, we need to compute the PSNR and BD rate in RGB domain when evaluating any proposed coding tools. For class F sequences, we use them as direct input to the base software to generate anchor and compute PSNR and BD rate in YUV domain when evaluating any coding tools.

# Related Contributions

The following contributions related to screen content coding are submitted to this JCT-VC meeting:

- JCTVC-L0301, AHG8: New 4:4:4 screen-content sequences for HEVC extension development, , H. Yu (Huawei), Robert Cohen (MERL), W. Gao, J. Ye, Y. Cao and X. Wang (Huawei), Anthony Vetro, Huifang Sun (MERL)

- JCTVC-L0302, AHG8: Results from lossless coding of the 4:4:4 screen content sequences, H. Yu, W. Gao, J. Ye, Y. Cao and X. Wang (Huawei)

- JCTVC-L0303, AHG8: P2M based dual-coder extension of HEVC, T. Lin, S. Wang, P. Zhang, K. Zhou (Tongji Univ.)

- JCTVC-L0313, AHG8: P2M and P2M+SAP as lossless coding tools for screen content coding, P. Zhang, T. Lin, S. Wang, K. Zhou (Tongji Univ.)

- JCTVC-L0317, AHG8: YUV444 and RGB screen content test sequences, S. Wang, T. Lin, K. Zhou (Tongji Univ.)

- JCTVC-L0362, AHG8: Cross-verification of JCTVC-L0313 on P2M and P2M+SAP as lossless coding tools for screen content coding, M. Zhou (TI)

The following five contributions are listed under AHG7 and are also related to AHG8:

- JCTVC-L0117, AHG7: Residual DPCM for HEVC lossless coding, S. Lee, I.-K. Kim, C. Kim (Samsung)

- JCTVC-L0161, AHG7: Intra prediction for lossless coding, Peter Amon, Andreas Hutter, Eugen Wige, André Kaup

- JCTVC-L0176, AHG7: Sample-based angular intra prediction for HEVC range extension, M. Zhou (TI)

- JCTVC-L0275, AHG7: Cross-verification report on JCTVC-L0176 entitled "Sample-based angular intra prediction for HEVC range extension", K.Tokumitsu, K.Chono (NEC)

- JCTVC-L0399, AHG7: Cross-check results of Sample-based angular intra prediction for HEVC range extension (JCTVC-L0176), W. Gao, J. Ye, H. Yu (Huawei)

- JCTVC-L0428, AHG8: Impact of transform skip on new screen content material, R. Cohen, A. Vetro (MERL)

# Recommendation

The Screen Content Coding AHG recommends reviewing all relevant input contributions.