| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0020 |
| --- | --- |

| Title: | CE1: Summary report of HEVC Range Extensions Core Experiment on square and non-square transforms for 4:2:2 chroma format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Joel Sole | Email: | joels@qti.qualcomm.com |
| Source: | CE coordinator |  |  |

_____________________________

# Introduction

A summary of Core Experiment 1 (CE1) on square and non-square transforms for 4:2:2 chroma format is reported. Three methods have been evaluated based on the range extensions common test conditions according to the CE description in JCTVC-K1121.

# Document List

## CE contributions and corresponding cross-checks

- JCTVC-L0182: CE1: Test 1 - Rectangular transform units for 4:2:2 (and AHG7 benchmarks), 	K. Sharman, N. Saunders, J. Gamei (Sony)

  - Cross-check: JCTVC-L0147 (Canon), JCTVC-L0331 (Qualcomm)

- JCTVC-L0146: CE1: Test 2 - Non 32x32 point transforms in chroma for Range Extensions, 	C. Rosewarne, V. Kolesnikov, M. Maeda (Canon)

  - Cross-check: JCTVC-L0320 (Hisilicon)

- JCTVC-L0148: CE1: Test 3 - Square transforms for Range Extensions, 	C. Rosewarne, V. Kolesnikov, M. Maeda (Canon)

  - Cross-check: JCTVC-L0159 (JVC Kenwood)

- JCTVC-L0333: CE1: Test 3 - Square transform blocks for 4:2:2, J. Sole, R. Joshi, M. Karczewicz (Qualcomm)

  - Cross-check: JCTVC-L0159 (JVC Kenwood), JCTVC-L0350 (BBC)

## Related non-CE contributions

- JCTVC-L0149: Non-CE1: Test 3 - Transform skip for chroma in 4:2:2 Range Extension, C. Rosewarne, V. Kolesnikov, M. Maeda (Canon)

- JCTVC-L0150: Non-CE1: Test 3 - Square transform deblocking for 4:2:2, C. Rosewarne, V. Kolesnikov, M. Maeda (Canon)

- JCTVC-L0151: Non-CE1: Test 3 - 32x32 transform in chroma for 4:2:2, V. Kolesnikov, C. Rosewarne, M. Maeda (Canon)

- JCTVC-L0351: Non-CE1: Square transform blocks for 4:2:2, J. Sole, R. Joshi, M. Karczewicz (Qualcomm), A. Gabriellini, M. Mrak (BBC)

# Summary of the tests

HEVC range extensions common conditions (JCTVC-K1006) are used for performance evaluation [1]. Three methods were tested [2]:

Method 1 (anchor): Non-square transforms  (JCTVC-L0182)

- Part of the HEVC range extensions software (rev 3055) and anchor for the other two methods

Method 2: Mix of square and non-square transforms (JCTVC-L0146)

- Avoids the usage of 32-point transforms for chroma by splitting 16x32 blocks in two 16x16 transforms

- Additional results avoiding  32-point transforms for chroma for 4:4:4 chroma formats provided

Method 3: Square transforms  (JCTVC-L0148 and JCTVC-L0333)

- Splits Nx2N rectangular blocks in two NxN transform blocks

- Two implementations for method 3 have been provided: one not supporting transfrom skip (JCTVC-L0148) and one supporting it (JCTVC-L0333)

- A bitstream produced by JCTVC- L0148 implementation can be decoded by JCTVC-L0351 implementation with matching md5sums and vice versa, so the decoders are normatively the same (aside from transform skipping). BD-rate differences are due to differences in the encoder.

# Summary of results

Results are obtained for the Main-tier (QP = 22, 27, 32, 27) and for the High-tier (QP = 17, 22, 27, 32). Cross-checkers confirm the results.

BD-rate for method 2

| YCbCr 4:2:2 | Main Tier |  |  |  | High Tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| All Intra HE |  | 0.1% | 0.9% | 0.9% | 0.1% | 0.5% | 0.5% |
| Random Access HE |  | 0.1% | 1.4% | 1.4% | 0.1% | 0.7% | 0.8% |
| Low Delay B HE |  | 0.1% | 2.2% | 2.4% | 0.1% | 1.0% | 1.3% |

BD-rate for method 3 (transform skip off as in JCTVC-L0148)

| YCbCr 4:2:2 | Main Tier |  |  |  | High Tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| All Intra HE |  | 0.9% | 3.0% | 4.6% | 0.7% | 1.8% | 2.5% |
| Random Access HE |  | 1.0% | 2.4% | 3.3% | 1.1% | 1.6% | 2.4% |
| Low Delay B HE |  | 1.0% | 0.6% | 1.0% | 1.0% | 1.0% | 1.4% |

Note: Anchor with RDOQ on

BD-rate for method 3 (transform skip on as in JCTVC-L0333)

| YCbCr 4:2:2 | Main Tier |  |  |  | High Tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| All Intra HE |  | 0.7% | 3.6% | 5.0% | 0.6% | 2.0% | 2.4% |
| Random Access HE |  | 0.4% | 4.3% | 5.4% | 0.3% | 2.4% | 3.6% |
| Low Delay B HE |  | 0.2% | 3.7% | 4.7% | 0.1% | 2.3% | 3.3% |

BD-rate for method 3 and RDOQ off (transform skip on as in JCTVC-L0333)

| YCbCr 4:2:2 | Main Tier |  |  |  | High Tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| All Intra HE |  | 0.5% | 3.3% | 5.4% | 0.3% | 1.8% | 3.1% |
| Random Access HE |  | 0.4% | 4.6% | 5.7% | 0.3% | 2.5% | 4.1% |
| Low Delay B HE |  | 0.2% | 4.2% | 4.9% | 0.2% | 2.4% | 3.6% |

# Discussion

Method 1 shows better performance than the other two methods. Also, it keeps the HEVC RQT structure. On the other hand, rectangular transforms require the modification of the QP value prior to quantisation and dequantisation and different scaling lists. This is not the case for square transforms. Also, method 2 and 3 do not require the introduction of 32-point transforms in the chroma path.

Irrespective of the transform shape, the three methods use the intra prediction applied to rectangular blocks of the HEVC range extensions software.

Non-CE1 contributions in this meeting propose variants on the above three methods.

# References

- D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions” Joint Collaborative Team on Video Coding, JCTVC-K1006, Shanghai, October 2012.

- J. Sole, “HEVC Range Extensions Core Experiment 1: Square and non-square transforms for 4:2:2 chroma format coding”, Joint Collaborative Team on Video Coding, JCTVC-K1121, October 2012.