| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0021 |
| --- | --- |

| Title: | Summary Report: Tool Experiment A1 (Upsampling Filter) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Tool experiment report |  |  |
| Author(s) or Contact(s): | Andrew Segall (Chair) Jianle Chen Jie Dong Elena Alshina | Email: | asegall@sharplabs.com cjianle@qti.qualcomm.com Jie.Dong@InterDigital.com elena_a.alshina@samsung.com |
| Source: | TE coordinators |  |  |

_____________________________

# Abstract

This document provides a summary of Tool Experiment A1.  The topic of the Tool Experiment is testing Upsampling filters for SHVC.

# Introduction

Between the 11th and 12th meetings of the JCT-VC, multiple proposals for the upsampling filters for the SHVC design were studied.  The purpose of this report is to summarize the activity and report results.

# Participants

| Nr. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 | Andrew Segall (Chair) Jianle Chen (Co-chair) Jie Dong (Co-chair) Elena Alshina (Co-chair) Do-Kyoung Madhukar Budagavi Ankur Saxena Zhan Ma Krishna Rapaka Jinwen Zan Shan Liu Shih-Ta Hsiang Tzu-Der (Peter) Chuang Yu-Wen Huang Zhenzhong Chen Cheung Auyeung Shuo Lu Chuo-Hao Yeo Kiran Misra Patrice Onno Edouard Francois Fabrice Leleannec Yan Ye Xianghui Wei | Sharp Qualcomm Interdigital Samsung TI TI Samsung Samsung Qualcomm Huawei MediaTek MediaTek MediaTek MediaTek MediaTek SONY SONY I2R Sharp Canon Canon Canon Interdigital Huawei | asegall@sharplabs.com cjianle@qti.qualcomm.com Jie.Dong@InterDigital.com elena_a.alshina@samsung.com d-kwon@ti.com madhukar@ti.com asaxena@sta.samsung.com zhan.ma@sta.samsung.com krapaka@qti.qualcomm.com JINWEN.ZAN@huawei.com shan.liu@mediatek.com shih-ta.hsiang@mediatek.com peter.chuang@mediatek.com yuwen.huang@mediatek.com zhenzhong.chen@mediatek.com Cheung.Auyeung@am.sony.com Shuo.Lu@jp.sony.com chyeo@i2r.a-star.edu.sg misrak@sharplab.com Patrice.onno@crf.canon.fr Edouard.francois@crf.canon.fr Fabrice.leleannec@crf.canon.fr Yan.Ye@InterDigital.com weixianghui@huawei.com |  |

# Summary of Proposals

During the TE process, some proposals (or configurations of proposals) were withdrawn.  Given that, the following tests were conducted.

- Test 4.1 (Samsung)

- Test 4.2 (InterDigital)

- Test 4.3 (Qualcomm)

- Test 4.4 (Samsung)

We note that the proponent for Test 4.2 provided results for a 6-bit, 7-bit and 6-bit luma version combined with the chroma filter proposed in JCTVC-K0378.  The 6-bit and 7-bit versions were cross-checked and verified.  The 6-bit luma version combined with the filter proposed in JCTVC-K0378 was provided consistent with the TE time line, but the cross verification was still in progress at the start of the JCT-VC meeting.

# Performance

The performance of the proposals were reported as follows:

| Test | Proponent | Avg. | BD-rate | x2 | Avg. | BD-rate | x1.5 | Enc.Time | Dec.Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # |  | Y | U | V | Y | U | V |  |  |
| test 4.1 | Samsung | -17.1 | -12.4 | -11.9 | -26.9 | -23.4 | -22.3 | 111 | 126 |
| test 4.2_6bit luma + K0378 chroma | InterDigital | -17.1 | -12.4 | -11.9 | -26.9 | -23.5 | -22.4 | 107 | 125 |
| test 4.3 | Qualcomm | -17.1 | -12.4 | -11.9 | -26.9 | -23.4 | -22.3 | 111 | 128 |
| test 4.4 | Samsung | -17.1 | -12.4 | -11.9 | -26.9 | -23.4 | -22.3 | 111 | 124 |

# Properties

Additional information about the proposals was also collected as follows:

| test | Proponent | Doc# | Filter Bit-depth |  | Number of Chroma Filters |
| --- | --- | --- | --- | --- | --- |
| # |  |  | Luma | Chroma |  |
| test 4.1 | Samsung | JCTVC-L0083 | 6 | 6 | 8 |
| test 4.2_6bit luma + K0378 chroma | InterDigital | JCTVC-L0054 | 6 | 6 | 8 |
| test 4.3 | Qualcomm | JCTVC-L0335 | 6 | 5 | 8 |
| test 4.4 | Samsung | JCTVC-L0083 | 6 | 6 | 3 |

# Conclusion

A summary of TE-A1 is provided that includes the performance of the proposed technologies.