| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0022 |
| --- | --- |

| Title: | Tool Experiment 2: Inter-layer Texture Prediction Signaling in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Tool experiment report |  |  |
| Author(s) or Contact(s): | Liwei Guo Yong He Do-Kyoung Kwon Jinwen Zan Haricharan Lakshman Jung Won Kang | Email: | liweig@qti.qualcomm.com Yong.He@InterDigital.com d-kwon@ti.com JINWEN.ZAN@huawei.com haricharan.lakshman@hhi.fraunhofer.de jungwon@etri.re.kr |
| Source: | TE coordinators |  |  |

_____________________________

# Abstract

The purpose of Tool Experiment 2 (TE 2) is to evaluate inter-layer texture prediction signaling. This document provides a summary of activities and results for this tool experiment.

# Introduction

The purpose of Tool Experiment 2 (TE 2) is to evaluate inter-layer texture prediction signaling. This tool experiment test was divided into 3 subtests. In subtest 1, different methods that use a flag to signal inter-layer texture prediction (Intra-BL flag approach) were investigated. In subtest 2, proposals for inserting the baselayer image in the reference frame lists (RefIdx approach) were explored. In subtest 3, a proposal for enhancement layer skipped slice was studied. This report provides a summary of activities and results for this tool experiment.

# Participants

| No. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Liwei Guo Vadim Seregin Jianle Chen Geert Van Der Auwera | Qualcomm | liweig@qti.qualcomm.com vseregin@qti.qualcomm.com cjianle@qti.qualcomm.com geertv@qti.qualcomm.com | Proponent |
| 2 | Yong He Yan Ye Xiaoyu Xiu | InterDigital | Yong.He@InterDigital.com Yan.Ye@InterDigital.com Xiaoyu.Xiu@InterDigital.com | Proponent |
| 3 | Do-Kyoung Kwon Madhukar Budagavi | Texas Instruments | d-kwon@ti.com madhukar@ti.com |  |
| 4 | Jinwen Zan Xianghui Wei Haoping Yu Haitao Yang | Huawei | JINWEN.ZAN@huawei.com weixianghui@huawei.com haoping.yu@huawei.com haitao.yang@huawei.com | Proponent |
| 5 | Haricharan Lakshman | HHI | haricharan.lakshman@hhi.fraunhofer.de |  |
| 6 | Jung Won Kang Jinho Lee Hahyun Lee | ETRI | jungwon@etri.re.kr jinosoul@etri.re.kr hanilee@etri.re.kr | Proponent |
| 7 | Kawamura Kei | KDDI | kei@kddilabs.jp |  |
| 8 | Tmoyuki Yamamoto Takeshi Tsukuba Andrew Segall Kiran Misra Jane Zhao | Sharp | yamamoto.tomoyuki@sharp.co.jp tsukuba.takeshi@sharp.co.jp asegall@sharplabs.com misrak@sharplabs.com jzhao@sharplabs.com | Proponent |
| 9 | Wenhao Zhang | Intel | wenhao.zhang@intel.com | Proponent |
| 10 | Edouard Francois Fabrice Le Léannec Patrice Onno | Canon | Fabrice.leleannec@crf.canon.fr Edouard.francois@crf.canon.fr Patrice.onno@crf.canon.fr | Proponent |
| 11 | Tzu-Der Chuang Yu-Wen Huang Shan Liu Polin Lai Shih-Ta Hsiang Ximin Zhang | MediaTek | Peter.chuang@mediatek.com yuwen.huang@mediatek.com shan.liu@mediatek.com polin.lai@mediatek.com shih-ta.hsiang@mediatek.com ximin.zhang@mediatek.com | Proponent |
| 12 | Chulkeun Kim Joonyoung Park Jungsun Kim | LG | chulkeun.kim@lge.com jy.park@lge.com jungsun.kim@lge.com | Proponent |
| 13 | O. Bici D. Bugdayci K. Ugur J. Lainema | Nokia | ext-oguz.bici@nokia.com ext-done.bugdayci@nokia.com Kemal.Ugur@nokia.com Jani.Lainema@nokia.com | Proponent |
| 14 | Kazushi Sato J. Xu S. Lu | Sony | Kazushi.Sato@jp.sony.com jun.xu@am.sony.com Shuo.Lu@jp.sony.com |  |
| 15 | Ankur Saxena Zhan Ma Tammy Lee Elena Alshina | Samsung | asaxena@sta.samsung.com zhan.ma@sta.samsung.com tammy.lee@samsung.com elena_a.alshina@samsung.com | Proponent |
| 16 | Y. H. Tan | I2R | yhtan@i2r.a-star.edu.sg |  |

# Summary of Proposals and Results

## Intra-BL flag approach

Table 1 Documents list for Subtest 1

| Test | Proposal No. | Proposal Title | Cross-checker |
| --- | --- | --- | --- |
| 3.1.1 (Qualcomm) | JCTVC-L0284 | TE2: Results of Test 3.1.1 on Inter-layer Texture Prediction Signaling | ETRI (JCTVC-L0061) Huawei (JCTVC-L0295) |
| 3.1.2 (Withdraw) |  |  |  |
| 3.1.3 (Huawei) | JCTVC-L0165 | TE2: Results of test 3.1.3 on CU based IntraBL signaling | KDDI(JCTVC-L0388) |
| 3.1.4 (Huawei) | JCTVC-L0164 | TE2: Results of test 3.1.4 on CU based IntraBL signaling with skip mode | InterDigital (JCTVC-L0214) |
| 3.1.5 (Intel) | JCTVC-L0285 | TE A2: Signalling of Inter-layer Skip and Direct | Sharp (JCTVC-L0285) |
| 3.1.6 (Withdraw) |  |  |  |
| 3.1.7 (MediaTek&LG) | JCTVC-L0069 | TE2-3.1.7: Inter-layer intra prediction signalling with residual skip flag | KDDI(JCTVC-L0387) |
| 3.1.8 (Nokia) | JCTVC-L0035 | TE2: Results of test 3.1.8 on PU based Intra BL signaling | LG (JCTVC-L0281) |
| 3.1.9 (Samsung) | JCTVC-L0110 | TE2 Subtest 1: Inter-layer Texture Prediction Signalling (test 3.1.9) | Nokia (JCTVC-L0130) MediaTek (JCTVC-L0341) |

In this section, proposals use a flag at CU or PU level to signal the inter-layer texture prediction.

Some common designs that are found in multiple proposals can be summarized as follows:

- Signal the flag at CU level (while there is one proposal signaling at PU level)

- Signal Intra-BL Skip flag

- Use DCT or DST for Intra-BL 4x4

- Use BS=2 or BS=1 for deblocking of Intra-BL unit

- Use cbf_root_flag to encode Intra-BL prediction residues

Table 2 Summary of technical details for tools in Subtest 1

| Test | Changes in Intra-BL flag coding |
| --- | --- |
| 3.1.1 (Qualcomm) | - Signaling level: CU - Intra-BL Skip: Yes - Intra-BL 4x4 transform: DCT (test A), DST (test B) - Deblocking BS: 1 (Intra-BL), 0 (Intra-BL Skip) - cbf_root_flag coding: No |
| 3.1.3 (Huawei) | - Signaling level: CU - Intra-BL Skip: No - Intra-BL 4x4 transform: DCT - Deblocking BS: 2 - cbf_root_flag coding: Yes |
| 3.1.4 (Huawei) | - Signaling level: CU - Intra-BL Skip: Yes - Intra-BL 4x4 transform: DCT - Deblocking BS: 2 - cbf_root_flag coding: Yes |
| 3.1.5 (Intel) | - Signaling level: CU - Intra-BL Skip: Yes - Intra-BL 4x4 transform: DCT(forward) / DST (backward) - Deblocking BS: 2 - cbf_root_flag coding: No |
| 3.1.7 (MediaTek & LG) | - Signaling level: CU - Intra-BL Skip: No - Intra-BL 4x4 transform: DST - Deblocking BS: 1 - cbf_root_flag coding: Yes |
| 3.1.8 (Nokia) | - Signaling level: PU except that no Intra-BL is allowed for: 1. Intra NxN PU; 2. the second unit in Nx2N/2NxN/AMP when the first one is Intra-BL. - Intra-BL Skip: Yes – LCU in I slice - Intra-BL 4x4 transform: DST (I slice), DCT(P/B slice) - Deblocking BS: 2 (I slice), 1(P/B slice) - cbf_root_flag coding: No (I slice), Yes (P/B slice) |
| 3.1.9 (Samsung) | - Signaling level: CU - Intra-BL Skip: Yes (test A), No (test B) - Intra-BL 4x4 transform: DCT - Deblocking BS: 2 - cbf_root_flag coding: Yes |

Experimental results are shown in the following table where BD-rate is calculated based on EL+BL bit rates. The anchor is HM 8.1 Simulcast. For the reference, results of SMuC 0.1.1 and SMuC 0.1.1 with Intra-BL 4x4 bug fix (two different fixes: using DCT and using DST) are also provided.

Table 3 Summary of results for tools in Subtest 1 (Anchor HM8.1 Simulcast)

|  |  | Avg. AI BD-rate (2X, 1.5X) |  |  |  |  | Avg. RA/LDP BD-rate (2X, 1.5X) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Proponent | Y | U | V | Enc. Time | Dec. Time | Y | U | V | Enc. Time | Dec.Time |
| 3.1.1 | Qualcomm_A | -28.0% | -28.1% | -28.2% | 115.2% | 101.1% | -19.3% | -12.6% | -11.5% | 108.8% | 130.8% |
|  | Qualcomm_B | -28.0% | -28.0% | -28.2% | 116.4% | 102.9% | -19.3% | -12.6% | -11.5% | 110.9% | 132.0% |
| 3.1.3 | Huawei | -27.9% | -27.8% | -28.0% | 105.5% | 88.9% | -19.4% | -12.5% | -11.4% | 106.6% | 118.7% |
| 3.1.4 | Huawei | -28.0% | -27.7% | -27.9% | 106.5% | 88.9% | -19.4% | -12.7% | -11.5% | 106.7% | 118.4% |
| 3.1.5 | Intel | -27.7% | -27.9% | -28.0% | 114.2% | 92.4% | -19.4% | -12.6% | -11.4% | 111.7% | 119.2% |
| 3.1.7 | MTK&LG | -28.0% | -28.0% | -28.2% | 110.3% | 88.9% | -19.4% | -12.7% | -11.5% | 106.9% | 117.2% |
| 3.1.8 | Nokia | -27.9% | -27.9% | -28.0% | 116.1% | 105.3% | -19.8% | -12.5% | -11.3% | 108.8% | 130.4% |
|  | Nokia (Intra-BL Skip only) | -27.7% | -28.0% | -28.0% | 122.7% | 138.6% |  |  |  |  |  |
| 3.1.9 | Samsung_A | -28.0% | -27.8% | -28.0% | 114.0% | 92.6% | -19.5% | -12.6% | -11.5% | 109.5% | 123.5% |
|  | Samsung_B | -27.9% | -27.8% | -28.0% | 115.5% | 93.0% | -19.4% | -12.5% | -11.4% | 109.5% | 121.2% |
|  | SMuC0.1.1 | -27.7% | -28.0% | -28.0% | 113.7% | 105.9% | -19.2% | -12.9% | -11.7% | 109.3% | 136.9% |
|  | SMuC0.1.1+DCT | -27.8% | -27.9% | -28.0% |  |  | -19.2% | -12.9% | -11.8% |  |  |
|  | SMuC0.1.1+DST | -27.9% | -27.9% | -28.0% |  |  | -19.2% | -12.9% | -11.7% |  |  |

## RefIdx Approach

Table 4 Documents list for Subtest 2

| Test | Proposal No. | Proposal Title | Cross-checker |
| --- | --- | --- | --- |
| 3.2.1 (InterDigital) | JCTVC-L0051 | TE2: Results of test 3.2.1 on inter-layer reference picture placement | Qualcomm (JCTVC-L0261) ETRI (JCTVC-L0062) Samsung (JCTVC-L0090) |
| 3.2.2 (ETRI) | JCTVC-L0060 | TE2 : Results of test 3.2.2 on inter-layer reference picture placement | Intel (JCTVC-L0306) and InterDigital (JCTVC-L0201) |
| 3.2.3 (Sharp) | JCTVC-L0251 | TE2-3.2.3 Infer Zero motion vector when referencing co-located base layer picture | TI (JCTVC-L0353) Canon (JCTVC-L0382) |

In this section, proposals insert up-sampled baselayer image as a reference picture. Inter-layer texture prediction is achieved by predicting from this so-called inter-layer reference picture (ILRP).

Some common designs that are found in multiple proposals can be summarized as follows:

- Adaptive placement of ILRP

- Use zero-MV for ILRP; the zero-MV may be signaled or inferred.

Table 5 Summary of technical details for tools in Subtest 2

| Test |  |
| --- | --- |
| 3.2.1 (InterDigital) | 5 different tests: - A: also called setting1 – RefIdx approach in SMuC 0.1.1 - B: also called setting2 – for temporal layer 0, ILRP inserted at last positions in both L0 and L1; for other temporal layers, ILRP is only inserted in L0 - C: B + forced uni- prediction for ILRP (encoder only constraint) - D: B + forced zero-MV prediction for ILRP (encoder only constraint) - E: B + forced zero-MV uni-prediction for ILRP (encoder only constraint) |
| 3.2.2 (ETRI) | - RefIdx assignment for ILRP: 1 for non-random access picures and 0 for random access pictures - Reference List assignment for ILRP: L0 and L1 for AI and RA, only L0 for LD-P |
| 3.2.3 (Sharp) | - Inferred zero-MV for ILRP (normative) |

Experimental results are shown in the following table where BD-rate is calculated based on EL+BL bit rates.

Table 6 Summary of results for tools in Subtest 2 (Anchor HM8.1 Simulcast)

|  |  | Avg. AI BD-rate (2X, 1.5X) |  |  |  |  | Avg. RA/LDP BD-rate (2X, 1.5X) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Proponent | Y | U | V | Enc. Time | Dec. Time | Y | U | V | Enc. Time | Dec.Time |
| 2.1 | InterDigital_A | -27.8% | -27.4% | -27.5% | 219.6% | 93.7% | -18.7% | -11.3% | -9.9% | 112.2% | 130.9% |
|  | InterDigital _B | -27.8% | -27.4% | -27.5% | 215.7% | 89.3% | -19.2% | -12.2% | -10.9% | 107.9% | 118.3% |
|  | InterDigital _C | -27.8% | -27.4% | -27.5% | 136.0% | 89.0% | -19.0% | -12.1% | -10.9% | 108.4% | 124.7% |
|  | InterDigital_D | -28.0% | -27.6% | -27.7% | 195.7% | 88.5% | -19.3% | -12.4% | -11.2% | 100.0% | 118.0% |
|  | InterDigital_E | -28.0% | -27.6% | -27.7% | 193.9% | 91.3% | -19.1% | -12.3% | -11.1% | 98.5% | 122.6% |
| 2.2 | ETRI | -27.8% | -27.4% | -27.5% | 239.9% | 87.1% | -18.8% | -11.1% | -9.7% | 108.5% | 116.5% |
| 2.3 | Sharp | -28.0% | -27.6% | -27.7% | 205.9% | 92.8% | -18.9% | -11.5% | -10.1% | 108.4% | 129.1% |
|  | SMuC 0.1.1 | -27.7% | -28.0% | -28.0% | 113.7% | 105.9% | -19.2% | -12.9% | -11.7% | 109.3% | 136.9% |
|  | SMuC0.1.1+DCT | -27.8% | -27.9% | -28.0% |  |  | -19.2% | -12.9% | -11.8% |  |  |
|  | SMuC0.1.1+DST | -27.9% | -27.9% | -28.0% |  |  | -19.2% | -12.9% | -11.7% |  |  |

### Inter- Slice-level approach

Table 7 Documents list for Subtest 3

| Test | Proposal No. | Proposal Title | Cross-checker |
| --- | --- | --- | --- |
| 3.3.1 (InterDigital) | JCTVC-L0055 | TE2: Results of test 3.3.1 Enhancement layer skipped slice | Huawei (JCTVC-L0296) |

In this section, skipping an entire slice in the enhancement layer is studied. The up-sampled base layer reconstructed picture is directly used as the enhancement layer reconstruction.

Table 8 Summary of technical details for tools in Subtest 3

| Test |  |
| --- | --- |
| 3.3.1 (InterDigital) | Skip entire enhancement layer slice, reconstructed slice is up-sampled base layer image - Test A: with SMuC Intra-BL flag scheme - Test B: with SMuC RefIdx scheme |

Experimental results are shown in the following table where BD-rate is calculated based on EL+BL bit rates.

Table 9 Summary of results for tools in Subtest 3 (Anchor HM8.1 Simulcast)

|  |  | Avg. AI BD-rate (2X, 1.5X) |  |  |  |  | Avg. RA/LDP BD-rate (2X, 1.5X) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Proponent | Y | U | V | Enc. Time | Dec. Time | Y | U | V | Enc. Time | Dec.Time |
| 3.3.1 | InterDigital_A | -27.7% | -28.0% | -28.0% | 105.2% | 93.4% | -19.2% | -12.9% | -11.7% | 90.1% | 133.2% |
|  | InterDigital_B | -27.8% | -27.4% | -27.5% | 204.1% | 81.2% | -18.7% | -11.3% | -9.9% | 85.4% | 124.1% |
|  | SMuC 0.1.1 | -27.7% | -28.0% | -28.0% | 113.7% | 105.9% | -19.2% | -12.9% | -11.7% | 109.3% | 136.9% |
|  | SMuC0.1.1+DCT | -27.8% | -27.9% | -28.0% |  |  | -19.2% | -12.9% | -11.8% |  |  |
|  | SMuC0.1.1+DST | -27.9% | -27.9% | -28.0% |  |  | -19.2% | -12.9% | -11.7% |  |  |

# Conclusion

In this TE, a number of tools related to inter-layer texture prediction signaling were studied. This report summarizes the technical features and the test results of these tools.

# Appendix – list of non-TE proposals

| Proposal No. | Proposal Title | Cross-checker |
| --- | --- | --- |
| JCTVC-L0174 | Non-TE2: Inter-layer reference picture placement | Qualcomm (JCTVC-L0261) |
| JCTVC-L0230 | Non-TE2: Improved inter-layer texture prediction signaling | Samsung (JCTVC-L0236) |
| JCTVC-L0134 | Non-TE 2: Constrained intra prediction at enhancement layer | MediaTek(JCTVC-L0345) |