| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14– 21 January 2013 |  |  | Document: JCTVC-L0024 |  |
| --- | --- | --- | --- | --- |
| Title: | TE-B4: Summary report of inter-layer filtering for SHVC |  |  |  |
| Status: | Input Document |  |  |  |
| Purpose: | Tool Experiment Summary |  |  |  |
| Author(s) or Contact(s): | Jianle Chen (primary) Andrew Segall Elena Alshina Shan Liu Jie Dong Joonyoung Park | Tel: Email: |  | +1-858-651-8028 cjianle@qti.qualcomm.com asegall@sharplabs.com elena_a.alshina@samsung.com shan.liu@mediatek.com jie.dong@interdigital.com jy.park@lge.com |
| Source: | TE coordinators |  |  |  |

_____________________________

# Abstract

This document summarizes the activities and test results performed in TE-B4 on inter-layer filtering. Nine tools from 7 companies have been evaluated on the test conditions defined in document JCTVC-K1104.

# Introduction

During the 11th JCT-VC meeting in Shanghai, multiple proposals for selectable, adaptive or additional inter-layer filters for the SHVC design were proposed. It was then decided to coordinate for further study and testing of these filters in this Tool Experiment (TE).

The goal of this TE is to evaluate the coding efficiency and complexity impact of:

- switchable/adaptive up-sampling filter

- An additional inter-layer filter

Five subtests were defined to evaluate the 9 tools of this TE.

- TE4.1 Switchable up-sampling filter

- TE4.2 Adaptive up-sampling filter

- TE4.3 Interlayer Sample Adaptive Offset filter

- TE4.4 Interlayer adaptive filter

- TE4.5 Interlayer de-blocking

# Participants List

Participants from 13 organizations listed in the table below participate the activities of this TE.

| Nr. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Krishna Rapaka Liwei Guo Wei Pu Jianle Chen Marta Karczewicz | Qualcomm | krapaka@qti.qualcomm.com liweig@qti.qualcomm.com wpu@qti.qualcomm.com cjianle@qti.qualcomm.com martak@qti.qualcomm.com | P, C |
| 2 | Andrew Segall Tomoyuki Yamamoto Kiran Misra Zhao, Jie | Sharp | asegall@sharplabs.com yamamoto.tomoyuki@sharp.co.jp misrak@sharplabs.com jzhao@sharplabs.com | P, C |
| 3 | Elena Alshina Ankur Saxena | Samsung | elena.alshina@gmail.com elena_a.alshina@samsung.com asaxena@sta.samsung.com | P, C |
| 4 | Shang Liu Mei Guo Ximin Zhang Polin Lai Shih-Ta Hsiang Tzu-Der (Peter) Chuang Yu-Wen Huang CW Hsu ChingYeh Chen Chiayang Tsai Zhenzhong Chen | MediaTek | shan.liu@mediatek.com mei.guo@mediatek.com ximin.zhang@mediatek.com polin.lai@mediatek.com shih-ta.hsiang@mediatek.com peter.chuang@mediatek.com yuwen.huang@mediatek.com cw.hsu@mediatek.com chingyeh.chen@mediatek.com chiayang.tsai@mediatek.com zhenzhong.chen@mediatek.com | P, C |
| 5 | Jie Dong Yuwen He Yan Ye | Interdigital | jie.dong@interdigital.com yuwen.he@interdigital.com yan.ye@interdigital.com | P, C |
| 6 | Le Leannec Fabrice Francois Edouard Onno Patrice | Canon | fabrice.leleannec@crf.canon.fr edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr | P, C |
| 7 | Madhukar Budagavi Do-Kyoung Kwon | TI | madhukar@ti.com d-kwon@ti.com |  |
| 8 | Chulkeun Kim Joonyoung Park | LGE | chulkeun.kim@lge.com jy.park@lge.com | P, C |
| 9 | Yi-jen Chiu Wenhao Zhang | Intel | yi-jen.chiu@intel.com wenhao.zhang@intel.com | P, C |
| 10 | Jinwen Zan Haitao Yang Haoping Yu | Huawei | jinwen.zan@huawei.com haitao.yang@huawei.com haoping.yu@huawei.com |  |
| 11 | Lu, Shuo Ikeda, Masaru Auyeung, Cheung | Sony | shuo.lu@jp.sony.com Masaru.Ikeda@am.sony.com Cheung.Auyeung@am.sony.com |  |
| 12 | Tan Yih Han | I2R | yhtan@i2r.a-star.edu.sg |  |
| 13 | Jan De Cock Sebastiaan Vanleuven Glenn Van Wallendael | Ugent | jan.decock@ugent.be sebastiaan.vanleuven@ugent.be Glenn.VanWallendael@elis.UGent.be |  |

# List of Contributions

Nine tools from 7 companies listed in the table below were evaluated and one test was withdrawn.

| Test | Proposal documents | Proposal Title | Cross-checking documents |
| --- | --- | --- | --- |
| TE4.1 | ## TE4.1 Switchable up-sampling filters |  |  |
| 4.1.1 | JCTVC-L0056 | TE4: Results of test 4.1.1 on selectable up-sampling filter (SUF) | JCTVC-L0339 |
| 4.1.2 | JCTVC-L0198 | TE4: Result of Test of Intra Mode Dependent Directional Filter | JCTVC-L0091 |
| TE4.2 | Adaptive up-sampling filter |  |  |
| 4.2.1 | JCTVC-L0309 | TE-B4: Results of test 4.2.1 on adaptive up-sampling filter | JCTVC-L0092 |
| 4.2.2 | withdraw |  |  |
| TE4.3 | Interlayer SAO |  |  |
| 4.3.1 | JCTVC-L0212 | TE4.3.1: Interlayer SAO | JCTVC-L0307 |
| 4.3.2 | JCTVC-L0085 | TEB4: Inter-layer SAO (test 4.3.2) | JCTVC-L0338 |
| TE4.4 | Interlayer adaptive filter |  |  |
| 4.4.1 | JCTVC-L0268 | TE4: Inter-layer adaptive filter | JCTVC-L0058 |
| 4.4.2 | JCTVC-L0075 | TE4.4: Inter-layer adaptive filter on up-sampled BL | JCTVC-L0218 |
| 4.4.3 | JCTVC-L0287 | TE B4: Inter-layer adaptive refine filtering | JCTVC-L0216 |
| TE4.5 | Interlayer de-blocking |  |  |
| 4.5.1 | JCTVC-L0133 | TE 4: Results of test on 4.5.1 Inter layer filtering of boundary between EL and BL | JCTVC-L0343 |

# Summary of Techniques

## Switchable up-sampling filters

In this subtest, up-sampling filter can be switched among a set of filters which are hard-coded in the system. In addition to the existing 8-tap filter, 2D non-separable up-sampling filters are employed by the proposed methods. The filter selection is indicated by an index signalled in slice head or derived based on the side information, such as intra prediction mode.

Two tools are evaluated in this subtest. The main features of each test are summarized in the table below.

| Test | Technique Summary |
| --- | --- |
| 4.1.1 JCTVC-L0056 | - Switch between 2D non-separable 7x7/6x7/7x6/6x6 (depending on phase) up-sampling filter and the existing 8-tap filter - 4 sets of additional non-separable 2D filters - Filter coefficient has 8 bits accuracy - Filter index is signaled in slice head - Only applies to Luma component in spatial scalability case - Both filter with symmetric and asymmetric coefficients are tested |
| 4.1.2 JCTVC-L0198 | - 2D non-separable 8x8/8x7/7x7 Intra-Mode-Dependent-Directional-Filters (IMDDF) is used to up-sample base layer intra mode coded blocks. Filter is determined based on directional intra prediction mode - Base-layer inter mode coded block is up-sampled by using the existing 8-tap filter - Slice level on/off - Only applies to Luma component in spatial scalability case |

## Adaptive up-sampling filters

In this subtest, the proposed tool derives the optimal up-sampling filter for each picture at encoder side and signaled it in the bit-stream. Only one tool is tested and its main features are summarized in the table below.

| Test | Technique Summary |
| --- | --- |
| 4.2.1 JCTVC-L0309 | - Up-sampling filter is switchable between adaptive filter and the existing filter - Adaptive filter has same length, coefficient accuracy with the existing filter - Filtering process is also applied to integer pixel position - Filter coefficients are signaled at picture level - Only applies to Luma component |

## Interlayer SAO

Inter layer Sample Adaptive Offset (SAO) is evaluated in this subtest. The proposed methods apply SAO as inter-layer filter, which is placed after the up-sampling filter to minimize the difference between up-sampled (if needed) reconstructed base layer and original enhancement layer picture.

Two tools are evaluated in this subtest. The main features of each test are summarized in the table below.

| Test | Technique Summary |
| --- | --- |
| 4.3.1 JCTVC-L0212 | - Picture level SAO - Each color component of one picture can be treated as one region or be split into four regions - Each region selects one SAO type - SAO parameters are coded in the first slice head of a picture |
| 4.3.2 JCTVC-L0085 | - Picture level SAO - 5 SAO types including 4 directions of Edge Offset and one Band Offset filters are sequentially applied to the up-sampled(if needed) base layer samples. - SAO parameters are coded in the first slice head of a picture |

## Interlayer adaptive filter

Inter layer adaptive filter is evaluated in this subtest. The proposed methods apply adaptive filter as inter-layer filter, which is placed after the up-sampling filter to minimize the distortion between up-sampled (if needed) reconstructed base layer and original enhancement layer picture. The proposed tool derives the optimal adaptive filter for each picture at encoder side and signaled it in the bit-stream.

Three tools are evaluated in this subtest. The main features of each test are summarized in the table below.

| Test | Technique Summary |
| --- | --- |
| 4.4.1 JCTVC-L0268 | - Filter shape: 5x5 with symmetric coefficients - Quad-tree based on/off signaling - Applied for both Luma and Chroma with independent on/off flag - Filter parameters are coded in the first slice head of a picture |
| 4.4.2 JCTVC-L0075 | - Filter shape: 7x7 Cross + 3x3 Square with symmetric coefficients - Single filter for each color component of one picture - Filter parameters are coded in the first slice head of a picture |
| 4.4.3 JCTVC-L0287 | - Filter shape: 5x5 Cross + 3x3 Square with symmetric coefficients - One filter for Luma component and one filter for Chroma component of one picture. - CU level on/off - Filter parameters are coded in the first slice head of a picture Filter |

## Interlayer de-blocking filter

In this test, smoothing filter is proposed for removing blocking artifacts between the prediction block from the base layer and the neighboring reconstructed region in the enhancement layer. Only one tool is tested and its main features are summarized in the table below.

| Test | Technique Summary |
| --- | --- |
| 4.5.1 JCTVC-L0133 | - Smoothing filtering across prediction block from the base layer and the neighboring reconstructed region in the enhancement layer - Boundary filtering on/off decision method is similar to that of de-blocking filter for Luma component in HEVC - A new simple smoothing filter is applied when boundary smoothing is on |

# Summary of Results

The test condition for this TE was specified in JCTVC-K1104, where all test configurations are mandatory to be tested. The BD-rate is calculated based on EL+BL bit rates with anchor as SMuC v0.1.1. The results of all TE-B4 tests are summarized in the table below. The detailed results are provided in the attached excel sheets.

| Test | Avg. BD-rate spatial (2X, 1.5X) |  |  |  |  | Avg. BD-rate SNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc. Time | Dec. Time | Y | U | V | Enc. Time | Dec. Time |
| 4.1.1 | -0.3% | 0.0% | 0.0% | 103% | 111% | 0.0% | 0.0% | 0.0% | 102% | 106% |
|  | -0.3% | 0.0% | 0.0% | 102% | 116% | 0.0% | 0.0% | 0.0% | 101% | 104% |
| 4.1.2 | -0.1% | -0.1% | 0.0% | 101% | 102% | 0.0% | 0.0% | 0.0% | 101% | 101% |
| 4.2.1 | -0.5% | 0.1% | 0.1% | 102% | 100% | -2.5% | 0.1% | 0.3% | 102% | 126% |
| 4.3.1 | -0.2% | -0.4% | -0.4% | 100% | 103% | -0.4% | -0.8% | -0.8% | 100% | 105% |
| 4.3.2 | -0.3% | -0.1% | -0.1% | 100% | 109% | -0.4% | 0.0% | 0.1% | 100% | 111% |
| 4.4.1 | -0.4% | -0.7% | -0.5% | 106% | 188% | -2.9% | -1.4% | -0.9% | 104% | 212% |
| 4.4.2 | -0.3% | -0.5% | -0.2% | 100% | 102% | -2.7% | -1.6% | -1.1% | 100% | 104% |
| 4.4.3 | -0.2% | -0.5% | -0.4% | 108% | 98% | -2.3% | -1.3% | -1.0% | 104% | 102% |
| 4.5.1 | -0.1% | 0.0% | 0.0% | 101% | 101% | -0.1% | 0.0% | 0.0% | 101% | 102% |

# List of non-TE proposals

The related non-TE proposals are listed in the table below.

| Proposal documents | Proposal Title | Authors |
| --- | --- | --- |
| JCTVC-L0076 | Non-TE4.4: Inter-layer adaptive filter on upsampled BL with CU on/off flags | C.-Y. Chen, C.-Y. Tsai, M. Guo, S. Liu, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-L0086 | non-TEB4: Band Offset correction for inter-layer texture prediction | E.Alshina, A.Alshin, J.H.Park (Samsung) |
| JCTVC-L0087 | On integer samples filtering during up-sampling | E.Alshina, A.Alshin, J.H.Park (Samsung) |
| JCTVC-L0107 | Inter Layer SAO | G. Laroche, T. Poirier, C. Gisquet, E. François, P. Onno (Canon) |
| JCTVC-L0195 | Non-TE4: Switchable De-ringing Filter for Inter-layer Prediction | Z. Ma, F. Fernadnes (Samsung) |
| JCTVC-L0233 | Multi-Type Inter Layer Sample Adaptive Offset Filter | W. Pu, J. Chen, K. Rapaka, M. Karczewicz (Qualcomm) |
| JCTVC-L0234 | High Frequency Pass Inter Layer Sample Adaptive Offset Filter | W. Pu, J. Chen, K. Rapaka, M. Karczewicz (Qualcomm) |
| JCTVC-L0252 | Non-TE4: Adaptive up-sampling of base layer picture using bilateral filters | J. Zhao, K. Misra, A. Segall (Sharp) |
| JCTVC-L0256 | Non-TE4: Inter-layer Adaptive Filters | M. Guo, P. Lai, S. Liu, C.-Y. Tsai, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-L0283 | Non-TE4: Inter-layer SAO | Z. Chen, S. Liu, X. Zhang, S.-T. Hsiang, C.-M. Fu, S. Lei (MediaTek) |
| JCTVC-L0290 | Non-TE B4: Inter-layer fixed refining filter | W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel) |

# Conclusion

In this TE, a number of tools related to inter-layer filtering process were studied.