| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0025 |
| --- | --- |

| Title: | TE5: Summary report of tool experiment on inter-layer syntax prediction using HEVC base layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Patrice Onno Shan Liu Tammy Lee Chulkeun Kim Haitao Yang Haricharan Laksman | Email: | vseregin@qti.qualcomm.com patrice.onno@crf.canon.fr shan.liu@mediatek.com tammy.lee@samsung.com chulkeun.kim@lge.com haitao.yang@huawei.com haricharan.lakshman@hhi.fraunhofer.de |
| Source: | TE coordinators |  |  |

_____________________________

# Introduction

This contribution summarizes the activities and test results performed in TE5 on inter-layer syntax prediction using HEVC base layer. The tools in TE5 were classified into four categories:

- TE5.1 Inter-layer intra mode prediction.

- TE5.2 Inter-layer motion prediction including merge and AMVP modes.

- TE5.3 Inter-layer inferred prediction mode (also known as InterBL or Base Mode). In this mode, co-located base layer block is divided into sub-blocks and motion information associated with each sub-block is used to form a prediction of the corresponding block in the enhancement layer.

- TE5.4 Motion mapping for HLS approach (also known as ”ref_idx” approach), where reconstructed upsampled base layer picture is inserted into DPB buffer and there is no low level block modifications.

# Document list

| Test | Proposal | Cross-checking documents |
| --- | --- | --- |
| TE5.1 | ## Inter-layer intra mode prediction |  |
| 5.1.1 | JCTVC-L0259 (MediaTek, LGE) “TE5.1.1: Inter-layer Intra mode prediction) | JCTVC-L0125 (Nokia) JCTVC-L0172 (Leibniz University Hannover) |
| 5.1.2 | JCTVC-L0224 (Qualcomm) “TE5 : Results of Test 5.1.2 on Inter-Layer Intra Mode Prediction” | JCTVC-L0359 (MediaTek) |
| 5.1.3 | JCTVC-L0031 (Nokia) “TE5: Results of test 5.1.3 on inter-layer intra mode prediction” | JCTVC-L0238 (Qualcomm) |
| 5.1.4 | JCTVC-L0113 (Samsung) “TE5 Subtest 1: Inter-layer intra mode prediction (test 5.1.4)” | JCTVC-L0242 (LGE) JCTVC-L0360 (MediaTek) |
| 5.1.5 | JCTVC-L0153 (Leibniz University Hannover) “TE5: Test Results of TE5.1.5” JCTVC-L0156 (Leibniz University Hannover) “Simplification of TE5.1.5 on inter-layer intra mode prediction” | JCTVC-L0160 (JVC Kenwood) JCTVC-L0163 (JVC Kenwood) |
| TE5.2 | Inter-layer motion vector prediction |  |
| 5.2.1 | JCTVC-L0070 (MediaTek, LGE) “TE5-5.2.1: Inter-layer motion vector prediction” | JCTVC-L0394 (Canon) |
| 5.2.2 | JCTVC-L0205 (Qualcomm) “TE5: Using base layer MV in Merge and AMVP modes (test 5.2.2)” | JCTVC-L0121 (Nokia) |
| 5.2.3 | JCTVC-L0065 (ETRI) “TE5: Results of test 5.2.3 on inter-layer motion vector prediction” | JCTVC-L0203 (InterDigital) JCTVC-L0274 (Sony) |
| 5.2.4 | JCTVC-L0288 (Intel) “TE C5: Inter-layer motion prediction” | JCTVC-L0168 (Huawei) |
| 5.2.5 | JCTVC-L0032 (Nokia) “TE5: Results of 5.2.5 on inter-layer motion vector prediction” | JCTVC-L0207 (Qualcomm) |
| 5.2.6 | JCTVC-L0101 (Canon) “TE5: Results of test 5.2.6 on the Inter-Layer Motion Vector Prediction” | JCTVC-L0344 (MediaTek) |
| 5.2.7 | JCTVC-L0164 (Huawei) “TE5: Results of TE5 tools from Huawei” | JCTVC-L0310 (Intel) |
| 5.2.8 | JCTVC-L0376 (KDDI) “TE5: Results of test 5.2.8 on inter-layer motion-vector prediction by the base-layer MV up-scaling and refinement using HEVC base layer“ | JCTVC-L0300 (Sharp) |
| 5.2.9 | JCTVC-L0297 (Sharp) “TE5: Results of test 5.2.9 on inter-layer motion prediction” | JCTVC-L0389 (KDDI) |
| 5.2.10 | JCTVC-L0185 (Vidyo, Samsung) “TE5: Results of test 5.2.10 on inter-layer motion vector prediction” |  |
| TE5.3 | Inter-layer inferred prediction mode (InterBL mode) |  |
| 5.3.1 | JCTVC-L0266 (Qualcomm) “TE5: Results of Test 5.3.1 on Inter-layer Inferred Prediction Mode” | JCTVC-L0066 (Samsung) JCTVC-L0142 (LGE) JCTVC-L0066 (ETRI) |
| 5.3.2 | JCTVC-L0289 (Intel) “TE C5: Inter-layer pattern/mode prediction” | JCTVC-L0168 (Huawei) |
| 5.3.3 | JCTVC-L0102 (Canon) “TE5: Results of test 5.3.3 on the Inter-Layer Prediction Mode (Base Mode)” | JCTVC-L0093 (Samsung) |
| 5.3.4 | JCTVC-L0115 (Samsung) “TE5 Subtest 3: Inter-layer inferred prediction mode (test 5.3.4)” | JCTVC-L0395 (Canon) |
| 5.3.5 | JCTVC-L0164 (Huawei) “TE5: Results of TE5 tools from Huawei” | JCTVC-L0311 (Intel) |
| TE5.4 | Motion mapping |  |
| 5.4.1 | JCTVC-L0052 (InterDigital) “TE5: Results on test 5.4.1 on motion field mapping” | JCTVC-L0168 (Huawei) |

# Summary

- TE5.1 Inter-layer intra mode prediction

In this section, proposed methods use intra prediction direction from the base layer (BL) in the intra mode coding of the enhancement layer (EL). Base layer intra mode can be used as one of the most probable modes or intra mode predictor in the enhancement layer. However, due to mode dependent coefficient scanning (MDCS) there is a dependency in EL coefficient parsing since intra mode of the base layer has to be derived and is involved at parsing stage. So, to avoid this dependency additional results are provided where MDCS is disabled for the EL intra-coded blocks.

The most common parts of the proposed methods can be summarized as following:

- Use BL intra direction as an MPM mode or intra mode predictor

- Disable MDCS to avoid parsing dependency.

More details can be found in the next table and the corresponding contribution documents.

| Test | Changes in intra mode coding |
| --- | --- |
| 5.1.1 | Luma intra mode coding: - Planar and DC modes are removed - Use BL intra mode derived from the center as an MPM, if BL intra mode is neither Planar nor DC - Total number of MPMs is one - Change in intra mode coding/parsing Chroma intra mode coding: - Planar and DC modes are removed - Three intra modes (VER, HOR and DM) instead of five (Planar, VER, HOR, DC and DM) are used - Change in intra mode coding/parsing To avoid parsing dependency, MDCS was disabled and additional results were provided. |
| 5.1.2 | Luma intra mode coding: - Replace the first MPM with the BL intra mode, if BL intra mode is neither DC nor equal to any already derived MPMs To avoid parsing dependency, MDCS was disabled and additional results were provided. |
| 5.1.3 | Luma intra mode coding: - The difference between BL intra mode or Planar mode is coded - Change in intra mode coding/parsing To avoid parsing dependency, MDCS was disabled and additional results were provided. |
| 5.1.4 | Luma intra mode coding: - Insert BL intra mode as a first MPM, if BL intra mode is distinct To avoid parsing dependency, MDCS was disabled and additional results were provided. |
| 5.1.5 | BL-MPM (test is withdrawn by the proponent) Luma intra mode coding: - Use BL intra mode as one more MPM inserted as forth MPM - BL MPM is context coded and one more context is added - Change in intra mode coding/parsing ILIPM Luma intra mode coding: - Use BL intra mode as intra mode predictor, HEVC intra mode coding is applied if EL intra mode is not equal to BL intra mode - BL intra mode is used if BL block is intra coded (**) - BL intra mode is used if EL PU size is twice of the co-located BL PU size (*) - Change in intra mode coding/parsing Additional test result was provided without condition (*). One more additional test result was provided in addition with disabled MDCS. The condition (**) doesn’t have an effect on AI test configuration. |

Experimental results of the above methods based on the BD-rate for EL+BL average among all the test cases are shown in the following graph.

- TE5.2 Inter-layer motion vector prediction

In this category, motion vector derived from the base layer picture is used as an additional candidate in Merge and AMVP candidates list. In most cases, the BL MV candidate derived from the center of the co-located block is added as first candidate into the candidate list. For enhancement layer Merge and AMVP modes, with including BL MV as a candidate the candidate, the list size can be increased by one. In some other cases more than one base layer MV can be included as merge motion vector candidates. Additional pruning can be applied between BL MV candidates as some spatial candidates. If BL MV is inserted before TMVP candidate, additional check can be performed for spatial MV candidate to guarantee TMVP being included in the candidate list. Roughly in have of the proposals, base layer MV is derived from the uncompressed base layer motion field to increase the accuracy of the derived motion vector.

The most common parts of the studied methods can be grouped as the following:

- Add base layer MV candidate into Merge/AMVP list

- Derive base layer MV from the center position of the co-located block using compressed or uncompressed motion field

- Include additional pruning or checks to address redundancy in the candidate list

- Increase the candidate list size by one or keep it unchanged as for single layer coding

More detailed description of the proposed methods can be found in the next table and the corresponding contribution documents.

| Test | Changes in Merge and AMVP modes |
| --- | --- |
| 5.2.1 | Merge modification: - Add center C3 BL MV as a first candidate - Add bottom right BL MV after second spatial candidate - Uncompressed motion field is used to derive BL MV - Pruning with BL MV candidate - Check to guarantee TMVP in the candidate list AMVP modification: - Add center C3 BL MV as first candidate - Uncompressed motion field is used to derive BL MV - Pruning with BL MV |
| 5.2.2 | Merge modification: - Add center C3 BL MV as a first candidate - Uncompressed motion field is used to derive BL MV - Pruning with BL MV candidate - Check to guarantee TMVP in the candidate list AMVP modification: - Add center C3 BL MV as a last candidate - Uncompressed motion field is used to derive BL MV Additional results without changing Merge and AMVP list generation were provided. BL MV is inserted as a first candidate into the complete candidate list. |
| 5.2.3 | Merge modification: - Add center C3 BL MV as a first candidate - Uncompressed motion field is used to derive BL MV - Pruning only between left candidate and BL MV candidate - Check to guarantee TMVP in the candidate list AMVP modification: - Add center C3 BL MV as first candidate - Uncompressed motion field is used to derive BL MV - Pruning with BL MV candidate |
| 5.2.4 | Merge modification: - Maximum number of candidates is 6 - Add bottom right BL MV as a first candidate - Pruning with BL MV candidate - Increased number of combined candidates AMVP modification: - Maximum number of candidates is 3 - Add bottom right BL MV as first candidate - Pruning with BL MV candidate |
| 5.2.5 | Merge modification: - Add center C3 BL MV as a first candidate - Uncompressed motion field is used to derive BL MV (*) - Pruning with BL MV candidate (*) - Check to guarantee TMVP in the candidate list - BL MV scaling with integer arithmetic Additional results without changes (*) were provided. |
| 5.2.6 | Modifications to the EL coding: - No motion compression for EL reference frames Merge modification: - Maximum number of candidates is 6 - TMVP derived only from the center C3 is the first candidate if the temporal collocated blocks are selected in the forward reference frame. - Pruning with TMVP candidate - Bottom left spatial candidate is added if count is less than 4 - Add bottom right or center C3 BL MV after spatial candidates - Four offset candidates are added after BL MV candidate - Candidate cannot be obtained from the block coded with Base Mode (*) AMVP modification: - Candidate cannot be obtained from the block coded with Base Mode (*) Additional results with enabled Base Mode (5.3.3) and changes (*) were provided. |
| 5.2.7 | AMVP modification: - Add center C3 BL MV as a first candidate - Temporal scaling of BL MV |
| 5.2.8 | Merge modification: - Maximum number of candidates is 6 - Add center C3 BL MV as a third candidate - BL MV derivation process - Derived based on 4x4 blocks - Unavailable motion field is copied from the neighboring blocks - MV refinement - Pruning with BL MV candidate AMVP modification: - Maximum number of candidates is 3 - Add center C3 BL MV before TMVP candidate - Similar to Merge mode BL MV derivation - Scale BL MV according to the temporal distance - Pruning with BL MV candidate |
| 5.2.9 | Merge modification: - Maximum number of candidates is 6 - Add bottom right or center C3 BL MV after TMVP candidate - Uncompressed motion field is used to derive BL MV - BL MV candidate derivation - Inter direction depends on EL slice type - Reference list can be switched according to the motion availability - If reference list switching happens check the reference index for validity - Temporal scaling following by spatial scaling for BL MV - BL MV clipping AMVP modification: - Maximum number of candidates is 3 - Add bottom right or center BL MV as a first candidate - Uncompressed motion field is used to derive BL MV - Similar to Merge mode BL MV derivation - Pruning with BL MV candidate |
| 5.2.10 | Merge modification: - Maximum number of candidates is 6 - Add center BL MV after TMVP candidate - Increased number of combined candidates AMVP modification: - Maximum number of candidates is 3 - Add center BL MV after TMVP candidate - Switch the list if the target reference picture is different from the BL reference picture |

Experimental results of the methods related to Merge and AMVP modifications studied in this section based on the BD-rate for EL+BL average among all mandatory test cases are shown in the following graph.

## TE5.3 Inter-layer inferred prediction mode (InterBL mode)

In this subset, base layer motion field is derived based on sub-blocks such as 4x4 or 8x8 or based on the partitioning obtained from the base layer co-located block. If motion field is not available for the sub-block, Intra-BL prediction can be applied or motion field can be copied from the neighbor already processed sub-block. In one method, the derived base layer MVs are used to find a residual between EL and BL reference pictures that will be added to the Intra-BL prediction to form a final prediction to the Base Mode. In some proposal deblocking filter is applied for Inter-BL mode. In another method, Inter-BL can be applied on PU basis and the mode is signaling with the merge index equal to zero.

The most common parts of the studied methods can be grouped as the following:

- Derive plurality of the motion vectors from the base layer based on sub-blocks (such as 4x4 or 8x8), or according to the partition structure derived from the base layer co-located block.

- Derive base layer MVs from compressed or uncompressed motion field

- Use Intra-BL prediction or copy the motion field from the neighbor blocks in case base layer co-located block is intra coded

- Apply deblocking filter for the new mode

More detailed description of the proposed methods can be found in the next table and the corresponding contribution documents.

| Test | Base Mode implementation |
| --- | --- |
| 5.3.1 | - Inferred mode is CU based - One context is added for new mode coding - For each sub-block, prediction is formed either by motion compensation or Intra-BL. - Sub-block size can be 4x4 or 8x8. - For MC basis units, MV if available is derived from the base layer. - Base layer motion compression can be postponed for BL MVs derivation |
| 5.3.2 | - It has parsing dependency on base layer depth - CU based mode - One context is added for new mode coding - Partition structure is derived from the base layer - Prediction is formed either by motion compensation or Intra-BL - Applied only for SNR and dyadic case |
| 5.3.3 | - CU based mode - One context is added for new mode coding - Partition and motion mapping is done do get upsampled information from the base layer - The prediction in this mode is done by adding the residue between EL and BL reference pictures, obtained by MC using derived base layer MVs, to the Intra-BL prediction - Base layer MVs are derived from uncompressed motion field |
| 5.3.4 | - CU based Inter-BL mode - Three contexts are added for new mode coding - Motion information is derived from the base layer based on 8x8 units - Motion information is obtained from the previous block in z-order, if base layer co-located block is intra coded - Zero motion can be used as initialization for the very first 8x8 block - Deblock is applied for Inter-BL on 8x8 basis |
| 5.3.5 | - CU based Inter-BL mode signaled with a new flag. Three contexts are added for new mode coding. - PU based Inter-BL mode signaled with merge index equal to zero - Motion information is derived from the base layer based on 8x8 units - If co-located BL block doesn’t have motion, it is derived from the neighbors or set to zero. Up to three neighbor blocks can be checked. - Base layer MVs are derived from uncompressed motion field - Deblock is applied for Inter-BL on 8x8 basis |

Experimental results of the methods studied in this section based on the BD-rate for EL+BL average among all test cases are shown in the following graph.

## TE5.4 Motion mapping

In this section, the enabling of the motion field for the inter-layer reference (ILR) is studied. Motion vector and reference indices are assigned for inter-layer reference picture using base layer motion field information.

More detailed description can be found in the next table and the corresponding contribution documents.

| Test | Motion mapping |
| --- | --- |
| 5.4.1 | - Up to four MVs and reference indices can be derived from the base-layer co-located blocks. - Median or average MV is assigned to the 4x4 block in the ILR picture out of up to four base-layer MVs. - The most frequently used reference index out of up to four indices is assigned to the 4x4 block in the ILR picture. - Base layer motion information can be from compressed or uncompressed motion field - In the current software implementation, the mapped ILR MVs are treated from a short term reference |

Experimental results based one the HLS-only or “reference index framework” anchor is summarized in the next table.

| Test | RA |  |  | LD-P |  |  | Average |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2x | 1.5x | SNR | 2x | 1.5x | SNR |  |
| Compressed base layer motion field | -1.10 | -1.73 | -1.96 | -0.21 | -0.46 | -0.79 | -1.04 |
| Uncompressed base layer motion field | -1.56 | -1.89 | -1.99 | -0.60 | -0.57 | -0.82 | -1.24 |

# Conclusion

In the studied methods, several experiments have been performed including different number of changes comparing to the single layer coding (HEVC). As in conclusion, the desired direction in the design for all subsets can be the following:

- Least number of changes comparing to the single layer coding

- Independent from the base layer parsing

# Appendix – list of related non-TE proposals

The following contributions related to TE5 have been additionally submitted and should be reviewed in relation with the TE5.

| Proposals | Title | Crosschecking documents |
| --- | --- | --- |
| JCTVC-L0040 (Nokia) | Non-TE5: On motion mapping between layers in SHVC |  |
| JCTVC-L0073 (I2R) | Non-TE5: On the effectiveness of temporal and base layer co-located motion vector prediction candidates |  |
| JCTVC-L0103 (Canon) | Non-TE5.3: Inter-layer inferred prediction mode and motion compensation restrictions in SHVC |  |
| JCTVC-L0105 (Canon) | Non-TE5.1: MPM derivation and coding in SHVC | JCTVC-L0403 (Sony) |
| JCTVC-L0106 (Canon) | Non-TE5.1: simplification of remaining modes coding in SHVC | JCTVC-L0402 (Sony) |
| JCTVC-L0156 (Leibniz Uni Hannover) | Simplification of TE5.1.5 on inter-layer intra mode prediction |  |
| JCTVC-L0220 (Sony) | Non-TE5: Inter-layer Intra MPM derivation for EL in SHVC | JCTVC-L0408 (Canon) |
| JCTVC-L0221 (Sony) | Non-TE5: Memory reduction for MV data of EL in SHVC | JCTVC-L0358 (InterDigital) |
| JCTVC-L0239 (LG,Mediatek) | Non-TE5:Assigning intra prediction mode to inter layer intra predicted blocks in enhance layer |  |
| JCTVC-L0260 (LG,Mediatek) | Non-TE5.1: Inter-layer Intra mode prediction | JCTVC-L0408 (Leibniz Uni Hannover) |
| JCTVC-L0336 (Qualcomm) | Non-TE5: on motion mapping in SHVC | JCTVC-L0365 (InterDigital) |
| JCTVC-L0398 (Sony) | NonTE5: Partition-dependent inter-layer motion vector prediction |  |