| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0032 |
| --- | --- |

| Title: | TE5: Results of 5.2.5 on inter-layer motion vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Done Bugdayci, Kemal Ugur Visiokatu 3, Tampere, Finland | Email: | ext-done.bugdayci@nokia.com; kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution presents an algorithm for the scalable extension of High Efﬁciency Video Coding (HEVC) standard, where the merge list construction of enhancement layer is modified to include co-located base layer motion vector as a motion vector predictor candidate in the merge list. Proposed algorithm modifies the enhancement layer merge list construction such that, motion vector predictor candidate associated with the base layer motion vector is always included as the first candidate to be processed in the merge list of enhancement layer. Furthermore, motion vector predictor from the base layer and motion vector predictor from another picture in the same layer (temporal motion vector predictor candidate) are always present in the list if they are both available and the list size is larger than two. Motion vector predictor candidate associated with base layer may or may not have a redundancy check with the other candidates present in the list, and results are presented for both cases. Uncompressed motion vectors taken from the location that is collocated to the enhancement layer center location are used. When the redundancy check is disabled, the algorithm reports average overall EL+BLBD-RD gain of 2.0% in luma for Random Access (RA) 2x scalability, 2.1% for RA 1.5x scalability,  2.2% for RA SNR scalability, 1.0% for LD-P 2x scalability, 1.0% for LD-P 1.5x scalability and 1.7% for LD-P SNR scalability.  When the redundancy check is enabled, the algorithm reports average overall EL+BL BD-RD gain of 2.3% in luma for Random Access (RA) 2x scalability, 2.4% for RA 1.5x scalability,  2.5% for RA SNR scalability, 1.3% for LD-P 2x scalability, 1.3% for LD-P 1.5x scalability and 2.0% for LD-P SNR scalability.

# Introduction

In a scalable video coding scheme, there is high correlation between the motions of base and enhancement layers. Therefore base layer motion vector can also be included as a motion vector predictor candidate in the enhancement layer motion vector coding known as merge mode. Handling of base layer motion vector predictor during list generation in the enhancement layer motion affects the coding efficiency.

# Algorithm Description

While coding enhancement layer motion vector, the motion vector prediction candidate list of an enhancement layer block is constructed in such a way that the base layer motion vector predictor and temporal motion vector predictor are always present in the candidate list if they are both available and the list size is larger than two. For example, when the number of candidates in merge list is set to 5, merge list construction process is modified as follows:

- Place the base layer MVP in the beginning of the list

- Select maximally 3 spatial MVP candidates and place them in the list.

- Place the temporal MVP to the list.

Base layer MVP may participate in redundancy check process with the other candidates to have a higher coding efficiency. Alternatively, base layer MVP may not participate in redundancy check process to reduce complexity.

We provide results for both cases when the base layer motion vector is compressed and uncompressed. In both cases, the location of the base layer block to take the motion vector information is collocated to the center location of the enhancement layer block as in TMVP. Base layer motion vector is scaled according to the scalability factor before added to the merge list.

# Simulation Results

Test Sequences, bitrates and coding conditions used in the simulations are in conformance with JCTVC-K1105. Anchor software is the software defined in JCTVC-K1105.

## BD-RD Results

Table 1 RA BD-rate (BL+EL) results with uncompressed motion and redundancy check enabled

|  | 2x |  |  | 1.5x |  |  | SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic | -2.1% | -2.6% | -2.7% | - | - | - | -2.7% | -2.9% | -3.7% |
| PeopleOnStreet | -3.3% | -8.3% | -7.7% | - | - | - | -2.9% | -10.1% | -9.9% |
| Kimono | -3.2% | -6.1% | -6.7% | -3.1% | -5.7% | -6.1% | -2.4% | -4.5% | -5.0% |
| ParkScene | -1.4% | -1.9% | -1.9% | -1.7% | -2.7% | -2.7% | -1.8% | -2.6% | -2.8% |
| Cactus | -2.4% | -3.3% | -3.4% | -2.7% | -3.3% | -4.0% | -2.5% | -3.4% | -4.1% |
| BasketballDrive | -2.9% | -5.4% | -5.7% | -3.0% | -4.6% | -5.4% | -3.0% | -5.8% | -5.5% |
| BQTerrace | -1.1% | -1.9% | -1.3% | -1.4% | -2.5% | -2.0% | -2.2% | -4.1% | -4.1% |
| Class A average | -2.7% | -5.4% | -5.2% | - | - | - | -2.8% | -6.5% | -6.8% |
| Class B average | -2.2% | -3.7% | -3.8% | -2.4% | -3.8% | -4.1% | -2.4% | -4.1% | -4.3% |
| Average | -2.3% | -4.2% | -4.2% | -2.4% | -3.8% | -4.1% | -2.5% | -4.8% | -5.0% |

Table 2 LD-P BD-rate (BL+EL) results with uncompressed motion and redundancy check enabled

|  | 2x |  |  | 1.5x |  |  | SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic | -1.3% | -1.8% | -1.9% | - | - | - | -2.8% | -2.9% | -3.1% |
| PeopleOnStreet | -1.9% | -4.9% | -4.5% | - | - | - | -2.1% | -5.6% | -5.4% |
| Kimono | -1.6% | -3.0% | -3.2% | -1.5% | -2.5% | -2.7% | -1.5% | -2.0% | -2.3% |
| ParkScene | -0.9% | -1.6% | -1.7% | -1.0% | -1.6% | -2.0% | -1.7% | -1.8% | -2.2% |
| Cactus | -1.2% | -1.6% | -1.7% | -1.4% | -1.7% | -2.2% | -1.7% | -2.0% | -2.3% |
| BasketballDrive | -1.4% | -2.6% | -2.6% | -1.3% | -2.0% | -2.2% | -1.7% | -2.8% | -2.7% |
| BQTerrace | -1.1% | -2.2% | -2.4% | -1.2% | -2.2% | -2.0% | -2.7% | -4.5% | -4.9% |
| Class A average | -1.6% | -3.4% | -3.2% | - | - | - | -2.5% | -4.2% | -4.2% |
| Class B average | -1.2% | -2.2% | -2.3% | -1.3% | -2.0% | -2.2% | -1.9% | -2.6% | -2.9% |
| Average | -1.3% | -2.5% | -2.6% | -1.3% | -2.0% | -2.2% | -2.0% | -3.1% | -3.3% |

Table 3 RA BD-rate (BL+EL) results with uncompressed motion and redundancy check disabled

|  | 2x |  |  | 1.5x |  |  | SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic | -1.7% | -2.5% | -2.6% | - | - | - | -2.4% | -2.9% | -3.6% |
| PeopleOnStreet | -3.0% | -8.0% | -7.5% | - | - | - | -2.6% | -9.9% | -9.8% |
| Kimono | -2.9% | -6.1% | -6.7% | -2.9% | -5.7% | -6.1% | -2.1% | -4.3% | -4.9% |
| ParkScene | -1.1% | -1.8% | -1.9% | -1.5% | -2.7% | -2.7% | -1.5% | -2.5% | -2.8% |
| Cactus | -2.1% | -3.2% | -3.3% | -2.5% | -3.3% | -4.0% | -2.2% | -3.3% | -4.0% |
| BasketballDrive | -2.6% | -5.2% | -5.5% | -2.8% | -4.5% | -5.3% | -2.8% | -5.5% | -5.4% |
| BQTerrace | -0.7% | -1.7% | -1.2% | -1.1% | -2.5% | -2.0% | -1.8% | -4.1% | -4.2% |
| Class A average | -2.3% | -5.3% | -5.1% | - | - | - | -2.5% | -6.4% | -6.7% |
| Class B average | -1.9% | -3.6% | -3.7% | -2.1% | -3.7% | -4.0% | -2.1% | -4.0% | -4.3% |
| Average | -2.0% | -4.1% | -4.1% | -2.1% | -3.7% | -4.0% | -2.2% | -4.7% | -4.9% |

Table 4 LD-P BD-rate (BL+EL) results with uncompressed motion and redundancy check disabled

|  | 2x |  |  | 1.5x |  |  | SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic | -0.9% | -1.6% | -1.7% | - | - | - | -2.4% | -2.7% | -3.0% |
| PeopleOnStreet | -1.7% | -4.6% | -4.2% | - | - | - | -1.9% | -5.5% | -5.3% |
| Kimono | -1.4% | -2.9% | -3.1% | -1.3% | -2.5% | -2.7% | -1.3% | -2.0% | -2.2% |
| ParkScene | -0.6% | -1.5% | -1.4% | -0.8% | -1.6% | -1.9% | -1.4% | -1.8% | -2.0% |
| Cactus | -0.8% | -1.4% | -1.5% | -1.1% | -1.4% | -1.9% | -1.3% | -1.7% | -2.0% |
| BasketballDrive | -1.2% | -2.4% | -2.5% | -1.1% | -2.1% | -2.0% | -1.5% | -2.7% | -2.6% |
| BQTerrace | -0.7% | -2.5% | -2.2% | -0.9% | -2.0% | -2.0% | -2.3% | -4.3% | -4.4% |
| Class A average | -1.3% | -3.1% | -3.0% | - | - | - | -2.2% | -4.1% | -4.1% |
| Class B average | -1.0% | -2.1% | -2.2% | -1.0% | -1.9% | -2.1% | -1.6% | -2.5% | -2.6% |
| Average | -1.0% | -2.4% | -2.4% | -1.0% | -1.9% | -2.1% | -1.7% | -3.0% | -3.1% |

Table 5 RA BD-rate (BL+EL) results with compressed motion and redundancy check enabled

|  | 2x |  |  | 1.5x |  |  | SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic | -0.7% | -1.0% | -1.0% | - | - | - | -2.1% | -2.1% | -2.7% |
| PeopleOnStreet | -1.0% | -2.9% | -2.8% | - | - | - | -2.0% | -6.6% | -6.6% |
| Kimono | -2.4% | -4.6% | -5.0% | -2.8% | -5.1% | -5.5% | -2.2% | -4.1% | -4.5% |
| ParkScene | -0.5% | -0.7% | -0.7% | -1.0% | -1.8% | -1.7% | -1.3% | -1.7% | -1.9% |
| Cactus | -1.1% | -1.7% | -1.8% | -1.9% | -2.3% | -2.8% | -2.0% | -2.6% | -3.1% |
| BasketballDrive | -1.9% | -3.4% | -3.6% | -2.5% | -3.7% | -4.3% | -2.7% | -4.8% | -4.7% |
| BQTerrace | -0.5% | -1.0% | -0.5% | -1.0% | -1.8% | -1.6% | -1.9% | -3.5% | -3.3% |
| Class A average | -0.9% | -2.0% | -1.9% | - | - | - | -2.0% | -4.4% | -4.7% |
| Class B average | -1.3% | -2.3% | -2.3% | -1.8% | -2.9% | -3.2% | -2.0% | -3.3% | -3.5% |
| Average | -1.2% | -2.2% | -2.2% | -1.8% | -2.9% | -3.2% | -2.0% | -3.6% | -3.8% |

Table 6 LD-P BD-rate (BL+EL) results with compressed motion and redundancy check enabled

|  | 2x |  |  | 1.5x |  |  | SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic | -0.1% | -0.4% | -0.4% | - | - | - | -1.9% | -1.8% | -2.0% |
| PeopleOnStreet | -0.1% | -1.0% | -0.9% | - | - | - | -1.1% | -3.0% | -2.9% |
| Kimono | -1.0% | -1.9% | -1.8% | -1.2% | -2.2% | -2.3% | -1.3% | -1.8% | -2.0% |
| ParkScene | -0.1% | -0.6% | -0.7% | -0.5% | -1.1% | -1.2% | -1.0% | -1.1% | -1.2% |
| Cactus | -0.1% | -0.4% | -0.3% | -0.7% | -1.1% | -1.1% | -1.1% | -1.2% | -1.6% |
| BasketballDrive | -0.7% | -1.4% | -1.4% | -0.9% | -1.5% | -1.5% | -1.3% | -2.2% | -2.0% |
| BQTerrace | -0.6% | -1.8% | -1.5% | -0.8% | -1.6% | -2.2% | -2.2% | -3.5% | -3.8% |
| Class A average | -0.1% | -0.7% | -0.6% | - | - | - | -1.5% | -2.4% | -2.4% |
| Class B average | -0.5% | -1.2% | -1.1% | -0.8% | -1.5% | -1.7% | -1.4% | -2.0% | -2.1% |
| Average | -0.4% | -1.1% | -1.0% | -0.8% | -1.5% | -1.7% | -1.4% | -2.1% | -2.2% |

Table 7 RA BD-rate (BL+EL) results with compressed motion and redundancy check disabled

|  | 2x |  |  | 1.5x |  |  | SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic | -0.4% | -1.0% | -1.0% | - | - | - | -1.7% | -2.0% | -2.6% |
| PeopleOnStreet | -0.7% | -2.7% | -2.6% | - | - | - | -1.7% | -6.4% | -6.4% |
| Kimono | -2.2% | -4.5% | -4.9% | -2.6% | -5.0% | -5.4% | -1.9% | -4.0% | -4.4% |
| ParkScene | -0.3% | -0.7% | -0.6% | -0.8% | -1.8% | -1.7% | -1.0% | -1.7% | -1.8% |
| Cactus | -0.9% | -1.5% | -1.6% | -1.7% | -2.3% | -2.8% | -1.7% | -2.5% | -3.0% |
| BasketballDrive | -1.7% | -3.3% | -3.4% | -2.3% | -3.6% | -4.2% | -2.4% | -4.8% | -4.5% |
| BQTerrace | -0.2% | -1.0% | -0.6% | -0.7% | -1.7% | -1.4% | -1.5% | -3.3% | -3.3% |
| Class A average | -0.6% | -1.8% | -1.8% | - | - | - | -1.7% | -4.2% | -4.5% |
| Class B average | -1.1% | -2.2% | -2.2% | -1.6% | -2.9% | -3.1% | -1.7% | -3.2% | -3.4% |
| Average | -0.9% | -2.1% | -2.1% | -1.6% | -2.9% | -3.1% | -1.7% | -3.5% | -3.7% |

Table 8 LD-P BD-rate (BL+EL) results with compressed motion and redundancy check disabled

|  | 2x |  |  | 1.5x |  |  | SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Traffic | 0.3% | -0.2% | -0.3% | - | - | - | -1.4% | -1.6% | -1.7% |
| PeopleOnStreet | 0.0% | -0.8% | -0.7% | - | - | - | -0.9% | -2.9% | -2.7% |
| Kimono | -0.8% | -1.9% | -1.7% | -1.0% | -2.1% | -2.1% | -1.1% | -1.7% | -1.8% |
| ParkScene | 0.2% | -0.4% | -0.4% | -0.2% | -0.9% | -1.2% | -0.7% | -1.0% | -1.1% |
| Cactus | 0.2% | -0.2% | -0.2% | -0.4% | -0.9% | -1.1% | -0.7% | -0.9% | -1.2% |
| BasketballDrive | -0.5% | -1.1% | -1.3% | -0.8% | -1.3% | -1.4% | -1.2% | -2.0% | -1.9% |
| BQTerrace | -0.2% | -1.7% | -1.2% | -0.5% | -1.6% | -1.6% | -1.8% | -3.4% | -3.5% |
| Class A average | 0.2% | -0.5% | -0.5% | - | - | - | -1.1% | -2.2% | -2.2% |
| Class B average | -0.2% | -1.0% | -1.0% | -0.6% | -1.3% | -1.5% | -1.1% | -1.8% | -1.9% |
| Average | -0.1% | -0.9% | -0.8% | -0.6% | -1.3% | -1.5% | -1.1% | -1.9% | -2.0% |

## Complexity Results

Timing results for the compressed motion vector case are accurate. However, timing results of uncompressed motion vector tests are not accurate due to heterogeneous cluster.

Table 9 Timing results for compressed motion vector case with respect to the anchor software

|  | Redundancy check enabled |  |  | Redundancy check disabled |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA 2x | RA 1.5x | RA SNR | RA 2x | RA 1.5x | RA SNR |
| Enc Time[%] | 95.0% | 93.0% | 96.0% | 95.5% | 93.6% | 95.7% |
| Dec Time[%] | 99.4% | 98.4% | 100.3% | 98.9% | 98.5% | 101.7% |
|  | LD-P 2x | LD-P 1.5x | LD-P SNR | LD-P 2x | LD-P 1.5x | LD-P SNR |
| Enc Time[%] | 96.5% | 94.1% | 96.6% | 96.8% | 94.4% | 97.0% |
| Dec Time[%] | 96.6% | 98.7% | 100.5% | 96.5% | 98.6% | 100.0% |

Table 10 Timing results for uncompressed motion vector case with respect to the anchor software

|  | Redundancy check enabled |  |  | Redundancy check disabled |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA 2x | RA 1.5x | RA SNR | RA 2x | RA 1.5x | RA SNR |
| Enc Time[%] | 75.8% | 76.7% | 76.4% | 73.4% | 72.7% | 74.5% |
| Dec Time[%] | 85.9% | 72.5% | 75.2% | 84.7% | 84.2% | 77.0% |
|  | LD-P 2x | LD-P 1.5x | LD-P SNR | LD-P 2x | LD-P 1.5x | LD-P SNR |
| Enc Time[%] | 80.0% | 79.4% | 80.9% | 78.7% | 77.5% | 79.4% |
| Dec Time[%] | 87.8% | 86.8% | 82.8% | 86.6% | 87.9% | 80.9% |

# Conclusion

This contribution presents a method to include base layer motion vector as motion vector predictor in merge list of enhancement layer(s) for scalable systems. Uncompressed base layer motion vector taken from the location that is collocated to the center location of the enhancement layer block is added to the first location of the merge list of the enhancement layer after scaling according to the scalability factor.  In addition, temporal motion vector predictor is kept in the list if it is available also by restricting the number of spatial candidates. Base layer motion vector may not have any redundancy check with other predictors in the list in order to reduce complexity or it may go under redundancy check to have higher compression efficiency.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).