| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0034_r1 |
| --- | --- |

| Title: | Cleanup for motion estimation process in HM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Houqiang Li |  |  |
| Source: | University of Science and Technology of China |  |  |

_____________________________

# Abstract

The combined list was removed because of the adoption of JCTVC-I0125. But there are still some lines of code related to the combined list in HM-9.0. This contribution proposes to remove the code related to combined list from HM software. The experimental results show that there is no impact on the low delay cases and the impact on random access cases is about 0.04% and −0.02% on average, in terms of Y BD-Rate for RA-Main and RA-HE10 respectively.

# Introduction

The combined list was removed because of the adoption of JCTVC-I0125. But there are still some lines of code related to the combined list in HM-9.0. This contribution proposes to remove the code related to combined list from HM software. The experimental results show that there is no impact on the low delay cases and the impact on random access cases is about 0.04% and −0.02% on average, in terms of Y BD-Rate for RA-Main and RA-HE10 respectively.

# Solution

The proposed motion estimation process is as follows:

- List 0: motion estimation for every reference picture to get MV and cost.

- List 1:

  - If there exists a same picture in list 0, use the MV obtained from list 0 to calculate cost;

  - Otherwise, perform motion estimation to get MV and cost

- Bi-directional: In HM, fast bi-directional motion estimation is used and only one iteration step is performed. In HM-9.0, the MV in list 0 is fixed and the MV in list 1 is re-estimated. In the proposed method, the MV in the list with smaller cost is fixed and the other MV is re-estimated.
We do not change the bi-directional prediction motion estimation process when pcCU->getSlice()->getMvdL1ZeroFlag() is true.

- Choose the best mode: For every picture in list 1, if there is a same picture in list 0, this picture cannot be used for uni-directional prediction from list 1.

The code related to combined list (and setNoBackPredFlag() / getNoBackPredFlag() ) is removed from the software.

# Experimental results

The experiments are performed based on HM-9.0 and the CTC specified in [1] is followed. The summary experimental results are shown in Table 1 and the detailed results together with the code (under the macro of COMBINED_LIST_CLEANUP) are also attached. In the r1 version of the source code, more lines are removed (mainly the lines related to read parameters from cfg file and command lines). And removing these lines will not change the R-D performance.

Table 1 Motion estimation cleanup

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.2% | -0.2% | -0.1% | -0.1% |
| Class B | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.2% |
| Class C | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | 0.0% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.04% | 0.08% | 0.17% | -0.02% | 0.04% | 0.07% |
|  | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.1% |
| Class F | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

The R-D performance of the low delay coding structures is identical to that of the HM-9.0 anchor.

# References

- JCTVC-K1100, “Common HM test conditions and software reference configurations,” Shanghai, CN, 10–19 Oct. 2012.