| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0035 |
| --- | --- |

| Title: | TE2: Results of test 3.1.8 on PU based Intra BL signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Kemal Ugur | Tel: Email: | jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document reports that PU level signaling of Intra BL mode in SHVC 0.1.1 provides objective bitrate reductions of -0.7% (AI 2x), -1.7% (AI 1.5x), -1.2% (RA 2x), -2.3% (RA 1.5x), -0.9% (RA SNR), -1.5% (LP 2x), -3.1% (LP 1.5x) and -1.2% (LP SNR) for enhancement layer coding (“EL only” results). Compared to the simulcast anchor the EL only bitrate reductions are reported to be -35.1%, -58.5%, -26.0%, -48.1%, -32.3%, -20.6%, -42.1% and -21.9%, respectively.

The document also further reports that the Intra Skip functionality studied in the test provides a -0.0% and -0.4% objective “EL only” gains for AI 2x and AI 1.5x configurations, respectively.

# Introduction

Tool Experiment 2 / 3.1.8 studied performance of PU based signaling of Intra BL. The solution contains also a method for signaling if an intra coded LCU is utilizing only Intra BL prediction and has no transform coded residual associated with it. We are providing results for the combined solution as requested by the TE. In addition we are also reporting the performance of the Intra Skip functionality as a stand-alone tool, in order to assess its contribution to the total performance.

# Results of the test

The tables below summarize performance of the tested solution. Simulation runtime information was generated by disabling YUV output in order to minimize external impacts to the runtimes. Full results are provided in the attached Excel sheets.

Table 1: Performance of TE2 3.1.8 with respect to simulcast anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -27.1% | -27.2% | -28.1% |  |  |  |  |  |  |
| Class B | -21.4% | -21.3% | -21.1% | -32.8% | -32.9% | -33.0% |  |  |  |
| Overall (EL+BL) | -23.0% | -23.0% | -23.1% | -32.8% | -32.9% | -33.0% |  |  |  |
| Overall (EL) | -35.1% | -35.3% | -35.5% | -58.5% | -58.0% | -58.3% |  |  |  |
| Enc Time[%] | 116.9% |  |  | 115.2% |  |  |  |  |  |
| Dec Time[%] | 107.1% |  |  | 103.5% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -18.9% | -7.8% | -9.4% |  |  |  | -21.9% | -11.7% | -11.4% |
| Class B | -16.0% | -8.6% | -7.1% | -26.6% | -20.4% | -18.9% | -19.8% | -10.8% | -7.3% |
| Overall (EL+BL) | -16.9% | -8.3% | -7.8% | -26.6% | -20.4% | -18.9% | -20.4% | -11.0% | -8.5% |
| Overall (EL) | -26.0% | -12.9% | -12.3% | -48.1% | -38.4% | -35.8% | -32.2% | -18.5% | -14.8% |
| Enc Time[%] | 108.8% |  |  | 109.9% |  |  | 107.1% |  |  |
| Dec Time[%] | 129.1% |  |  | 127.4% |  |  | 94.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -14.2% | -1.6% | -2.8% |  |  |  | -14.5% | -6.7% | -5.6% |
| Class B | -12.4% | -5.7% | -4.1% | -22.7% | -16.6% | -14.8% | -13.3% | -4.5% | -0.1% |
| Overall (EL+BL) | -12.9% | -4.5% | -3.7% | -22.7% | -16.6% | -14.8% | -13.6% | -5.1% | -1.7% |
| Overall (EL) | -20.6% | -7.2% | -6.1% | -42.1% | -31.8% | -28.6% | -21.9% | -10.1% | -5.3% |
| Enc Time[%] | 108.1% |  |  | 108.3% |  |  | 107.0% |  |  |
| Dec Time[%] | 133.5% |  |  | 131.5% |  |  | 95.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 2: Performance of TE2 3.1.8 with respect to SHVC 0.1.1 anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -0.2% | -0.1% | -0.2% |  |  |  |  |  |  |
| Class B | -0.4% | -0.1% | -0.2% | -0.4% | 0.2% | 0.0% |  |  |  |
| Overall (EL+BL) | -0.4% | -0.1% | -0.2% | -0.4% | 0.2% | 0.0% |  |  |  |
| Overall (EL) | -0.7% | -0.1% | -0.2% | -1.7% | 1.5% | 1.0% |  |  |  |
| Enc Time[%] | 101.0% |  |  | 102.3% |  |  |  |  |  |
| Dec Time[%] | 100.1% |  |  | 96.8% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.6% | 0.1% | 0.0% |  |  |  | -0.5% | 0.6% | 0.6% |
| Class B | -0.6% | 0.9% | 0.9% | -0.7% | 0.2% | 0.0% | -0.5% | 0.7% | 1.0% |
| Overall (EL+BL) | -0.6% | 0.7% | 0.6% | -0.7% | 0.2% | 0.0% | -0.5% | 0.7% | 0.9% |
| Overall (EL) | -1.2% | 1.2% | 1.0% | -2.3% | 0.4% | -0.5% | -0.9% | 1.5% | 1.7% |
| Enc Time[%] | 98.9% |  |  | 95.5% |  |  | 98.7% |  |  |
| Dec Time[%] | 93.8% |  |  | 89.5% |  |  | 93.4% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.7% | 0.7% | 0.4% |  |  |  | -0.6% | 0.9% | 1.0% |
| Class B | -0.8% | 1.6% | 1.4% | -1.0% | 0.9% | 0.9% | -0.7% | 1.4% | 1.9% |
| Overall (EL+BL) | -0.8% | 1.4% | 1.1% | -1.0% | 0.9% | 0.9% | -0.6% | 1.2% | 1.6% |
| Overall (EL) | -1.5% | 2.5% | 1.9% | -3.1% | 2.6% | 1.7% | -1.2% | 2.2% | 2.5% |
| Enc Time[%] | 99.7% |  |  | 96.1% |  |  | 100.5% |  |  |
| Dec Time[%] | 93.1% |  |  | 87.9% |  |  | 93.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Supplemental results

The tables below summarize performance of the Intra Skip method as a stand-alone tool when integrated in SHVC 0.1.1. Simulation runtime information was generated by disabling YUV output in order to minimize external impacts to the runtimes. Full results are provided in the attached Excel sheets.

Table 3: Performance of the Intra Skip mode with respect to the SHVC 0.1.1 anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (EL+BL) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (EL) | 0.0% | 0.0% | 0.0% | -0.4% | 0.4% | 0.4% |
| Enc Time[%] | 100.2% |  |  | 100.5% |  |  |
| Dec Time[%] | 98.5% |  |  | 95.4% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Algorithm description

Same partitioning structure as HEVC is used. In addition, the proposed method allows block based prediction of sample values from base layer in PU granularity in inter slices and CU granularity in intra slices. This is done by using a new syntax element, called pred_base_flag (which is entropy coded similar to skip_flag). If pred_base_flag is 1, the associated prediction unit performs the prediction by copying the co-located reconstructed samples from the upsampled base layer. pred_base_flag is used at the CU level in I slices and at PU level in inter slices (B and P slices). The signaling and utilization of the flag is slightly different for intra and inter slices. This is done to be able to exploit the rectangular prediction block granularity in inter PU’s maximally. Below figure shows examples of possible prediction modes allowed by this design:

Figure 1 Example of PU based sample prediction from base

Intra slices

The intra_skip_flag is present for the largest CU size in Intra slices. In the case the flag is set to 1, no other syntax elements are sent for the CU, the CU is fully predicted using the upsampled base layer and the residual decoding process is omitted. In the case the flag has a value of 0, parsing continues with the remaining syntax elements.

The pred_base_flag is placed before any intra prediction information in a CU. When the flag is 1, no other intra prediction syntax elements are parsed and corresponding luma and chroma prediction blocks are predicted using the upsampled base layer. When the flag is 0, rest of the intra prediction information is parsed as usual. Transform coding, intra mode prediction and deblocking filter consider Intra BL blocks to be DC predicted blocks.

Inter slices

For an inter PU, pred_base_flag is parsed before any of the syntax elements in the PU. When the flag is 1, no other syntax is parsed for the PU and corresponding samples of luma and chroma prediction blocks are generated by upsampling the base layer picture. When the flag is 0, rest of the syntax elements are parsed as usual. For a skip CU, the pred_base_flag indicates whether the PU in the skip CU is copied from the base layer or if the HEVC merge process is invoked. Similarly to the case of 2Nx2N merge, there is no need to signal the presence of residual coding for 2Nx2N base prediction in the case of the CU is not signaled as skip coded (in these cases there is always residual coding associated with the modes mentioned). In the case there are two PUs in a CU and the first one is Intra_BL coded, the flag is omitted for the second PU to avoid redundancy. The flag is not parsed for intra PUs in an inter slice and is inferred to be zero instead. Transform coding, intra mode prediction and deblocking filter consider Intra BL blocks to be inter coded.

# Proposed specification text

The CU and PU syntax is proposed to be changed as follows:

coding_unit( x0, y0, log2CbSize ) {

…

if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) {

if(slice_type = = I && log2CbSize = = Log2MaxCbSize )

intra_skip_flag

if( intra_skip_flag )

Return

if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize )

pcm_flag

if( pcm_flag ) {

num_subsequent_pcm

NumPCMBlock = num_subsequent_pcm + 1

while( !byte_aligned( ) )

pcm_alignment_zero_bit

pcm_sample( x0, y0, log2CbSize )

} else {

pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : 0

for( j = 0; j <= pbOffset; j = j + pbOffset )

for( i = 0; i <= pbOffset; i = i + pbOffset ) {

if( slice_type = = I && PartMode = = PART_2Nx2N )

pred_base_flag[ x0 + i ][ y0+ j ]

if ( ! pred_base_flag[ x0 + i ][ y0+ j ] )

Prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ]

}

for( j = 0; j <= pbOffset; j = j + pbOffset )

for( i = 0; i <= pbOffset; i = i + pbOffset ) {

if ( ! pred_base_flag[ x0 + i ][ y0+ j ] ){

if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] )

mpm_idx[ x0 + i ][ y0+ j ]

Else

rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ]

}

}

if ( ! pred_base_flag[ x0 + i ][ y0+ j ] )

intra_chroma_pred_mode[ x0 ][ y0 ]

}

…

if( !pcm_flag ) {

if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) && !(PartMode = = PART_2Nx2N && pred_base_flag [x0][y0]))

no_residual_syntax_flag

if( !no_residual_syntax_flag ) {

MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter )

transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 )

}

}

}

}

prediction_unit( x0, y0, nPbW, nPbH, numPU, indPU, x0first, y0first ) {

if( !( numPU == 2 && indPU == 1 && pred_base_flag[ x0first ][ y0first ] ) )

pred_base_flag[ x0 ][ y0 ]

if ( ! pred_base_flag[ x0 ][ y0 ] ){

…

}

}

intra_skip_flag when set to 1 indicates that the coding unit contains one prediction unit of type 2Nx2N, the prediction performed by copying the co-located reconstructed samples of the upsampled base layer and the residual decoding is omitted. When intra_skip_flag is not present, it is inferred to be 0.

pred_base_flag [ x0 ][ y0 ] specifies whether the associated prediction unit performs the prediction by copying the co-located reconstructed samples of the upsampled base layer. When pred_base_flag[x0][y0] is not present, it is inferred to be 0.

# Patent rights declaration(s)

Nokia may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).