| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0038 |
| --- | --- |

| Title: | TE3: Results of test 4.6.3 on base enhanced motion compensated prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Kemal Ugur | Tel: Email: | jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document reports that utilizing base layer enhanced motion compensation provides objective bitrate reductions of -3.2% (RA 2x), -5.8% (RA 1.5x), -3.1% (RA SNR), -4.1% (LP 2x), -7.5% (LP 1.5x) and -5.0% (LP SNR) for enhancement layer coding (“EL only” results). The proposed method is reported to improve the enhancement layer temporal prediction by adding a base layer component to the prediction. The base layer component is expected to approximate the new information appearing to (or disappearing from) the corresponding block in the base layer. This component is derived by performing motion compensation on the base layer with the motion information identical to the enhancement layer and deducting the result from the reconstructed base layer signal.

# Introduction

Tool Experiment 3 / 4.6.3 studied performance of base enhanced motion compensated prediction (BEMCP). The method is motivated by the assumption that the already coded base layer texture can be utilized to improve the motion compensation process. This is proposed to be done by adding a base layer component to the enhancement layer motion compensated prediction as described in section 3 of this document.

# Results of the test

The tables below summarize performance of the tested solution. Simulation runtime information was generated by disabling YUV output in order to minimize external impacts to the runtimes. Full results are provided in the attached Excel sheets.

Table 1: Performance of TE3 4.2.1 with respect to SHVC 0.1.1 anchor

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.8% | -5.1% | -5.2% |  |  |  | -1.3% | -4.6% | -5.1% |
| Class B | -1.7% | -3.0% | -3.2% | -2.5% | -4.5% | -4.9% | -1.8% | -4.0% | -4.6% |
| Overall (EL+BL) | -1.7% | -3.6% | -3.8% | -2.5% | -4.5% | -4.9% | -1.6% | -4.2% | -4.7% |
| Overall (EL) | -3.2% | -6.9% | -7.2% | -5.8% | -10.6% | -11.6% | -3.1% | -8.2% | -9.1% |
| Enc Time[%] | 121.6% |  |  | 112.6% |  |  | 114.0% |  |  |
| Dec Time[%] | 103.2% |  |  | 109.5% |  |  | 103.6% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.4% | -4.6% | -4.5% |  |  |  | -2.3% | -4.3% | -4.4% |
| Class B | -2.3% | -2.1% | -2.1% | -3.4% | -3.8% | -3.6% | -3.1% | -3.6% | -4.0% |
| Overall (EL+BL) | -2.3% | -2.9% | -2.8% | -3.4% | -3.8% | -3.6% | -2.9% | -3.8% | -4.2% |
| Overall (EL) | -4.1% | -5.4% | -5.5% | -7.5% | -8.5% | -8.6% | -5.0% | -6.7% | -7.4% |
| Enc Time[%] | 117.4% |  |  | 108.9% |  |  | 114.4% |  |  |
| Dec Time[%] | 102.5% |  |  | 100.0% |  |  | 102.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Algorithm description

The base-layer enhanced motion compensated prediction (BEMCP) is a tool improving the enhancement layer temporal prediction by adding a base layer component to the prediction. The base layer component approximates the new information appearing to (or disappearing from) the corresponding block in the base layer. The base layer component is derived by performing motion compensation on the base layer with the motion information identical to the enhancement layer and deducting the result from the reconstructed base layer signal. The codec supports adding no, full or half of the base layer component to the enhancement layer prediction – the first approach corresponding to the traditional motion compensated prediction. The reconstruction process for the three modes can be given as:

P0(x,y) = P’(x,y)

P1(x,y) = Clip ( P’(x,y) + B(x,y) - B’(x,y) )

P2(x,y) = Clip ( P’(x,y) + ( ( B(x,y) - B’(x,y) ) >> 1 ) )

where P’(x,y) corresponds a predicted pixel from enhancement layer motion compensation, B’(x,y) a predicted pixel utilizing enhancement layer motion information but upsampled base layer reference frames and B(x,y) the upsampled reconstructed base layer block co-locating with the enhancement layer block. The figure below illustrates the process for creating the BEMCP prediction with full base layer component added to the prediction.

# Proposed specification text

The PU syntax is proposed to be changed as follows:

prediction_unit( x0, y0, nPbW, nPbH ) {

if( skip_flag[ x0 ][ y0 ] ) {

if( MaxNumMergeCand > 1 )

merge_idx[ x0 ][ y0 ]

} else { /* MODE_INTER */

merge_flag[ x0 ][ y0 ]

if( merge_flag[ x0 ][ y0 ] ) {

if( MaxNumMergeCand > 1 )

merge_idx[ x0 ][ y0 ]

} else {

if( slice_type = = B )

inter_pred_idc[ x0 ][ y0 ]

if( inter_pred_idc[ x0 ][ y0 ] != Pred_L1 ) {

if( num_ref_idx_l0_active_minus1 > 0 )

ref_idx_l0[ x0 ][ y0 ]

mvd_coding( x0, y0, 0 )

mvp_l0_flag[ x0 ][ y0 ]

}

if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) {

if( num_ref_idx_l1_active_minus1 > 0 )

ref_idx_l1[ x0 ][ y0 ]

if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = Pred_BI) {

MvdL1[ x0 ][ y0 ][ 0 ] = 0

MvdL1[ x0 ][ y0 ][ 1 ] = 0

} else

mvd_coding( x0, y0, 1 )

mvp_l1_flag[ x0 ][ y0 ]

}

}

}

base_enh_mcp[ x0 ][ y0 ]

}

base_enh_mcp [ x0 ][ y0 ] specifies the type of base enhanced motion compensated prediction (BEMCP) to apply for the prediction unit. It can have three values: 0, 1 or 2. 0 indicates normal motion compensation, 1 indicates base enhance motion compensation with full base layer component added to the enhancement layer prediction and 2 indicates base enhance motion compensation with half of the base layer component added to the enhancement layer prediction. base_enh_mcp[x0][y0] is only present in the enhancement layers of scalable bitstreams. When base_enh_mcp[x0][y0] is not present, it is inferred to be 0.

[New section 8.5.2.3]

8.5.2.3	Decoding process for inter prediction samples in enhancement layers

Inputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

–	a variable nCS specifying the size of the current luma coding block,

–	variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

–	luma motion vectors mvL0 and mvL1,

–	chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags, predFlagL0 and predFlagL1,

–	upsampled base layer sample arrays bL, bCb and bCr.

Outputs of this process are:

–	a (nCSL)x(nCSL) array predSamplesL of luma prediction samples, where nCSL is derived as specified in 8.5.2.2,

–	a (nCSC)x(nCSC) array predSamplesCb of chroma prediction samples for the component Cb, where nCSC is derived as specified in 8.5.2.2,

–	a (nCSC)x(nCSC) array predSamplesCr of chroma residual samples for the component Cr, where nCSC is derived as specified in 8.5.2.2.

The decoding process of section 8.5.2.2 is invoked to create preliminary enhancement layer prediction sample arrays predEnhL, predEnhCb and predEnhCb. If base_enh_mcp [ xC ][ yC ] is larger than 0, base layer prediction sample arrays predBaseL, predBaseCb and predBaseCb are also created by invoking the decoding process of 8.5.2.2 but substituting the reference picture sample arrays refPicLXL, refPicLXCb, and refPicLXCr with upsampled base layer reference pictures corresponding to the reference index refIdxLX in 8.5.2.2.1.

Depending on the value of base_enh_mcp [ xC ][ yC ] the following applies.

- If base_enh_mcp [ xC ][ yC ] is equal to 0:

predSamplesL[ x ][ y ] = predEnhL[ x ][ y ]

predSamplesCb[ x ][ y ] = predEnhCb[ x ][ y ]

predSamplesCr[ x ][ y ] = predEnhCr[ x ][ y ]

- Otherwise if base_enh_mcp [ xC ][ yC ] is equal to 1:

predSamplesL[ x ][ y ] = Clip ( predEnhL[ x ][ y ] + bL[ x ][ y ] - predEnhL[ x ][ y ] )

predSamplesCb[ x ][ y ] = Clip ( predEnhCb[ x ][ y ] + bCb[ x ][ y ] – predEnhCb[ x ][ y ] )

predSamplesCr[ x ][ y ] = Clip ( predEnhCr[ x ][ y ] + bCr[ x ][ y ] – predEnhCr[ x ][ y ] )

- Otherwise:

predSamplesL[ x ][ y ] = Clip ( predEnhL[ x ][ y ] + (bL[ x ][ y ] - predEnhL[ x ][ y ]) >> 1 )

predSamplesCb[ x ][ y ] = Clip ( predEnhCb[ x ][ y ] + (bCb[ x ][ y ] – predEnhCb[ x ][ y ]) >> 1)

predSamplesCr[ x ][ y ] = Clip ( predEnhCr[ x ][ y ] + (bCr[ x ][ y ] – predEnhCr[ x ][ y ]) >> 1 )

# Patent rights declaration(s)

Nokia may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).