| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0039 |
| --- | --- |

| Title: | On SHVC RAP pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This contribution contains the following proposals related to 1) enhancement layer CRA pictures, 2) BLA and IDR pictures, and 3) cross-layer impacts of RAP pictures:

- On EL CRA pictures:

  - CRA NAL unit type can be used when nuh_layer_id is greater than 0.

  - Inter-layer prediction is allowed for CRA NAL units with nuh_layer_id greater than 0, while inter prediction is disallowed.

  - CRA NAL units need not be aligned across layers. In other words, a CRA NAL unit type can be used for all VCL NAL units with a particular value of nuh_layer_id while another NAL unit type can be used for all VCL NAL units with another particular value of nuh_layer_id in the same access unit.

- On IDR and BLA pictures:

  - BLA pictures shall have nuh_layer_id equal to 0.

  - IDR pictures may have nuh_layer_id greater than 0 and they may be inter-layer predicted while inter prediction is disallowed.

  - IDR pictures shall be present in an access unit either in no layers or in all layers, i.e. an IDR nal_unit_type indicates a complete IDR access unit where decoding of all layers can be started.

  - NAL unit types BLA_W_DLP and BLA_N_LP with nuh_layer_id greater than 0 are used to indicate a stepwise layer access (STLA) picture (STLA_W_DLP and STLA_N_LP, respectively), which is otherwise identical to an IDR picture with nuh_layer_id greater than 0 but needs not be aligned across layers.

- On cross-layer impacts:

  - After a BLA picture at the base layer, the decoding of an enhancement layer is started when the enhancement layer contains a RAP picture and the decoding of all of its reference layers has been started.

  - When the decoding of an enhancement layer starts from a CRA picture, its RASL pictures are handled similarly to RASL pictures of a BLA picture.

  - Layer down-switching or unintentional loss of reference pictures is identified from missing reference pictures, in which case the decoding of the related enhancement layer continues only from the next RAP picture on that enhancement layer.

# Introduction

In some discussions among JCT-VC experts, a design choice of re-using the NAL unit type values of the RAP pictures with nuh_layer_id greater than 0 to indicate enhancement layer random access points has been discussed. In this contribution, the proponents explored the mentioned design choice in greater detail and made detailed decisions on some aspects of enhancement layer RAP picture characteristics. Moreover, a detailed high-level decoding process specification was authored that realizes the proposed enhancement layer RAP functionality.

The contribution is organized as follows: Section 2 describes the motivation for each proposal listed in the abstract. Section 3 presents the proposed specification text for the decoding process. Section 4 provides illustrative examples of the enhancement layer RAP behavior.

# Motivation for the Proposals

## EL CRA pictures

a. CRA NAL unit type can be used when nuh_layer_id is greater than 0.

Motivation: nuh_layer_id can be used to make a distinction between an EL-CRA picture and a conventional BL-CRA picture. There is no reason to spend NAL unit type values for EL-IDR/EL-CRA pictures.

b. Inter-layer prediction is allowed for CRA NAL units with nuh_layer_id greater than 0, while inter prediction is disallowed.

Motivation: Similar design was used in SVC and assumed in many SHVC CfP responses.

c. CRA NAL units need not be aligned across layers. In other words, a CRA NAL unit type can be used for all VCL NAL units with a particular value of nuh_layer_id while another NAL unit type can be used for all VCL NAL units with another particular value of nuh_layer_id in the same access unit.

Motivation: This allows the use of different RAP periods in different layers. For example, more frequent RAP pictures can be used in the base layer, where they are cheaper due to a smaller spatial resolution. Such flexibility was allowed in SVC too.

## IDR and BLA pictures

a. BLA pictures shall have nuh_layer_id equal to 0.

Motivation: At a splice point, the first access unit of the latter coded video sequence shall have an IDR picture or a BLA picture at the base layer in order to enable correct decoding. A simple requirement “BLA pictures shall have nuh_layer_id equal to 0” is sufficient to be able to omit decoding of non-base layers until they contain a RAP picture. Consequently, splicers only need to change the NAL unit type of base-layer NAL units when performing the splicing operation.

b. IDR pictures may have nuh_layer_id greater than 0 and they may be inter-layer predicted while inter prediction is disallowed.

Motivation: Similar design was used in SVC and assumed in many SHVC CfP responses.

c. IDR pictures shall be present in an access unit either in no layers or in all layers, i.e. an IDR nal_unit_type indicates a complete IDR access unit where decoding of all layers can be started.

Motivation: A video parameter set and a sequence parameter set (potentially applicable to more than one layer) may be activated at the start of a coded video sequence. In multi-loop decoding operation a start of a coded video sequence should be such where the decoding of all layers is initialized. For VPS and SPS activation it is important to be able to conclude a start of a new coded video sequence already when decoding the first slice of the base layer picture.

A design to require the presence of IDR pictures in all views within an access unit was taken in MVC, which uses a multi-loop decoding operation.

d. NAL unit types BLA_W_DLP and BLA_N_LP with nuh_layer_id greater than 0 are used to indicate a stepwise layer access (STLA) picture (STLA_W_DLP and STLA_N_LP, respectively), which is otherwise identical to an IDR picture with nuh_layer_id greater than 0 but needs not be aligned across layers.

Motivation: A STLA picture may activate a new layer SPS, while a CRA picture with nuh_layer_id greater than 0 never activates a new layer SPS.

STLA pictures allow different RAP periods in different layers. For example, more frequent RAP pictures can be used in the base layer, where they are cheaper due to a smaller spatial resolution. Such flexibility was allowed in SVC too. Without STLA pictures, different RAP periods could be allowed only through CRA pictures, which cannot be used to activate a new layer SPS.

## Cross-layer impacts

a. After a BLA picture at the base layer, the decoding of an enhancement layer is started when the enhancement layer contains a RAP picture and the decoding of all of its reference layers has been started.

Motivation: A BLA picture with nuh_layer_id equal to 0 has originally been a CRA picture with nuh_layer_id equal to 0 but the NAL unit type has been changed in a splicing operation. As there is no requirement to have STLA or CRA access units across all layers in the same access unit, there may be temporally predicted pictures with nuh_layer_id greater than 0 in the access unit that contains the BLA picture with nuh_layer_id equal to 0 and subsequent access units in decoding order. These temporally predicted pictures cannot typically be decoded correctly as some of their reference pictures are not present in the spliced bitstream. Consequently, they should be handled like TFD pictures for a BLA picture or for a CRA picture that starts a bitstream.

b. When the decoding of an enhancement layer starts from a CRA picture, its RASL pictures are handled similarly to RASL pictures of a BLA picture.

Motivation: When the decoding of an enhancement layer starts from a CRA picture, some of the reference pictures for RASL pictures have not been decoded and hence the RASL pictures have to be specifically handled.

c. Layer down-switching or unintentional loss of reference pictures is identified from missing reference pictures, in which case the decoding of the related enhancement layer continues only from the next RAP picture on that enhancement layer.

Motivation: Multi-loop decoding operation facilitates layer down-switching at any access unit. Hence, the normative decoding operation should support such flexibility in layer down-switching. The reference picture set design enables detecting of down-switching or unintentional reference picture losses, and hence the decoding operation can be based on the information obtained from the RPS.

# Proposed Specification Text

To be more exact, the following types of changes are proposed to clause 8.1 (or similar clause for SHVC Annex). The font color indicates for which proposal of this sub-section the proposed change is associated with:

- purple: general changes needed for scalability support

- red: item 3.a: After a BLA picture at the base layer, the decoding of an enhancement layer is started when the enhancement layer contains a RAP picture and the decoding of all of its reference layers has been started.

- green: item 3.b: When the decoding of an enhancement layer starts from a CRA picture, its RASL pictures are handled similarly to RASL pictures of a BLA picture.

- blue: item 3.c: Layer down-switching or unintentional loss of reference pictures is identified from missing reference pictures, in which case the decoding of the related enhancement layer continues only from the next RAP picture on that enhancement layer.

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in subclause Error: Reference source not found.

- The processes in subclause Error: Reference source not found specify decoding processes using syntax elements in the slice segment layer and above:

–	Variables and functions relating to picture order count are derived in subclause Error: Reference source not found (which only needs to be invoked for the first slice segment of a picture).

–	The decoding process for reference picture set in subclause Error: Reference source not found is invoked, wherein reference pictures may be marked as "unused for reference" or "used for long-term reference" (which only needs to be invoked for the first slice segment of a picture).

–	When CurrPic is an IDR picture, LayerInitialisedFlag[ nuh_layer_id ] is set equal to 1.

–	When the current picture is a BLA picture or is a CRA picture that is the first picture in the bitstream, the variable LayerInitialisedFlag[ 0 ] is set equal to 1, the variable LayerInitialisedFlag[ i ] is set equal to 0 for all values of i from 1 to 63, inclusive, and the decoding process for generating unavailable reference pictures specified in subclause Error: Reference source not found is invoked (which only needs to be invoked for the first slice segment of a picture).

–	When CurrPic is either a CRA picture or a STLA picture and LayerInitialised[ nuh_layer_id ] is equal to 0 and LayerInitialisedFlag[ refLayerId ] is equal to 1 for all values of refLayerId equal to ref_layer_id[ nuh_layer_id ][ j ], where j is in the range of 0 to num_direct_ref_layers[ nuh_layer_id ] – 1, inclusive, the following applies:

–	LayerInitialisedFlag[ nuh_layer_id ] is set equal to 1.

–	When CurrPic is a CRA picture and nuh_layer_id is greater than 0, the decoding process for generating unavailable reference pictures specified in subclause 8.3.3 is invoked.

–	LayerInitialisedFlag[ nuh_layer_id ] is set equal to 0, when all of the following are true:

–		CurrPic is a non-RAP picture.

–		LayerInitialisedFlag[ nuh_layer_id ] is equal to 1.

–		One or more of the following is true:

–	Any value of RefPicSetStCurrBefore[ i ] is equal to "no reference picture", with i in the range of 0 to NumPocStCurrBefore − 1, inclusive.

–	Any value of RefPicSetStCurrAfter[ i ] is equal to "no reference picture", with i in the range of 0 to NumPocStCurrAfter − 1, inclusive.

–	Any value of RefPicSetLtCurr[ i ] is equal to "no reference picture", with i in the range of 0 to NumPocLtCurr − 1, inclusive.

–	When LayerInitialisedFlag[ nuh_layer_id ] is equal to 1, slices of the picture are decoded as specified in this subclause. When LayerInitialisedFlag[ nuh_layer_id ] is equal to 0, slices of the picture are not decoded.

–	PicOutputFlag is set as follows:

–	If LayerInitialisedFlag[ nuh_layer_id ] is equal to 0, PicOutputFlag is set equal to 0.

–	Otherwise, if the current picture is a RASL picture and the previous RAP picture with the same non-zero nuh_layer_id in decoding order is a CRA picture and the value of LayerInitialisedFlag[ nuh_layer_id ] was equal to 0 at the start of the decoding process of that CRA picture, PicOutputFlag is set equal to 0.

–	Otherwise, if the current picture is a RASL picture with nuh_layer_id equal to 0 and the previous RAP picture with nuh_layer_id equal to 0 in decoding order is a BLA picture or is a CRA picture that is the first coded picture in the bitstream, PicOutputFlag is set equal to 0.

–	Otherwise, PicOutputFlag is set equal to pic_output_flag.

–	At the beginning of the decoding process for each P or B slice, the decoding process for reference picture lists construction specified in subclause Error: Reference source not found is invoked for derivation of reference picture list 0 (RefPicList0), and when decoding a B slice, reference picture list 1 (RefPicList1).

–	After all slices of the current picture have been decoded, the decoded picture is marked as "used for short-term reference".

- The processes in subclauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found specify decoding processes using syntax elements in the coding tree unit layer and above.

# Illustrative Examples

## Spliced bitstream

This example illustrates a concatenated bitstream starting from a splice point. The output order is from left to right and the decoding order and prediction references are illustrated with arrows. Let us assume that the enhancement layer (EL) has nuh_layer_id equal to 1.

The access unit following the splice point had been generated by converting a CRA picture at the base layer to a BLA picture (BLA_N_LP) during the splicing operation. No RAP picture is present for the enhancement layer of the same access unit.

The decoding process notices the BLA_N_LP picture on the base layer and sets LayerInitialisedFlag[ 0 ] to 1 LayerInitialisedFlag[ 1 ] to 0.

As the access unit containing BLA_N_LP picture on the base layer does not contain a RAP picture on the enhancement layer, the EL picture is not decoded.

The decoding then continues from the next access unit (TRAIL_R on the BL, CRA_NUT on the EL). As the enhancement layer contains a RAP picture, it (CRA_NUT on the EL) is decoded and LayerInitialisedFlag[ 1 ] is set to 1.

The CRA_NUT on the EL is followed in decoded order by skipped leading pictures (RASL), which according to the HEVC decoding process are decoded but for which the unavailable reference pictures are generated. Consequently, the decoded RASL pictures are unlikely to be correct in content and also they are not output.

After that (for access units containing either TRAIL_R or TSA pictures), the decoding process operates normally, i.e. both layers are decoded.

## Random access or bitstream switch to a position where only BL contains CRA

This example illustrates a decoded bitstream when a player has seeked to a new position in a bitstream and restarts playback from that position. Equivalently, this example could illustrate a situation when a DASH client has switched to another SHVC representation for bitrate adaptation. The example illustrates the switch-to bitstream and its decoding operation. The first access unit of the switch-to bitstream contains a CRA picture on the base layer but not on the enhancement layer. The player or DASH client sets HandleCraAsBlaFlag equal to 1 for the first access unit, hence the decoding operation of the presented bitstream is identical to the decoding of the example in the previous sub-section (provided that the CRA picture in the base layer has no leading pictures – otherwise, RADL pictures of the BL CRA picture can be decoded while RASL pictures of the BL CRA can be omitted).

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).