| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0040 |
| --- | --- |

| Title: | Non-TE5: On motion mapping between layers in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kemal Ugur | Tel: Email: | +358 50 486 0857 kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

TE5 tests motion mapping concept to improve the coding efficiency of a potential high-level syntax only approach for SHVC. This contribution shows the results of an alternative motion mapping algorithm that is argued to be simpler than the one tested in TE5.  The tested algorithm utilizes information from a single block from base layer, instead of multiple blocks, to derive motion information of enhancement layer. The experimental results show that there is 0.0% bitrate difference compared to the algorithm tested in TE5.4.1.

# Introduction

In TE 5.4.1, motion mapping concept is tested to allow using motion information of co-located picture in base layer as reference for enhancement layer TMVP. Some blocks in EL can overlap multiple blocks in BL and in those cases motion information of EL block is derived using the corresponding four blocks from BL (see Figure-1). The mapping algorithm works as follows:

- motion vectors are derived by taking the median of those blocks if all or three of those motion blocks are available. If only two are available, then the average values motion vectors are taken. If only one is available, then that one is used. If none of the motion information is available, the mapped motion becomes zero.

- for ref_idx, histogram of the four motion blocks is taken and the index that occurs the most is chosen.

Figure 1 Illustration of overlapping motion blocks (JCTVC-K0034)

This contribution simplifies this by always copying the motion information (MV and ref_idx) of top-left block from BL if available, otherwise setting the corresponding motion information to 0. In other words, for the block u4 in EL, only the motion information from b0 is utilized (instead of using b0, b1, b2, b3)

# Results

Results are compared to TE5.4.1 algorithm in the below table. In SNR and 2x cases, enhancement layer block does not overlap multiple blocks in base layer, so both the proposal and TE 5.4.1 shows identical results.

The tests were run in a heterogeneous cluster and the running times are not accurate.

Figure 2 Results of the proposed algorithm compared to TE5.4.1

Figure 3 Results of proposed algorithm compared to not using motion mapping

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).