| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0041 |
| --- | --- |

| Title: | AHG4: Performance evaluation of HEVC on still picture coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kemal Ugur Jani Lainema | Tel: Email: | +358 50 486 0857 kemal.ugur@nokia.com jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution presents results of several experiments that aim to analyze the still picture coding performance of HEVC compared with JPEG, JPEG-XR and JPEG-2000. This is done by coding first picture of the standard test-set as still picture using the above codecs at various bitrates and measuring the BD-rate difference. From the objective results obtained using the JCT-VC testset, following conclusions could be drawn:

- The improvement HEVC brings over other still pictures is larger at high resolutions, compared to smaller resolutions and larger at low bitrates, compared to higher bitrates.

- When all the results are averaged, it takes around 2.2X more bits to code same picture with JPEG, 1.5X  more bits to code the same picture with JPEG-XR and 1.8X more bits to code the same picture with JPEG-2000.

- HEVC provides similar lossless coding capability compared to JPEG-XR but better than JPEG-2000.

# Introduction

Draft HEVC standard includes a profile for coding still pictures. In this contribution, we aim to present the coding efficiency performance of this profile and compare it with other still picture codecs, JPEG, JPEG-XR and JPEG-2000. This is done by coding first picture of the standard test-set as still picture using the above codecs at various bitrates and measuring the BD-rate difference

The bitstreams are generated by aiming to maximize the objective performance, using the available reference software of the standards, when available. The exact configurations and command line parameters are provided in Appendix

# Objective results

## High bitrate test

For this test, we first code the test set using HM common conditions where QP is set-up at: 12,17,22,27. The PSNR of the resulting pictures are recorded and pictures are coded so that the PSNR’s match for other codecs. The results are given below.

JPEG:

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 69.2% | 283.6% | 242.6% |
| Class B | 67.0% | 359.1% | 474.5% |
| Class C | 74.8% | 128.0% | 210.6% |
| Class D | 69.6% | 230.8% | 91.6% |
| Class E | 105.9% | 344.1% | 315.5% |
| Overall | 76.0% | 268.4% | 278.5% |

JPEG-XR:

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 29.2% | 41.6% | 41.0% |
| Class B | 39.1% | 55.0% | 66.9% |
| Class C | 34.3% | 46.9% | 50.3% |
| Class D | 32.9% | 44.7% | 46.4% |
| Class E | 46.7% | 81.0% | 88.7% |
| Overall | 36.8% | 53.8% | 59.4% |
|  | 36.8% | 51.4% | 59.3% |
| Class F | 103.4% | 112.6% | 118.3% |

JPEG-2000

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 92.2% | 82.0% | 68.6% |
| Class B | 87.9% | 92.0% | 146.5% |
| Class C | 61.9% | 173.3% | 163.7% |
| Class D | 58.3% | 118.1% | 153.0% |
| Class E | 138.6% | 77.9% | 59.6% |
| Overall | 84.5% | 112.4% | 128.6% |
|  | 82.3% | 121.4% | 139.5% |
| Class F | #VALUE! | #VALUE! | #VALUE! |

## Medium bitrate test

For this test, we first code the test set using HM common conditions where QP is set-up at: 22,27,32,37. The PSNR of the resulting pictures are recorded and pictures are coded so that the PSNR’s match for other codecs. The results are given below.

JPEG:

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 87.1% | 167.9% | 166.7% |
| Class B | 123.6% | 224.7% | 251.2% |
| Class C | 121.7% | 230.1% | 236.7% |
| Class D | 109.6% | 180.4% | 242.0% |
| Class E | 169.5% | 217.5% | 210.1% |
| Overall | 123.7% | 208.5% | 229.7% |
|  | 123.5% | 207.5% | 229.1% |
| Class F | 222.7% | 406.9% | 351.4% |

JPEG-XR

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 43.8% | 66.9% | 67.9% |
| Class B | 62.2% | 103.7% | 106.7% |
| Class C | 52.6% | 77.9% | 83.1% |
| Class D | 47.1% | 71.4% | 75.0% |
| Class E | 73.0% | 119.6% | 122.8% |
| Overall | 56.5% | 89.3% | 92.8% |
|  | 56.4% | 89.2% | 92.4% |
| Class F | 117.7% | 133.9% | 141.1% |

JPEG-2000

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 81.1% | 33.7% | 25.1% |
| Class B | 98.4% | 29.7% | 35.8% |
| Class C | 67.4% | 65.7% | 58.9% |
| Class D | 63.0% | 53.8% | 63.6% |
| Class E | 119.6% | 49.1% | 42.1% |
| Overall | 85.3% | 46.7% | 47.0% |
|  | 85.1% | 47.6% | 48.6% |
| Class F | 128.0% | 275.2% | 117.2% |

## Low bitrate test

For this test, we first code the test set using HM common conditions where QP is set-up at: 32,37,42,47. The PSNR of the resulting pictures are recorded and pictures are coded so that the PSNR’s match for other codecs. The results are given below.

JPEG:

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 123.1% | 247.8% | 302.3% |
| Class B | 177.9% | 341.7% | 392.2% |
| Class C | 135.2% | 257.8% | 282.6% |
| Class D | 122.2% | 211.6% | 269.8% |
| Class E | 218.6% | 346.7% | 339.5% |
| Overall | 156.7% | 284.5% | 321.9% |
|  | 156.8% | 283.8% | 322.7% |
| Class F | 224.6% | 376.1% | 414.4% |

JPEG-XR:

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 61.5% | 227.2% | 298.4% |
| Class B | 75.8% | 358.2% | 432.4% |
| Class C | 62.4% | 298.1% | 326.2% |
| Class D | 53.6% | 276.6% | 278.3% |
| Class E | 90.0% | 370.7% | 393.8% |
| Overall | 68.7% | 314.3% | 353.2% |
|  | 68.6% | 304.4% | 324.0% |
| Class F | 101.3% | 267.8% | 291.4% |

JPEG-2000

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 72.3% | 30.0% | 18.7% |
| Class B | 80.9% | 34.3% | 49.0% |
| Class C | 75.2% | 74.3% | 69.4% |
| Class D | 78.7% | 77.1% | 88.3% |
| Class E | 103.1% | 81.1% | 64.5% |
| Overall | 81.9% | 60.0% | 61.5% |
|  | 81.9% | 60.3% | 62.9% |
| Class F | 98.0% | 156.7% | 171.2% |

## Lossless coding

Lossless coding is quite useful for still picture applications, for example in medical imaging. Here we test HEVC lossless coding performance and compare it with JPEG-XR and JPEG2000.

|  | Ratio over HEVC lossless coding |  |
| --- | --- | --- |
| Sequence | JPEG-XR | JPEG-2000 |
| Traffic | 0.940282 | 1.432876 |
| PeopleOnStreet | 0.946981 | 1.39589 |
| Kimono | 0.952121 | 1.426793 |
| ParkScene | 0.943484 | 1.427948 |
| Cactus | 0.983422 | 1.486488 |
| BasketBallDrive | 1.035024 | 1.507558 |
| BQTerrace | 1.028034 | 1.446572 |
| BasketballDrill | 1.068806 | 1.550847 |
| BQMall | 1.020084 | 1.458874 |
| PartyScene | 0.986234 | 1.414783 |
| RaceHorses_big | 0.979853 | 1.37922 |
| BasketBallPass | 1.024931 | 1.420952 |
| BQSquare | 1.061919 | 1.39568 |
| BlowingBubbles | 0.981402 | 1.42266 |
| RaceHorses_s | 0.959498 | 1.373169 |
| FourPeople | 1.003635 | 1.423272 |
| Johnny | 1.053955 | 1.494122 |
| KristenAndSara | 1.055418 | 1.495869 |
| BasketBallDrillText | 1.105048 | 1.570421 |
| ChinaSpeed | 1.412287 | 1.723284 |
| SlideEditing | 1.952294 | 1.996417 |
| SlideShow | 2.027442 | 1.760459 |
| Average | 1.114643 | 1.500189 |
| Average (no Class F) | 1.006849 | 1.448631 |

## Summary and Conclusions

Table 1 Average increase in bitrate compared to HEVC (excluding Class-F)

|  | JPEG | JPEG-XR | JPEG2000 |
| --- | --- | --- | --- |
| High bitrate | +76.0% | +36.8% | +84.5% |
| Medium bitrate | +123.7% | +56.5% | +85.3% |
| Low bitrate | +156.7% | +68.7% | +81.9% |
| Average | +118.8% | +54.0% | +83.9% |
| Lossless | X | +0.0% | +44.9% |

From the objective results obtained using the JCT-VC testset, following conclusions could be drawn:

- The improvement HEVC brings over other still pictures is larger at high resolutions, compared to smaller resolutions.

- The improvement HEVC brings over other still pictures is larger at low bitrates, compared to higher bitrates.

- When all the results are averaged, it takes around 2.2X more bits to code same picture with JPEG, 1.5X  more bits to code the same picture with JPEG-XR and 1.8X more bits to code the same picture with JPEG-2000.

- HEVC provides similar lossless coding capability compared to JPEG-XR but better than JPEG-2000.

# Appendix

Here we provide details of the software and associated command line parameters used to obtain JPEG, JPEG-XR and JPEG2000 results. The codecs were configured to operate in maximizing objective rate-distortion performance.

## JPEG-XR

The ITU-T Rec. T.835 (01/2010) JPEG XR Reference Software is used with the following command line parameters:

-c -o out.jxr -a 0 -f YUV420 -q $qp -d -P 111 -r -W $WIDTH -H $HEIGHT -M 19 $SEQ_FILE

## JPEG-2000

The ITU-T Rec. T.804 JPEG-2000 JasPer reference software is used with the following command line parameters:

jasper -f in.pnm -F out.jp2 -T jp2 -O rate=$qp

## JPEG

ImageMagick convert application is used to encode the images in JPEG, with the following command line options:

convert -size ${WIDTH}x${HEIGHT} yuv:$SEQ_FILE -sampling-factor 4:2:0 -quality $qp out.jpg