| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0044v6 |
| --- | --- |

| Title: | AHG9: HEVC HRD cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Adarsh K. Ramasubramonian Ying Chen 5775 Morehouse Drive San Diego, CA 92121-1714 USA Sachin Deshpande 5750 NW Pacific Rim Blvd Camas, WA 98607 USA Kimihiko Kazui 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com 1-360-817-8486 sdeshpande@sharplabs.com +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com |
| Source: | Qualcomm Incorporated, Sharp Labs of America, Inc., FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

This contribution proposes some cleanup changes on HEVC HRD. The proposals are summarized in the contribution document, and the proposed spec text changes, marked in relative to JCTVC-L0030, are enclosed in the same zip file containing the contribution document.

In version 2 of this document, in order to enable further delay reduction, the derivation of the CPB removal time for decoding units (DUs) as specified in Equation C-14 is changed such that no clock sub-tick alignment adjustment is made when the low delay flag is equal to 1 and the nominal CPB removal time is earlier than the final CPB arrival time, rather the DU CPB removal time in this case is derived as equal to the final CPB arrival time.

In version 3 of this document, the solution for the issue on the CPB operations related to CRA and BLA pictures is modified such that the proposed offset is also applied to the DPB output times, by adding the offset back to the DPB output time of each of the access units following the CRA or BLA access unit in decoding order, to maintain the delta between the DPB output times of any two pictures within a coded video sequence idential to the delta between the capturing times.

In version 4 of this document, for the subject in sub-section 1.2.2 of JCTVC-L044, a simplifed list of changes (compared to version 3) is proposed that only consists of the following:

- Subclause C.2.3 (Timing of decoding unit removal and decoding of decoding unit) is changed such that 1) For AU-level operation, derivation of DU-level timing is not invoked; 2) AU-level timing derivation does not depend on DU-level CPB parameters; 3) Derivation of AU-level CPB removal time is not needed for DU-level operation and derivation of DU-level CPB removal time is not needed for AU-level operation, and both equations are simplified.

In version 5 of this document, text changes marked with user names "HRD#2" and "HRD#3" are both proposed in combination to solve the AU-level and DU-level CPB removal time alignment issue.

In version 6 of this document, regarding the subject in subsection 1.1, instead of applying the signalled delay offset to shift forward in time the DPB output times of the pictures following the CRA or BLA picture in decoding order, the offset is applies to shift backward in time the DPB output time of the CRA or BLA picture. This way, after discarding the RASL picture, not only decoding is continous, but also output is continuous. The text for this subject is included in the attachment marked with user name"HRD#1".

# Summary of the proposed changes

## On CPB operations related to CRA and BLA pictures

Currently, when access unit (AU) 0, where the HRD is initialized, is a CRA or BLA AU, if the associated RASL AUs are present, the default initial CPB removal delay and delay offset are used, otherwise the alternative initial CPB removal delay and delay offset are used.

However, the change of the initial CPB removal delay and delay offset (compared to the case with the associated RASL AUs being present) does not change the time duration between the nominal decoding times of AU0 and the first non-RASL AU following AU0 in decoding order. For example, assume that there were 10 RASL pictures immediately following AU0 in decoding order and immediately followed by trailing pictures in the original bitstream, the CBP removal time is equal to nominal CPB removal time for each AU, and the time duration between the decoding times of any two consecutive AUs is one clock tick when the original bitstream is decoded. According to the current spec, the time duration between the decoding times of AU0 and the first trailing picture would be the same (11 clock ticks) regardless of whether the RASL pictures are discarded.

In this case, decoding of consecutive pictures is continuous for the original bitstream, but not when the RASL pictures are discarded. This would incur additional end-to-end delay. In addition, as the bits of the trailing pictures would flow into the CPB earlier than the case with the associated RASL AUs being present, while decoding of the first trailing picture needs to wait such a long time period, the CPB may easily overflow, unless the CBP size is sufficiently increased to compensate the effect.

Such a problem would be cumulative if there are one or more BLA access units that follow access unit 0 in decoding order for which the associated RASL access units are discarded (i.e. not transmitted as derminted by a streaming server).

To solve this problem, the following change is proposed.

A CPB removal delay offset is signalled for each CRA or BLA access unit. For each CRA or BLA access unit for which the associated RASL pictures are not present, the CPB removal time of each of the access units following the CRA or BLA access unit in decoding order is shifted earlier by the CPB removal delay offset.

In version 3 of this document, the above offset is also applied to the DPB output times, by adding the offset back to the DPB output time of each of the access units following the CRA or BLA access unit in decoding order, to maintain the delta between the DPB output times of any two pictures within a coded video sequence idential to the delta between the capturing times.

In version 6 of this document, regarding the subject in subsection 1.1, instead of applying the signalled delay offset to shift forward in time the DPB output times of the pictures following the CRA or BLA picture in decoding order, the offset is applies to shift backward in time the DPB output time of the CRA or BLA picture. This way, after discarding the RASL picture, not only decoding is continous, but also output is continuous.

The text changes are included in the attachment to this document under user name "HRD#1".

## On sub-picture level CPB operations

HEVC includes the support of sub-picture based CPB operations to enable reduced codec delay, sometimes referred to as ultralow delay. The CPB may operate at either access unit (AU) level or sub-picture level, depending on whether sub-picture level CPB operation is preferred by a decoder (which is specified by an external means not specified in the HEVC spec) and whether sub-picture CPB parameters are present (in the bitstream or through external means not specified in the HEVC spec). If both conditions are true, the CPB operates at sub-picture level (and in this case each decoding unit (DU) is a subset of an AU); otherwise the CPB operates at AU level (and in this case each DU is an AU).

For simplicity, in the following descriptions, we also refer to sub-picture level operations and parameters as DU-level operations and parameters.

Eqn. C-15 is as follows:

DpbOutputTime( n ) = CpbRemovalTime( n ) + ClockTick * pic_dpb_output_delay( n )	(C-1)

From the equation, we can see that the output time of a decoded picture is equal to the decoding time (i.e. CPB removal time) of the last DU (i.e. the AU itself for AU-level operation) plus the signalled DPB output delay. Therefore there are two possible approaches that may be utilized to reduce the picture output delay in DU-level operation:

- to reduce the signalled value of pic_dpb_output_delay; and

- to shift earlier the decoding time.

Currently, when DU-level CPB parameters are present, the low delay flag is equal to 1, and the final CPB arrival time is later than the nominal CPB removal time, it is required that the CPB removal time of the last DU of an AU for DU-level operation is equal to the CPB removal time of the AU for AU-level operation. As part of this requirement, the derivation of AU-level CPB timing depends on signalled and some derived DU-level CPB parameters.

We see the following issues associated with this requirement:

- It disallows shifting earlier the decoding time of the last DU for DU-level operation compared to the decoding time of the AU for AU-level operation, i.e. disallows taking advantage of the second approach to achieve ultralow delay.

- The AU-level HRD behavior is not the same as in AVC if DU-level CPB parameters are present and is the same otherwise. Consequently, the AU-level HRD behavior is not the same between the case when sub-picture-level CPB parameters are present and the case when sub-picture-level CPB parameters are not present. However, it seems desirable to keep the AU-level HRD behavior the same as in AVC regardless of whether DU-level CPB parameters are present.

- Decoders that wish to perform only AU-level HRD operations cannot ignore DU-level CPB parameters, when present, and also have to derive some DU-level CPB parameters. However, it seems desirable if such decoders can ignore DU-level CPB parameters, when present.

Also, since there is only one pic_dpb_output_delay signalled for each picture for both AU-level and sub-picture-level operations, the first approach is not utilized either by the current design.

Consequently, as such the sub-picture level HRD feature does not work for its purpose.

To solve these problems, two sets of changes, as described below, are proposed.

### First set of changes

The first set of changes takes advantage of the first approach, i.e. to reduce the picture output delay by reducing the signalled value of pic_dpb_output_delay.

When sub_pic_cpb_params_present_flag is equal to 1, one additional DPB output delay is signalled for each AU in the picture timing SEI message. This additional DPB output delay is used in for derivation of the DPB output time for sub-picture based HRD operations.

The text changes are included in the attachment to this document under user name "HRD#2".

### Second set of changes

The second set of changes remove the requirement on alignment of AU and DU CPB removal times as mentioned above, in order to be able to utilize the second approach, i.e. to shift earlier the decoding time, and to keep the AU-level HRD behavior the same as in AVC, and to enable decoders that wish to perform only AU-level HRD operations to ignore DU-level CPB parameters, when present.

The changes are summarized as follows:

- Subclause C.2.3 (Timing of decoding unit removal and decoding of decoding unit) is changed such that 1) For AU-level operation, derivation of DU-level timing is not invoked; 2) AU-level timing derivation does not depend on DU-level CPB parameters; 3) Derivation of AU-level CPB removal time is not needed for DU-level operation and derivation of DU-level CPB removal time is not needed for AU-level operation, and both equations are simplified.

- Item 10 in subclause C.4 (Bitstream conformance) is removed.

- The condition for the presence of du_cpb_removal_delay_increment_minus1[ i ] in the picture timing SEI message syntax is changed, and the semantics are changed accordingly.

- The semantics of the decoding unit information SEI message is changed.

In version 2 of this document, in order to enable further delay reduction, the derivation of the CPB removal time for decoding units (DUs) as specified in Equation C-14 is changed such that no clock sub-tick alignment adjustment is made when the low delay flag is equal to 1 and the nominal CPB removal time is earlier than the final CPB arrival time, rather the DU CPB removal time in this case is derived as equal to the final CPB arrival time.

The text changes are included in the attachment to this document under user name "HRD#3".

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).