| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0045v3 |
| --- | --- |

| Title: | AHG9: HEVC SEI messages cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Ying Chen Adarsh K. Ramasubramonian 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com 1-858-658-5804 aramasub@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes some cleanup changes as well as raises some issues to be discussed and addressed on SEI messages in HEVC. The proposed changes are in the contribution document, and the proposed spec text changes, marked in relative to JCTVC-L0030, are enclosed in the same zip file containing the contribution document.

To address comments received during the initial review of this contribution, version 2 of this contribution provides

- two alterntaive solutions for the issue of two different solutions for the issue of unclearly specified scope of non-nested SEI messages for the group to choose, both with the target that non-nested SEI messages correspond to the whole bitstream for HRD purposes

- modified texts for enabling of SEI messages to reside between VCL NAL units within an access unit

Version 3 of this document includes changed restrictions on nesting of SEI messages per the discussion on Jan. 22, 2013.

# Summary of the proposed changes

## On nested and non-nested SEI messages

For a nested SEI message (an SEI message contained in a scalable nesting SEI message), the applicable operation points, layers or sub-layers are specified by the semantics of the scalable nesting SEI message. However, the applicable operation points, layers or sub-layers of a non-nested SEI message are currently not specified. One consequence is that non-nested buffering period SEI messages, picture timing SEI messages and decoding unit information SEI messages are never selected for use in HRD operations.

### SEI messages providing parameters for HRD operations

Buffering period SEI messages, picture timing SEI messages, and decoding unit information SEI messages are the three types of SEI messages that provide parameters for HRD operations. Among the three, picture timing information SEI message also provides frame/field information.

Two alterntaive solutions for the issue of unclearly specified scope of non-nested buffering picture SEI messages, picture timing SEI messages, and decoding unit information SEI messages, both with the target that non-nested SEI messages correspond to the whole bitstream for HRD purposes, are provided below.

The common part of the two solutions is that non-nested buffering picture SEI messages, picture timing SEI messages, and decoding unit information SEI messages apply to the operation point that corresponds to the whole bitstream.

#### Solution 1

The first solution consists of adding the following bitstream conformance requirements or changes:

- When payloadType is equal to 0 (buffering period), 1 (picture timing), or 130 (decoding unit information) for a non-nested SEI message, the non-nested SEI message applies to the operation point that has OpTid equal to the greatest value of nuh_temporal_id_plus1 among all VCL NAL units in the whole bitstream, and that has OpLayerIdSet containing all values of nuh_reserved_zero_6bits of all VCL NAL units in the whole bitstream.

- All SEI NAL units conaining non-nested buffering picture SEI messages, picture timing SEI messages, or decoding unit information SEI messages are discarded during any bitstream extraction.

Reason, implication, explanation and discussion related to this contraint:

  - If not discarded in bitstream extractions, and a CBR mode is indicated for the CPB operation, then after some enhancement layers or sub-layers are discarded, the HRD conformance would be broken and consequently extracted sub-bitstream would be a non-conforming bitstream.

  - An implication of the discarding is that, frame/field information, when included in picture timing SEI messages, would be discarded during the bitstream extraction process too. This is one reason why it might be better to include the frame/field information in another SEI message.

  - A "smart" bitstream extractor may include non-nested buffering picture SEI messages, picture timing SEI messages, and decoding unit information SEI messages in the extracted sub-bitstream, provided that the SEI messages applicable to the sub-bitstream were present as nested SEI messages in the original bitstream.

  - This is a good spot for a last-chance discussion of whehter to mix frame/field information with picture timing information, just to make sure that the decision for the mixing is indeed a fully-informed decision.

    - When different picture timing SEI messages are provided for different operation points, frame/field information would the same in all picture timing SEI messages associated with a particular access unit, i.e. there can be a lot of repetition of frame/field information for a same picture. This does not really matter for those who don't care about either interlaced souce or field picture coding at all. But those who care about either interlaced souce or field picture coding should pay a little bit attention.

- SEI NAL units containing non-nested buffering picture SEI messages, picture timing SEI messages, or decoding unit information SEI messages are not allowed to contain SEI messages others than buffering picture SEI messages, picture timing SEI messages, and decoding unit information SEI messages. (this seems to be a good constraint regardless of which solution to go)

  - The motivation is to avoid other SEI messages that do not provide HRD operations related parameters being also discarded duing bitstream extractions.

  - An implication of this constraint is that an SEI NAL unit cannot contain a non-nested active parameter sets SEI message followed by a non-nested buffering period SEI message.

#### Solution 2

The second solution consists of adding the following bitstream conformance requirements or changes:

- When payloadType is equal to 0 (buffering period), 1 (picture timing), or 130 (decoding unit information) for a non-nested SEI message, the non-nested SEI message applies to the operation point that has OpTid equal to the greatest value of nuh_temporal_id_plus1 among all VCL NAL units in the whole bitstream, and that has OpLayerIdSet containing all values of nuh_reserved_zero_6bits of all VCL NAL units in the whole bitstream. (same as in solution 1)

- Change the semantics of nuh_reserved_zero_6bits from

nuh_reserved_zero_6bits shall be equal to 0. Other values of nuh_reserved_zero_6bits may be specified in the future by ITU-T | ISO/IEC. For purposes other than as specified in Annex Error: Reference source not found, decoders shall ignore (i.e. remove from the bitstream and discard) all NAL units with values of nuh_reserved_zero_6bits not equal to 0.

to

nuh_reserved_zero_6bits shall be equal to 0. Other values of nuh_reserved_zero_6bits may be specified in the future by ITU-T | ISO/IEC. For SEI NAL units containing non-nested buffering picture SEI messages, picture timing SEI messages, or decoding unit information SEI messages, although the value of nuh_reserved_zero_6bits is required to be equal to 0 in this version of this Specification, decoders shall allow other values of nuh_reserved_zero_6bits greater than 0 in the syntax. For purposes other than as specified in Annex Error: Reference source not found, decoders shall ignore (i.e. remove from the bitstream and discard) all NAL units with values of nuh_reserved_zero_6bits not equal to 0 except SEI NAL units containing non-nested buffering period SEI messages, picture timing SEI messages, or decoding unit information SEI messages.

### General nesting and non-nesting issues not specific to SEI messages providing parameters for HRD operations

The following changes are proposed:

- Add definitions of nested SEI messages and non-nested SEI messages

- Allow non-nested buffering period SEI messages, picture timing SEI messages and decoding unit information SEI messages to be selected for use in HRD operations

- The applicable operation points, layers or sub-layers for non-nested SEI messages are as follows:

–	If payloadType is equal to 0 (buffering period), 1 (picture timing), or 130 (decoding unit information), the non-nested SEI message applies to the operation point that has OpTid equal to the greatest value of nuh_temporal_id_plus1 among all VCL NAL units in the bitstream, and that has OpLayerIdSet containing all values of nuh_reserved_zero_6bits of all VCL NAL units in the bitstream.

–	Otherwise, when payloadType is equal to 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 128, 131, or 134 (i.e. one of the SEI messages that have payloadType not equal to 0, 1, or 130, and that are allowed to be nested SEI messages), the non-nested SEI message applies to the layer for which the VCL NAL units have nuh_reserved_zero_6bits equal to the nuh_reserved_zero_6bits of the SEI NAL unit containing the SEI message (note that in Version 1 nuh_reserved_zero_6bits is required to be equal to 0 for any NAL unit), and the TemporalId of the SEI NAL unit containing the SEI message shall be equal to the TemporalId of the access unit containing the SEI message.

- The following restrictions on nesting of SEI messages are proposed:

–	A scalable nesting SEI message shall not be nested in a scalable nesting SEI message.

–	An active parameter sets SEI message shall not be nested in a scalable nesting SEI message.

–	When a scalable nesting SEI message contains buffering period SEI message, picture timing SEI message or decoding unit infomation SEI message, the scalable nesting SEI message shall not contain any other SEI message with payloadType not equal to 0 (buffering period), 1 (picture timing), or 130 (decoding unit information).

–	When a buffering period SEI message, picture timing SEI message or decoding unit infomation SEI message is present as a nested SEI message, the same SEI message (with the same content) shall not be present as a non-nested SEI message.

Some of the text changes are included in the attachment to this document under user name "SEI cleanups#1 ".

## Enabling SEI message repetition within the same access unit

The changes proposed here relate to the folloiwng editing notes:

When any prefix SEI NAL units are present, they shall not follow the last VCL NAL unit of the access unit. [Ed. (GJS): Can the currently-specified prefix SEI NAL units (that have a whole-picture scope) be placed between the VCL NAL units of the picture?]

When an SEI NAL unit that contains a post-filter hint SEI message and has nuh_reserved_zero_6bits equal to 0 is present, the SEI NAL unit shall precede, in decoding order, the first VCL NAL unit in the access unit. [Ed. (GJS): How is it necessary to say that?]

Restrictions like "When an SEI NAL unit that contains a post-filter hint SEI message and has nuh_reserved_zero_6bits equal to 0 is present, the SEI NAL unit shall precede, in decoding order, the first VCL NAL unit in the access unit.", when present, are removed from the semantics of all types of SEI messages. Thus, non-sub-picture-level SEI messages can also be repeated within the same access unit.

This also allows scalable nesting SEI message to be after first VCL NAL unit in an AU, and hence be used for nesting of sub-picture level SEI messages after the first VCL NAL unit in an AU.

The text changes are included in the attachment to this document under user name "SEI cleanups#2".

## Temporal scalability consideration for some SEI messages

### Recovery point SEI message and region refresh information SEI message

The current semantics of the recovery point SEI message and the region refresh information SEI message have an issue with temporal scalability. For example, when the recovery point picture indicated by the recovery point SEI message has higher TemporalId than the picture containing the recovery point SEI message and in an extracted sub-bitstream wherein the indicated recovery point picture has been discarded hence does not exists.

To solve this problem, the semantics of the recovery point SEI message is changed with a different definition of the recovery point, along with other changes accordingly, and the semantics of the region refresh SEI message is changed with a different definition of gdrPicSet, along with other changes accordingly.

The text changes are included in the attachment to this document under user name "SEI cleanups#3".

### Progressive refinement segment start SEI message

For the progressive refinement segment start SEI message, the syntax element num_refinement_steps_minus1 plus 1 specifies the number of pictures in the tagged set of consecutive coded pictures that are the refinement of the picture associated with the SEI message. However, if the bitstream is temporally scalable and a bitstream subset may be extracted from the entire bitstream, then in the bitstream subset, the actual number of pictures in the tagged set of consecutive coded pictures would be less than as indicated by the syntax element num_refinement_steps_minus1.

To solve this problem, the syntax element num_refinement_steps_minus1 is replaced by pic_order_cnt_delta, to clearly specify which pictures are quality refinements of the current picture in a scalable way.

The text changes are included in the attachment to this document under user name "SEI cleanups#3".

## Some SEI messages with syntax-specified persistency scope

The following SEI messages include a repetition-period-like syntax element (e.g. pan_scan_rect_repetition_period in the pan-scan rectangle SEI message syntax) to specify the persistency scope:

- pan-scan rectangle

- film grain characteristics

- tone mapping information

- frame packing arrangement

- display orientation

For example, the semantics of pan_scan_rect_repetition_period is as follows:

pan_scan_rect_repetition_period specifies the persistence of the pan-scan rectangle SEI message and may specify a picture order count interval within which another pan-scan rectangle SEI message with the same value of pan_scan_rect_id or the end of the coded video sequence shall be present in the bitstream. The value of pan_scan_rect_repetition_period shall be in the range of 0 to 16384, inclusive. When pan_scan_cnt_minus1 is greater than 0, pan_scan_rect_repetition_period shall not be greater than 1.

pan_scan_rect_repetition_period equal to 0 specifies that the pan-scan rectangle information applies to the current decoded picture only.

pan_scan_rect_repetition_period equal to 1 specifies that the pan-scan rectangle information persists in output order until any of the following conditions are true:

–	A new coded video sequence begins.

–	The bitstream ends.

–	A picture in an access unit containing a pan-scan rectangle SEI message with the same value of pan_scan_rect_id is output having PicOrderCntVal greater than PicOrderCnt( CurrPic ).

pan_scan_rect_repetition_period greater than 1 specifies that the pan-scan rectangle information persists until any of the following conditions are true:

–	A new coded video sequence begins.

–	The bitstream ends.

–	A picture in an access unit containing a pan-scan rectangle SEI message with the same value of pan_scan_rect_id is output having PicOrderCntVal greater than PicOrderCnt( CurrPic ) and less than or equal to PicOrderCnt( CurrPic ) + pan_scan_rect_repetition_period. [Ed. (YK): This is probably not the design intent. I guess it should be the last picture in output order among those that fulfil the condition. Similar for several other SEI messages.]

pan_scan_rect_repetition_period greater than 1 indicates the presence of another pan-scan rectangle SEI message with the same value of pan_scan_rect_id for a picture in an access unit that is output having PicOrderCntVal greater than PicOrderCnt( CurrPic ) and less than or equal to PicOrderCnt( CurrPic ) + pan_scan_rect_repetition_period, unless the bitstream ends or a new coded video sequence begins without output of such a picture.

[Ed. (YK): The description of the persistency scope is in a way for description of decoding process or decoder behavior. It might better to describle it as a bitstream property instead. Similar for several other SEI messages.]

There are two issues with the semantics, which apply to all the list SEI messages, as mentioned in the above highlighted editing notes.

These issues, particularly the first one, should be discussed and addressed during this meeting.

## Non-syntax-specified persistency scopes for some SEI messages

The following SEI messages belong to those for which the persistency scopes are not specified by syntax, and have some issues as noted in editing notes in Table D-1 (excerpted):

Table D-1 – Persistence scope of prefix SEI messages (informative)

| - SEI message | - Persistence scope |
| --- | --- |
| Scene information | - The access unit containing the SEI message and up to but not including the next access unit, in decoding order, that contains a scene information SEI message [Ed. (GJS): Seems incorrect. That's a big problem for a splicer if it reaches beyond the end of the coded video sequence.] |
| Post-filter hint | - The access unit containing the SEI message [Ed. (GJS): Current semantics of this message are vague about scope.] |

These issues, particularly the first one, should be also be discussed and addressed during this meeting.

## On pan_scan_cnt_minus1 equal to 2

The semantics of pan_scan_cnt_minus1 is as follows:

pan_scan_cnt_minus1 equal to 2 indicates that three pan-scan rectangles are specified that apply to the decoded pictures that are within the persistency scope of the current SEI message and that are associated with picture timing SEI messages having pic_struct equal to 5 or 6. The first rectangle applies to the first field of the frame in output order, the second rectangle applies to the second field of the frame in output order, and the third rectangle applies to a repetition of the first field as a third field in output order, where the output order of fields in one frame is as shown in Error: Reference source not found for pic_struct equal to 5 or 6. [Ed. (YK): Is it possible for the first rectangle and the third rectangle, which are for the same field, be different? If not, then pan_scan_cnt_minus1 shall never be equal to 2.]

The issue mentioned in the editing note should be discussed and addressed at this meeting.

## On the SOP description SEI message

There are a few issues on the SOP description SEI message, as noted in the following listed editing notes, should be discussed and addressed during this meeting:

- The SOP [Ed. (GJS): Undefined term] containing the access unit that contains the SEI message

- SOP SEI semantics: [Ed.(GJS): Define SOP – what is it?]

- SOP SEI semantics: [Ed. TK. This was changed since nal_ref_flag has been removed. Perhaps sop_desc_nal_ref_flag[ i ] can be removed if no longer useful.]

- SOP SEI semantics: [Ed. (GJS): This paragraph seems normative. Improve phrasing. Does it contradict other statements within the annex?]

## Signalling of frame/field information

The following issues on signalling of frame/field information, currently in the picture timing SEI message, should be discussed and addressed during this meeting:

- It might be better to put the pic_struct, progressive_source_idc, and duplicate_flag into a separate SEI message (e.g. named frame field information SEI message), because the picture timing info applies to operation points or bitstream subsets while the frame/field info applies to individual pictures, and these are not really picture timing info but frame/field info.

- Currently the CPB delay parameters are present even if CpbDpbDelaysPresentFlag is equal to 0.

- Some restrictions on the value of progressive_source_idc for different pic_struct values are missing.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).