| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0046v1 |
| --- | --- |

| Title: | AHG9: Indication of interlaced and frame-packed video in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution re-proposes the proposal in JCTVC-K0119 with some updates addressing comments and suggestions received from the review of JCTVC-K0119 in Shanghai and the latest related changes to the draft HEVC specification.

The proposal is as follows.

- To change general_reserved_zero_16bits and sub_layer_reserved_zero_16bits[ i ], coded as u(16), to general_reserved_zero_48bits and sub_layer_reserved_zero_48bits[ i ], respectively, coded as u(48).

- To signal the indication of whether a coded video sequence contains non-frame-packed pictures only using one bit in general_reserved_zero_48bits and sub_layer_reserved_zero_48bits[ i ].

- To signal the indication of whether a coded video sequence contains progressive frame pictures only using another bit in general_reserved_zero_48bits and sub_layer_reserved_zero_48bits[ i ].

With this change, it is asserted that it would be convenient for systems specifications to directly use the 12-byte profile/tier/level information as defined in the HEVC coding specification without change as one of the most important parameters for session negotiation or content selection, without the various interoperability problems that have been experienced for the frame-packed 3D stereoscopic video support in AVC, in the contexts of video decoders as well as the developments of systems and applications specifications.

# Introduction

## The interoperability problem associated with the frame-packed 3D stereoscopic video support in AVC

The support of frame-packed stereoscopic 3D video, through the frame packing arragnement (FPA) SEI message, was introduced into AVC after the first version of AVC was finalized. The following interoperablity problems have been identified:

- The backward compatibility problem: Decoders implemented according to earlier versions of AVC do not recognize FPA SEI messages and thus would ignore the FPA SEI messages in a bitstream and decode and output the decoded pictures as if the video was not a frame-packed stereoscopic 3D video. Consequently, the resulting video quality can be seriously distorted, generating lousy user experience.

- The don't-care-SEI problem: Even for decoders implemented according to a version of AVC that has the support of frame-packed stereoscopic 3D video, some conforming decoders may be implemented in way to ignore all SEI messages or only to handle a subset of them, e.g. buffering period SEI messages and picture timing SEI messages. Such decoders would also ignore the FPA SEI messages in a bitstream, and the same seriously distorted video quality can happen.

The problem also manifested itself in video application systems standardization. For example, in the recently (August 2012) specified support of frame-packed stereoscopic 3D video based on AVC in 3GPP dynamic adaptive streaming over HTTP (3GP-DASH, 3GPP TS 26.247), packet-switched streaming (PSS, 3GPP TS 26.234), and multimedia broadcast and multicast service (MBMS, 3GPP TS 26.346), totally different mechanims had to be specified to avoid the above problems. Specifically, to solve such problems, a new profile was be defined in 3GP-DASH, a new capability exchange attribute for the streaming component was defined in PSS, and a new feature tag was defined in MBMS.

Although a solution for each of the above three service specifciations was found, it is stil unclear, at least to the authors, how to solve the problem for 3GPP multimedia telephone service over IMS (MTSI, 3GPP TS 26.114). It is possible to define a new SDP attribute to make it work for new devices, but that cannot solve the backwards compatibility problem therein, as a legacy device would simply not be able to recognize the new SDP attribute and have to ignore it.

## The interoperability problem associated with of frame-packed 3D stereoscopic video support in HEVC

The situation is much better for HEVC, as the FPA SEI message is being included into the first version of HEVC. However, the don't-care-SEI problem still exists.

## The interoperability problem associated with of interlaced video support in HEVC

An HEVC bitstream may contain the following types of coded pictures:

- Progressive frames coded in frame pictures

- Interlaced fields coded in frame pictures

- Interlaced fields coded in field pictures

- Fields extracted from progressive frames coded in field pictures

These picture types are indicated through the field_seq_flag in VUI and the progressive_source_idc in the picture timing SEI messages. A problem similar to the frame-packed stereoscopic 3D video problem, but probably to a lesser extent, exists herein.

Lots of video clients or players do not implement de-interlacing or other signal processing capabilities to appropriately handle picture types other than pictures that are progressive frames coded in frame pictures. Due to that SEI messages are not required to be recognized or processed by conforming decoders, a client or player with a conforming HEVC decoder that does not recognize picture timing SEI messages would ignore the picture timing SEI messages in such a bitstream and decode and output the decoded pictures as if the bitstreams only contained pictures that are progressive frames coded in frame pictures. Consequently, the resulting video quality can be sub-optimal. Furthermore, even for a client or a player with conforming HEVC decoder that does recognize and is able to process picture timing SEI messages, all access units must be inspected to check the absence of picture timing SEI messages and all the present picture timing SEI messages have to be parsed and interpreted before a conclusion can be drawn that all pictures are progressive frames coded in frame pictures.

# Proposal

The syntax and semantics of the profile, tier and level syntax and semantics are proposed to be changed as follows (with the changed parts highlighted in yellow).

| profile_tier_level( profilePresentFlag, maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| if( profilePresentFlag ) { |  |
| general_profile_space | u(2) |
| general_tier_flag | u(1) |
| general_profile_idc | u(5) |
| for( i = 0; i < 32; i++ ) |  |
| general_profile_compatibility_flag[ i ] | u(1) |
| general_progressive_frames_only_flag | u(1) |
| general_non_packed_only_flag | u(1) |
| general_reserved_zero_46bits | u(46) |
| } |  |
| general_level_idc | u(8) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) { |  |
| if( profilePresentFlag ) |  |
| sub_layer_profile_present_flag[ i ] | u(1) |
| sub_layer_level_present_flag[ i ] | u(1) |
| if( profilePresentFlag && sub_layer_profile_present_flag[ i ] ) { |  |
| sub_layer_profile_space[ i ] | u(2) |
| sub_layer_tier_flag[ i ] | u(1) |
| sub_layer_profile_idc[ i ] | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| sub_layer_profile_compatibility_flag[ i ][ j ] | u(1) |
| sub_layer_progressive_frames_only_flag[ i ] | u(1) |
| sub_layer_non_packed_only_flag[ i ] | u(1) |
| sub_layer_reserved_zero_46bits[ i ] | u(46) |
| } |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |
| } |  |

...

general_progressive_frames_only_flag equal to 1 indicates that in the coded video sequence all the pictures are progressive frames coded in frame pictures. general_progressive_frames_only_flag equal to 0 indicates that in the coded video sequence there may be frame pictures containing interlaced fields, field pictures containing interlaced fields, or field pictures containing fields extracted from progressive frames.

general_non_packed_only_flag equal to 1 indicates that there is no frame packing arrangement SEI message in the coded video sequence. general_non_packed_only_flag equal to 0 indicates that there may be frame packing arrangement SEI messages in the coded video sequence.

general_reserved_zero_46bits shall be equal to 0 in bitstreams conforming to this Specification. Other values for general_reserved_zero_46bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of general_reserved_zero_46bits.

...

sub_layer_profile_space[ i ], sub_layer_tier_flag[ i ], sub_layer_profile_idc[ i ], sub_layer_profile_compatibility_flag[ i ][ j ], sub_layer_progressive_frames_only_flag[ i ], sub_layer_non_packed_only_flag[ i ], sub_layer_reserved_zero_46bits[ i ], and sub_layer_level_idc[ i ] have the same semantics as general_profile_space, general_tier_flag, general_profile_idc, general_profile_compatibility_flag[ j ], general_progressive_frames_only_flag, general_non_packed_only_flag, general_reserved_zero_46bits, and general_level_idc, respectively, but apply to the representation of the sub-layer with TemporalId equal to i.

When not present, the value of sub_layer_tier_flag[ i ] is inferred to be equal to 0.

NOTE 1 – It is possible that sub_layer_tier_flag[ i ] is not present and sub_layer_level_idc[ i ] is present. In this case, a default value of sub_layer_tier_flag[ i ] is needed for interpretation of sub_layer_level_idc[ i ].

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).