| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0047v2 |
| --- | --- |

| Title: | AHG9: Indication of parameter sets properties in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Adarsh K. Ramasubramonian 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-658-5804 aramasub@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

Version 1 of this contribution proposes to signal, in the VPS, whether full random accessibility is supported for a coded video sequence, and whether parameter set update exists in a coded video sequence.

In both the draft HEVC file format (becoming a part of ISO/IEC 14496-15, which currently includes the file format specifications for AVC and its SVC and MVC extensions) and MPEG-2 systems extension for HEVC carriage, full random accessibility (i.e. all required parameter sets are available when random accessing from a RAP picture by completely discarding earlier access units in decoding order) is required. Thus, if it is known that full random accessibility is supported, the composer for HEVC files or MPEG-2 TS content does not have to thoroughly inspect the bitstream (including parsing all slice headers at least to find out the PPS IDs) to find it out.

Knowing that parameter set update is not present, senders and receivers would know that they need not take additional measures to avoid hard-to-detect loss of updated parameter sets, and decoders know that a parameter set with an existing ID is simply a repetition by bit-by-bit comparison.

Version 2 of this contribution proposes to signal, in the active parameter sets SEI message, whether full random accessibility is supported the coded video sequence, and whether parameter set update exists in the coded video sequence, and the name of the SEI message is changed to parameter sets information SEI message.

- Proposed syntax and semantics changes in version 1

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_reserved_three_2bits | u(2) |
| vps_reserved_zero_6bits | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_full_random_access_flag | u(1) |
| vps_no_param_set_update_flag | u(1) |
| vps_reserved_0x3fff_14bits | u(14) |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| bit_rate_pic_rate_info( 0, vps_max_sub_layers_minus1 ) |  |
| ... |  |
| } |  |

...

vps_full_random_access_flag equal to 1 specifies that full random accessibility is enabled in the coded video sequence, i.e. it is possible to perform random access at the position of each RAP access unit in the coded video sequence by discarding all access units preceding the RAP access unit in decoding order and to correctly decode the RAP picture and the following pictures in decoding order other than the RASL pictures associated with the RAP picture. In this case, for each RAP access unit in the coded video sequence, when all access units preceding the RAP access unit in decoding order are discarded, each video parameter set, sequence parameter set or picture parameter set is available when it needs to be activated. vps_full_random_access_flag equal to 0 specified that full random accessibility may or may not be enabled in the coded video sequence.

vps_no_param_set_update_flag equal to 1 specifies that there is no parameter set update in the coded video sequence, i.e. each video parameter set in the coded video sequence that has the same value of vps_video_parameter_set_id as the previous video parameter set in decoding order is an exact copy of the previous video parameter set in decoding order, each sequence parameter set in the coded video sequence that has the same value of sps_seq_parameter_set_id as the previous sequence parameter set in decoding order is an exact copy of the previous sequence parameter set in decoding order, and each picture parameter set in the coded video sequence that has the same value of pps_pic_parameter_set_id as the previous picture parameter set in decoding order is an exact copy of the previous picture parameter set in decoding order. vps_no_param_set_update_flag equal to 0 specifies that the there may or may not be parameter set update in the coded video sequence.

vps_reserved_0x3fff_14bits shall be equal to 0x3FFF in bitstreams conforming to this version of this Specification. Other values for vps_reserved_0x3fff_14bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of vps_reserved_0x3fff_14bits.

NOTE 1 – It is anticipated that in future scalable or 3D video coding extensions of this Specification, this field specifies the byte offset of the next set of fixed-length coded information in the video parameter set NAL unit, starting from the beginning of the NAL unit. Video parameter set information for non-base layer or view starts from a byte-aligned position of the video parameter set NAL unit, with fixed-length coded information that is essential for session negotiation and/or capability exchange. The byte offset would then help to locate and access those essential information in the video parameter set NAL unit without the need of entropy decoding, which may not be possible for some network entities that may desire to access only the information in the video parameter set that is essential for session negotiation and/or capability exchange.

...

# Proposed syntax and semantics changes in version 2

Parameter sets information SEI message syntax

| parameter_sets_info( payloadSize ) { | Descriptor |
| --- | --- |
| active_vps_id | u(4) |
| full_random_access_flag | u(1) |
| no_param_set_update_flag | u(1) |
| num_sps_ids_minus1 | ue(v) |
| for( i = 0; i <= num_sps_ids_minus1; i++ ) |  |
| active_seq_parameter_set_id[ i ] | ue(v) |
| } |  |

Parameter sets information SEI message semantics

The parameter sets information SEI message indicates which video parameter set is active for the VCL NAL units of the access unit associated with the SEI message. The SEI message may also provide information on which sequence parameter set is active for the VCL NAL units of the access unit associated with the SEI message, and other information related to parameter sets.

When an SEI NAL unit that contains an active parameter sets SEI message and has nuh_reserved_zero_6bits equal to 0 is present, the SEI NAL unit shall precede, in decoding order, the first VCL NAL unit in the access unit.

active_vps_id identifies [Ed. (GJS): How? Clarify what syntax element this is referring to.] the video parameter set that is active for the VCL NAL units of the access unit associated with the SEI message. The value of active_vps_id shall be in the range of 0 to 15, inclusive.

full_random_access_flag equal to 1 indicates that full random accessibility is enabled in the coded video sequence, i.e. it is possible to perform random access at the position of each RAP access unit in the coded video sequence by discarding all access units preceding the RAP access unit in decoding order and to correctly decode the RAP picture and the following pictures in decoding order other than the RASL pictures associated with the RAP picture. In this case, for each RAP access unit in the coded video sequence, when all access units preceding the RAP access unit in decoding order are discarded, each video parameter set, sequence parameter set or picture parameter set is available when it needs to be activated. full_random_access_flag equal to 0 indicates that full random accessibility may or may not be enabled in the coded video sequence.

no_param_set_update_flag equal to 1 indicates that there is no parameter set update in the coded video sequence, i.e. each video parameter set in the coded video sequence that has the same value of vps_video_parameter_set_id as the previous video parameter set in decoding order is an exact copy of the previous video parameter set in decoding order, each sequence parameter set in the coded video sequence that has the same value of sps_seq_parameter_set_id as the previous sequence parameter set in decoding order is an exact copy of the previous sequence parameter set in decoding order, and each picture parameter set in the coded video sequence that has the same value of pps_pic_parameter_set_id as the previous picture parameter set in decoding order is an exact copy of the previous picture parameter set in decoding order. no_param_set_update_flag equal to 0 indicates that the there may or may not be parameter set update in the coded video sequence.

num_sps_ids_minus1 plus 1 specifies the number of sequence parameter sets that are active for the VCL NAL units of the access unit associated with the active parameter sets SEI message. The value of num_sps_ids_minus1 shall be in the range of 0 to 15, inclusive. In bitstreams conforming to this version of this Specification, num_sps_ids_minus1 shall be equal to 0. However, decoders shall allow other values to appear in the num_sps_ids_minus1 syntax.

active_seq_parameter_set_id[ i ] specifies the sps_seq_parameter_set_id of the i-th sequence parameter set that is active for the VCL NAL units of the access unit associated with the SEI message. [Ed. (GJS): Vague. What does it mean for i equal to 0? What does it mean for i not equal to 0? What does it mean at all?] The value of active_seq_parameter_set_id[ i ] shall be in the range of 0 to 15, inclusive. [Ed. (GJS): Does that constraint apply to the values for i > 0?]

Changes in other places

Changes all instances of "active parameter sets SEI message" in other places to "parameter sets information SEI message".

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).