| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0048v1 |
| --- | --- |

| Title: | AHG9: A minor NAL unit header change in HEVC for multi-standard extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution is a follow-up proposal of JCTVC-K0121. It is proposed to switch the positions of the two syntax elements in the NAL unit header in the HEVC specification: nuh_reserved_zero_6bits and nal_unit_type. It is asserted that the change enables future multi-standard HEVC extensions for which bitstreams containing multiple layers with AVC base layer can be directly fed to and handled by legacy AVC decoders.

# Introduction

At the previous meeting, two approaches were proposed for carrying NAL units of non-HEVC base layer of multi-standard bitstreams, in JCTVC-K0121 and JCTVC-K0206, respectively.

The approach in JCTVC-K0121 uses a bit in the NAL unit header to distinguish between AVC and HEVC NAL units, and requires switching the positions of the two syntax elements nuh_reserved_zero_6bits and nal_unit_type in the NAL unit header.

The approach in JCTVC-K0206 uses HEVC NAL units with one particular NAL unit type for encapsulation of non-HEVC base layer NAL units, and does not affect the version 1 HEVC specification as long as there is reserved NAL unit type that can be used in the future.

It was agreed that the JCTVC-K0206 approach seems better due to that it also support base layer per other standards such as MPEG-2 Video.

It is desirable if a multi-standard bitstream containing multiple layers with AVC base layer can be directly fed to and successfully decoded by an AVC decoder, through identifying and ignoring HEVC-based enhancement layer NAL units as reserved NAL unit types.

However, this is not possible with the current HEVC NAL unit header design, due to the different syntax elements, different lengths and different syntax element layout between the two NAL unit header designs, thus an AVC decoder may understand HEVC NAL units as AVC NAL units. This leads to unexpected parsing and decoding behavior, such as crash. Consequently legacy AVC decoders cannot be directly used in broadcast or multicast application scenarios wherein the base layer and at least one enhancement layer are transmitted in one transmission session (e.g. RTP session). Rather, the receiving system has to be equipped with a bitstream extractor to extract a clean AVC bitstream to feed to the decoder. This also applies to the approach in JCTVC-K0206.

Note also that

- Switching of the positions of the two syntax elements nuh_reserved_zero_6bits and nal_unit_type in the NAL unit header does not affect the applicability of the approach in JCTVC-K0206.

- It is possible to use the approach in JCTVC-K0121 for the cases with AVC base layer, and to use the approach in JCTVC-K0206 for the cases with base layer of other codecs than AVC and HEVC.

Therefore, we propose the following:

- Switch the positions the two syntax elements nuh_reserved_zero_6bits and nal_unit_type in the NAL unit header in HEVC version 1 specification, just to enable the possibility of support the approach in JCTVC-K0121 in the future.

- Let the exact approach for carrying non-HEVC base layer NAL units be decided during the development of multi-standard extensions, if any.

# Proposed change

## Syntax

Texts not highlighted are the same as in the latest HEVC draft, and those highlighted in yellow are additions, and those strikethroughs in red fonts are removals, compared to the latest HEVC draft.

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nuh_reserved_zero_6bits | u(6) |
| nal_unit_type | u(6) |
| nuh_reserved_zero_6bits | u(6) |
| nuh_temporal_id_plus1 | u(3) |
| } |  |

## Semantics

No change, except for changing of the order of the semantics.

The third bit of the nuh_reserved_zero_6bits, which is the fourth bit of the NAL unit header, can be required to be equal to 1 in multi-standard bitstream and thus can be used to distinguish between NAL units of HEVC and NAL units of AVC.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).