| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0052 |
| --- | --- |

| Title: | TE C5: Motion field mapping |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yong He, Yan Ye 9710 Scranton R-D, #250 San Diego, CA 92121 USA | Tel: Email: | 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com Yong.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

This document presents the experimental results of the motion field mapping tool included in Tool Experiment C5, which aims to improve the performance of temporal motion vector prediction (TMVP) for the enhancement-layer coding of scalable video coding extension of HEVC. The proposed tool is carried out at picture level, which maps base-layer motion information to form the motion field of inter-layer reference (ILR) picture. This allows the ILR picture to be used as the co-located picture for the TMVP of the enhancement-layer picture to be coded. Experimental results show that when using the un-compressed motion field of base-layer, the proposed motion field mapping tool can reportedly achieve  1.6%, 1.9%, 2%, 0.6%, 0.6% and 0.8% Y BD-rate savings for RA-2x, RA-1.5x, RA-SNR, LD-P-2x, LD-P-1.5x and LD-P-SNR, respectively, compared to the “EL+BL” anchors of the “ref_idx” framework in the reference software; when using compressed base-layer motion field, the corresponding Y BD-rate savings are 1.1%, 1.7%, 2%, 0.2%, 0.5% and 0.8%, respectively.

# Algorithm description

The proposed motion field mapping tool is based on the “ref_idx” framework of SMuC 0.1.1 and carried out at the picture level. More specifically, the proposed tool maps the base-layer motion information to form motion field for the inter-layer reference (ILR) picture which is then inserted as one additional entry to the reference picture lists of the enhancement-layer picture. This allows the ILR picture to be used as a collocated picture for enhancement-layer TMVP.

In our current implementation, motion field mapping is performed based on units of smallest Pus (SPUs). Either the un-compressed motion field or compressed motion field from the base-layer could be used. Taking spatial scalability with 1.5x up-sampling ratio as an example, Figure1 shows the relationship between the SPUs of the ILR picture (blocks depicted with red solid lines) and the SPUs of the reconstructed base-layer picture (blocks depicted with blue dashed lines). As shown in Figure1, depending on the up-sampling ratio and the SPU’s coordinates in the picture, each SPU in the ILR picture could correspond to various numbers and proportions of the base-layer SPUs. The proposed motion field mapping method estimates the motion field for each of the SPUs in the ILR picture, including its reference picture index and its motion vector (MV), using motion information from their corresponding base-layer SPUs.

Figure 1. Example relationship between the SPUs of inter-layer reference picture and the reconstructed base-layer picture

## Reference picture index mapping

To create its motion field, it is assumed that the ILR picture has the same temporal prediction structure as its corresponding base-layer picture. As in Error: Reference source not found, denoting the base-layer picture and enhancement-layer picture as BL and EL, the ILR’s virtual reference picture lists are generated by copying the same entries from BL’s corresponding reference picture list entries.

Figure 2. Example of virtual reference picture lists of inter-layer reference picture

When an inter-layer SPU is covered by just one base-layer SPU, for example, block “u0” in Figure1, its reference index is copied from the base-layer SPU. When an inter-layer SPU is covered by more than one base-layer SPUs, for example, block “u4” in Figure, the majority rule is applied to assign reference picture index to the inter-layer SPU; specifically, the inter-layer SPU will inherit the reference picture index used most frequently by its corresponding base-layer SPUs. To break possible ties, if multiple reference picture indices have the same usage, the reference index representing a reference picture with the shortest POC distance to the current picture will be selected.

## Motion vector mapping

To estimate MVs of one inter-layer SPU, medium filter is applied on the corresponding SPUs of the base-layer. The reference picture index of the inter-layer SPU, denoted as refILR, is decided according to Sec. 1.1. The motion vector  for the inter-layer SPU, denoted as MVILR, is determined by applying median filter to the corresponding base-layer SPUs that have the same reference index as refILR, using (1),

MVILR=N⋅mediumi∈BMViBL, 	B=j:refjBL=refILR		(1)

where MViBL is the MV of the corresponding base layer SPUs, N is the up-sampling ratio (for example, N = 1.5 or 2), and B is the subset of base layer SPUs with reference index equal to refILR. The x and y component of the motion vector are calculated separately using (1).

## Base-layer motion field

It should be mentioned that the motion field of inter-layer reference pictures can be mapped either from the original (un-compressed) motion field or from compressed motion field of base-layer, by the proposed motion field mapping algorithm. The corresponding experimental results using un-compressed or compressed base-layer motion field will be presented in this document.

## Syntax and semantics

To indicate if the proposed motion field mapping algorithm is applied to inter-layer reference pictures, a one-bit flag is added into sequence parameter set (SPS) of enhancement-layer [2].

# Experiments

## Experimental conditions

The proposed algorithm is implemented based on SMuC0.1.1 “ref_idx” framework software released by AhG on November 2, 2012, with common test conditions for SHVC Tool Experiments.

To evaluate the proposed motion field mapping method, the performance of the “ref_idx” framework in the reference software is used as the anchor, where the inter-layer reference picture is placed at the end of the reference picture lists. This means that in the reference software, the enhancement-layer TMVP will always use a temporal enhancement-layer reference picture as the collocated picture. Therefore, non-normative change is made to slice header signaling in reference software to always use the inter-layer reference picture for enhancement-layer TMVP. In this way, the performance with and without the proposed motion field mapping method can be fairly compared.

The decoding timing results are measured from the same PC, while the encoding timing results are measured from a heterogeneous cluster system.

## Experimental results

### Motion field mapping using un-compressed base-layer motion field

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.5% | -3.7% | -4.0% |  |  |  | -1.6% | -4.1% | -4.6% |
| Class B | -1.6% | -3.2% | -3.6% | -1.9% | -3.3% | -3.8% | -2.2% | -4.1% | -4.6% |
| Overall (EL+BL) | -1.6% | -3.4% | -3.7% | -1.9% | -3.3% | -3.8% | -2.0% | -4.1% | -4.6% |
| Overall (EL) | -2.9% | -6.0% | -6.7% | -4.8% | -7.7% | -8.9% | -3.9% | -7.3% | -8.3% |
| Enc Time[%] | 79.5% |  |  | 75.0% |  |  | 78.1% |  |  |
| Dec Time[%] | 105.9% |  |  | 124.0% |  |  | 110.7% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.8% | -1.6% | -1.7% |  |  |  | -1.1% | -1.7% | -1.9% |
| Class B | -0.5% | -0.9% | -0.9% | -0.6% | -0.7% | -0.9% | -0.7% | -1.1% | -1.3% |
| Overall (EL+BL) | -0.6% | -1.1% | -1.1% | -0.6% | -0.7% | -0.9% | -0.8% | -1.3% | -1.5% |
| Overall (EL) | -1.0% | -1.9% | -2.0% | -1.2% | -1.3% | -1.9% | -1.4% | -2.1% | -2.4% |
| Enc Time[%] | 80.3% |  |  | 80.3% |  |  | 82.4% |  |  |
| Dec Time[%] | 113.0% |  |  | 120.0% |  |  | 109.5% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

### Motion field mapping using compressed base-layer motion field

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.7% | -2.4% | -2.6% |  |  |  | -1.6% | -3.9% | -4.5% |
| Class B | -1.2% | -2.7% | -3.1% | -1.7% | -3.0% | -3.5% | -2.1% | -4.0% | -4.5% |
| Overall (EL+BL) | -1.1% | -2.6% | -3.0% | -1.7% | -3.0% | -3.5% | -2.0% | -4.0% | -4.5% |
| Overall (EL) | -2.1% | -4.7% | -5.3% | -4.5% | -7.2% | -8.4% | -3.8% | -7.2% | -8.1% |
| Enc Time[%] | 83.7% |  |  | 80.5% |  |  | 80.6% |  |  |
| Dec Time[%] | 106.0% |  |  | 124.0% |  |  | 110.3% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.6% | -0.6% |  |  |  | -1.0% | -1.6% | -1.7% |
| Class B | -0.2% | -0.5% | -0.5% | -0.5% | -0.7% | -0.8% | -0.7% | -1.1% | -1.2% |
| Overall (EL+BL) | -0.2% | -0.6% | -0.5% | -0.5% | -0.7% | -0.8% | -0.8% | -1.2% | -1.4% |
| Overall (EL) | -0.4% | -1.0% | -1.0% | -1.0% | -1.4% | -1.6% | -1.3% | -2.0% | -2.2% |
| Enc Time[%] | 84.4% |  |  | 79.9% |  |  | 84.4% |  |  |
| Dec Time[%] | 113.0% |  |  | 120.1% |  |  | 109.5% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

### Observation

First of all, it can be observed that with the proposed motion field mapping method, it is more beneficial to use the ILR picture as the co-located picture for enhancement-layer TMVP, compared to using a temporal reference picture as the co-located picture. More specially, the proposed motion field mapping method using un-compressed base-layer motion field can achieve 1.6%, 1.9%, 2%, 0.6%, 0.6% and 0.8% BD-rate savings for RA-2x, RA-1.5x, RA-SNR, LD-P-2x, LD-P-1.5x and LD-P-SNR, respectively, compared to the “EL+BL” anchors of the “ref_idx” framework.

Second, un-compressed base-layer motion field is more beneficial than compressed base-layer motion field to the performance of the motion field mapping method, which can provide additional 0.5%, 0.2%, 0%, 0.4%, 0.1% and 0% BD-rate savings for RA-2x, RA-1.5x, RA-SNR, LD-P-2x, LD-P-1.5x and LD-P-SNR, respectively.

# Conclusion

In this contribution, the performance of the motion field mapping tool is investigate for Tool Experiment C5. Simulation shows that the proposed motion field mapping tool can improve the TMVP performance of the “ref_idx” framework of SMuC0.1.1. More specifically, when using the un-compressed motion field of base-layer, the proposed motion field mapping tool can reportedly achieve 1.6%, 1.9%, 2%, 0.6%, 0.6% and 0.8% Y BD-rate savings for RA-2x, RA-1.5x, RA-SNR, LD-P-2x, LD-P-1.5x and LD-P-SNR, respectively, compared to the “EL+BL” anchors of the “ref_idx” framework of SMuC0.1.1; when using compressed base-layer motion field, the corresponding Y BD-rate savings are 1.1%, 1.7%, 2%, 0.2%, 0.5% and 0.8%, respectively.

# Patent rights declaration(s)

InterDigital Communications, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer. Document no JCTVC-K1105. October 2012.

- B. Bross, W-J. Han, J-R. Ohm, G. J. Sullivan, High Efficiency Video Coding (HEVC) Text Specification Draft 8. Document no JCTVC-J1003. July 2012.