| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0053 |
| --- | --- |

| Title: | Modified motion vector signaling for ref_idx framework |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yan Ye, Yuwen He, Yong He 9710 Scranton R-D, #250 San Diego, CA 92121 USA | Tel: Email: | 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yan.Ye@InterDigital.com Yuwen.He@InterDigital.com Yong.He@InterDigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

This contribution is related with TE A2 3.2.1: inter-layer reference picture placement [1]. The goal is to further simplify the motion vector signaling of the case in TE A2 3.2.1, ref_idx setting 2 + zeroMV, by skipping the unnecessary overhead of signaling motion vectors (MVs) when inter-layer reference (ILR) picture is used for prediction of the enhancement-layer picture. Simulation shows when skipping MV signaling at both rate-distortion (R-D) decision process and entropy coding process, 0.2%, 0.2%, 0.2%, 0.3%, 0.3% and 0.3% Y BD-rate savings are achieved for RA-2x, RA-1.5x, RA-SNR, LD-P-2x, LD-P-1.5x and LD-P-SNR respectively. When skipping MV signaling only at entropy coding process, the corresponding Y BD-rate savings are 0.2%, 0.2%, 0.2%, 0.2%, 0.2% and 0.3% respectively.

# Algorithm description

The current SHVC software SMuC0.1.1 supports ref_idx based signaling of inter-layer prediction, where ILR picture is treated as an additional entry of the reference picture lists of the enhancement-layer picture. In additional, it is observed in [1] that forcing the enhancement-layer picture to use zero MV when making reference to the ILR picture can further improve the performance of the ref_idx signaling. Given fixed zero MVs for the ILR picture, it is not necessary to send extra overhead to signal the corresponding MVs when the ILR picture is selected as reference. Correspondingly, in this contribution, all MV related signaling, i.e., motion vector predictor (MVP) and motion vector difference (MVD), is skipped when the current enhancement-layer PU refers to the ILR picture for prediction.

Two set of tests have been configured. In Test Setting 1, the MV signaling related with ILR picture is skipped at both the R-D decision process and entropy encoding process at the encoder. In Test Setting 2, the MV signaling related with ILR picture is only skipped at the entropy encoding process, but maintained at the R-D decision process at the encoder.

# Syntax and semantics

| prediction_unit( x0, y0, nPbW, nPbH ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| if( layer_id && IsILRPic( L0, ref_idx_l0[ x0 ] [y0 ] ){ |  |
| MvdL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL0[ x0 ][ y0 ][ 1 ] = 0 |  |
| }else |  |
| mvd_coding( x0, y0, 0 ) |  |
| if( layer_id && IsILRPic( L0, ref_idx_l0[ x0 ] [y0 ] ){ |  |
| MvpL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvpL0[ x0 ][ y0 ][ 1 ] = 0 |  |
| }else |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = Pred_BI) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else{ |  |
| if( layer_id && IsILRPic( L1, ref_idx_l1[ x0 ] [y0 ] ){ |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| }else |  |
| mvd_coding( x0, y0, 1 ) |  |
| } |  |
| if( layer_id && IsILRPic( L1, ref_idx_l1[ x0 ] [y0 ] ){ |  |
| MvpL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvpL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| }else |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

layer_id specifies the current layer index. The function IsILRPic(LX, refIdx) specifies if the reference picture with reference picture index refIdx from reference picture list LX is the ILR picture (TRUE) or not (FALSE).

# Experiments

The proposed algorithm is implemented based on the software TE A2 3.2.1 [1]. The best case ref_idx setting 2 + zero MV is used as the anchor.

All the pictures of the Class-A and Class-B that were considered in the SHVC Call for Proposals, with all the AI, RA and LD-P configurations for both 2x, 1.5x and SNR scalability scenarios, are tested.

The QP combinations that base layer QPs are 26 and 30, and the difference between the base layer QP and enhancement QP are -2, 0, 2 and 4, are tested for the proposed algorithm.

## Test Setting 1

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall (EL+BL) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall (EL) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 99.1% |  |  | 95.4% |  |  |  |  |  |
| Dec Time[%] | 100.9% |  |  | 96.3% |  |  |  |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.3% | -0.3% |  |  |  | -0.3% | -0.3% | -0.2% |
| Class B | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% |
| Overall (EL+BL) | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | 0.0% |
| Overall (EL) | -0.4% | -0.3% | -0.3% | -0.5% | -0.2% | -0.2% | -0.4% | -0.2% | -0.1% |
| Enc Time[%] | 89.5% |  |  | 96.4% |  |  | 87.2% |  |  |
| Dec Time[%] | 93.0% |  |  | 96.4% |  |  | 88.0% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.2% | -0.2% |  |  |  | -0.4% | -0.2% | 0.0% |
| Class B | -0.3% | 0.1% | -0.1% | -0.3% | 0.0% | 0.0% | -0.3% | 0.0% | 0.3% |
| Overall (EL+BL) | -0.3% | 0.0% | -0.1% | -0.3% | 0.0% | 0.0% | -0.3% | 0.0% | 0.2% |
| Overall (EL) | -0.6% | -0.1% | -0.2% | -0.6% | -0.1% | -0.1% | -0.5% | -0.1% | 0.2% |
| Enc Time[%] | 103.3% |  |  | 103.4% |  |  | 108.6% |  |  |
| Dec Time[%] | 100.4% |  |  | 101.3% |  |  | 108.7% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

## Test Setting 2

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall (EL+BL) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall (EL) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 101.1% |  |  | 93.0% |  |  |  |  |  |
| Dec Time[%] | 101.2% |  |  | 93.2% |  |  |  |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.3% | -0.3% |  |  |  | -0.2% | -0.3% | -0.3% |
| Class B | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.3% | -0.2% |
| Overall (EL+BL) | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.3% | -0.3% |
| Overall (EL) | -0.3% | -0.4% | -0.4% | -0.4% | -0.4% | -0.4% | -0.4% | -0.5% | -0.5% |
| Enc Time[%] | 91.4% |  |  | 92.1% |  |  | 89.3% |  |  |
| Dec Time[%] | 94.3% |  |  | 93.8% |  |  | 90.4% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.4% | -0.4% |  |  |  | -0.3% | -0.4% | -0.3% |
| Class B | -0.2% | -0.2% | -0.3% | -0.2% | -0.3% | -0.3% | -0.2% | -0.4% | -0.3% |
| Overall (EL+BL) | -0.2% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% | -0.3% | -0.4% | -0.3% |
| Overall (EL) | -0.4% | -0.5% | -0.5% | -0.4% | -0.5% | -0.6% | -0.4% | -0.7% | -0.5% |
| Enc Time[%] | 107.7% |  |  | 99.2% |  |  | 109.6% |  |  |
| Dec Time[%] | 104.8% |  |  | 98.2% |  |  | 109.2% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

It can been seen that Test Setting 1 shows 0.2%, 0.2%, 0.2%, 0.3%, 0.3% and 0.3% BD-rate savings in Y for RA-2x, RA-1.5x, RA-SNR, LD-P-2x, LD-P-1.5x and LD-P-SNR respectively, while the corresponding BD-rate savings are 0.2%, 0.2%, 0.2%, 0.2%, 0.2% and 0.3% respectively. In addition, for all the cases, there is always chroma gain except for LD-P SNR in Test Setting 1 where 0.2% loss is observed for V.

# Conclusion

In this contribution, the motion vector signaling of TE A2 3.2.1 is simplified when the ILR picture is used as reference for the enhancement layer picture and the MVs are fixed to be zero. Simulation results show that the proposed method can further improve the performance for ref_idx framework of SMuC0.1.1, both for luma and chroma components.

# Patent rights declaration(s)

InterDigital Communications, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- X. Xiu, Y. He, Y. He and Y. Ye, “TE A2: Inter-layer reference picture placement”, JCTVC document JCTVC-L0051, Geneva, Switzerland, January, 2013

- Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC. Document no JCTVC-K1102. October 2012.