| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0054 |
| --- | --- |

| Title: | TE1 4.2 Fixed Upsampling Filters for SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Dong and Yan Ye 9710 Scranton Rd, Ste 250, San Diego, CA 92121, USA | Tel: Email: | +1 858 210 4809 jie.dong@interdigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

This contribution proposes the fixed upsampling filters for 1.5X and 2X spatial scalability of SHVC. The filter coefficients are first derived by the Gaussian windowed Sinc function, and approximated to integers with certain precisions. Here, two precisions, i.e., 7-bit and 8-bit integer filter coefficients, are tested and reported.

# Introduction

During the 11th JCT-VC meeting in Shanghai, multiple proposals for upsampling filters for the SHVC design were proposed.  It was then decided to coordinate for further study and testing of these filters in TE1. The upsampling filters proposed in this contribution, originally proposed in InterDigital’s response to SHVC CfP [1], are included in TE1, and the description is in Section 4.2 of TE1 document [2].

According to the description in Section 4.2 of [2], the exact filters as proposed in [1] should be studied and tested, which are the 8-bit filters presented in this contribution. At the same time, the same filters with lower precision, referred to as 7-bit filters, are also requested to be studied and tested. Section 2 introduces the features of the upsampling filters, and provides the values of filter coefficients at 7-bit and 8-bit precisions. Section 3 shows the experimental results based on the test condition specified by TE1.

# Proposed Upsampling Filters

The proposed upsampling filters are 2-D separable, and the coefficients are derived from Gaussian windowed Sinc function. The values of filter coefficients are shown in Table 1 (8-bit) and Table 2 (7-bit). As can be seen, the magnitudes of coefficients in Table 2 are roughly the half of those in Table 1, which means the filters shown in Table 1 and Table 2 are the same except the precision.

Table 1 8-bit upsampling filter coefficients

| Luma | 2X |  | [-3 | 8 | -21 | 80 | 80 | -21 | 8 | -3]/128 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.5X |  | [-5 | 11 | -24 | 107 | 53 | -18 | 8 | -4]/128 |
|  |  |  | [-4 | 8 | -18 | 53 | 107 | -24 | 11 | -5]/128 |
| Chroma | 2X | Vertical | [-17 | 100 | 56 | -11]/128 |  |  |  |  |
|  |  |  | [-3 | 15 | 126 | -10]/128 |  |  |  |  |
|  |  | Horizontal | [-15 | 79 | 79 | -15]/128 |  |  |  |  |
|  | 1.5X | Vertical | [-20 | 122 | 38 | -12]/128 |  |  |  |  |
|  |  |  | [-20 | 71 | 101 | -24]/128 |  |  |  |  |
|  |  |  | [-4 | 12 | 128 | -8]/128 |  |  |  |  |
|  |  | Horizontal | [-23 | 113 | 54 | -16]/128 |  |  |  |  |
|  |  |  | [-16 | 54 | 113 | -23]/128 |  |  |  |  |

Table 2 7-bit upsampling filter coefficients

| Luma | 2X |  | [-1 | 4 | -11 | 40 | 40 | -11 | 4 | -1]/64 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.5X |  | [-2 | 5 | -12 | 54 | 26 | -9 | 4 | -2]/64 |
|  |  |  | [-2 | 4 | -9 | 26 | 54 | -12 | 5 | -2]/64 |
| Chroma | 2X | Vertical | [-8 | 50 | 28 | -6]/64 |  |  |  |  |
|  |  |  | [-2 | 8 | 63 | -5]/64 |  |  |  |  |
|  |  | Horizontal | [-8 | 40 | 40 | -8]/64 |  |  |  |  |
|  | 1.5X | Vertical | [-10 | 61 | 19 | -6]/64 |  |  |  |  |
|  |  |  | [-10 | 35 | 51 | -12]/64 |  |  |  |  |
|  |  |  | [-2 | 6 | 64 | -4]/64 |  |  |  |  |
|  |  | Horizontal | [-11 | 56 | 27 | -8]/64 |  |  |  |  |
|  |  |  | [-8 | 27 | 56 | -11]/64 |  |  |  |  |

# Experimental Results

The proposed upsampling filters are integrated into SMuC 0.1.1 and tested under the common test conditions specified by TE1. Compared with Simulcast, the performances of the spatial scalable coding with the proposed 8-bit and 7-bit upsampling filter are shown in Tables 3 and 4, respectively.

Table 3 Average BD-rate performance of 8-bit upsampling filters

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -26.5% | -26.6% | -27.4% |  |  |  |
| Class B | -20.9% | -20.8% | -20.6% | -32.5% | -32.6% | -32.6% |
| Overall (EL+BL) | -22.5% | -22.5% | -22.5% | -32.5% | -32.6% | -32.6% |
| Overall (EL) | -34.3% | -34.5% | -34.6% | -58.0% | -57.6% | -57.7% |
| Enc Time[%] | 101.5% |  |  | 98.9% |  |  |
| Dec Time[%] | 85.7% |  |  | 85.6% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -18.1% | -7.5% | -9.0% |  |  |  |
| Class B | -15.4% | -9.0% | -7.6% | -26.0% | -19.9% | -18.3% |
| Overall (EL+BL) | -16.1% | -8.6% | -8.0% | -26.0% | -19.9% | -18.3% |
| Overall (EL) | -24.9% | -13.2% | -12.6% | -47.0% | -37.2% | -34.5% |
| Enc Time[%] | 87.3% |  |  | 89.2% |  |  |
| Dec Time[%] | 131.1% |  |  | 136.6% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -13.2% | -1.5% | -2.6% |  |  |  |
| Class B | -11.6% | -6.5% | -4.8% | -21.6% | -15.5% | -13.8% |
| Overall (EL+BL) | -12.1% | -5.0% | -4.2% | -21.6% | -15.5% | -13.8% |
| Overall (EL) | -19.3% | -7.9% | -6.7% | -40.1% | -29.2% | -26.1% |
| Enc Time[%] | 90.6% |  |  | 96.5% |  |  |
| Dec Time[%] | 111.9% |  |  | 118.1% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 4 Average BD-rate performance of 7-bit upsampling filters

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -26.9% | -26.9% | -27.7% |  |  |  |
| Class B | -21.0% | -21.0% | -20.8% | -32.6% | -32.7% | -32.7% |
| Overall (EL+BL) | -22.7% | -22.7% | -22.8% | -32.6% | -32.7% | -32.7% |
| Overall (EL) | -34.7% | -34.8% | -35.0% | -58.2% | -57.8% | -57.9% |
| Enc Time[%] | 108.9% |  |  | 100.8% |  |  |
| Dec Time[%] | 91.1% |  |  | 87.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -18.4% | -7.6% | -9.0% |  |  |  |
| Class B | -15.5% | -9.0% | -7.7% | -26.1% | -20.0% | -18.4% |
| Overall (EL+BL) | -16.3% | -8.6% | -8.0% | -26.1% | -20.0% | -18.4% |
| Overall (EL) | -25.2% | -13.2% | -12.6% | -47.3% | -37.4% | -34.7% |
| Enc Time[%] | 89.3% |  |  | 91.2% |  |  |
| Dec Time[%] | 131.1% |  |  | 136.6% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -13.6% | -1.5% | -2.6% |  |  |  |
| Class B | -11.7% | -6.6% | -4.7% | -21.7% | -15.7% | -13.9% |
| Overall (EL+BL) | -12.2% | -5.1% | -4.1% | -21.7% | -15.7% | -13.9% |
| Overall (EL) | -19.5% | -8.0% | -6.6% | -40.3% | -29.6% | -26.4% |
| Enc Time[%] | 93.9% |  |  | 92.0% |  |  |
| Dec Time[%] | 115.1% |  |  | 113.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

In Addition to Tables 3 and 4, the proposed 7-bit luma and chroma upsampling filters are tested separately, and the results are shown in Tables 5 and 6, respectively.

Table 5 Average BD-rate performance of the proposed 7-bit luma filter and chroma filter in JCTVC-K0378

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -27.0% | -27.2% | -28.0% |  |  |  |
| Class B | -21.1% | -21.2% | -21.0% | -32.6% | -33.1% | -33.1% |
| Overall (EL+BL) | -22.8% | -22.9% | -23.0% | -32.6% | -33.1% | -33.1% |
| Overall (EL) | -34.8% | -35.2% | -35.3% | -58.2% | -58.5% | -58.6% |
| Enc Time[%] | 102.4% |  |  | 103.7% |  |  |
| Dec Time[%] | 86.7% |  |  | 88.4% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -18.5% | -7.8% | -9.3% |  |  |  |
| Class B | -15.5% | -9.2% | -7.9% | -26.2% | -20.4% | -18.9% |
| Overall (EL+BL) | -16.4% | -8.8% | -8.3% | -26.2% | -20.4% | -18.9% |
| Overall (EL) | -25.2% | -13.6% | -12.9% | -47.3% | -38.1% | -35.4% |
| Enc Time[%] | 86.8% |  |  | 85.7% |  |  |
| Dec Time[%] | 105.6% |  |  | 105.1% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -13.6% | -1.9% | -3.0% |  |  |  |
| Class B | -11.7% | -6.9% | -5.1% | -21.8% | -17.1% | -15.2% |
| Overall (EL+BL) | -12.3% | -5.5% | -4.5% | -21.8% | -17.1% | -15.2% |
| Overall (EL) | -19.6% | -8.6% | -7.2% | -40.4% | -32.3% | -29.2% |
| Enc Time[%] | 87.0% |  |  | 90.7% |  |  |
| Dec Time[%] | 107.4% |  |  | 114.1% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 6 Average BD-rate performance of the proposed 7-bit chroma filter and luma filter in JCTVC-K0378

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -26.9% | -26.9% | -27.7% |  |  |  |
| Class B | -21.0% | -21.0% | -20.8% | -32.6% | -32.6% | -32.6% |
| Overall (EL+BL) | -22.7% | -22.7% | -22.8% | -32.6% | -32.6% | -32.6% |
| Overall (EL) | -34.7% | -34.8% | -35.0% | -58.1% | -57.5% | -57.6% |
| Enc Time[%] | 104.2% |  |  | 97.7% |  |  |
| Dec Time[%] | 88.3% |  |  | 84.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -18.4% | -7.6% | -9.0% |  |  |  |
| Class B | -15.5% | -9.0% | -7.7% | -26.1% | -19.9% | -18.3% |
| Overall (EL+BL) | -16.3% | -8.6% | -8.0% | -26.1% | -19.9% | -18.3% |
| Overall (EL) | -25.2% | -13.2% | -12.6% | -47.2% | -37.2% | -34.5% |
| Enc Time[%] | 83.1% |  |  | 85.7% |  |  |
| Dec Time[%] | 102.3% |  |  | 105.9% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -13.6% | -1.5% | -2.6% |  |  |  |
| Class B | -11.7% | -6.6% | -4.7% | -21.9% | -15.7% | -13.8% |
| Overall (EL+BL) | -12.2% | -5.1% | -4.1% | -21.9% | -15.7% | -13.8% |
| Overall (EL) | -19.5% | -8.0% | -6.6% | -40.5% | -29.5% | -26.2% |
| Enc Time[%] | 84.5% |  |  | 92.3% |  |  |
| Dec Time[%] | 105.5% |  |  | 114.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Conclusion

This contribution proposes the fixed upsampling filters for 1.5X and 2X spatial scalability of SHVC. The filter coefficients are first derived by the Gaussian windowed Sinc function, and approximated to integers with certain precisions of 7-bit and 8-bit. Considering the performance and complexity, we suggest adopting the 7-bit fixed upsampling filters into test model for further investigation and development.

# Patent rights declaration(s)

InterDigital Communications, LLC, may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Reference

[1] J. Dong, et al, “Description of scalable video coding technology proposal by InterDigital Communications,” JCTVC-K0034, Shanghai, China, Oct. 2012.

[2] A. Segall, J. Chen, J. Dong, E. Alshina, “Description of Tool Experiment A-1: Upsampling filter,” JCTVC-K1101, Shanghai, China, Oct. 2012.