| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0058 |
| --- | --- |

| Title: | TE4: Cross-verification results of test 4.4.1 on inter-layer adaptive filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Dong and Yan Ye 9710 Scranton Rd, Ste 250, San Diego, CA 92121, USA | Tel: Email: | +1 858 210 4809 jie.dong@interdigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

This document is a cross-check report on SHARP's proposal on adaptive filter for ILR pictures, described in Section 4.4.1 of TE4 document. It is verified that the encoding/decoding results completely match those reported by the proponent, and the description also matches the implementation in the software.

# Introduction

SHARP’S contribution proposes to apply a 5×5 non-separable filter with point-symmetry to the upsampled base layer reconstructed pictures. The 5×5 filter is trained on a slice basis using least minimum mean squared error (LMMSE) estimator and can achieve the minimum MSE between the original EL picture and filtered upsampled BL reconstructed picture. In a slice, the area where the filter is applied is represented by a quad-tree structure with maximum depth equal to 3.

In this cross-check, the source code and BD-rate results of the proposal are examined and verified.

# Experimental Results

The implementation of the proposal is based on SMuC v0.1.1, and the modified places in the software are indicated by the macros “K0032_AF”. The modified source code matches the description in the proposal. The cross-check experiments were conducted on 64-bit Linux cluster under the common test condition of TE4. The results exactly match what are reported by the proponent. The detailed data are shown in the enclosed spreadsheet.

# Conclusion

It was verified that the simulation results matched exactly those reported by the proponent and the description in the proposed adaptive filter matches the implementation in the software.