| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0059 |
| --- | --- |

| Title: | Chroma Enhancement for ILR Picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Dong, Yuwen He, Yan Ye 9710 Scranton Rd, Ste 250, San Diego, CA 92121, USA | Tel: Email: | +1 858.210.4809 jie.dong@interdigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

In the SHVC framework with ref_idx signaling, the quality of ILR pictures has a significant impact on the coding efficiency of the enhancement layer. This contribution proposes to enhance the chroma planes of the ILR picture using the corresponding information from the luma plane. More specifically, for each chroma pixel, an appropriate offset calculated based on the values of surrounding 3×4 luma pixels is added. Compared with the ref_idx framework in SMuC v0.1.1, the chroma BD-rate reduction for {AI 2x, AI 1.5x, RA 2x, RA 1.5x, RA SNR, LP 2x, LP 1.5x, LP SNR} is {-8.0%, -10.5%, -9.7%, -13.1%, -10.0%, -6.0%, -9.0%, -6.9%} (BL+EL) or {-14.8%, -29.5%, -16.3%, -28.6%, -18.6%, -10.3%, -19.9%, -12.1%} (EL only).  At the same time, luma BD-rate is also improved, varying from -0.2% to -0.8% (BL+EL) or -0.3% to -2.1% (EL only).

# Introduction

In one branch of the SMuC v0.1.1 software, inter-layer reference (ILR) picture is generated by processing (such as upsampling for spatial scalability) the base layer reconstruction picture and inserted into the reference list(s) of the corresponding enhancement layer picture as an additional reference picture. Whether an enhancement layer PU is using inter-layer texture prediction is detected by checking whether its ref_idx is associated with the ILR picture. This signaling approach is named “ref_idx”.

Using the ref_idx framework, the quality of ILR pictures has a significant impact on the coding efficiency of the enhancement layer, and deserves much research effort to improve it. It is observed that with medium to high QPs, the chroma planes of ILR pictures, having much more delicate edges than luma plane due to very small dynamic range, are severely blurred, whereas luma plane still has fair to good quality. The poor chroma quality of ILR picture significantly degrades the efficiency of enhancement layer chroma coding.

In order to take advantages of the high correlation among the luma and chroma planes, such as content, structure, and motion, and the better quality of the associated luma plane, we propose to enhance the chroma planes of the ILR picture using the corresponding information from the luma plane. More specifically, for each chroma pixel, an appropriate offset calculated based on the values of surrounding 3×4 luma pixels is added. Compared with SMuC v0.1.1 ref_idx_framework turned on, the proposed algorithm achieves significant BD-rate savings for enhancement layer chroma coding.

# Proposed Chroma Enhancement for ILR Picture

Error: Reference source not found shows the relative luma and chroma pixel positions in 4:2:0 color format. Each chroma pixel (red dot) in an ILR picture is enhanced by adding an appropriate offset, where the offset is the result of filtering the surrounding 3×4 luma pixels (white squares) by a high-pass filter. By doing this, the chroma edges lost during base layer compression are well restored using the high frequency components from the corresponding luma plane.

High-Pass Filter Design

Fig. 1 Relative Luma and Chroma pixel positions in 4:2:0 color subsampling format.

On the encoder side, for each enhancement layer slice, two high-pass filters are designed for Cb and Cr planes of the ILR picture, respectively, using Least Minimum Mean Squared Error (LMMSE) estimator. The objective, formulated by (1) and (2) below, is to find the optimal high-pass filters hCb,opt and hCr,opt, such that MSE between the original enhancement layer chroma plane and the enhanced chroma plane in the ILR picture is minimized.

For Cb: hCb,opt=argminhCbEj=−21i=−11hCb(i,j)Y(2x−i,2y−j)+Cb(x,y)−SCb(x,y)2          (1)

For Cr:  hCr,opt=argminhCrEj=−21i=−11hCr(i,j)Y(2x−i,2y−j)+Cr(x,y)−SCr(x,y)2          (2)

In (1) and (2), Y, Cb, and Cr are the three planes in the given ILR picture; SCb and SCr are the original enhancement layer Cb and Cr planes to be coded.

Obviously, hCb,opt and hCr,opt have real-valued coefficients, and therefore need quantizing before transmission.

## Quantizing and Signaling the High-Pass Filter Coefficients

Without the loss the generality, here we only explain how to quantize and signal hCb,opt. Each coefficient hCb,opt(i,j) is quantized by a 16-level uniform quantizer, of which the quantization step size, controlled by two parameters QCb and NCb, is equal to QCb/2NCb. Then the integer high-pass filter, denoted as fCb, is obtained, of which the dynamic range is from -8 to 7 (4-bit representation). So the original real-valued high-pass filter hCb,opt is approximated in the following way.

hCb,opt(i,j)=fCb(i,j)×QCb2NCb                                                       (3)

All the filter coefficients and parameters are transmitted in the slice header, including a binary flag indicating whether this tool is used for each chroma plane. Altogether, for each of the two chroma planes, 65 bits are needed to signal the side information when chroma enhancement is enabled:

- 1-bit flag

- 3×4 filter coefficients, each of which with 4-bit precision

- QCb represented by 11 bits (10 bits for magnitude and 1 bit for sign)

- NCb represented by 5 bits

For higher level syntax, we added one binary flag to SPS to indicate whether the proposed tool is used in the current coded video sequence.

## Process of Chroma Enhancement

Like Section 2.2, only the enhancement for Cb plane of the ILR picture is introduced here. To enhance a Cb pixel with position (x,y) (take the red circle in Fig. 1 as an example), high-pass filter fCb is first applied to the surrounding 3×4 luma pixels to generate the intermediate result z(x,y), as shown in (4).

z(x,y)=j=−21i=−11fCb(i,j)Y(2x−i,2y−j)                                        (4)

Then, z(x,y) is scaled by QCb/2NCb to the normal range, denoted as o(x,y). The integer realization of this scaling step is shown as below.

- If z(x,y)×QCb>0, o(x,y) is calculated as z(x,y)×QCb+2NCb−1>>NCb.

- Otherwise, o(x,y) is calculated as −−z(x,y)×QCb+2NCb−1>>NCb.

The enhanced Cb pixel, denoted as Cbenh(x,y), is obtained by Cbenh(x,y)=Cb(x,y)+o(x,y).

# Experimental Results

The proposed chroma enhancement algorithm is integrated into SMuC v0.1.1 and tested under the common test conditions. The summary of the testing results is shown in Table 1, and more details can be found in the enclosed spreadsheet. In short, compared with the ref_idx framework in SMuC v0.1.1, the chroma BD-rate reduction for {AI 2x, AI 1.5x, RA 2x, RA 1.5x, RA SNR, LP 2x, LP 1.5x, LP SNR} is {-8.0%, -10.5%, -9.7%, -13.1%, -10.0%, -6.0%, -9.0%, -6.9%} (BL+EL) or {-14.8%, -29.5%, -16.3%, -28.6%, -18.6%, -10.3%, -19.9%, -12.1%} (EL only).  At the same time, luma BD-rate is also improved, varying from -0.2% to -0.8% (BL+EL) or -0.3% to -2.1% (EL only).

As an example, Error: Reference source not found shows the difference of the chroma plane before and after the proposed chroma enhancement is applied. Error: Reference source not found(a) is the Cr plane of the first ILR picture in ParkScene, obtained by directly upsampling the corresponding base layer Cr plane without using the chroma enhancement, where the base layer QP is 30. It is severely blurred due to compression, in comparison with the original Cr plane, as shown in Error: Reference source not found(c). In Error: Reference source not found(b), where chroma enhancement is applied, blocking artifacts are significantly alleviated and edges are restored, especially those areas highlighted by black lines. Compared with the original Cr plane, the PSNR values of Error: Reference source not found (a) and (b) are 30.79 dB and 31.69 dB, respectively.

# Conclusion

This contribution proposes to enhance the chroma planes of the ILR picture using the corresponding information from the luma plane. Specifically, for each chroma pixel, an appropriate offset calculated based on the values of surrounding 3×4 luma pixels is added. Compared with the ref_idx framework in SMuC v0.1.1, the chroma BD-rate reduction for {AI 2x, AI 1.5x, RA 2x, RA 1.5x, RA SNR, LP 2x, LP 1.5x, LP SNR} is {-8.0%, -10.5%, -9.7%, -13.1%, -10.0%, -6.0%, -9.0%, -6.9%} (BL+EL) or {-14.8%, -29.5%, -16.3%, -28.6%, -18.6%, -10.3%, -19.9%, -12.1%} (EL only).  At the same time, luma BD-rate is also improved, varying from -0.2% to -0.8% (BL+EL) or -0.3% to -2.1% (EL only). Considering its high efficiency in improving the EL chroma coding, we suggest adopting this technique into test model for further investigation and future SHVC development.

Table 1 Summary of BD-rate reduction compared with SMuC v0.1.1 with ref_idx_framework turned on

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -0.9% | -8.3% | -6.9% |  |  |  |  |  |  |
| Class B | -0.8% | -7.0% | -9.3% | -0.8% | -9.3% | -11.8% |  |  |  |
| Overall (EL+BL) | -0.8% | -7.3% | -8.6% | -0.8% | -9.3% | -11.8% |  |  |  |
| Overall (EL) | -1.6% | -14.0% | -15.7% | -2.1% | -27.1% | -31.9% |  |  |  |
| Enc Time[%] | 102.5% |  |  | 102.7% |  |  |  |  |  |
| Dec Time[%] | 105.5% |  |  | 107.8% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.5% | -13.1% | -7.8% |  |  |  | -0.4% | -12.4% | -6.9% |
| Class B | -0.3% | -8.5% | -10.4% | -0.3% | -12.0% | -14.2% | -0.3% | -9.8% | -10.7% |
| Overall (EL+BL) | -0.4% | -9.8% | -9.6% | -0.3% | -12.0% | -14.2% | -0.3% | -10.5% | -9.6% |
| Overall (EL) | -0.6% | -16.9% | -15.8% | -0.6% | -27.4% | -29.9% | -0.5% | -19.6% | -17.6% |
| Enc Time[%] | 108.1% |  |  | 103.9% |  |  | 94.8% |  |  |
| Dec Time[%] | 115.5% |  |  | 109.5% |  |  | 104.2% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.2% | -9.8% | -4.9% |  |  |  | -0.2% | -9.3% | -5.3% |
| Class B | -0.2% | -4.8% | -6.0% | -0.2% | -7.8% | -10.2% | -0.2% | -6.3% | -7.3% |
| Overall (EL+BL) | -0.2% | -6.2% | -5.7% | -0.2% | -7.8% | -10.2% | -0.2% | -7.1% | -6.7% |
| Overall (EL) | -0.4% | -10.9% | -9.6% | -0.4% | -18.7% | -21.1% | -0.3% | -12.6% | -11.6% |
| Enc Time[%] | 104.5% |  |  | 108.8% |  |  | 101.2% |  |  |
| Dec Time[%] | 113.5% |  |  | 116.7% |  |  | 109.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Patent rights declaration(s)

InterDigital Communications, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

(a)

(b)

(c)

Fig. 2 Cr plane of the first ILR picture in ParkScene with base layer QP=30. (a) Before chroma enhancement (30.79 dB); (b) after chroma enhancement (31.69 dB); (c) the original Cr plane.