| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0060 |
| --- | --- |

| Title: | TE 2 : Results of test 3.2.2 on inter-layer reference picture placement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hahyun Lee Jung Won Kang Jinho Lee Jin Soo Choi 218 Gajeongno, Yuseong-gu Daejeon, 305-700, Korea | Tel: Email: | +82-42-860-6138 hanilee@etri.re.kr jungwon@etri.re.kr jinosoul@etri.re.kr jschoi@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

# Abstract

This document provides the experimental results for Tool Experiment 2 3.2.2 which is an inter-layer reference picture placement method initially proposed by JCTVC-K0037. Compared to the HM8.1 simulcast, the proposal reports overall EL+BL luma BD-rate gains of 22.8% for AI 2x spatial scalability, 32.8% for AI 1.5x spatial scalability, 15.2% for RA 2x spatial scalability, 25.5% for RA 1.5x spatial scalability, 18.6% for RA SNR scalability, 12.4% for LD-P 2x spatial scalability, 22.3% for LD-P 1.5x spatial scalability, 13.1% for LD-P SNR scalability.

# Introduction

During the 11th JCT-VC meeting in Shanghai, Tool Experiment 2 (TE 2) [1] was set up for testing inter- layer texture prediction signaling for SHVC. TE 2 includes inter-layer reference picture placement, and the related proposal [2] is described in TE 2 3.2.2

# Inter-layer reference picture placement

The reconstructed picture of the base layer is used as the additional reference picture for the corresponding picture of the enhancement layer. For inter-layer texture prediction, the reconstructed picture of the base layer is up-sampled (if necessary) and is inserted into reference picture lists, L0 and L1, for the corresponding picture of the enhancement layer.

The process of reference picture list construction of the enhancement layer (encoder and decoder)

- An inter-layer reference picture (ILRP) is generated by reconstructing and up-sampling (if necessary) the corresponding picture of the base layer

- The reference picture lists, L0 and L1, are constructed as signaled in slice header of the enhancement layer

- For non-random access picture (B slice), the ILRP, that is assigned to the reference index of “1”, is inserted into both reference picture list 0 and reference picture list 1 of the enhancement layer while reference index of reference picture whose reference index was originally greater than or equal to “1” is increased by 1 if there is any

- For non-random access picture (P slice), the ILRP, that is assigned to the reference index of “1”, is inserted into reference picture list 0 of the enhancement layer was originally greater than or equal to “1” is increased by 1 if there is any

- For random access picture, the ILRP, that is assigned to the reference index of “0”, is inserted into both reference picture list 0 and reference picture list 1 of the enhancement layer or only reference picture list 0 of the enhancement layer

Fig. 1 shows an example of reference picture construction of non-random access picture whose slice type is B slice in the enhancement layer.

Fig 1. Example of reference picture construction of the enhancement layer

# Experimental results

The proposed method was performed based on the ‘ref_idx’ framework of SMuC software version 0.1.1 under experimental conditions defined in TE 2 [1].

The encoding of anchor and proposed method was executed on a 64-bit Linux cluster. The decoding of anchor and proposed method was executed on a single 64-bit Linux PC without output of decoded YUV data.

For RA, AI case, the inter-layer reference picture is inserted into both list L0 and L1 of the enhancement-layer pictures and the command “-use-rap-b” is set as 1. For LD-P case, the inter-layer reference picture is inserted into only list L0 of the enhancement layer pictures and the command “-use-rap-b” is set 0

Table1 represents the experimental results of the proposed method for AI configuration. Table 2 and Table 3 show the experimental results of the proposed method for RA and LD-P configuration, respectively. More detailed results can be found in the Excel attachment.

Table 1 Performance of test 3.2.2 for Al 2x and 1.5x spatial scalability

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -26.9% | -26.2% | -27.1% |  |  |  |
| Class B | -21.2% | -20.1% | -20.0% | -32.8% | -32.9% | -33.1% |
| Overall (EL+BL) | -22.8% | -21.9% | -22.0% | -32.8% | -32.9% | -33.1% |
| Overall (EL) | -34.9% | -33.7% | -33.9% | -58.5% | -58.2% | -58.7% |
| Enc Time[%] | 264.7% |  |  | 215.1% |  |  |
| Dec Time[%] | 89.7% |  |  | 84.4% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 2 Performance of test 3.2.2 for RA 2x, 1.5x, and SNR (optional) spatial scalability

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -17.4% | -4.6% | -6.2% |  |  |  | -20.5% | -8.3% | -7.4% |
| Class B | -14.3% | -5.2% | -3.2% | -25.5% | -18.9% | -17.0% | -17.8% | -6.0% | -1.4% |
| Overall (EL+BL) | -15.2% | -5.1% | -4.1% | -25.5% | -18.9% | -17.0% | -18.6% | -6.7% | -3.1% |
| Overall (EL) | -23.5% | -8.0% | -6.8% | -46.5% | -36.1% | -32.9% | -29.4% | -12.0% | -6.8% |
| Enc Time[%] | 115.2% |  |  | 110.7% |  |  | 109.2% |  |  |
| Dec Time[%] | 116.4% |  |  | 111.3% |  |  | 94.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 3 Performance of test 3.2.2 for LD-P 2x, 1.5x, and SNR (optional) spatial scalability

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -13.6% | -0.9% | -2.1% |  |  |  | -13.9% | -6.0% | -4.9% |
| Class B | -11.9% | -5.1% | -3.2% | -22.3% | -16.5% | -14.7% | -12.8% | -4.4% | -0.1% |
| Overall (EL+BL) | -12.4% | -3.9% | -2.9% | -22.3% | -16.5% | -14.7% | -13.1% | -4.9% | -1.5% |
| Overall (EL) | -19.8% | -6.2% | -4.9% | -41.6% | -31.7% | -28.4% | -21.1% | -9.8% | -5.0% |
| Enc Time[%] | 103.6% |  |  | 104.3% |  |  | 101.1% |  |  |
| Dec Time[%] | 121.2% |  |  | 116.9% |  |  | 96.1% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Conclusions

In this contribution, we proposed the inter-layer reference picture placement method as one of inter-layer texture prediction signaling methods for SHVC. The results show overall EL+BL luma BD-rate gains of 22.8% for AI 2x spatial scalability, 32.8% for AI 1.5x spatial scalability, 15.2% for RA 2x spatial scalability, 25.5% for RA 1.5x spatial scalability, 18.6% for RA SNR scalability, 12.4% for LD-P 2x spatial scalability, 22.3% for LD-P 1.5x spatial scalability, 13.1% for LD-P SNR scalability.

# Patent rights declaration(s)

ETRI (Electronics and Telecommunications Research Institute) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] L. Guo, Y. He, D. Kwon, J. Zan, H. Lakshman, J. Kang “Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC”, JCTVC-K1102, Oct. 2012, Shanghai, China.

[2] J. W. Kang, H. Lee, J. Lee, J. S. Choi, J. W. Kim, J. Nam, H. Choi, D. Sim “Description of scalable video coding technology proposal by ETRI and Kwangwoon Univ.”, JCTVC-K0037, Oct. 2012, Shanghai, China.