| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0061 |
| --- | --- |

| Title: | TE 2: Cross-check results of test 3.1.1 on CU level Intra BL Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hahyun Lee Jung Won Kang Jinho Lee 218 Gajeongno, Yuseong-gu Daejeon, 305-700, Korea | Tel: Email: | +82-42-860-6138 hanilee@etri.re.kr jungwon@etri.re.kr jinosoul@etri.re.kr |
| Source: | ETRI |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results of TE2 test 3.1.1 on CU level Intra BL Mode proposed by Qualcomm. Test B configuration was tested. All the BD-rate results matched exactly those provided by Qualcomm.

# Introduction

In TE2 test 3.1.1 test B configuration, the followings are implemented on SMuC version 0.1.1 software

- Intra-BL Skip is signaled at the CU level. Intra-BL Skip flag is first signaled. If Intra-BL Skip flag is 1, no further information needs to be transmitted. If Intra-BL Skip flag is 0, intra-BL flag is signaled.

- The de-blocking boundary strength for Intra-BL and Intra-BL Skip mode is modified as follows. In case both blocks are coded with intra-BL skip mode, the edge is not processed; otherwise, if at least one of the blocks is coded as normal intra mode, the BS is set to 2; otherwise, if at least one of the blocks is coded in intra-BL mode, then the BS for the luma edge is set to 1, and the BS is set to 2 for the chroma edge.

- For 4x4 TU in Intra-BL, both forward and backward transforms are DST.

# Cross-check results

Proponent provided the source code and simulation results. The cross-check of TE2 test 3.1.1 is carried out by setting the macro definitions in the provided code as in Table 1.

Table 1 macro definitions

#define INTRA_BL_SKIP 1 #define SVC_DBF_INTRABL_BS 1 ///< de-blocking boundary strength adjustment Intra-BL #define INTRA_BL_BUGFIX 1 #if INTRA_BL_BUGFIX #define INTRA_BL_DCT_FIX 0 ///< fix : 0: use DST forward transform: 1: use DCT inverse transform #endif

All the BD-rate results exactly matched the results provided by proponent. Table 2 shows the cross-check results. More detailed results can be found in the Excel attachment.

No YUV files were written when the decoding times were measured. Encoding was done on a heterogeneous Linux cluster. Decoding was done sequentially on a single Linux PC.

Table 2 Cross-check results of TE2 3.1.1 test B configuration

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -27.2% | -27.4% | -28.3% |  |  |  |
| Class B | -21.5% | -21.4% | -21.3% | -33.0% | -33.0% | -33.1% |
| Overall (EL+BL) | -23.1% | -23.1% | -23.3% | -33.0% | -33.0% | -33.1% |
| Overall (EL) | -35.3% | -35.5% | -35.8% | -58.7% | -58.1% | -58.5% |
| Enc Time[%] | 113.9% |  |  | 111.4% |  |  |
| Dec Time[%] | 98.0% |  |  | 93.0% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -18.5% | -7.9% | -9.5% |  |  |  | -21.6% | -12.1% | -11.8% |
| Class B | -15.7% | -9.2% | -7.8% | -26.3% | -20.0% | -18.6% | -19.6% | -10.8% | -7.9% |
| Overall (EL+BL) | -16.5% | -8.8% | -8.3% | -26.3% | -20.0% | -18.6% | -20.2% | -11.2% | -9.0% |
| Overall (EL) | -25.5% | -13.6% | -13.0% | -47.6% | -37.4% | -35.0% | -31.9% | -18.7% | -15.6% |
| Enc Time[%] | 109.2% |  |  | 111.7% |  |  | 106.3% |  |  |
| Dec Time[%] | 119.1% |  |  | 115.0% |  |  | 96.6% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -13.6% | -2.0% | -3.2% |  |  |  | -14.0% | -7.3% | -6.3% |
| Class B | -11.9% | -6.7% | -4.9% | -22.0% | -16.3% | -14.6% | -12.9% | -5.2% | -1.7% |
| Overall (EL+BL) | -12.4% | -5.4% | -4.4% | -22.0% | -16.3% | -14.6% | -13.2% | -5.8% | -3.0% |
| Overall (EL) | -19.7% | -8.3% | -7.1% | -40.8% | -30.9% | -27.8% | -21.2% | -11.0% | -7.0% |
| Enc Time[%] | 106.2% |  |  | 110.7% |  |  | 103.5% |  |  |
| Dec Time[%] | 124.5% |  |  | 121.4% |  |  | 99.2% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Conclusion

The results of TE2 test 3.1.1 Test B configuration presented by Qualcomm (JCTVC-L0284) are verified by ETRI