| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0063 |
| --- | --- |

| Title: | TE 3: Cross-check results of test 4.6.2.1 on Generalized Residual Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hahyun Lee Jung Won Kang Jinho Lee 218 Gajeongno, Yuseong-gu Daejeon, 305-700, Korea | Tel: Email: | +82-42-860-6138 hanilee@etri.re.kr jungwon@etri.re.kr jinosoul@etri.re.kr |
| Source: | ETRI |  |  |

_____________________________

# Abstract

This document provides the cross-check results of TE 3 test 4.6.2.1 on Generalized Residual Prediction proposed by Qualcomm. Test 2 and Test 5 configurations were tested. All the BD-rate results matched those provided by the proponent.

# Introduction

In test 2, GRP with three weighting factors (w=0, 0.5, 1) is applied to inter CUs and skip mode CUs and the following fast GRP weighting factor selection method is enabled.

- Firstly, the current CU Depth is larger than the maximum CU depth of left, above, and above-right CU, only one weighting factor (w=0) is checked

- Secondly, for non-merge modes at temporal level higher than one, two weighting factor (w=0, 0.5) are checked

- Finally, for CUs with non-2Nx2N, two weighting factor (w=0, 0.5) are checked

In test 5, based on test 2, additional motion estimation is conducted in differential domain for weighting factors 0.5 and 1

# Cross-check results

### 2.1 Test 2 configuration

Proponent provided the source code and simulation results. The cross-check of Test 2 configuration is carried out by setting the macro definitions in the provided code as in Table 1.

Table 1 macro definitions

#define GRP_TEST1 0 // GRP weighting (0, res-0.5) with no fast algorithm #define GRP_TEST2 1 // GRP weighting (0, res-0.5, 1) with fast algorithm #define GRP_TEST3 0 // GRP weighting (0, res-0.5, 1) with fast algorithm and GRP up-sampling filter enabled

All the BD-rate results exactly matched with the results provided by proponent. Table 2 shows the cross-check results. More detailed results can be found in the Excel attachment.

No YUV files were written when the decoding times were measured. Encoding was done on a heterogeneous Linux cluster. Decoding was done sequentially on a single Linux PC.

Table 2 Cross-check results of TE3 4.6.2.1 test 2

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.9% | -5.0% | -5.2% |  |  |  | -1.6% | -4.8% | -5.3% |
| Class B | -1.9% | -3.6% | -3.7% | -2.8% | -5.2% | -5.6% | -2.3% | -4.7% | -5.1% |
| Overall (EL+BL) | -1.9% | -4.0% | -4.2% | -2.8% | -5.2% | -5.6% | -2.1% | -4.7% | -5.2% |
| Overall (EL) | -3.5% | -7.5% | -7.8% | -6.5% | -12.5% | -13.3% | -3.9% | -9.2% | -9.9% |
| Enc Time[%] | 118.6% |  |  | 109.5% |  |  | 113.0% |  |  |
| Dec Time[%] | 102.8% |  |  | 103.2% |  |  | 105.1% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.6% | -4.6% | -4.2% |  |  |  | -2.7% | -4.4% | -4.0% |
| Class B | -2.8% | -2.8% | -2.5% | -3.9% | -4.3% | -3.5% | -3.6% | -4.1% | -3.8% |
| Overall (EL+BL) | -2.7% | -3.3% | -3.0% | -3.9% | -4.3% | -3.5% | -3.4% | -4.2% | -3.8% |
| Overall (EL) | -4.7% | -6.1% | -5.7% | -8.6% | -9.9% | -8.5% | -5.8% | -7.3% | -6.8% |
| Enc Time[%] | 120.2% |  |  | 115.1% |  |  | 118.4% |  |  |
| Dec Time[%] | 103.4% |  |  | 101.3% |  |  | 104.8% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

### 2.2 Test 5 configuration

Proponent provided the source code and simulation results. The cross-check of Test 5 configuration is carried out by setting the macro definitions in the provided code as in Table 3.

Table 3 macro definition

#define GRP_TEST1 0 // GRP weighting (0, res-0.5) with no fast algorithm #define GRP_TEST2 0 // GRP weighting (0, res-0.5, 1) with fast algorithm #define GRP_TEST3 0 // GRP weighting (0, res-0.5, 1) with fast algorithm and GRP up-sampling filter enabled #define GRP_TEST4 0 // Test1 + diff domain ME (with no fast algorithm) #define GRP_TEST5 1 // Test2 + diff domain ME (with fast algorithm)

All the BD-rate results exactly matched with the results provided by proponent. Table 4 shows the cross-check results. More detailed results can be found in the Excel attachment.

No YUV files were written when the decoding times were measured. Encoding was done on a heterogeneous Linux cluster. Decoding was done sequentially on a single Linux PC.

Table 4 Cross-check results of TE3 4.6.2.1 test 5

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.3% | -5.7% | -5.7% |  |  |  | -2.1% | -5.5% | -5.9% |
| Class B | -2.4% | -4.0% | -4.1% | -3.2% | -5.5% | -5.8% | -2.9% | -5.3% | -5.7% |
| Overall (EL+BL) | -2.4% | -4.5% | -4.5% | -3.2% | -5.5% | -5.8% | -2.6% | -5.4% | -5.8% |
| Overall (EL) | -4.3% | -8.2% | -8.4% | -7.5% | -12.9% | -13.5% | -4.9% | -10.1% | -10.8% |
| Enc Time[%] | 155.8% |  |  | 138.6% |  |  | 138.1% |  |  |
| Dec Time[%] | 103.9% |  |  | 104.0% |  |  | 106.5% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -3.4% | -5.6% | -5.1% |  |  |  | -3.6% | -5.2% | -4.8% |
| Class B | -3.4% | -3.3% | -2.9% | -4.7% | -4.8% | -4.0% | -4.6% | -4.8% | -4.5% |
| Overall (EL+BL) | -3.4% | -4.0% | -3.5% | -4.7% | -4.8% | -4.0% | -4.3% | -4.9% | -4.5% |
| Overall (EL) | -5.7% | -7.1% | -6.6% | -10.3% | -10.8% | -9.5% | -7.2% | -8.3% | -7.8% |
| Enc Time[%] | 157.8% |  |  | 146.0% |  |  | 146.1% |  |  |
| Dec Time[%] | 105.0% |  |  | 102.5% |  |  | 106.8% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Conclusion

The results of TE 3 test 4.6.2.1 Test 2 and Test 5 configurations presented by Qualcomm (JCTVC-L0078) are confirmed by ETRI